# 3GPP TSG-RAN WG4 Meeting #104-e	R4-2212799

# Electronic Meeting, 15th - 26th August, 2022

Source: 	vivo

Title: 	11.2.3

Agenda Item:	Impacts on RF requirements for 700/800/900MHz CA band combinations

Document for:	Approval

Introduction

A new SI: Study on enhancement for 700/800/900MHz band combinations was approved for Rel-18 [1]. This SI was aimed at investigating the feasibility and solutions to enable CA_n8-n20-n28, CA_n5-n8, CA_n5-n28. Also, the objective was to identify potential impacts on RF requirements for these low band CA combinations. In this contribution, we give our initial thoughts on RF requirements for 700/800/900MHz CA band combinations.

Discussion

Framework for inter-band CA with 2UL bands

In this SI, the following CA band combinations are considered for smartphone UE factor, CA_n8-n20-n28, CA_n5-n8, CA_n5-n28 as shown in Table 1.

Table 1: Inter-band CA operating bands

| CA Band Combinations | Uplink configuration | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n20-n28 | CA_n8-n20 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | CA_n8-n28 | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | CA_n20-n28 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| CA_n5-n8 | CA_n5-n8 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  |  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| CA_n5-n28 | CA_n5-n28 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  |  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

These band combinations can fall into the framework of inter-band CA for 2/3 DL bands with 2UL bands. In NR Rel-16 and Rel-17, we’ve studied NR inter-band CA for 2/3 DL bands with 2UL bands, and the technical reports were in reference with TR 38.716-02-00 and TR 38.716-03-02. The inter-band CA study framework created in Rel-16/Rel-17 can still apply to 700/800/900 MHz CA band combinations. For the CA bandwidths studied in this SI, these following RF requirements framework can still apply:

- Operating bands for CA

- Channel bandwidths per operating band for CA

- Co-existence studies

- ∆TIB and ∆RIB values

- REFSENS requirements

Before we further discuss these items, we should come to a consensus on the typical antenna/RF architectures for these band combinations. ∆TIB and ∆RIB values highly depend on the RF architecture. The co-existence studies should be specific to 2UL bands transmission. Based on the outcome of co-existence studies, we can decide whether MSD requirements are needed.

Proposal 1: The following RF requirements framework can still apply for 700/800/900 MHz NR inter-band CA:

- Operating bands for CA

- Channel bandwidths per operating band for CA

- Co-existence studies

- ∆TIB and ∆RIB values

- REFSENS requirements

Co-existence study for CA_n8-n20-n28, CA_n5-n8, CA_n5-n28

For CA_n5-n28, the 2UL bands coexistence study is shown in Table 2.

Table 2: Band n5 and Band n28 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 703 | 748 |
| 2nd harmonics frequency limits | 2* fx_low | 2* fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3* fx_low | 3* fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 2109 | 2244 |
| 4th harmonics frequency limits | 4* fx_low | 4* fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 2812 | 2,992 |
| 5th harmonics frequency limits | 5* fx_low | 5* fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 3515 | 3,740 |
| 6th harmonics frequency limits | 6* fx_low | 6* fx_high | 6*fy_low | 6*fy_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 4218 | 4,488 |
| 7th harmonics frequency limits | 7* fx_low | 7* fx_high | 7*fy_low | 7*fy_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 4921 | 5,236 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 76 | 146 | 1,527 | 1597 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 900 | 995 | 557 | 672 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2351 | 2446 | 2230 | 2345 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 152 | 292 | 3054 | 3,194 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1,724 | 1844 | 1260 | 1420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3175 | 3295 | 2933 | 3093 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1988 | 1963 | 2693 | 2548 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3636 | 3841 | 3999 | 4144 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 596 | 411 | 1141 | 976 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3757 | 3942 | 3878 | 4043 |

For CA band combination CA_n5-n28, IMD component is outside the band. There is no potential interference for DL n5 and DL n28. Therefore, no extra MSD is needed for DL n5 and DL n28.

Observation 1: For CA_n5-n28, with 2UL bands transmission, there is no potential interference for DL n5 and n28.

For CA_n5-n8, the DL part of n5 overlaps with the UL part of n8 UL. It is suggested for RAN4 to identify the frequency restriction for CA_n5-n8.

Figure 1. Frequency Layout for CA_n5-n8

For CA band combination CA_n8-n20-n28 with UL CA_n8-n20 configuration, the coexistence study is in Table 3. We can observe that IMD3 of band n8 and n20 would fall into n8 DL and n20 DL. Further study on MSD requirement is needed after RF architecture assumption is decided.

Table 3: Band n8 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 832 | 862 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 998 | 749 | 826 |

For CA band combination CA_n8-n20-n28 with UL CA_n8-n28 configuration, the coexistence study is in Table 4. We can observe that IMD3 of band n8 and n28 would not interfere with the DL reception for this CA band combination. Thus, no MSD requirement is needed.

Table 4: Band n8 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |

For CA band combination CA_n8-n20-n28 with UL CA_n20-n28 configuration, the coexistence study is in Table 5. We can observe that IMD3 of band n20 and n28 would fall into n8 DL. Further study on MSD requirement is needed after RF architecture assumption is decided.

Table 5: Band n20 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 703 | 748 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 916 | 1021 | 544 | 664 |

Observation 2: For CA_n8-n20-n28 with UL CA_n8-n20 configuration, IMD3 of band n8 and n20 would fall into n8 DL and n20 DL.

Observation 3: For CA_n8-n20-n28 with UL CA_n8-n28 configuration, IMD3 of band n8 and n28 would not interfere with the DL reception for this CA band combination.

Observation 4: For CA_n8-n20-n28 with UL CA_n20-n28 configuration, IMD3 of band n20 and n28 would fall into n8 DL.

Conclusion

This contribution discusses the potential impacted RF requirements for 700/800/900MHz low band CA. The following observations and proposals are made:

Proposal 1: The following RF requirements framework can still apply for 700/800/900 MHz NR inter-band CA:

- 	Operating bands for CA

- 	Channel bandwidths per operating band for CA

- 	Co-existence studies

- 	∆TIB and ∆RIB values

- 	REFSENS requirements

Observation 1: For CA_n5-n28, with 2UL bands transmission, there is no potential interference for DL n5 and n28.

Observation 2: For CA_n8-n20-n28 with UL CA_n8-n20 configuration, IMD3 of band n8 and n20 would fall into n8 DL and n20 DL.

Observation 3: For CA_n8-n20-n28 with UL CA_n8-n28 configuration, IMD3 of band n8 and n28 would not interfere with the DL reception for this CA band combination.

Observation 4: For CA_n8-n20-n28 with UL CA_n20-n28 configuration, IMD3 of band n20 and n28 would fall into n8 DL.

- References

[1] RP-221016, New SI: Study on enhancement for 700/800/900MHz band combinations, CATT, RAN#95e.