# 3GPP TSG-RAN WG4 Meeting #104-e	R4-2212800

# Electronic Meeting, 15th - 26th August, 2022

Source: 	vivo

Title: 	Considerations for simplification of specification structure for V2X band combinations

Agenda Item:	11.3.3

Document for:	Approval

Introduction

In RAN#96 meeting, a revised SID: Study on simplification of band combination specification for NR and LTE was approved [1]. The objective related to Uu and PC5 band combinations was added:

- A simplified approach aiming to allow operation of any PC5 configuration (LTE PC5, NR PC5, CA on PC5) with any Uu configuration (any LTE CA/DC, EN-DC, NR DC) should be investigated in order to minimise the specification efforts for such automotive relevant combinations.

In this contribution, we summarize the PC5 and Uu band combinations in Rel-16 and Rel-17 and provide our views on this kind of band combinations.

Discussion

For PC5 and Uu band combinations, we had a definition for them as con-current operation in RAN4’s specifications:

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

In Rel-16 NR V2X and Rel-17 NR SL enhancement, these kinds of Uu and PC5 band combinations are classified as these categories:

- Inter-band con-current V2X operating bands (TS 38.101-1&3)

  - NR Uu+NR PC5 (TS 38.101-1)

  - LTE Uu+NR PC5(TS 38.101-3)

  - NR Uu+LTE PC5(TS 38.101-3)

- Intra-band con-current V2X operating bands （TS 38.101-1）

Even in Rel-17 WI “NR_LTE_V2X_PC5_combos” [2], only the inter-band con-current V2X operating bands are considered, which are within the above categories. The NR Uu and NR PC5 band combinations involves one RAT and they are included in TS 38.101-1. The LTE Uu+NR SL and NR Uu +LTE SL band combinations are included in TS 38.101-3 since two RATs are involved.

For NR Uu and NR PC5 band combinations, they are noted as ‘V2X_ (NR Uu band)- (NR SL band)’. The example band combinations are:

- V2X_n1-n47, V2X_n5-n47, V2X_n8-n47……

For LTE Uu and NR PC5 band combinations, they are noted as ‘V2X_(LTE Uu band)-(NR PC5 band)’.

- V2X_20_n38……

For NR Uu and LTE PC5 band combinations, they are noted as ‘V2X_(NR Uu band)-(LTE PC5 band)’.

- V2X_ n71_47

For intra-band con-current V2X operating bands, there is only one band combination included now. That is ‘V2X_n79-n79’, which means Uu and PC5 are in the same band n79.

In this SI FS_SimBC, the objective involves any PC5 and any Uu band combinations. First of all, PC5 on CA was in the scope of Rel-18 SL evolution [3] as follows:

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#97)

For SL CA part, it is still put on hold from the Rel-18 SL evolution WI. Even if we included the SL CA in the Rel-18 SL evolution WI, RAN4 will start the Rel-18 SL work until 2023. Therefore, we suggest to exclude PC5 on CA from this SI FS_SimBC.

For Uu configuration, any LTE CA/DC, EN-DC, NR-DC was included in SI FS_SimBC. However, we never studied CA/DC+PC5 band combination in Rel-16 NR V2X or Rel-17 SL enhancement. Furthermore, this kind of CA/DC + PC5 band combinations are not even in the scope of Rel-18 SL evolution. Therefore, we suggest not to consider them.

Proposal 1: The Uu and PC5 band combinations studied in SI FS_SimBCs should be restricted as follows:

- Inter-band con-current V2X operating bands (TS 38.101-1&3)

  - NR Uu+NR PC5 (TS 38.101-1)

  - LTE Uu+NR PC5(TS 38.101-3)

  - NR Uu+LTE PC5(TS 38.101-3)

- Intra-band con-current V2X operating bands （TS 38.101-1）

Conclusion

This contribution discusses Uu and PC5 band combinations and provides one suggestion on the scope of Uu and PC5 band combinations studied in SI FS_SimBCs. The following proposal is made:

Proposal 1: The Uu and PC5 band combinations studied in SI FS_SimBCs should be restricted as follows:

- Inter-band con-current V2X operating bands (TS 38.101-1&3)

  - NR Uu+NR PC5 (TS 38.101-1)

  - LTE Uu+NR PC5(TS 38.101-3)

  - NR Uu+LTE PC5(TS 38.101-3)

- Intra-band con-current V2X operating bands （TS 38.101-1）

- References

[1] RP-221790, Revised SID: Study on simplification of band combination specification for NR and LTE, ZTE, RAN#96.

[2]RP-213297, Revised WID: Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band, CATT, RAN#94-e.

[3]RP-213678, New WID on NR sidelink evolution, OPPO, LG Electronics, RAN#94-e.