3GPP TSG-RAN WG4 Meeting#104-e                	R4-2212831

Electronic Meeting, 15 August – 26 August 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUCCH demodulation performance of Rel-17 NR coverage enhancements

Agenda item: 	9.16.2.2

Document for: 	Discussion

Introduction

This document extends the discussion on the NR Coverage enhancements BS demodulation requirements introduced in RAN4 #101bis-e summarized in [1]. The discussion continued in RAN4#102-e in [2], and in RAN4#103-e in [3].

The agreements reached during the last meeting regarding NR Coverage Enhancements BS Demodulation work are captured on the WFs [4], [5] and [6]. The major open topics being PUCCH enhancements.

This paper presents Nokia’s views on the open issues related to the NR coverage enhancements BS demodulation work, extending the previous discussion introduced in [1], [7] and [8].

PUCCH enhancements

DMRS configuration for PUCCH format 3

In [9], the simulation results show at most a difference of 0,5 dB between DMRS 1+0 and DMRS 1+1 configurations. The best performance is achieved using DM-RS 1+1. Hence, we think that testing only DM-RS 1+1 configuration should be sufficient to define PUCCH demod requirements.

Our simulation results show that DM-RS 1+1 shows better performance than DM-RS 1+0.

RAN4 to cover either DM-RS 1+1 or both DM-RS 1+1 and DM-RS 1+0 for PUCCH requirements.

Conclusion

In this contribution, we have provided our view in DM-RS configuration in case of JCE PUCCH.

Observations and proposals are derived from this discussion are presented as follows:

Observation 1:Our simulation results show that DM-RS 1+1 shows better performance than DM-RS 1+0.

Proposal 1:RAN4 to cover either DM-RS 1+1 or both DM-RS 1+1 and DM-RS 1+0 for PUCCH requirements.

References

- R4-2203117 NR_cov_enh_Demod_NWM

- R4-2207449 Email discussion summary for [102-e][329] NR_cov_enh_Demod

- R4-2210530 Email discussion summary for [103-e][327] NR_cov_enh_Demod

- R4-2203031 WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2207211 WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2210667 WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2205490 PUCCH demodulation performance of Rel-17 NR coverage enhancements

- R4-2207743 PUCCH demodulation performance of Rel-17 NR coverage enhancements

- R4-2212832 PUCCH demodulation performance of Rel-17 NR coverage enhancements: simulation results