3GPP TSG-RAN WG4 Meeting#104-e                	R4-2212865

Electronic Meeting, 15 – 26 August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM transmit timing and timing advance issues for Rel-17 NR NTN

Agenda item:	9.11.5.2

Document for:	Discussion

Introduction

This contribution identifies and discusses several technical issues with the transmit and timing advance. Based on our discussions, we provide our views and suggested corrections to the text in TS 38.133,

Discussion

Transmit Timing Adjustments.

The current text provided in 38.133, Release 17.6.0 [2], states the following:

TS 38.133 Section 7.2C (…) The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Tc. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0. c (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. (…) When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell, the updating of NTA,common and the updating of NTA,UE-specific, except when the timing advance in clause 7.3C is applied.

The yellow marked sentence may lead to a behaviour by the UE different of the intended by the agreements in other RAN groups. The sentence states that (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Tc  corresponds to the difference in UE transmission timing and the downlink timing after the UE applies a  NTA relative to a Timing Advance Command (TAC). However, TAC is issued by the gNB on a per-need basis, the interval between them may be very large, whereas the agreements of UE pre-compensation of UL timing in RAN1 require the UE to update the values of NTA,common, NTA,UE-specific according to the orbital trajectory provided in the ephemeris and with the high-order parameters for the common delay drift.

As there are no update requirements currently set for NTA,common and NTA,UE-specific, it is NOT clearly specified that these values must be updated between TA commands.

Therefore, we propose:

Proposal 1 : Modify the requirements such that the reference for (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Tc   accounts for updates in  NTA,common and NTA,UE-specific.

Proposal 2: RAN4 to decide what is the reference point in time for updated values of NTA,common and NTA,UE-specific:

Option 1: The beginning of a DL frame at the UE side.

The proposed text modification to the specification is provided in a companion CR [3].

Previous agreements in other RAN groups also established that the update of NTA,common and NTA,UE-specific shall only be performed if the UE has a running validity timer for the NTN SIB, otherwise the UE must be precluded from transmitting.

Proposal 3: Include the requirements for the validity timer in the specifications.

UE Timing Advance

The updating of UE timing is captured in the timing adjustment accuracy requirements (7.1C.2.1) and the requirements for UE upon receiving a Time Advance Command (7.3.C2). However, in NTN two fundamental components for the timing advance calculation were introduced: NTA,common and NTA,UE-specific . There must be specification of the expected values for NTA,common and NTA,UE-specific , and their updating procedures within the validity timer, such that meaningful tests can account for the NTN agreements on UE pre compensation behavior.

Proposal 4: Introduce requirements for  NTA,common.

Proposal 5: Introduce requirements for  NTA,UE-specific.

The proposed text modification to the specification is provided in a companion CR [3].

Conclusion

The contribution has identified and discussed several technical issues with the transmit timing and timing advance in the specification. We therefore propose the following:

Proposal 1: Modify the requirements such that the reference for (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Tc   accounts for updates in  NTA,common and NTA,UE-specific.

Proposal 2: RAN4 to decide what is the reference point in time for updated values of NTA,common and NTA,UE-specific:

Option 1: The beginning of a DL frame at the UE side.

Proposal 3: Include the requirements for the validity timer in the specifications.

Proposal 4: Introduce requirements for NTA,common.

Proposal 5: Introduce requirements for NTA,UE-specific.

References

[1]    R4-2210612, WF on timing requirements and performance requirements for NR NTN, Xiaomi

[2]   TS 38.133, NR; Requirements for support of radio resource management, V17.6.0.

[3]   R4-2212853, CR to TS 38.133: Adding requirements for timing advance for satellite access, Nokia, Nokia Shanghai Bell