3GPP TSG-RAN WG4 Meeting#104-e                	R4-2212885

Electronic Meeting, 15 – 26 August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM tests for RLM for Rel-17 NR NTN

Agenda item:	9.11.6.6

Document for:	Discussion

Introduction

The WI description for RAN4 work related to NR over NTN was presented in [1]. Following the progress of RAN4 on the topic, it was approved an extension for the WI activities, in order to address eventual open issues. The WI exception list provided in [2] includes RAN4 RRM requirements, with provision for clarifications and addressing possible inconsistencies.  This contribution proposes further clarification for issues regarding the consistency of GNSS-related operation by UEs in NTN.   In [3] several issues are listed, of which we discuss several in the following sections.

Discussion

The procedure in current test scenarios for Radio Link Monitoring, described in Section A6.5.1, provide a baseline for the definition of the test scenarios for Radio Link Monitoring for NTN UEs. The test cases require the UE to be connected to the test cell, configured with CSI reporting and, in some cases, the time alignment timer is extended to infinity for long tests (in order to prevent the UE to go out-of-sync).

A.6.5.1.1.1 Test Purpose and Environment The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1. In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.1.1-1. The test parameters are given in Tables A.6.5.1.1.1-2, A.6.5.1.1.1-3, and A.6.5.1.1.1-4 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 1. (…) A.6.5.1.2.1 Test Purpose and Environment (…) Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. (…) A.6.5.1.3.1 Test Purpose and Environment (….) Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. (…) A.6.5.1.4.1 Test Purpose and Environment (…) Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. (…) A.6.5.1.5.1 Test Purpose and Environment (…) Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting of 5ms. In the test, DRX configuration is not enabled. (…) A.6.5.1.6.1 Test Purpose and Environment (…) Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. (…) A.6.5.1.7.1 Test Purpose and Environment (…) Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. (…) A.6.5.1.8.1 Test Purpose and Environment (…) Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. (…)

In NTN scenarios, there is an additional requirement for a UE to be considered “synchronized” to a cell: the UE must have a valid GPS position, the UE must have read the satellite ephemeris and there is a running validity timer for the ephemeris information. If the UE does not fulfill these criteria, the UE is not able to comply with UL transmissions, such as the CSI reporting.

Therefore, we suggest RAN4 to consider include description of what a fully synchronized UE must achieve for testing purposes.

Proposal 1: RAN4 to discuss the “fully synchronized” criteria for UE testing regarding ephemeris and validity timer acquisition.

Proposal 2: RAN4 to discuss the ephemeris validity timer to be used during test procedures, so that the UE timing alignment is maintained during the test.

Conclusion

Proposal 1: RAN4 to discuss the “fully synchronized” criteria for UE testing regarding ephemeris and validity timer acquisition.

Proposal 2: RAN4 to discuss the ephemeris validity timer to be used during test procedures, so that the UE timing alignment is maintained during the test.

References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-220209, Rel-17 Work Item Exception for NR-NTN-solutions, Thales

- R4-2210612, WF on timing requirements and performance requirements for NR NTN, Xiaomi

- TS 38.133, Release 17.6.0, NR; Requirements for support of radio resource management, 3GPP