3GPP TSG RAN WG4 Meeting #104-e	R4-2213050

Electronic Meeting, August 15 - August 26, 2022

Agenda item:			9.21.2.1

Source:	vivo

Title:	Further discussion on performance requirements for PDC enhancement

Document for:	Discussion

- Introduction

In the RAN4#103-e meeting, RRM performance requirements for propagation delay compensation enhancements were extensively discussed. The agreements and remaining open issues were captured in WF [1].

Issue 3-1: Define UE Rx-Tx time difference measurement requirement for PRS under fading channel conditions? RAN4 shall focus on specify performance requirements for AWGN condition as first priority: - UE Rx-Tx time difference measurement requirement for PRS - gNB Rx-Tx time difference measurement requirement for SRS - UE Rx-Tx time difference measurement requirements for TRS FFS whether additional requirements for fading condition shall be specified or not Define gNB Rx-Tx time difference measurement requirement for SRS under fading channel conditions? - RAN4 should not define gNB Rx-Tx time difference measurement requirement for SRS under fading channel conditions Issue 3-3: The channel model used when defining UE Rx-Tx time difference measurement requirements for TRS? Way forward/Agreement: further discuss - At least define UE Rx-Tx time difference measurement requirement for TRS under AWGN channel conditions - Continue to discuss whether RAN4 additionally should define UE Rx-Tx time difference measurement requirement for TRS under fading channel conditions Moderator comment: see GTW agreement for Issue 3-1. Issue 3-4: Should RAN4 define requirements for UE Rx-Tx time difference measurement requirements for TRS under one channel model or both channel models (fading and AWGN)? Way forward: further discuss - At least define UE Rx-Tx time difference measurement requirement for TRS under AWGN channel conditions - Continue to discuss whether RAN4 should define UE Rx-Tx time difference measurement requirement for both channel models (fading and AWGN) Issue 3-5: RAN4 to use same BW groups from PRS to define accuracy for TRS based PDC? - RAN4 will use same BW groups from PRS to define accuracy for TRS based PDC Issue 3-6: RAN4 to use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx? - RAN4 will use same group delay calibration margin from PRS to accuracy for TRS based UE Rx-Tx Issue 3-7: RAN4 to introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17? - RAN4 will introduce new SRS type in TS 38.133 A.3.24 for usagePDC-r17 Issue 3-8: Test case list for measurement core requirements Issue 3-9: Test case list for measurement accuracy requirements - RAN4 agree to use one set of test cases to verify both the delay and accuracy of PDC measurement Way forward/Agreement: further discuss - On the CSI-RS RMC, we agree that we need a new RMC for TRS based PDC. TRS RMC is defined in A.3.17 instead of A.3.30? - Remove 60kHz SCS for both FR1 and FR2 because 60kHz SCS is not tested in any existing test case? - Include sub-tests for two different PRS/TRS BWs because of different accuracies?

In this contribution, we further provide our views on remaining open issues of accuracy requirements for PDC enhancement.

- Discussion

It was agreed that RAN4 shall focus on specify performance requirements for AWGN condition as first priority:

- UE Rx-Tx time difference measurement requirement for PRS

- gNB Rx-Tx time difference measurement requirement for SRS

- UE Rx-Tx time difference measurement requirements for TRS

It is FFS whether additional requirements for fading condition shall be specified or not

Both gNB-side RTT PDC and UE-side RTT PDC are supported based on RAN2 agreements.

- RAN2 confirms that gNB-side RTT Propagation Delay Compensation is supported. - For the separate signalling procedures for UE-side RTT PDC, provision of measurement configuration indicates that UE measures the Rx-Tx time difference, and provision of gNB Rx-Tx time difference to UE implicitly activates RTT-based PDC calculation at the UE side. - For a UE that has been configured to perform RTT measurements but has not been provided with gNB Rx-Tx time difference measurement to calculate PDC, the UE applies the reference time, without UE-side PDC

Regardless of UE-side PDC or gNB-side PDC, final time compensation accuracy is dependent on accuracy of both UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement. Therefore, similar requirements should be specified for both gNB Rx-Tx time difference measurement and UE Rx-Tx time difference measurement.

Since RTT-based PDC is also useful under fading channels, it would be better to specify requirements with fading channels. With 4 samples are used in the measurement core requirements, only accuracy requirements are needed for fading channels for UE Rx-Tx time difference measurement.

Same principle is applied to PRS based and TRS based UE Rx-Tx time difference measurement.

Proposal 1: Accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on PRS and gNB Rx-Tx time difference measurement based on SRS, with measurement of 4 samples.

Proposal 2: Accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on TRS and gNB Rx-Tx time difference measurement based on SRS, with measurement of 4 samples.

- Summary

In this contribution, we further provided our views on the remaining open issues of accuracy requirements for PDC enhancement. Based on analysis following observations and proposals are present.

Proposal 1: Accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on PRS and gNB Rx-Tx time difference measurement based on SRS, with measurement of 4 samples.

Proposal 2: Accuracy requirements under fading channel are specified for both UE Rx-Tx time difference measurement based on TRS and gNB Rx-Tx time difference measurement based on SRS, with measurement of 4 samples.

- References

- R4-2210618	WF on NR_IIOT_URLLC_enh, Nokia