3GPP TSG-RAN WG4 Meeting # 104-e 		R4-2213075

Electronic Meeting, August 15 - 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	BS demodulation performance for NB-IoT 16QAM

Agenda item:	9.24.6.1.2

Document for:	Discussion

Introduction

The Rel-17 work item for additional enhancements for NB-IoT and LTE-MTC was approved at TSG RAN#88-e with latest WID revision [1]. At RAN4 #103-e, the technical discussion on BS demodulation requirements for NB-IoT was continued [2] and simulation assumptions for NPUSCH format 1 were finalized [3]:

Table 5-3: Summary of simulation assumption for NPUSCH format 1

| Parameter | Value |
| --- | --- |
| Number of antennas | 1T2R |
| Propagation condition | ETU 1Hz Low |
| SCS | 15KHz |
| Frequency Resource | 12 tones |
| Modulation order | 16QAM |
| Number of Repetition | 1 |
| TB Scheduling | single-TB scheduling |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| (IRU, ITBS), TBS | (0,15), 280bits |
| RV | {0,2,0,2} |
| Max number of HARQ transmission | 4 |

This contribution provides our BS demodulation performance proposals for alignment and for the specification.

NPUSCH format 1 16QAM performance

Link-level simulations have been performed based on the agreed simulation assumptions in [3] with real channel and noise estimation.

Simulation results for SNR required at 70% of maximum throughput are summarized in Table 1.

Table 1: SNR at 70% of maximum TP of proposed new FRCs for 16QAM in NB-IoT uplink

| Reference channel | 12 tones / 280 bits |
| --- | --- |
| SNR [dB] @TP=70%, ETU1, with HARQ | 5.7 |

Include simulation result of 5.7 dB to final simulation summary for NPUSCH format 1 16QAM alignment results.

Regarding performance with impairments, we propose to apply a margin of 2.5 dB as defined in [4], clause 7.3 dynamic range.

Use impairment margin of 2.5 dB for NPUSCH format 1 16QAM requirements.

Conclusion

This contribution has provided our BS demodulation performance proposals for alignment and for the specification for NB-IoT 16QAM [1].

Following proposals are submitted for discussion:

- Include simulation result of 5.7 dB to final simulation summary for NPUSCH format 1 16QAM alignment results.

Use impairment margin of 2.5 dB for NPUSCH format 1 16QAM requirements.

References

- RP-211340, WID revision: Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN #92e

- R4-2210534, Email discussion summary for [103-e][331] NB-IOT_MTC_Demod, Moderator (Huawei, HiSilicon), RAN4 #103-e

- R4-2210674, WF on Rel-17 NB-IOT and eMTC performance requirements, Huawei, RAN4 #103-e

- TR 38.817-02 V15.9.0, General aspects for Base Station (BS) Radio Frequency (RF) for NR