3GPP TSG-RAN4 Meeting #104-e	R4-2213173

# Electronic meeting, Aug. 15th – Aug. 26th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.133 Test case for R17 UL TCI state switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-8-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Rel-17, core requirement for unified TCI state switching delay is introduced. Therefore the corresponding test case for validating the feature of UE TCI state switching should be introceduced in TS38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the test case for UE UL TCI state switching indicated by a separate UL TCI of unified TCI (TCI-UL-State-r17). The test defined in this CR is in the scenario of EN-DC FR2 PSCell with the configuration that the TCI state is known but the pathlossReferenceRS is not maintained. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The test case for UL TCI state switching is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | - A.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | THARQ defined in TS38.213 clause 9.2.3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st version |  |  |  |  |  |  |  |  |

==========================Start of 1st change =============================

A.5.5.X	Uplink TCI state switch delay

A.5.5.X.2	MAC-CE based active uplink TCI state switching

A.5.5.X.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switching for a known TCI state

A.5.5.X.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the MAC-CE based uplink TCI state switch delay requirement defined in clause 8.16.3 by a UE capable of beam correspondence without the need for UL beam sweeping. Supported test configurations are shown in Table A.5.5.X.2.1.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1), and one NR PSCell (Cell 2) as given in Table A.5.5.X.2.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.2-1 and cell-specific parameters of NR PSCell is specified in Table A.5.5.X.2.1.2-1 below. The OTA related test parameters for FR2 is shown in Table A.5.5.X.2.1.2-2.

Table A.5.5.X.2.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.5.5.X.2.1.1-2: General test parameters for spatial relation switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel No. |  | Channel 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel No. |  | Channel 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| L1-RSRP reporting period | slot | 160 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0, SSB1 | L1-RSRP measurements of SSB0 and SSB1. |
| Number of reported RS |  | 2 | L1-RSRP reporting of measurements on SSB0 and SSB1. |
| T1 | s | 2 |  |
| T2 | s | 2 |  |

A.5.5.X.2.1.2	Test parameters

Before the test starts,

-	PDCCHs indicating new transmissions shall be sent continuously on PSCell (Cell 2) to ensure that the UE would have continuous ACK/NACK sending by PUCCH;

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC);

-	UE is provided with TCI-UL-State-r17 and UE’s higher layer signalling unifiedTCI-StateType-r17 in IE MIMOParam-r17 is set to separate;

-	UE is configured with 2 different UL TCI states for PSCell, PUCCH UL TCI state 0 (associated with SSB0) and UL TCI state 1 (associated with SSB1), by using RRC signalling ul-TCI-StateList-r17 in IE BWP-UplinkDedicated, in Cell 2 before starting the test;

-	UE is indicated in UL TCI state 0 as the active PUCCH UL TCI state.

The test consists of two time periods, T1 and T2. During T1 only SSB with which PUCCH UL TCI state 0 is associated is transmitted. At the beginning of T2, the SSB corresponding to UL TCI state 1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The test consists of two time periods, T1 and T2. During T1, SSBs with which PUCCH UL TCI state 0 and UL TCI state 1 are associated are transmitted. At the beginning of T2, in slot n which is within 1280ms of UE providing L1-RSRP report with results for both SSB0 and SSB1, UE receives a MAC-CE command indicating a switch to transmit PUCCH with UL TCI state 1. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

Index of CSI-RS1 is configured for UE as pathlossReferenceRS-Id-r17 which is indicated in TCI-UL-State-r17 of UL TCI state 1. CSI-RS1 is QCL’d typeD with SSB1 and configured with higher layer parameter repetition set to on.

Table A.5.5.X.2.1.2-1: NR Cell specific test parameters for UL TCI state switch

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.1 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| CSI-RS Configuration |  | CSI-RS.3.4A TDD |
| Uplink TCI State 0 |  | UL-TCI.State.0 |
| Uplink TCI State 1 |  | UL-TCI.State.1 |
| TRS Configuration |  | TRS.2.1 TDD |
| reportQuantity for SSB |  | ssb-Index-RSRP-Index-r17 |
| reportConfigType for SSB |  | periodic |
| reportQuantity for CSI-RS |  | cri-RSRP-Index-r17 |
| reportConfigType for CSI-RS |  | periodic |
| timeRestrictionForChannelMeasurements |  | configured |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.5.5.X.2.1.2-2: OTA related test parameters for uplink spatial relation switch

| Parameter | Unit | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |
|  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup 3 according to clause A.3.15.3 |  |  |  |
|  |  | AoA1 |  | AoA2 |  |
| Assumption for UE beams Note 6 |  | Rough for SSB reception |  |  |  |
| NocNote 1 | dBm/15 kHz | -92.1 |  |  |  |
| NocNote 1 | dBm/SCS | -83.1 |  |  |  |
| Ês/Noc | dB | 1 | 1 | 1 | 1 |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -82.1 | -82.1 | -82.1 | -82.1 |
| IoNote2,Note6 | dBm/95.04 MHz Note4 | -50.6 | -50.6 | -50.6 | -50.6 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |

A.5.5.9.1.1.2	Test Requirements

The test verifies that UE can be scheduled on PSCell on UL TCI state 0 and UL TCI state 1. The test also verifies the active UL TCI state switch time in PSCell meeting the requirement defined in 8.16.3. Specifically,

Before T2, UE shall send L1-RSRP report with results for SSB1.

After receiving MAC-CE command in slot n in T2, UE shall:

-	be able to continue to transmit PUCCH on UL TCI state 0 till slot n + THARQ + 3Nslotsubframe,µ;

-	be able to start transmitting PUCCH on UL TCI state 1 from slot n + THARQ + 3Nslotsubframe,µ + (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length and onwards.

==========================End of 1st change =============================