3GPP TSG-RAN4 Meeting #104-e	R4-2213174

# Electronic meeting, Aug. 15th – Aug. 26th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | dCR | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.133 RRM test configurations for FeMIMO test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-8-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Rel-17, core requirement for unified TCI state switching delay is introduced. Corresponding test case for validating the feature of UE TCI state switching should be introceduced in TS38.133. For the UL TCI state switching tests, new configurations for UL TCI state should be introduced for test parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introducing UL TCI state configurations for the corresponding tests. - Introducing new CSI-RS configuration as pathloss RS for UL TCI state switching tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UL TCI state configurations for the tests are missing. - No proper CSI-RS configuration as pathloss reference signal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | - A.3.16 and A.3.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st version |  |  |  |  |  |  |  |  |

==========================Start of 1st change =============================

A.3.14	CSI-RS configurations

< Unchanged parts are omitted >

A.3.14.2	TDD

< Unchanged parts are omitted >

Table A.3.14.2-3A: CSI-RS Reference Measurement Channels for SCS=120kHz

|  | CSI-RS.3.1A TDD | CSI-RS.3.2A TDD | CSI-RS.3.3A TDD | CSI-RS.3.4A TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic | periodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 | 1 |
| repetition | off | off | off | on |
| aperiodicTriggeringOffset | n.a. | 6 | n.a. | n.a. |
| trs-Info | n.a. | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 | 1 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 | 2 for resource #1 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot160 | n.a. | slot80 | slot160 |
| Offset | 8 | n.a. | 16 | 16 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |  |

==========================Start of 1st change =============================

==========================Start of 2nd change =============================

A.3.16	TCI State Configuration

A.3.16.1	Introduction

This clause provides the configurations for TCI states towards either SSB or CSI-RS. The TCI states defined in this clause are configured in each test when applicable to indicate that certain DL signals are QCL’ed with the referenceSignal configured in the TCI states.

A.3.16.2	TCI states

TCI state configurations listed in this clause are for separate downlink TCI states.

Table A.3.16.2-1: TCI States

| Parameter | TCI.State.0 | TCI.State.1 | TCI.State.2 | TCI.State.3 |
| --- | --- | --- | --- | --- |
| tci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeC | typeC | typeA | typeA |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD |
| referenceSignal | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. |  |  |  |  |

A.3.16.3	UL TCI states

TCI state configurations listed in this clause are for separate uplink TCI states configured by ul-TCI-ToAddModList-r17.

Table A.3.16.3-1: UL TCI States

| Parameter | UL-TCI.State.0 |  | UL-TCI.State.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tci-UL-State-Id | Id0 |  | Id1 |  |  |  |  |  |
| referenceSignal Note1 |  | SSB0 |  | SSB1 |  |  |  |  |  |
| pathlossReferenceRSNote1 | n.a. |  | CSI-RS1 |  |  |  |  |  |
| Note 1: referenceSignal and pathlossReferenceRS configurations towards which the TCI states are configured are defined in a test-specific manner. |  |  |  |  |  |  |  |  |  |

==========================Start of 2nd change =============================