3GPP TSG-RAN WG4 Meeting #104-e	R4-2213310

Electronic Meeting, 15 – 26 Aug, 2022

Title:	R18 Discussion on MSD improvement signalling

Source:	OPPO

Agenda item:	11.6.4.2

Document for:	Approval

- Introduction

In Rel-18, lower MSD is one of the targets for FR1 enhancement WI, and the objectives from WID [1] is as below. This paper will discuss the signalling aspects.

Investigate the feasibility of lower MSD for inter-band CA/EN-DC/DC combinations - Select a limited set of band combinations (2-4 combinations) to cover all types of MSD (harmonic, harmonic mixing, IMD and cross band isolation) - Study how the MSD performance can be improved for the example band combinations - Study of MSD improvement with different MSD sources (harmonics, IMD2/3/4/5, cross band isolation and harmonic mixing) - Study the feasibility of and options for allowing a UE to signal improved lower MSD performance capability for combinations where MSD is allowed - Aim to conclude the study phase by RAN#99, and further discuss in RAN#99 how to handle the objective based on the study progress.

- Discussion

MSD signaling was discussed widely as a candidate for the MSD improvement topic, and it didn’t intend to study how the MSD can be improved, instead just define another lower MSD in the spec, and ask UE to signal whether it can achieve that. Then NW based on the reported MSD to decide whether or not to configure a band combination.

This approach received concerns on the usage of this MSD signaling for several reasons like:

- MSD requirement just represent the worst case which doesn’t necessarily mean UE cannot work at all with a certain band combination for example when UE is in the cell center, or when UE Tx power is not high, etc. Therefore, using MSD to decide whether UE can work with a band combination is not always proper;

- How NW will use this reported MSD signaling is unknow;

Observation 1:    MSD probably cannot be used as the only condition to decide whether NW configure or not configure a band combination since it also depends on other factors like the DL signal levels and UE Tx power.

Proposal 1:         Propose to clarify how MSD signaling will be used by NW if UE report large or low MSD values.

Besides, it is not proper to define another set of requirements that is much larger (e.g.10dB, 20dB, 30dB) than minimum requirements this will make the spec quite wired. And it is well understood that RAN4 requirements are minimum requirements, and UE can always do better. Instead of define a much better requirement in RAN4 and ask UE to report whether it can achieve that, it would be more flexible to be a configured threshold by NW and request UE to report it can achieve or not. However, this kind of approach may also need to clarify the NW behavior when it got the message from UE.

Observation 2:    Defining two quite different sets of MSD in RAN4 spec may cause confusion about the minimum requirements. Comparingly, by NW configured MSD threshold and UE report its ability would be more flexible.

Proposal 2:         Consider NW configured MSD reporting threshold and UE indicate its ability instead of defining another MSD requirement in RAN4 spec.

- Conclusion

In this paper, we discussed the possible MSD signaling approaches, and got following observations and proposals.

Observation 1:    MSD probably cannot be used as the only condition to decide whether NW configure or not configure a band combination since it also depends on other factors like the DL signal levels and UE Tx power.

Proposal 1:         Propose to clarify how MSD signaling will be used by NW if UE report large or low MSD values.

Observation 2:    Defining two quite different sets of MSD in RAN4 spec may cause confusion about the minimum requirements. Comparingly, by NW configured MSD threshold and UE report its ability would be more flexible.

Proposal 2:         Consider NW configured MSD reporting threshold and UE indicate its ability instead of defining another MSD requirement in RAN4 spec.

- References

[1] RP-221496, New WID on Further RF requirements enhancement for NR frequency range 1 (FR1)