3GPP TSG-RAN4 Meeting #104-e	R4-2213369

# Electronic meeting, Aug 15-26, 2022

Title:		On remaining RF requirements on n263

Source:					Huawei, HiSilicon

Agenda Item:			9.14.3.1

Document for:		Discussion and Endorsement

Introduction

There are still a few RF requirements for band n263 not fully determined after RAN4 #103-e meeting. This paper provides proposals for completing the RF requirements.

Discussion

UL gap for TX power management

The requirements for UL gap for Tx power management was introduced in RAN4#103e meeting. Currently the requirements don't distinguish between FR2-1 and FR2-2. Considering FR2-2 was not studied when defining the requirements, it’s would be reasonable to exclude FR2-2 in Rel-17.

Proposal 1: The UL gap for Tx power measurement doesn’t apply to FR2-2 in Rel-17.

Dynamic power requirements

The minimum output power for PC 1, 2 and 3 are still TBD. OFF power requirements are same as FR2-1 but within brackets. Based on our analysis on this stage, the FR2-1 requirements for each power class could be reused.

Proposal 2: Reuse FR2-1 requirements for minimum output power and OFF power.

EVM

At RAN4#103 meeting, following agreement was made for EVM

Issue 2: Scaling min EVM power level for BW > 400 MHz

Proposal 1: Scale minimum EVM power level based on BW for > 400 MHz

Agreement: agree proposal 1.

According to the agreement, the power scaling doesn’t apply to BW <400MHz, however in current specification, the EVM power is also scaled for 100MHz.

Proposal 3: The min EVM power for 100MHz reuses values from FR2-1 and is not scaled.

ACLR

In RAN4#101, there was agreement for ACLR as below [1]:

Agreement: Specify 99% OBW requirements in CCBW and specify ACLR

- Waive the ACLR in the test if the ACLR requirement is less stringent than OBW requirement

Considering 99% OBW requirements could be approximately translated to 23dB ACLR, and is more stringent than agreed ACLR requirements for n263, the test of ACLR could be waived.

Proposal 4: Waive the test of ACLR for band n263.

REFSENS

In RAN4#102-e, the REFSENS for band n263 PC1 was agreed [2] but not implemented in the specification.

- PC1 REFSENS

proposed WF:

Two proposals, one based on 64 elements, and one based on 32 elements.

| PC1 REFSENS | PC1 | elements |
| --- | --- | --- |
| QCOM | -88.3 | 64 |
| Intel | -76.2 | 32 |

Company comments

QCOM and Intel; The two proponents can compromise to -79 dBm in 400 MHz

Company B:

Tentative Agreement: -79 dBm in 400 MHz

Proposal 5: Implement the agreed PC1 REFSENS in the specification.

In the subclause of PC1 REFSENS, the uplink configuration for REFSENS is specified for each band and each applicable bandwidth respectively. The uplink configuration table is also referred by other power classes. For band 263, the uplink configuration needs to be specified for 100MHz, 400MHz, 800MHz, 1600MHz and 2000MHz. Considering there is not a common applicable SCS for all the bandwidths, the uplink configuration could be specified for each bandwidth on the minimum applicable SCS.

Proposal 6: Specify the uplink configuration for band n263 as in Table 2.6-1.

Table 2.6-1

| Operating band | NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n262 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n263 | N.A | 64 | N.A | 256 | N.A | N.A | N.A | 120 kHz | TDD |
|  | N.A | N.A | N.A | N.A | 120 | 243 | N.A | 480 kHz | TDD |
|  | N.A | N.A | N.A | N.A | N.A | N.A | 144 | 960 kHz | TDD |

ACS

In RAN4#101-bis-e, the UE ACS requirements was agreed as 21dB based on the co-existence study. This needs to be captured in TS 38.101-2.

Proposal 7: The ACS of band n263 is 21dB. The measurement bandwidth is the same as minimum output power.

In-band blocking

For FR2-1, In-band blocking was specified such that:

- Requirements are specified for all the bandwidths

- The IBB requirement is the same as ACS

The principle could also apply to FR2-2.

Proposal 8: The IBB of band n263 is specified for all supported bandwidths.

Proposal 9: The IBB requirement of band n263 is the same as ACS requirement

Conclusion

This document provides view for remaining band specific RF requirements for n263. With proposals summarized as below, the expected changes in the specification are provided in document R4-2213366 and R4-2213367.

Proposal 1: The UL gap for Tx power measurement doesn’t apply to FR2-2 in Rel-17.

Proposal 2: Reuse FR2-1 requirements for minimum output power and OFF power.

Proposal 3: The min EVM power for 100MHz reuses values from FR2-1 and is not scaled.

Proposal 4: Waive the test of ACLR for band n263.

Proposal 5: Implement the agreed PC1 REFSENS in the specification.

Proposal 6: Specify the uplink configuration for band n263 as in Table 2.6-1.

Proposal 7: The ACS of band n263 is 21dB. The measurement bandwidth is the same as minimum output power.

Proposal 8: The IBB of band n263 is specified for all supported bandwidths.

Proposal 9: The IBB requirement of band n263 is the same as ACS requirement

Reference

[1] R4-2120062	WF on 60GHz UE RF requirements, RAN4#101-e

[2] R4-2206536	WF on 60 GHz UE RF, RAN4#102-e

[3] R4-2210578	WF on 60 GHz UE Requirements, RAN4#103e