3GPP TSG-RAN WG4 Meeting # 104-e		R4-2213381

# Electronic Meeting, August 15 – August 26, 2022

Source:	ZTE Corporation

Title:	draft reply LS on UL Tx switching across 3 or 4 bands in Rel-18

Agenda Item:	11.15.2

Document for:	Discussion

# Introduction

The WID on multi-carrier enhancements for NR includes an objective for UL Tx switching across up to 3 or 4 bands operation [1]:

- Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Note: Extension of TX switching for 2 bands to multiple TAG configurations is included in the scope. The work is limited to RAN4. - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

The discussion has already been started in RAN1 and an LS to RAN4 was sent out by conveying the RAN1 agreements and question on the potential increase of switching period and UE complexity in comparison with the case of 2 bands [2]:

Agreement - Companies are encouraged to investigate pros and cons of following possible mechanisms for dynamic Tx carrier switching across the configured bands, and RAN1 strives for the down-selection at RAN1#110 - Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via UL grant and/or RRC configuration for UL transmission - Alt.2: NW indicates 2 bands out of the configured bands (3 or 4 bands) via DCI or MAC-CE, and dynamic Tx carrier switching between indicated bands is same as Rel-17 - Alt.3: One anchor band is selected among configured bands (3 or 4 bands), and dynamic Tx carrier switching can be performed only from the anchor band to a non-anchor band and from a non-anchor band to the anchor band - Note: Other mechanisms are not precluded Based on the above observations and agreement, RAN1 also agreed to send this LS to respectfully ask RAN4’s feedback on the potential increase of switching period and UE’s complexity in case of UL Tx switching across 3 or 4 bands in comparison to 2 bands. RAN4’s prompt feedback will be appreciated and would be helpful for potential down-selection at RAN1#110. Additionally, RAN1 would like to also ask RAN4’s feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands. - “When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period.”

The paper addresses these questions from RAN4 perspective, and based on our understanding we propose the draft reply LS accordingly as seen in the Annex section.

# Discussion

Fig. 1, Rel-16/17 UL Tx switching

In Rel-16 and Rel-17, UL Tx switching is specified to support the switching between 2 bands with 1T-2T/2T-2T and for both 2 CCs and 3 CCs as shown in Fig. 1, where each row corresponding one Tx chain, and each column corresponding to the states of the two Tx chains. And both switching options, i.e., switched-UL and dual-UL, are supported.

Rel-18 sees and addresses the demand for increasing the number of bands involved from 2 to 3 or 4. However, the maximum number of concurrent UL transmissions still remains unchanged, i.e., 2. This restriction leads to the fact that UL Tx switching in Rel-18 has to be dependent on Rel-16/17 UL Tx switching operation between two selected bands, and requires an additional operation in comparison with that in Rel-16/17. The additional operation is the selection of one band pair within the concerned band combination. Fig. 2 illustrates the Rel-18 UL Tx switching.

The additional band pair selection naturally means the swapping of band pairs, thus involves an additional potential switching period when the running band pair is changed. Note that the band pair switching period only happens once per band pair change, and plays no impact on conventional Rel-16/17 UL Tx switching. Therefore, the requirement might be introduced only when the band pair swapping is frequent enough, which may require further study in RAN4.

Observation 1: A one-time additional switching period associated with each band pair swapping is required, and RAN4 may study further whether or not to introduce any additional requirement.

According to the LS from RAN1 [2], there are three alternatives considered in association with the selection of band pairs from a band combination from RAN4 perspective:

- Alt. #1: in one band combination, swapping between any of band pair is supported.

- Alt. #2: in one band combination, switching between an explicitly indicated band pair is supported.

- Alt. #3: in one band combination, there is one anchor band defined, and only band pairs having a common band can be swapped.

Fig. 2, Rel-18 UL Tx switching across >2 bands

For Alt. #1, one possible way is to apply the same switching period for all band pairs. However, if there is one band pair where Rel-16/17 UL Tx switching is not supported, then this band pair will stop supporting Rel-18 UL Tx switching for the band combination, thus a restriction is imposed to form a band combination supporting Rel-18 UL Tx switching: for a band combination intended for Rel-18 Tx switching, any of band pair within the band combination should be able to perform Rel-16/17 Tx switching. The restriction leads to the cost of flexibility and applicability.

Observation 2: For Alt. #1, if applying one identical switching period for all band pairs in a band combination, there may be a restriction on forming a band combination supporting Rel-18 UL Tx switching at cost of flexibility and applicability.

Another way is to apply one switching period to all the band pairs in the band combination, and since the real UL Tx switching is based on Rel-16/17, Rel-16/17 requirements should apply, then the band-combination-wise switching period should be at least the maximum of switching periods among all band pairs in the band combination. Obviously, this may degrade the performance thus offset the potential benefits of switching more than 2 bands.

Observation 3: For Alt. #1, applying one identical switching period for any band pair in the band combination may offset the potential performance benefits of UL Tx switching across more than 2 bands.

For Alt. #2, network controls and configures which band pair is chosen at a time for Rel-16/17 UL Tx switching according the reported UE capability, and there is no restriction like Alt. #1. And the switching period for the configured band pair can apply same as Rel-16/17, thus it may harvest all potential benefits from UL Tx switching across more than 2 bands in terms of performance and applicability.

Observation 4: Alt. #2 outperforms in terms of performance and applicability where the switching period for a band pair same as Rel-16/17 may apply.

For Alt. #3, an anchor band is defined in one band combination, and the band pair swapping must happen between two band pairs having a common band (i.e., non-anchor band). In addition to the flexibility loss, several questions may be raised:

- For one band combination supporting Rel-18 UL Tx switching, is the anchor band in this band combination fixed?

- If fixed, how to address the potential demands for different anchor bands from different operators?

- If not fixed, how to determine the anchor band and can it be dynamically changed during one connection?

Actually, these questions lead to one fact that Alt. #3 is supported if Alt. #2 is supported. Alt. #2 can support Alt. #3 if an anchor band is introduced.

Observation 5: Alt. #3 is a sub-case of Alt. #2.

Fig. 3 Band pair swapping with one common band and switching option change

Regarding the last question on the assumption raised in the LS, i.e., “When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period.”, it applies for the Rel-16/17 UL Tx switching, however, for the additional band pair swapping in Rel-18, it could be different for some cases. Fig. 3 shows one case:

- A band pair swapping from band pair A+B to A+C

- Switching options between Switched-UL and Dual-UL

- The end state of band pair A+B is one Tx chain for band A, the other Tx chain for band B

- The initial state of band pair A+C is one Tx chain for band A, the other Tx chain for band C

In this case, the second Tx chain is triggered to switch from band B to C with the band pair change from A+B to A+C, and the first Tx chain may continue to work on band A during the band pair swapping.

Observation 6: The UE assumption/behavior in case of the UL Tx switching across 3 or 4 bands may not apply for some case with band pair swapping.

Based on the above discussion, we draft the reply LS as seen in the Annex.

Proposal 1: Send the reply LS as in Annex to RAN1.

# Conclusion

In this paper, we have the following observations and proposal for the reply LS to RAN1 on UL Tx switching across 3 or 4 bands in Rel-18:

Observation 1: A one-time additional switching period associated with each band pair swapping is required, and RAN4 may study further whether or not to introduce any additional requirement.

Observation 2: For Alt. #1, if applying one identical switching period for all band pairs in a band combination, there may be a restriction on forming a band combination supporting Rel-18 UL Tx switching at cost of flexibility and applicability.

Observation 3: For Alt. #1, applying one identical switching period for any band pair in the band combination may offset the potential performance benefits of UL Tx switching across more than 2 bands.

Observation 4: Alt. #2 outperforms in terms of performance and applicability where the switching period for a band pair same as Rel-16/17 may apply.

Observation 5: Alt. #3 is a sub-case of Alt. #2.

Observation 6: The UE assumption/behavior in case of the UL Tx switching across 3 or 4 bands may not apply for some case with band pair swapping.

Proposal 1: Send the reply LS as in Annex to RAN1.

# Reference

RP-221435 Revised WID on Multi-carrier enhancements, NTT DoCoMo

R1-220552 LS on UL Tx switching across 3 or 4 bands in Rel-18, RAN1

# Annex

Title: draft reply LS on UL Tx switching across 3 or 4 bands in Rel-18 Response to: Release: Rel-18 Work Items: NR_MC_enh-Core Source: RAN WG4 To: RAN WG1 CC: Contact Person: #### Name: Aijun Cao E-mail Address: cao.aijun@zte.com.cn 1. Overall Description: RAN4 respectfully thanks to RAN1’s LS on UL Tx switching across 3 or 4 bands in Rel-18 with the RAN1 agreements already made. RAN4 discusses the questions raised in the LS in RAN4#104-e, and agrees that a one-time additional switching period associated with each band pair swapping is required, and RAN4 will study further whether or not to introduce any additional requirement. Furthermore, RAN4 reaches to the following common understandings on the three alternative schemes: - For Alt. #1, if applying the same switching period as that in Rel-16/17 for a band pair, there is a restriction on forming a band combination supporting Rel-18 UL Tx switching at cost of flexibility and applicability. If applying one identical switching period for any band pair in the band combination may offset the potential performance benefits of UL Tx switching across more than 2 bands. - Alt. #3 is a sub-case of Alt. #2 - Alt. #2 outperforms in terms of performance and applicability where the same switching period for a band pair as Rel-16/17 may apply. Regarding the UE assumption/behavior in case of the UL Tx switching across 3 or 4 bands: - “When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period.” RAN4 identifies some case where the assumption may not apply during band pair swapping. One example is: - A band pair swapping from band pair A+B to A+C - Switching options between Switched-UL and Dual-UL - The end state of band pair A+B is one Tx chain for band A, the other Tx chain for band B - The initial state of band pair A+C is one Tx chain for band A, the other Tx chain for band C 2. Actions: To RAN WG1 RAN WG4 would like to respectfully ask RAN WG1 to take into account the above feedback for UL Tx switching across 3 or 4 bands. 4. Date of Next RAN WG4 Meetings: TSG-RAN WG4 Meeting #104bis-e 10th – 19th October 2022 Online TSG-RAN WG4 Meeting #105 14th – 18th November 2022 Canada