3GPP TSG-RAN WG4 Meeting #104-e	R4-2213418

Electronic Meeting, 15 – 26 Aug, 2022

Title:	Consideration on dual-panel test method of FR2 OTA

Source:	OPPO

Agenda item:	11.5.2

Document for:	Approval

- Introduction

The new study item of NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements was approved in RAN #95-e meeting. And the test methods for multi-panel reception and 4DL layer are agreed to take those defined in TR 38.810 and TR 38.884 as the baseline. [1]

The paper presents our views on test methods in TR 38.810 and TR 38.884 for multi-panel simultaneous reception.

- Discussion

The test methodologies defined in TR 38.810 and TR 38.884 are briefly reviewed.

In TR 38.810, four permitted test methods are defined as FR2 UE RF testing methodology in Clause 5.2, i.e. Direct Far Field (DFF), Direct Far Field setup simplification for centre of beam measurements (simplified DFF), Indirect Far Field (IFF), Near Field to Far Field Transform (NFTF). These four test methods are designed and suitable for single AoA test scenario for RF test purpose. In Clause 6.2, the baseline measurement setup for RRM test is shown below, which is capable of providing two AoAs. DFF, simplified DFF and IFF can be applied for UE RRM baseline measurement setup. However, the absolute position of the probes is left up to system vender’s implementation.

In TR 38.884, three enhanced test methods are permitted for FR2 RF measurements, i.e. Combined Far-Field/Near-Field (CFFNF), Combined Far-Field/Direct-Near-Field (CFFDNF) and Combined Far-Field/delta-Near-Field (CFFdeltaNF). These three test methods are defined to solve the issues of high DL power and low UL power. And they are not capable for supporting two AoAs so far.

From the above, the permitted test methods defined in both TR 38.810 and TR 38.884 are not suitable for the measurement of multi-panel simultaneous reception, i.e. at least two AoAs emulated around the DUT.

Observation 1:       The permitted test methods defined in both TR 38.810 and TR 38.884 are not suitable for the measurement of multi-panel simultaneous reception, i.e. at least two AoAs emulated around the DUT.

Besides TR 38.810 and TR 38.884, TR 38.827 (i.e. Study Item for NR MIMO OTA) has provided a test method setup for FR2 MIMO reception performance verification, that equipped with 6 probes distributed on the spherical surface, as below figure.

The probe locations in Channel Model Coordinate System are listed below. The largest angle difference of the probes on Phi axis is 50 degree, which is an advantage for building a multi-AoA channel environment. Therefore, the test method setup for FR2 MIMO OTA in TR 38.827 can also be included as the baseline together with those in TR 38.810 and TR 38.884.

Proposal 1:             The test method setup for FR2 MIMO OTA in TR 38.827 can also be included as the baseline together with those in TR 38.810 and TR 38.884.

Considering two panels receiving simultaneously, two different AoAs are required. Due to UE’s implementation, the performance of dual-panel reception depends on the pair of AoAs. Therefore, the two AoAs used for multi-panel simultaneous reception need to be further studied.

Proposal 2:             The two AoAs used for multi-panel simultaneous reception need to be further studied.

- Conclusions

This paper discussed about our views on test methods in TR 38.810 and TR 38.884 for multi-panel simultaneous reception. And got the following conclusions.

Proposal 1:             The test method setup for FR2 MIMO OTA in TR 38.827 can also be included as the baseline together with those in TR 38.810 and TR 38.884.

Proposal 2:             The two AoAs used for multi-panel simultaneous reception need to be further studied.

- References

[1] RP-220988, New SI: Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements, Qualcomm Incorporated

[2] TR 38.810, NR; Study on test methods

[3] TR 38.884, Study on enhanced test methods for Frequency Range 2 (FR2) NR User Equipment (UE)

[4] TR 38.827, Study on enhanced test methods for Frequency Range 2 (FR2) NR User Equipment (UE)