# 3GPP TSG-RAN WG4 Meeting #104-e   	R4-2213464

# Electronic Meeting, 15th – 26th Aug., 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for relaxed RLM test for FR1 PSCell configured with SSB-based RLM RS in EN-DC mode (A.4.5.1.X) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 agreed to introduce requirements for radio link monitoring relaxation |  |  |  |  |  |  |  |  |
|  |  | . |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the relaxed FR1 radio link monitoring requirements for FR1 PSCell configured with SSB-based RLM RS in EN-DC mdoe. Comparing with legacy RLM test cases in DRX mode, the changing part includes: - Extension for T1 agreed in last meeting - Set SNR2 = SNR1 so that criteria can still be fulfilled during T2 - Extend N310 number to 2 so that UE behaviour with non-relaxated indication period during T3 can be tested - Extend the D1 and corresponding T3 period - D1 = 15*4*40ms + (N310-1)* 60ms + 40 ms =2500ms - T3= D1 +40ms=2540ms - Setting of good serving cell quality criterion and low mobility criterion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for relaxed RLM test for FR1 PSCell configured with SSB-based RLM RS in EN-DC mode is missed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.4.5.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------------- Beginning of Change ------------------------------------------------------------

A.4.5.1.X	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS and UE fulfilling relaxed measurement criterion

A.4.5.1.X.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the relaxed FR1 radio link monitoring requirements specified in clause 8.1.2.4 for UE fulfilling good serving cell quality criterion and low mobility criterion, if configured.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.4.5.1.X.1-1. The test parameters are given in Tables A.4.5.1.X.1-2 and A.4.5.1.X.1-3. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.4.5.1.X.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.4.5.1.X.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.5.1.X.1-2: General test parameters for FR1 out-of-sync testing in DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 1 |
| Active PSCell |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 2 |
| Duplex mode |  | Config 1, 4 |  | FDD |
|  |  | Config 2, 3, 5, 6 |  | TDD |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |
|  |  | Config 2, 5 |  | 10: NRB,c = 52 |
|  |  | Config 3, 6 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.1.1 |
| TDD |  | Config 1, 4 |  | Not Applicable |
| Configuration |  | Config 2, 5 |  | TDDConf.1.1 |
|  |  | Config 3, 6 |  | TDDConf.2.1 |
| CORESET |  | Config 1, 4 |  | CR.1.1 FDD |
| Reference |  | Config 2, 5 |  | CR.1.1 TDD |
| Channel |  | Config 3, 6 |  | CR.2.1 TDD |
| SSB |  | Config 1, 4 |  | SSB.1 FR1 |
| Configuration |  | Config 2, 5 |  | SSB.1 FR1 |
|  |  | Config 3, 6 |  | SSB.2 FR1 |
| SMTC |  | Config 1, 2, 4, 5 |  | SMTC.1 |
| Configuration |  | Config 3, 6 |  | SMTC.1 |
| PDSCH/PDCCH |  | Config 1, 2, 4, 5 |  | 15 kHz |
| subcarrier spacing |  | Config 3, 6 |  | 30 kHz |
| PRACH |  | Config 1, 2, 4, 5 |  | Table A.3.8.2.1-1 |
| Configuration |  | Config 3, 6 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync | DCI format |  |  | 1-0 |
| transmission parameters | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 2 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI |  | Config 1, 4 |  | CSI-RS.1.1 FDD |
| reporting |  | Config 2, 5 |  | CSI-RS.1.1 TDD |
|  |  | Config 3, 6 |  | CSI-RS.2.1 TDD |
| CSI-RS for |  | Config 1, 4 |  | TRS.1.1 FDD |
| tracking |  | Config 2, 5 |  | TRS.1.1 TDD |
|  |  | Config 3, 6 |  | TRS.1.2 TDD |
| T1 |  |  | s | 5.2 |
| T2 |  |  | s | 0.68 |
| T3 |  |  | s | 2.54 |
| D1 |  |  | s | 2.5 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.4.5.1.X.1-3: Cell specific test parameters for FR1 (Cell 2) for out-of-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on | Config 1, 4 | dB | 1 | 1 | -15 |
| RLM-RS | Config 2, 5 |  | 1 | 1 | -15 |
|  | Config 3, 6 |  | 1 | 1 | -15 |
|  | Config 1, 4 | dBm/15kHz | -98 |  |  |
|  | Config 2, 5 |  | -98 |  |  |
|  | Config 3, 6 |  | -98 |  |  |
|  | Config 1, 4 | dBm/SCS | -98 |  |  |
|  | Config 2, 5 |  | -98 |  |  |
|  | Config 3, 6 |  | -95 |  |  |
| goodServingCellEvaluationRLM |  | dB | 0 |  |  |
| s-SearchDeltaP-Connected |  | dB | 3 |  |  |
| t-SearchDeltaP-Connected |  | s | 5 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.4.5.1.X.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. |  |  |  |  |  |

Figure A.4.5.1.X.1-1: SNR variation for out-of-sync testing

A.4.5.1.X.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal in Cell 2 no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

------------------------------------------------------------ End of Change ----------------------------------------------------------------