3GPP TSG-RAN WG4 Meeting #104-e	R4-2213564

Electronic Meeting, 15 – 26 August, 2022

Title: 	Discussion on MG-less measurement in feMG

Source: 	Huawei, HiSilicon

Agenda item:	11.10.3

Document for:	Discussion

Introduction

In RAN#95-e, a new Rel-18 WI [1] on further enhancement of MG is approved.

One objective of the WI is to support measurement without MG, including inter-frequency and intra-frequency measurement based on NeedForGaps, and inter-RAT NR and LTE measurement.

- Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc. - Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

In this paper we will provide our initial views on MG-less measurement in feMG WI.

Discussion

Requirements for NeedForGaps

The signalling of NeedForGaps was defined in Rel-16 as part of RAN2 TEI work. During Rel-16/17 time, RAN4 has discussed the requirements for NeedForGaps but could reach consensus. As a result, the Rel-18 feMG WI includes the objective to define requirements for NeedForGaps.

The most controversial issue in defining the requirements for NeedForGaps is whether interruption is allowed when UE reports ‘no-gap’. Some companies thinks no interruption should be allowed, and the feature should work in the same way as in LTE, while other companies thinks interruption should be allowed because in typical UE implementations use of spare RF chain for measurement would require some interruption due to e.g. RF ON/OFF or BW re-tuning.

We do not have strong view on this issue but slightly prefer that interruption is not allowed. The reason is that RAN4 has already defined NCSG in Rel-17 to cope with the scenario where UE requires interruption to perform the measurement, i.e. UE can report ‘nogap-ncsg’ in NeedForGapNCSG, and NW would configure NCSG to enable UE to do the measurement. In our view, the need to define another framework based on NeedForGaps and interruption to address the same UE implementation is not very necessary considering that

- The interruption is less flexible than NCSG. With NCSG, NW can control both the periodicity and the location (offset) of the interruption occurrences, but with interruptions NW can only follow the spec to determine where interruption may occur.

- The NeedForGaps requirements is defined in Rel-18. This means a Rel-17 UE with additional spare RF chains can already benefit from MG-less by utilizing NCSG and does not have to wait until Rel-18.

Proposal 1: Interruption is not allowed for UE reporting ‘no-gap’ for NeedForGapsInfoNR.

If Proposal 1 is agreeable, defining the requirements for the case where UE reports ‘no-gap’ for NeedForGaps can be quite straightforward. It can re-use the requirements defined in Rel-17 for the case where UE reports ‘nogap-noncsg’ for NeedForGapNCSG.

Proposal 2: For the case where UE reports ‘no-gap’ for NeedForGapsInfoNR, re-use the requirements defined in Rel-17 for the case where UE reports ‘nogap-noncsg’ for NeedForGapNCSG-InfoNR.

MG-less inter-RAT measurement

Inter-RAT NR measurement

In Rel-16, RAN2 has introduced NeedForGap in Rel-16 also for inter-RAT NR measurement when UE is in LTE SA, i.e. interRAT-NeedForGapsNR. Similar as intra-frequency and inter-frequency NR measurement, RAN4 has not defined requirements for interRAT-NeedForGapsNR. It is meaningful to define requirements this case.

The requirements for inter-RAT measurement with interRAT-NeedForGapsNR can be similar to NR intra-frequency or inter-frequency measurement with NeedForGaps. RAN4 also needs to discuss whether interruption is allowed when UE reports interRAT-NeedForGapsNR, and the conclusion does not have to be same as that for NeedForGaps because 1) in LTE SA we do not have NCSG framework for inter-RAT NR measurement, and 2) UE implementation may be different for intra-/inter-frequency and inter-RAT measurement. We suggest RAN4 to further study whether interruption is allowed when UE reports interRAT-NeedForGapsNR.

Proposal 3: RAN4 to define requirements for inter-RAT NR measurement without MG based on interRAT-NeedForGapsNR. FFS whether interruption is allowed when UE reports interRAT-NeedForGapsNR.

Inter-RAT LTE measurement

For inter-RAT LTE measurement without MG, during the WID discussion there are different views which scenarios RAN4 should address, and there are two scenarios considered:

- Scenario 1: UE has spare RF chain for the measurement

- Scenario 2: the LTE carrier is within the active BWP of NR serving cell

In our view, scenario 1 can be already addressed by Rel-17 NCSG framework. Since NCSG can be used for inter-RAT LTE measurement, RAN2 has introduced NeedForNCSG-EUTRA in Rel-17, and RAN4 has also defined corresponding requirements in clause 9.4 of 38.133. Therefore, there is no need to define additional requirements for inter-RAT LTE measurement based on spare RF chain in Rel-18.

Scenario 2 is a new scenario where LTE and NR are sharing the same carrier. In our view, it is meaningful to support inter-RAT LTE measurement without MG in this scenario. It is noted that in Rel-17 the CRS-IM has been defined in demodulation session, and in some cases it could rely on inter-RAT LTE measurement to benefit from the all possible NW deployments. Therefore, it is meaningful to define requirements for inter-RAT LTE measurement without MG for the scenario where LTE carrier is within active BWP of NR serving cell. RAN4 can further discuss the exact requirements such as CSSF, measurement period, scheduling restriction for this scenario.

Proposal 4: RAN4 to define requirements for inter-RAT LTE measurement without MG for the scenario where LTE carrier is within active BWP of NR serving cell.

Conclusions

In this paper we provided our initial views on MG-less measurement in feMG WI.

Proposal 1: Interruption is not allowed for UE reporting ‘no-gap’ for NeedForGapsInfoNR.

Proposal 2: For the case where UE reports ‘no-gap’ for NeedForGapsInfoNR, re-use the requirements defined in Rel-17 for the case where UE reports ‘nogap-noncsg’ for NeedForGapNCSG-InfoNR.

Proposal 3: RAN4 to define requirements for inter-RAT NR measurement without MG based on interRAT-NeedForGapsNR. FFS whether interruption is allowed when UE reports interRAT-NeedForGapsNR.

Proposal 4: RAN4 to define requirements for inter-RAT LTE measurement without MG for the scenario where LTE carrier is within active BWP of NR serving cell.

Reference

- RP-221018, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation