3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213680

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	10.19.4

Source: 	ZTE Corporation

Title: 	Discussion on BS RF requirements for APT600MHz

Document for:	Approval

Introduction

The topic of APT 600MHz have already been extensively discussed during R17 time phase. At that time, it was a SID, which mainly focus on feasibility analyses for options B1, B2 and B2a, where the feasibility studies and conclusions can be found in the TR38.860[1]. In this contribution, we want to share some initial views on BS RF requirements.

- Specify a new NR FDD operating band to include band numbering and core requirements to operate in APT region by considering that:

  - UE transmit: 663 - 703 MHz, BS transmit: 612 MHz – 652 MHz,

  - To support the following channel bandwidths and SCS as in the Table 2 below

  - UL channel bandwidth is limited to 20 MHz to limit the MSD as was the case with n71

  - The use of UE power class 3 (23dBm).

Table 2: Channel bandwidth and SCS parameters

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |
| 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |  |

- Develop a technical specification for the APT 600 MHz band for option B1 (as defined in table 1) for the APT region 600 MHz band with a view to developing the normative text on:

- Operating band, channel bandwidth and system parameters- see Table 1 and Table 2

- BS and UE RF core requirement taking into account potential coexistence issues

- RRM requirement

Discussion

Table 1. Summary of related RF requirement

| Tx part |  |
| --- | --- |
| Base station output power | All of these requirements are defined as band or channel bandwidth agnostic except for output power dynamics, therefore it’s not expected to have any impacts with the introduction of APT600MHz. |
| Output power dynamics |  |
| Transmit ON/OFF power |  |
| Transmitted signal quality |  |
| OBW |  |
| Tx intermodulation |  |
| ACLR | To follow the requirement of band n71 |
| Operating band unwanted emissions | For OBUE requirement, to follow OBUE requirements of band n71. For Fobue requirement, to follow the existing requirement in TS 38.104 |
| Transmitter spurious emissions | To add the additional spurious emissions requirements for APT600MHz To add the co-location with other base station for APT600MHz |
| Rx part |  |
| REFSENS | No requirement impacts and just to follow the requirements of band n71 |
| Dynamic range requirement | No requirement impacts and to follow the requirements of n71 |
| ACS, | No requirement impacts and to follow the requirements of n71 |
| Blocking requirement | No requirement impacts and to follow the requirements of n71s |
| OOBB | No requirement impacts and to follow the requirements of n71 |
| RX IMD | No requirement impacts and to follow the requirements of n71s |
| Rx spurious emission | No requirement impacts and to follow the requirements of n71 |
| ICS | No requirement impacts and to follow the requirements of n71 |

Conclusions

In this contribution, we want to share some initial views on BS RF requirements.and proposals are made as following:

Proposal 1: to use the proposals in table for APT600MHz BS RF requirements.

References

- TR38.860, Study on Extended 600 MHz NR band, v17.0.0

- RP-221778, New WID on APT 600 MHz NR band, Spark NZ, Qualcomm Incorporated

- TR36.775, US 600 MHz Band for LTE, v15.0.0