3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213712

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	9.5.1

Source: 	ZTE Corporation

Title: 	Further discussion on the supported bands for NR repeater

Document for:	Approval

Introduction

In the previous discussion for the supported bands for NR repeater, in high level, it’s supposed that all operating band in TS 38.104 should be supported for NR repeater, however after the review the specified RF requirement as specified in TS38.106 [1], there are still some ambiguity and misalignment of its applicable frequency range. In the last RAN4 meeting, the ambiguity of band n46, n96 and n102 was excluded. However we realized that there is still some ambiguity for other bands e.g. n104.

Discussion

For the supported operating band for NR repeater, it was specified as following, from the requirement itself, it could be known that all FR1 and FR2-1 should be supported including licensed band, unlicensed band, SDL and SUL band.

NR repeater is designed to operate in the operating bands in FR1 and FR2-1 defined in TS 38.104 [2].

However when checking the ACLR requirement and unwanted emission requirements defined for NR repeater, all requirements are inherited from BS operating in the licensed operating band.

In the last RAN4 meeting, band n104 was introduced in TS 38.104, it could found that the existing ACLR/UEM requirement from TS 38.106 is not applicable for band n104. Therefore we propose to define new set of requirements for band n104 in TS38.106 in Rel-17.

Proposal 1: to define the repeater in band n104 since the existing requirement defined in TS 38.106 is not applicable for it.

Conclusions

In this contribution, we want to share some further consideration on the supported operating band for NR repeater  and proposals are made as following:

Proposal 1: to define the repeater in band n104 since the existing requirement defined in TS 38.106 is not applicable for it.

References

- TS 38.106 17.0.0

- R4-220xx, draft CR to TS 38.106: clarification on the supported band for NR repeater, ZTE Corporation

- R4-2210740,CR to TS38.104 the introduction of 6425-7125MHz,ZTE,Sanechips Huawei, HiSilicon

- R4-2211224,Introduction of NR licensed band 6425- 7125 MHz,Qualcomm Incorporated, Skyworks Solutions, Inc