3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213718

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	9.5.5.3

Source: 	ZTE Corporation

Title: 	Discussion on FR2 NR repeater test configuration

Document for:	Approval

Introduction

In last RAN4 meeting, Core requirement of NR repeater has been completed. Based on the approved Rel-17 RAN4 led repeater WID RP-212129, the conformance testing for both NR repeater should be completed in Sep, 2022. In this contribution, we provide some further views on conformance testing requirement for FR2 NR repeater.

Sub-topic#1-4: FR2 set-up baseline

Recommended agreement:

- For EIRP based measurement, option 1 (R4-2209606) could be starting point;

- Reverberation chamber is not applicable for FR2 repeater conformance testing; FFS for CATR, Near field chamber

- ACRR/out of band gain requirement, the output should be done with TRP measurement

- FFS on TRP measurement of FR2 repeater on OTA chamber;

Open issues:

Further discuss the following issues:

- TRP measurement with input antenna rotating together with repeater ;

- Testing chamber size with testing antenna ;

- Two calibration antenna for repeater within the OTA chamber ;

- Other approach should not be precluded e.g. moving measurement antenna or measure with measure with multiple test antennas.

Sub-topic# 1-5: FR2 set-up specific issues compared with baseline

Open issues:

Further discuss the following issues:

- TDD off power measurement;

- Potential issue/solution: Input signal may need to turn off.

- spurious emissions testing

- Potential issue/solution: isolation between input and output may need to be considered.

- Feasibility of the DUT moving together testing antenna

- The impact of feeding signal to measured signal, such as two calibrated directional paths in the chamber, the signal power impact, etc.

Discussion

- 2.1. Test setup for FR2 NR repeater

For the conformance testing for NR repeater, the 1st issue need to be addressed is how to establish the measurement setup for NR repeater conformance testing. Since for FR2 NR repeater type 1-C is similar as LTE repeater or UTRA repeater, therefore we propose the use the measurement setup in TS 38.141-2 as baseline with some updates for FR2 NR repeater as following:

### 2.1.1. Test setup for FR2 NR repeater

C.1	OTA output power EIRP, OTA Frequency stability, OTA Error Vector Magnitude and OTA Transmit ON/OFF power

Figure C.1-1: Measuring system set-up for maximum EIRP testing

Note 1:  The repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.1-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

C.2	Out of band gain

Figure C.2-1: Measuring system set-up for out of band gain

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.2-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

C.3	Unwanted emission: Operating band unwanted emission and ACLR

Figure C.3-1: Measuring system set-up for unwanted emission: ACLR, Operating band unwanted emission and spurious emission requirement

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.3-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.4-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, etc.).

C.4	Input intermodulation

Figure C.5-1: Measuring system set-up for input intermodulation.

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.5-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

C.5	Adjacent Channel Rejection Ratio

Figure C.5-1: Measuring system set-up for Adjacent Channel Rejection Ratio

Note 1:  That repeater is a bi-directional device. The signal generator may need protection.

Note 2:  The OTA chamber shown in figure C.6-1 is intended to be generic and can be replaced with any suitable OTA chamber (Far field anechoic chamber, CATR, Near field chamber, etc.)

Proposal 1: propose to use the above measurement setup from C.1 to C.5 as baseline for FR2 NR repeater;

### 2.1.2. Potential testing concern

- TRP measurement with input antenna rotating together with repeater ;

At least we don’t see the problem for TRP measurement with input antenna rotating together with repeater. Rotating desktop could be relatively larger to accommodate the input antenna and repeater. In addition, if the repeater is placed within the near filed of input antenna, then some other calibration for pathloss need to considered compared with far field pathloss calibration.

Observation 1: no problem, if the repeater is placed within the near filed of input antenna, then some other calibration for pathloss need to considered compared with far field pathloss calibration

- Testing chamber size with testing antenna ;

If the input antenna is quite small, like small array antenna assumed, then it should be okay to be placed within the chammber .

Observation 2: If the input antenna is quite small, like small array antenna assumed, then it should be okay to be placed within the chammber

- Two calibration antenna for repeater within the OTA chamber

For FR2 repeater, since its need to receive the input signal firstly, then amplify and forward its received signal to the target node. There was some concerns raised in the past that input signal generator within the chamber would have some impacts on repeater output signal measurement. From our understanding, it is true that input signal for repeater placed within the chamber could also been detected by measurement equipment, however it should be known that input signal should be quite lower than compared with output power if repeater is set to the maximum gain mode. In addition, as described in the previous section, if input signal generator could form very narrow beam towards the repeater, then its transmission would also been blocked a lot without causing the impact on reception at the measurement equipment.

For calibration between input antenna and repeater input side, maybe this could be done separately than the calibration between repeater output and measurement antenna.

Observation 3: input signal generator for the repeater within chamber should have marginal impact on the reception of repeater output since its power should be much lower compared with repeater output power.

- TDD off power measurement;

- Potential issue/solution: Input signal may need to turn off.

Observation 4: . for repeater TDD off power measurement, to turn off the input signal is one feasible way.

- spurious emissions testing

- Potential issue/solution: isolation between input and output may need to be considered.

Observation 5: . for spurious emission testing, isolation between input and output might be needed since the power level of spurious emission is quite lower. Or input signal’ spurious emission requirement could be removed from the total spurious emission measured at the repeater output if input signal’ spurious emission is measured again in the chamber, however it might be quite time consuming since spurious emission testing is most timing consuming test case.

- 2.2. Measurement uncertainty

FFS

- 2.3. Test procedure

Regarding the test procedures for FR1 NR repeater, there are two remaining issues left for further discussion in [7].

In the following section, for EVM conformance testing under ALC condition, from our understanding, it’s reasonable to have test under the ALC and non-ALC condition to ensure the EVM performance otherwise some clipping or other incorrect approach handling might cause signal quality within channel degradation.

Proposal 2: to use option 3 for the EVM conformance testing.

For RX IM requirement for repeater, currently we think that option 1 is sufficient for test its function if RX IM falling into the channel unless there is other technical reasons to need multiple testing.

Proposal 3: to use option 1 for the Rx IM conformance testing

Conclusions

In this contribution, we want to share some further considerations on those remaining open issues of conducted repeater and proposals are made as following:

Proposal 1: propose to use the above measurement setup from C.1 to C.6 as baseline for FR2 NR repeater;

Observation 1: for TRP measurement with input antenna rotating together with repeater, it should be no problem, if the repeater is placed within the near filed of input antenna, then some other calibration for pathloss need to considered compared with far field pathloss calibration

Observation 2: for Testing chamber size with testing antenna, if the input antenna is quite small, like small array antenna assumed, then it should be okay to be placed within the chammber

Observation 3: for Two calibration antenna for repeater within the OTA chamber, input signal generator for the repeater within chamber should have marginal impact on the reception of repeater output since its power should be much lower compared with repeater output power. In addition, for calibration between input antenna and repeater input side, maybe this could be done separately than the calibration between repeater output and measurement antenna.

Observation 4: . for repeater TDD off power measurement, to turn off the input signal is one feasible way.

Observation 5: . for spurious emission testing, isolation between input and output might be needed since the power level of spurious emission is quite lower. Or input signal’ spurious emission requirement could be removed from the total spurious emission measured at the repeater output if input signal’ spurious emission is measured again in the chamber, however it might be quite time consuming since spurious emission testing is most timing consuming test case.

Proposal 2: to use option 3 for the EVM conformance testing..

Proposal 3: to use option 1 for the Rx IM conformance testing.

References

- TS 38.106 17.0.0

- TS 36.143 17.0.0

- R4-2210631,WF on NR FR1/FR2 repeater measurement system set-up, ZTE, Revised to R4-2211223

- R4-2211223,WF on NR FR1/FR2 repeater measurement system set-up, ZTE, approved

- R4-2211150,WF on NR FR1/FR2 repeater MU and TT, Huawei, approved.

- R4-2211151, WF on NR FR1/FR2 repeater declarations, Nokia

- R4-2211152,WF on NR FR1/FR2 repeater test procedures and test issues, Ericsson

- R4-2211153, WF on NR FR1/FR2 repeater test specs draft rules and TP splits, CATT, approved.

- R4-2210845,Spec skeleton for TS 38.115-1, CATT, approved.

- R4-2210846,Spec skeleton for TS 38.115-2 v.0.0.1, ZTE Corporation. Approved.