3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213754

# Electronic Meeting, August 15 - 26, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Input on feasibility of LB-LB-LB band combinations

Agenda Item:	11.2.2 Feasibility investigation for simultaneous reception and transmissions [FS_NR_700800900_combo_enh]

Document for:	Approval

# Introduction

In Release 18 SI for enhanced 700/800/900MHz band combinations [1], 2/3band DL with 2band UL combinations are requested in the low-band range. In this contribution, we provide some input on the challenges to be solved, potential impact on architectures and limitations in smartphones form factor.

# Discussion

## Study objectives from [1]

Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands and simultaneous reception on two or three bands for the band combination of 700, 800 and 900MHz spectrum for smart phone form factor

  - The following band combinations will be considered. And the feasibility study on three band combination will start after the completion of feasibility study of all the fallback band combinations.

    - CA_n8-n20-n28 with uplink configurations of CA_n8-n20, CA_n8-n28, CA_n20-n28, and the fallback modes

    - CA_n5-n8 with uplink configuration of CA_n5-n8, and the fallback modes

Note1: Spectrum restrictions should be studied to solve overlap of band n5 downlink and band n8 uplink

Note2: The current filter is used as the baseline. Further study whether or not to have new solutions.

    - CA_n5-n28 (full range) with uplink configuration of CA_n5-n28.

Summary of band combinations considered in the SI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n8-n20-n28 | CA_n8-n20, CA_n8-n28, CA_n20-n28 | Vodafone, Telecom Italia, Orange, Deutsche Telekom |
| CA_n5-n8 | CA_n5-n8 | China Telecom, Spark NZ, China Unicom |
| CA_n5-n28 | CA_n5-n28 | Spark NZ |

  - The following aspects need be studied

    - UE architecture including n-plexing, PA

    - Study feasibility of low band wideband antenna

    - Performance due to impacts including inter-modulation products

    - Method to manage the inter-modulation product impacts

Note: Revisit in RAN#98 whether additional aspects need to be added.

  - Power class 3 (PC3) is considered in this study

Identify potential impacts to relevant RAN4 requirements.

## Study of the different cases

One of the critical aspects of LB/LB and LB/LB/LB combinations is that there are only very small gaps between the different UL and DL bands discussed in the SI as shown on Figure 1a and 1b, thus many DL bands may be subject to de-sense.

Figure 1a: 700-860MHz spectrum with distances between band 5, 20 and 28

Figure 1b: 790-960MHz spectrum with distances between band 5, 8, 20 and 28

On top of small duplex distances in the order of 10MHz, the distances between bands in the combinations are in the order of a few MHz to 20MHz, with some overlapping cases.

For CA_n8-n20-n28:

- Our understanding is that n28 is only n28A and based on n28A-n20 triplexer

- Band n8 UL is only 18MHz above n20 DL while its duplex distance is 10MHz

- Additionally, NR CBW in UL and DL have increased thus creating more opportunity for de-sense:

  - Band 8 CBW: 20MHz UL, 35MHz DL (this is the entire band)

  - Band 20 CBW: 20MHz UL, 20MHz DL

  - Band 28A CBW: 30MHz UL, 30MHz DL (this is the entire sub-band)

  - In most cases the UL CBW is in the same order as the UL-DL distance, while the DL CBW occupies the entire band, which results in large MSDs, especially if the UL requires high number of resources which is anticipated as it must carry Ack/Nack for 3 DL bands at larger DL CBW since high DL throughput is the target of the SI

- Finally, there are additional MSD issues to be considered related to the IMDs of the two UL configurations:

  - CA_n8-n20: IMD3 fall in all DL bands n8, n20, n28A,

  - CA_n8-n28: no IMD issue thanks to the distance between the two UL

  - CA_n20-n28: IMD3 fall in band n8 DL

- With the above considerations, it is unlikely that higher throughput is obtained because of the reduced SNR in the DL bands.

Proposal for CA_n8-n20-n28:

- Only n28A spectrum is supported based on n28A UL+(n28A+n20) DL + n20 UL tri-plexing as a starting point

- Maximum UL and DL CBW per band must be defined

For CA_n5-n28:

- Band n28 is understood as full band but it is unclear if this assumes full band duplexer or not. The full band n28 duplexer is already challenging on its own to envisage a n28F+n5 quad-plexer

- The gap between n5 UL and n28 DL is 20MHz and with 20MHz UL CBW in band 5, large MSD in band n28F DL can be anticipated.

- The dual UL IMDs fall outside of band n5 DL

- Given the band arrangement, one possible approach for the study is a dual tri-plexing:

  - Main antenna: n28UL+n28DL+n5DL

  - Diversity antenna: n28DL+n5UL+n5DL

Proposal for CA_n5-n28:

- n28F spectrum is supported but architectures based on both n28F duplexer and n28A+B dual duplexer approach are studied

- Maximum UL and DL CBW per band must be defined

- As a starting point a two-antenna tri-plexing with UL on both antennas is preferred

For CA_n5-n8:

- Band n5 DL overlaps with band 8 UL, in our understanding this combination would only be feasible for restricted spectrum in some regions:

  - Band n5 DL 825-835MHz and band 8 UL 890-915MHz in China

    - Even with a dedicated filter design, the gap is only 10MHz while the n8 duplex distance is also 10MHz

    - It is unlikely that standard band n5 and n8 duplexer can be reused and this would be a China only solution

    - IMD3 of CA_n5-n8 UL fall in band n8 DL

    - Quad-plexing is challenging and high MSD can be anticipated with 20MHz UL in n8

  - Band n5 DL 825-840MHz and band 8 UL 890-915MHz in New Zealand, but with only 5MHz gap and the need for a dedicated duplexer design this seems not feasible.

- It is unclear what is the overall ecosystem for such an approach and whether the likely poor performance is worth developing a country specific solution.

Proposal for CA_n5-n8:

- Only the China restricted spectrum is studied: band n5 DL 825-835MHz and band 8 UL 890-915MHz

- Maximum UL and DL CBW per band must be defined

- Use of standard n5 and n8 duplexer is unlikely and even with additional antennas the tight coupling between LB antennas will not isolate the filters sufficiently: the study should include dedicated filter design with all its limitations for wide support

Additionally, all cases require dual UL in LB, on top of generating IMDs that will result in large MSDs, this means that either the second UL is n-plexed on:

- The main antenna: then the sum of the two PA noise may de-sense the DL bands further

- The diversity antenna: then the diversity path will see higher UL noise

- Or a third antenna: in general, this third antenna is borrowed from a higher band and has poor efficiency for a low band especially for UL

- Unless we have a clear view of the PA/Architecture options to be studied, it will be difficult to perform a thorough study of the multiple MSD, UL coupling (antenna coupling at LB is probably less than 10dB for a smartphone)

Proposal on PA/Antenna Architecture:

- Baseline architecture should be agreed for 2nd PA being on Main, diversity or third antenna for all cases and a generic approach is targeted for two LB on one side and three LB on the other side

- Coupling between the two or three antennas should be agreed for the 700MHz to 960MHz range in a smartphone context to assess the different cases

# Conclusions

In this contribution, we quickly evaluated the issues for the example combinations given in the study and make the following proposal to further clarify the target.

Proposal for CA_n8-n20-n28:

- Only n28A spectrum is supported based on n28A UL+(n28A+n20) DL + n20 UL tri-plexing as a starting point

- Maximum UL and DL CBW per band must be defined

Proposal for CA_n5-n28:

- n28F spectrum is supported but architectures based on both n28F duplexer and n28A+B dual duplexer approach are studied

- Maximum UL and DL CBW per band must be defined

- As a starting point a two-antenna tri-plexing with UL on both antennas is preferred

Proposal for CA_n5-n8:

- Only the China restricted spectrum is studied: band n5 DL 825-835MHz and band 8 UL 890-915MHz

- Maximum UL and DL CBW per band must be defined

- Use of standard n5 and n8 duplexer is unlikely and even with additional antennas the tight coupling between LB antennas will not isolate the filters sufficiently: the study should include dedicated filter design with all its limitations for wide support

Proposal on PA/Antenna Architecture:

- Baseline architecture should be agreed for 2nd PA being on Main, diversity or third antenna for all cases and a generic approach is targeted for two LB on one side and three LB on the other side

- Coupling between the two or three antennas should be agreed for the 700MHz to 960MHz range in a smartphone context to assess the different cases

# References

- RP-221464 Revised SI: Study on enhancement for 700/800/900MHz band combinations for NR, CATT, RAN#96