3GPP TSG-RAN WG4 Meeting#104-e      	R4-2213757

E-meeting, 15 – 26 Aug, 2022

Title:	Reply LS to RAN5 LS on ModifiedMPR-Behaviour clarification for different power classes

Source:	Nokia, Qualcomm Inc, Skyworks Inc

Agenda item:	9.4.1.2

Document for:	Approval

# 1	Introduction

This contribution discusses RAN5 LS on ModifiedMPR-Behaviour clarification for different power classes and proposes reply LS to RAN5.

# 2	Discussion

When defining test specification for FR2 MPR enhancements, RAN5 faced some concerns as listed below:

- For Rel-15 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE supports modifiedMPR-Behaviour bit 0 UE capability?

Proposed answer: Yes it is.

- For Rel-15 PC2 and 4 UEs, is modifiedMPR-Behaviour bit 0 capability applicable?

Proposed answer: Yes it is.

- For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? Also, is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true?

Proposed answer: The changes introduced to PC3 MPR in v16.2.0 are optional to REL16 UE

- For Rel-16 PC2, 4 and 5 UEs, is the PC3 MPR as defined in 38.101-2 v16.2.0 applicable? Also, is modifiedMPR-Behaviour bit 0 capability applicable?

Proposed answer: MPR as defined in v16.2.0 is applicable also to Rel-16 PC2 and PC4 UEs if they indicate with modifiedMPR-Behaviour bit 0 that they support this feature. PC5 was defined during REL17.

- Is any kind of Rel-16 UE supposed to support MPR as defined in 38.101-2 version v16.11.0?

Proposed answer: There are two CRs implemented to MPR section between 16.11.0 and 16.2.0:

- R4-2111524 #99e (QCOM/SKWS):            CR to 16.7.0: editorial change to address RAN5 concerns on ambiguity for PC3 MPRnarrow.

- R4-2207884 #103e (KS/SKWS):                  CR to 16.11.0: technical correction on RBstart equation for PC3 MPRnarrow.

As first CR is editorial and second is necessary correction there were no modifiedMPR-Behaviour introduced. When REL16 UE indicates that it supports MPR as defined in v16.2.0 it means that it supports MPR as defined in latest version of REL16 38.101-2. which is currently 16.12.0.

- For Rel-17 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE signals modifiedMPR-Behaviour bit 0=true?

Proposed answer: Rel17 38.101 Annex-H is not correct. It should state This bit SHALL be set to 1 instead of MAY. There is a RAN4 CR in this [1]

- For Rel-17 PC3 UE, what is the MPR requirement if the UE signals modifiedMPR-Behaviour bit 0=false?

Proposed answer: REL17 UE may not set this bit 0 to false, see [1]

# 3	Conclusion

Approve the LS in Annex.

# Annex: Draft LS

3GPP TSG-RAN WG4 Meeting#104-e      	R4-220

E-meeting, 15 – 26 Aug, 2022

Title:	Reply LS on ModifiedMPR-Behaviour clarification for different power classes

Response to:

Release:	Release 16

Work Item:	NR_RF_FR2_req_enh-UEConTest

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:	N/A

Contact Person:

#### Name:	Petri Vasenkari

E-mail Address:petri.j.vasenkari@nokia.com@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Answers to RAN5 questions

- For Rel-15 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE supports modifiedMPR-Behaviour bit 0 UE capability?

Answer: Yes it is.

- For Rel-15 PC2 and 4 UEs, is modifiedMPR-Behaviour bit 0 capability applicable?

Answer: Yes it is.

- For Rel-16 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 mandatory or optional? Also, is the Rel-16 UE expected to signal modifiedMPR-Behaviour bit 0=true?

Answer: The changes introduced to PC3 MPR in v16.2.0 are optional to REL16 UE

- For Rel-16 PC2, 4 and 5 UEs, is the PC3 MPR as defined in 38.101-2 v16.2.0 applicable? Also, is modifiedMPR-Behaviour bit 0 capability applicable?

Answer: MPR as defined in v16.2.0 is applicable also to Rel-16 PC2 and PC4 UEs if they indicate with modifiedMPR-Behaviour bit 0 that they support this feature. PC5 was defined during REL17.

- Is any kind of Rel-16 UE supposed to support MPR as defined in 38.101-2 version v16.11.0?

Answer: There are two CRs implemented to MPR section between 16.11.0 and 16.2.0:

- R4-2111524 #99e (QCOM/SKWS):            CR to 16.7.0: editorial change to address RAN5 concerns on ambiguity for PC3 MPRnarrow.

- R4-2207884 #103e (KS/SKWS):                  CR to 16.11.0: technical correction on RBstart equation for PC3 MPRnarrow.

As first CR is editorial and second is necessary correction there were no modifiedMPR-Behaviour introduced. When REL16 UE indicates that it supports MPR as defined in v16.2.0 it means that it supports MPR as defined in latest version of REL16 38.101-2. which is currently 16.12.0.

- For Rel-17 PC3 UE, is the MPR as defined in 38.101-2 v16.2.0 applicable if the UE signals modifiedMPR-Behaviour bit 0=true?

Answer: Rel17 38.101 Annex-H is not correct. It should state This bit SHALL be set to 1 instead of MAY. There is a RAN4 CR in this [1]

- For Rel-17 PC3 UE, what is the MPR requirement if the UE signals modifiedMPR-Behaviour bit 0=false?

Answer: REL17 UE may not set this bit 0 to false, see [1]

Additionally RAN4 has agreed two CRs related to this topic. One for Rel-16 [1] to clarify applicability of new MPR and second one [2] to make new MPR mandatory for Rel-17 UEs.

2. Actions:

To RAN5 group.

ACTION: 	RAN4 respectfully requests RAN5 group to discuss the answer and evaluate the RAN4 CRs and provide feedback if any.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN4 Meeting#104bis 10th – 19th October 2022	Online

TSG-RAN4 Meeting#105	 14h – 18th November 2022	Canada

# References

[1]

[2]