3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213790

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Simulation results on RedCap PDSCH

Source: 	Huawei, HiSilicon

Agenda item:       9.18.5.1.1

Document for:	Discussion

Background

As per [1], RAN 4 listed initial test cases for simulation. In this paper, we provide our simulation results for RedCap PDSCH.

Simulation results for PDSCH test

The BLER-SNR curve are captured in Figure 2-1 and summary of simulation results are shown in Table 2-1 to 2-3.

Figure 2-1: BLER-SNR curve for PDSCH requirements

Table 2-1: Summary of simulation results for FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | Ideal SNR(dB)@ 70% of max TP | Impairment SNR(dB)@ 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | 0.8 | 2.3 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | 9.3 | 10.8 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | 13.4 | 14.9 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | 5.3 | 6.8 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse | Reuse |
| 2Rx UE | 10MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | Reuse | Reuse |

Table 2-2: Summary of simulation results for TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | Ideal SNR(dB)0% of max TP | Impairment SNR(dB)@ 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | 1.2 | 2.7 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | 9.3 | 10.8 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | 13.9 | 15.4 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | -2.5 | -1.0 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | 5.4 | 6.9 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | 16.2 | 17.7 |
| 2Rx UE | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | 22.4 | 23.9 |

Table 2-3: Summary of simulation results for TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | Ideal SNR(dB)0% of max TP | Impairment SNR(dB)@ 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse | Reuse |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse | Reuse |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse | Reuse |

Conclusion

In this paper, we provide our simulation results for RedCap PDSCH.

Reference

[1]   R4-2210672 WF on Redcap demodulation and CQI reporting requirements. Ericsson