3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213798

# Electronic Meeting, Aug.15th -26th ,2022

Title: 	Simulation results and discussions on remain issues on Rel-17 NPDSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       10.8.6.2.1

Document for:	Discussion

Background

In this paper, we provide our simulation results for NPDSCH based on the WF [1].

Simulation results

The simulation results for NPDSCH is captured in Figure 1 based on the simulation assumptions in appendix, the target SNR is captured in Table 2-1.

Figure 2-1: Simulation results for 16 QAM with (IBS, ISF) = (21, 7)

Table 2-1: Summary of simulation results for NPDSCH with 16QAM

|  | Ideal results (dB) | Impairment results (dB) |
| --- | --- | --- |
| HD-FDD | 17.3 | 18.8 |
| TDD | 17.4 | 18.9 |

Scheduling pattern

The scheduling pattern should be derived by the procedure of minimizing the test timing. The proposed scheduling pattern for HD-FDD is captured in Figure 2-2 and TDD is captured in Figure 2-3:

For HD-FDD, we propose to use Idelay=1, G=8, αoffset=0 and Rmax=1.

Figure 2-2: Proposed scheduling pattern for HD-FDD

In [1], TDD pattern in the simulation assumption table. We propose to use uplink downlink configuration 4 and special subframe configuration 4 to keep align with legacy test.

Proposal 1: Use uplink downlink configuration 4 and special subframe configuration 4 for TDD test.

For TDD, considering the 2 UL slot overhead, we assume PDSCH scheduling delay is 6ms (4ms+2ms for UL slot)  and assume the feedback delay is 8ms, the maximum total time is 21ms, to minimum the test time, we also propose to use Idelay=1, G=1, αoffset=0 and Rmax=8.

Figure 2-3: Proposed scheduling pattern for TDD

Proposal 2: Use Idelay=1, G=8, αoffset=0 and Rmax=1 for scheduling pattern and PDCCH search space configuration

Conclusion

In this paper, we provide our simulation results for Rel-17 NPDSCH with 16QAM and give our views on the remain test configurations. The proposals are:

Proposal 1: Use uplink downlink configuration 4 and special subframe configuration 4 for TDD test.

Proposal 2: Use Idelay=1, G=8, α_offset=0 and Rmax=1 for scheduling pattern and PDCCH search space configuration

Reference

[1]   R4-2210674 WF for perf requirements for Rel-17 NB-IOT and eMTC _final. Huawei, HiSilicon

Appendix

| Parameter | Value |
| --- | --- |
| Duplex mode | HD-FDD and TDD |
| Operation mode | Standalone |
| Carrier type | Non-anchor |
| Number of antennas | 1T1R |
| Number of NRS port | 1 |
| Propagation condition | EPA5 Low |
| SCS | 15KHz |
| Modulation order | 16QAM |
| Number of NPDCCH repetition | 1 (no repetition) |
| Number of NPDSCH repetition | 1 (no repetition) |
| TB Scheduling | single-TB scheduling |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| (ISF, ITBS), TBS | (21, 7), 4968bits |
| RV | {0,2,0,2} |
| Max number of HARQ transmission | 4 |
| The ratio of EPRE of NPDSCH in symbols with and without NRS to NRS | 0dB |
| Test metric | 70% max TP |