3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213800

# Electronic Meeting, Aug.15th -26th ,2022

Title: 	Simulation results on NPUSCH with 16QAM

Source: 	Huawei, HiSilicon

Agenda item:       9.24.6.1.2

Document for:	Discussion

Background

As per [1], we reached following agreement for NPUSCH test in last meeting.

Number of allocated subcarriers - Only consider 12 tones Number of scheduled RUs and TBS - (IRU, ITBS)=(0,15), TBS=280 bits

In this paper, we provide our simulation results based on these agreements.

Simulation results

The simulation results based on simulation assumptions in Table 5-1 in Appendix derived in [1] are captured in Figure 2-1:

Figure 2-1: Simulation results for NPUSCH format 1 with 16QAM

The ideal result SNR is 7.8dB and impairment result is 9.3dB

Observation 1: The ideal result is 7.8dB and impairment result is 9.3dB

Conclusion

In this paper, we provide our simulation results for NPUSCH format 1 with 16QAM, the observation is:

Observation 1: The ideal result is 7.8dB and impairment result is 9.3dB

Reference

[1]   R4-2210674 Way forward for performance requirements of Rel-17 NB-IOT and eMTC

Appendix

Table 5-1 Simulation assumptions for NPUSCH format 1

| Parameter | Value |
| --- | --- |
| Number of antennas | 1T2R |
| Propagation condition | ETU 1Hz Low |
| SCS | 15KHz |
| Frequency Resource | 12 tones |
| Modulation order | 16QAM |
| Number of Repetition | 1 |
| TB Scheduling | single-TB scheduling |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| (IRU, ITBS), TBS | (0,15), 280bits |
| RV | {0,2,0,2} |
| Max number of HARQ transmission | 4 |