3GPP TSG-RAN WG4 Meeting # 104e	R4-2213806

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Simulation results for FR2-2 PDSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.2.1

Document for:	Discussion

Background

As per [1], RAN 4 listed some PDSCH test cases for simulation and interested companies can provide simulation results in this meeting. In this paper, we provide our initial simulation results

Discussions

The common simulation assumptions are captured in Table 2-1.

Table 2-1: Common Simulation assumptions for FR2-2 PDSCH requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | - For 120kHz SCS: DDDSU S=10D:2G:2U - For 480kHz SCS: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 2 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 for 120kHz and 16 for 480kHz |
| Phase noise model |  |  | Set 2 in TR 38.808 |
| Receiver type |  |  | For case except 120kHz/400MHz: - CPE compensation For case with 120kHz/400MHz: - CPE compensation - ICI compensation with 1 tap filter - ICI compensation with 2 tap filter - ICI compensation with 3 tap filter |
| The number of slots between final repetition of PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 (Note 1) |
| Note 1: ACK/NACK feedback is generated for PDSCH on slot i, where mod(i,4) = 1, where i is the slot index per frame; i = {0,...,79} |  |  |  |

The simulation results are captured in Figure 2-1:

Figure 2-1: Throughput-BLER curve for PDSCH performance

The summary of simulation results are captured in Table 2-2:

Table 2-2: Summary of simulation results

| CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | Receiver type | Target SNR |
| --- | --- | --- | --- | --- | --- |
| 100/120 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low | No PN | -2.5 |
|  |  |  |  | PN with CPE compensation | -2.4 |
| 100/120 | MCS4/Rank1 | TDLA10-650 | 2x2 ULA Low | No PN | -2.3 |
|  |  |  |  | PN with CPE compensation | -2.2 |
| 100/120 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low | No PN | 5.5 |
|  |  |  |  | PN with CPE compensation | 5.9 |
| 100/120 | MCS13/Rank2 | TDLD10-200 | 2x2 ULA Low | No PN | 8.4 |
|  |  |  |  | PN with CPE compensation | 8.8 |
| 100/120 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | No PN | 7.9 |
|  |  |  |  | PN with CPE compensation | 8.4 |
| 400/480 | MCS4/Rank1 | TDLA10-200 | 2x2 ULA Low | No PN | -2.3 |
|  |  |  |  | PN with CPE compensation | -2.2 |
| 400/480 | MCS4/Rank1 | TDLA10-650 | 2x2 ULA Low | No PN | -2.1 |
|  |  |  |  | PN with CPE compensation | -2.0 |
| 400/480 | MCS13/Rank1 | TDLA10-200 | 2x2 ULA Low | No PN | 6.2 |
|  |  |  |  | PN with CPE compensation | 6.0 |
| 400/480 | MCS13/Rank2 | TDLD10-200 | 2x2 ULA Low | No PN | 9.5 |
|  |  |  |  | PN with CPE compensation | 9.2 |
| 400/480 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | No PN | 8.5 |
|  |  |  |  | PN with CPE compensation | 7.8 |
| 400/120 (Note 2) | MCS20/Rank1 | TDLA10-200 | 2x2 ULA Low | No PN | 11.5 |
|  |  |  |  | PN with CPE compensation | 13.1 |
|  |  |  |  | PN with ICI-1 compensation | 13.2 |
|  |  |  |  | PN with ICI-2 compensation | 13.2 |
|  |  |  |  | PN with ICI-3 compensation | 13.2 |
| 400/120 (Note 2) | MCS22/Rank1 | TDLA10-200 | 2x2 ULA Low | No PN | 13.6 |
|  |  |  |  | PN with CPE compensation | 16.1 |
|  |  |  |  | PN with ICI-1 compensation | 16.1 |
|  |  |  |  | PN with ICI-2 compensation | 16.1 |
|  |  |  |  | PN with ICI-3 compensation | 15.9 |

Conclusion

In this paper, we provide our simulation results for FR2-2 PDSCH performance requirements.

Reference

[1]   R4-2210665 WF on UE demodulation performance requirements definition for 52.6 - 71 GHz. Qualcomm Incorporated