3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213808

# Electronic Meeting, Aug.9th –20th ,2022

Title: 	Simulation results on PDCCH and PBCH requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.2.2

Document for:	Discussion

Background

As per [1], RAN4 listed some test cases for PDCCH simulation results alignment and interested companies can bring simulation results in this meeting. In this paper, we provide our initial simulation results for PDCCH. Additionally, we also provide some initial PBCH simulation results.

Discussions

PDCCH requirements

The BLER-SNR curve for PDCCH simulation results are captured in Figure 2-1 and summary of PDCCH simulation results are captured in Table 2-1:

Table 2-1: Summary of PDCCH simulation results

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA10-200 | 1_0 | 40 | 1x2 Low | 3.5 |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 1.0 |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low | -3.1 |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low | -6.0 |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 0.3 |

Figure 2-1: Simulation results for PDCCH

PBCH requirements

The BLER-SNR curve for PBCH simulation results are captured in Figure 2-2 and summary of PBCH simulation results are captured in Table 2-2:

Table 2-2: Summary of PBCH simulation results

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Antenna configuration and correlation matrix | SSB index | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNRBB (dB) |
| 1 | 100 / 120 | TDLA10-200 | 1 x 2 Low | Known | 1 | -9.7 |
| 2 | 100 / 120 | TDLA10-200 | 1 x 2 Low | Unknown | 1 | -9.2 |
| 3 | 400 / 480 | TDLA10-200 | 1 x 2 Low | Known | 1 | -9.6 |
| 4 | 400 / 480 | TDLA10-200 | 1 x 2 Low | Unknown | 1 | -7.7 |

Figure 2-2: Simulation results for PBCH

Conclusion

In this paper, we provide the initial simulation results for PDCCH and PBCH.

Reference

[1]   R4-2210665 WF on UE demodulation performance requirements definition for 52.6 - 71 GHz. Qualcomm Incorporated