3GPP TSG-RAN WG4 Meeting # 104e	R4-2213816

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Discussions on FR2-2 PRACH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.3.3

Document for:	Discussion

Background

As per [1], there are some open issues for FR2-2 PRACH demodulation requirements. In this paper, we provide our views on these open issues.

Discussions

Scope of PRACH requirements Define the following PRACH performance requirements: - False alarm probability requirements - PRACH miss detection requirements Sequence length PRACH sequence length: - 120 kHz: - 1151, 571, 139 - 480 kHz: - 571, 139 - 960 kHz: - 139 PRACH formats PRACH formats: A2, B4, C2 Frequency offset - 7100 Hz Time error tolerance Time error tolerance: - 0.07us for AWGN - FFS for multi-path fading (Ncs, logical sequence index, v) - Consider the following PRACH parameters as starting point: Ncs = 69, logical sequence index = 0, v =0 as starting point - Other options are not precluded

Discussions

NCS

One issue is how to select Ncs, based on our understanding, Ncs depends on cell size or max detectable time offset which can be calculated as follows:

Tmax=NCSLRA*SCS

The procedure to select NCS is to make the same cell size as defined in FR2-1 PRACH requirements. In Rel-15 FR2-1 PRACH requirements, NCS=69 was used. Hence we propose the following:

-SCS=120, LRA=139, NCS=69
      -SCS=120, LRA=571, NCS=285
      -SCS=120, LRA=1151, NCS=575
      -SCS=480, LRA=139, NCS=69
      -SCS=480, LRA=571, NCS=285

Proposal 1: Use following configuration for PRACH requirements:

-SCS=120, LRA=139, NCS=69
      -SCS=120, LRA=571, NCS=285
      -SCS=120, LRA=1151, NCS=575
      -SCS=480, LRA=139, NCS=69
      -SCS=480, LRA=571, NCS=285

Time error tolerance

One issue is time error tolerance:

Time error tolerance: - 0.07us for AWGN - FFS for multi-path fading

The time error tolerance for multi-path fading channel should depend on the delay spread of last tap defined in delay profiles. However, the simplified channel model still be discussing. Hence we propose to postponing discussion of this issue until simplified FR2-2 channel model is determined.

Proposal 2: Postponing discussion of time error tolerance until simplified FR2-2 channel model is determined.

Conclusion

In this paper we provide our views on FR2-2 PRACH performance requirements. The proposals are:

Proposal 1: Use following configuration for PRACH requirements:

-SCS=120, LRA=139, NCS=69
      -SCS=120, LRA=571, NCS=285
      -SCS=120, LRA=1151, NCS=575
      -SCS=480, LRA=139, NCS=69
      -SCS=480, LRA=571, NCS=285

Proposal 2: Postponing discussion of time error tolerance until simplified FR2-2 channel model is determined.

Reference

[1]   R4-2210664 WF on general and BS aspects for FR2-2 demodulation requirements. Intel Corporation