3GPP TSG-RAN WG4 Meeting#104-e                	R4-2213893

Electronic Meeting, 15 – 26 August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results of L1 and L3 measurement accuracy for FR2 HST scenarios

Agenda item:	9.7.3.1

Document for:	Discussion

Introduction

This contribution presents our L1 and L3 simulation results according to the agreed simulation assumptions in [1].

Discussion

L3 simulation results

Pursuant to the WF [1], the L3 simulation assumptions are provided in Tables 1 and 2.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 30 GHz |
| DRX | No |
| RRH transmit antenna configuration for SS blocks | 1 tx or single layer transmissions |
| UE receive antenna configuration | 2 rx |
| Data and control channel subcarrier spacing | SSB SCS 240 kHz/120 kHz, data SCS 120 kHz |
| Measurement period (in number of measurement samples) | 5 (other values should be considered such as 1 and 3) |
| - Subcarrier spacing SSB | 240 kHz/120 kHz |
| - Number of SS blocks per SS burst set, K | 1 up to 8 is possible (a subset can be used) |
| - SS burst periodicity | 5 ms and other values, e.g., 10ms, 20 ms |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | [CP/2] |
| Es/IoT | dB | 6.4 | 1.25; (SINR: -6dB) |
| Propagation conditions | - | AWGN with 19444 Hz offset between desired and interfering cells |  |

Tables 3-4 provide the SS-RSRP and SS-RSRQ measurement accuracy for the 5th, 50th and 95th percentile, which is calculated using the following equation.

Delta SS-RSRP   = (estimated SS-RSRP – ideal SS-RSRP) 	[dB]

Delta SS-RSRQ   = (estimated SS-RSRQ – ideal SS-RSRQ) 	[dB]

Table 3: Absolute SS-RSRP measurement accuracy for AWGN with and without 19444 Hz frequency offset

| SCS | Propagation condition | 5th percentile (dB) | 50th percentile (dB) | 95th percentile (dB) |
| --- | --- | --- | --- | --- |
| 120 kHz | AWGN 19444 Hz | -1.09 | -0.07 | 0.77 |
|  | AWGN | -0.57 | 0.3 | 1.1 |
| 240 kHz | AWGN 19444 Hz | -0.75 | 0.2 | 1.07 |
|  | AWGN | -0.57 | 0.34 | 1.11 |

Table 4: Absolute SS-RSRQ measurement accuracy for AWGN with and without 19444 Hz frequency offset

| SCS | Propagation condition | 5th percentile (dB) | 50th percentile (dB) | 95th percentile (dB) |
| --- | --- | --- | --- | --- |
| 120 kHz | AWGN 19444 Hz | -0.8 | 0.15 | 0.87 |
|  | AWGN | -0.3 | 0.53 | 1.22 |
| 240 kHz | AWGN 19444 Hz | -0.45 | 0.4 | 1.21 |
|  | AWGN | -0.32 | 0.53 | 1.23 |

Observation 1: From our simulation results, there is no degradation of SS-RSRP and SS-RSRQ accuracy with 19444 Hz carrier frequency offset as compared to without carrier frequency offset for both unidirectional and bidirectional scenarios.

Proposal 1: For Rel-17 FR2 HST unidirectional and bidirectional scenarios, the Rel-15 FR2 SS-RSRP and SS-RSRQ accuracy requirements are reused.

Figure 1 shows the simulation results of SS-SINR accuracy vs SNR values ranging from -10 dB to 25 dB for bi-directional deployment scenarios. As can be observed in the figure, the SS-SINR accuracy deteriorates for SNR ≥ 10 dB. For the region where SNR ≥ 10 dB, the SS-SINR accuracy becomes unbounded.

Observation 2: For unidirectional deployment scenarios, there is no performance degradation in SS-SINR accuracy. For bi-directional deployment scenarios, the SS-SINR accuracy becomes unbounded as SNR increases.

Based on the above observation, we propose to adopt the same methodology as FR1 HST in specifying the SS-SINR accuracy requirement by defining an SNR upper bound. This means, the SINR accuracy requirement is only defined for SNR values below the upper bound.

Proposal 2: For Rel-17 FR2 HST unidirectional scenarios, the Rel-15 FR2 SS-SINR accuracy requirements can be reused. However, for Rel-17 FR2 HST bidirectional scenarios, the Rel-15 FR2 SS-SINR accuracy requirements can be limited to SNR ≤ 10 dB.

Figure 1: Delta SS-SINR vs SNR for bidirectional deployment scenarios

L1 simulation results

Pursuant to the WF [1], the L1 simulation assumptions are provided in Tables 3 and 4.

Table 5: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 | 30 GHz |
| channel | AWGN |
| BS transmit antenna configuration | 1 tx |
| UE receive antenna configuration | 2 rx |
| Subcarrier spacing | 120 kHz |
| Carrier frequency offset | 19444 Hz |
| Frequency tracking | TRS with periodicity = 10 ms |

Table 6: L1-SINR accuracy evaluation simulation assumptions for CMR only

| Simulation parameters | Comments/values |
| --- | --- |
| Data channel subcarrier spacing | The same as CSI-RS subcarrier spacing |
| Measurement period (in number of measurement samples) | 1 sample |
| CSI-RS periodicity | 80 slots |
| Number of PRBs | 48 |
| Density | 3 |
| Side condition (SNR) on CMR | -3 dB |

Table 7: L1-SINR accuracy evaluation simulation assumptions for SSB-based CMR + NZP-IMR

| Simulation parameters | Comments/values |
| --- | --- |
| Data channel subcarrier spacing | The same as SSB and CSI-RS subcarrier spacing |
| Measurement period (in number of measurement samples) for CMR/IMR | 1 sample |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| periodicity | CMR periodicity (80 slots) = IMR periodicity |
| Ideal SINR | -3dB |
| Number of PRBs for IMR | 48 |
| Density for IMR | 3 |
| Side condition (SNR) on CMR | 0 dB |
| Side condition (SNR) on IMR | 0 dB |

Table 8: L1-SINR accuracy evaluation simulation assumptions for SSB-based CMR + CSI-IM

| Simulation parameters | Comments/values |
| --- | --- |
| Data channel subcarrier spacing | The same as SSB and CSI-RS subcarrier spacing |
| Measurement period (in number of measurement samples) for CMR/IMR | 1 sample |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-IM |
| periodicity | CMR periodicity (80 slots) = IMR periodicity |
| Ideal SINR | -3dB |
| Number of PRBs for IMR | 48 |
| Density for IMR | 3 |
| Side condition (SNR) on CMR | -3 dB |
| Side condition (SNR) on IMR | N/A |

Table 9: L1-SINR accuracy evaluation simulation assumptions for CSI-RS-based CMR + NZP-IMR

| Simulation parameters | Comments/values |
| --- | --- |
| Data channel subcarrier spacing | The same as SSB and CSI-RS subcarrier spacing |
| Measurement period (in number of measurement samples) for CMR/IMR | 1 sample |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| periodicity | CMR periodicity (80 slots) = IMR periodicity |
| Ideal SINR | -3dB |
| Number of PRBs for IMR | 48 |
| Density for CMR/IMR | 3 |
| Side condition (SNR) on CMR | 0 dB |
| Side condition (SNR) on IMR | 0 dB |

Table 10: L1-SINR accuracy evaluation simulation assumptions for CSI-RS-based CMR + CSI-IM

| Simulation parameters | Comments/values |
| --- | --- |
| Data channel subcarrier spacing | The same as SSB and CSI-RS subcarrier spacing |
| Measurement period (in number of measurement samples) for CMR/IMR | 1 sample |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-IM |
| periodicity | CMR periodicity (80 slots) = IMR periodicity |
| Ideal SINR | -3dB |
| Number of PRBs for IMR | 48 |
| Density for CMR/IMR | 3 |
| Side condition (SNR) on CMR | -3 dB |
| Side condition (SNR) on IMR | N/A |

Tables 11-13 provide the L1-SINR measurement accuracy for the 5th, 50th and 95th percentile, which is calculated using the following equation.

Delta L1-SINR   = (estimated L1-SINR – ideal L1-SINR) 	[dB]

Table 11: Absolute L1-SINR measurement accuracy for AWGN with 19444 Hz frequency offset (frequency tracking disabled)

| CMR | IMR | 5th percentile (dB) | 50th percentile (dB) | 95th percentile (dB) |
| --- | --- | --- | --- | --- |
| CSI-RS | N/A | -1.47 | 0.13 | 1.55 |
| SSB | NZP-IMR | -1.74 | -0.13 | 1.38 |
|  | CSI-IM | -1.71 | -0.1 | 1.3 |
| CSI-RS | NZP-IMR | -1.53 | -0.01 | 1.32 |
|  | CSI-IM | -1.32 | -0.02 | 1.14 |

Table 12: Absolute L1-SINR measurement accuracy for AWGN with 19444 Hz frequency offset (frequency tracking enabled for bidirectional scenarios)

| CMR | IMR | 5th percentile (dB) | 50th percentile (dB) | 95th percentile (dB) |
| --- | --- | --- | --- | --- |
| CSI-RS | N/A | -0.82 | 0.64 | 1.94 |
| SSB | NZP-IMR | -1.06 | 0.53 | 2.07 |
|  | CSI-IM | -1.07 | 0.47 | 1.98 |
| CSI-RS | NZP-IMR | -0.88 | 0.53 | 1.91 |
|  | CSI-IM | -0.77 | 0.48 | 1.61 |

Table 13: Absolute L1-SINR measurement accuracy for AWGN with 0 Hz frequency offset

| CMR | IMR | 5th percentile (dB) | 50th percentile (dB) | 95th percentile (dB) |
| --- | --- | --- | --- | --- |
| CSI-RS | N/A | -0.41 | 0.37 | 1.29 |
| SSB | NZP-IMR | -1.04 | 0.45 | 1.75 |
|  | CSI-IM | -1.1 | 0.44 | 1.78 |
| CSI-RS | NZP-IMR | -0.81 | 0.57 | 1.88 |
|  | CSI-IM | -0.7 | 0.58 | 1.71 |

For L1-SINR measurement accuracy under Rel-17 FR2 HST, simulation results are presented in Tables 11-13 for consideration in RAN4.

The general assumptions in Table 3 are applicable to L1-RSRP evaluation and the specific simulation parameter assumptions derived from Table 6 are given in the table below.

Table 14: SSB-based L1-RSRP accuracy evaluation simulation assumptions

| Simulation parameters | Comments/values |
| --- | --- |
| Data channel subcarrier spacing | The same as CSI-RS subcarrier spacing |
| Measurement period (in number of measurement samples) | 1 sample |
| SSB periodicity | 80 slots |
| Side condition (SNR) on CMR | -3 dB |

Tables 15-17 provide the L1-RSRP measurement accuracy for the 5th, 50th and 95th percentile, which is calculated using the following equation.

Delta L1-RSRP   = (estimated L1-RSRP – ideal L1-RSRP) 	[dB]

Table 15: Absolute L1-RSRP measurement accuracy for AWGN with 19444 Hz frequency offset (frequency tracking disabled)

| CMR | IMR | SINR | 5th percentile (dB) | 50th percentile (dB) | 95th percentile (dB) |
| --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | -3 dB | -1.17 | 0.14 | 1.24 |
| SSB | N/A | -3 dB | -1.54 | 0.04 | 1.44 |

Table 16: Absolute L1-RSRP measurement accuracy for AWGN with 19444 Hz frequency offset (frequency tracking enabled)

| CMR | IMR | SINR | 5th percentile (dB) | 50th percentile (dB) | 95th percentile (dB) |
| --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | -3 dB | -0.7 | 0.48 | 1.53 |
| SSB | N/A | -3 dB | -0.98 | 0.42 | 1.73 |

Table 17: Absolute L1-RSRP measurement accuracy for AWGN with 0 Hz frequency offset

| CMR | IMR | SINR | 5th percentile (dB) | 50th percentile (dB) | 95th percentile (dB) |
| --- | --- | --- | --- | --- | --- |
| CSI-RS | N/A | -3 dB | -0.63 | 0.54 | 1.55 |
| SSB | N/A | -3 dB | -1.11 | 0.43 | 1.64 |

For L1-RSRP measurement accuracy under Rel-17 FR2 HST, simulation results are presented in Tables 15-17 for consideration in RAN4.

Conclusion

We have presented the simulation results for L1 and L3 measurement accuracy under FR2 HST scenarios.

Observation 1: From our simulation results, there is no degradation of SS-RSRP and SS-RSRQ accuracy with 19444 Hz carrier frequency offset as compared to without carrier frequency offset for both unidirectional and bidirectional scenarios.

Proposal 1: For Rel-17 FR2 HST unidirectional and bidirectional scenarios, the Rel-15 FR2 SS-RSRP and SS-RSRQ accuracy requirements are reused.

Observation 2: For unidirectional deployment scenarios, there is no performance degradation in SS-SINR accuracy. For bi-directional deployment scenarios, the SS-SINR accuracy becomes unbounded as SNR increases.

Proposal 2: For Rel-17 FR2 HST unidirectional scenarios, the Rel-15 FR2 SS-SINR accuracy requirements can be reused. However, for Rel-17 FR2 HST bidirectional scenarios, the Rel-15 FR2 SS-SINR accuracy requirements can be limited to SNR ≤ 10 dB.

- For L1-SINR measurement accuracy under Rel-17 FR2 HST, simulation results are presented in Tables 11-13 for consideration in RAN4.

- For L1-RSRP measurement accuracy under Rel-17 FR2 HST, simulation results are presented in Tables 15-17 for consideration in RAN4.

References

[1]    R4-2210609, WF on RRM performance requirement for FR2 HST, Samsung