3GPP TSG-RAN4 Meeting #104-e	R4-2213931

# Online, , 15th  - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2555 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TRP Specific BFR requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current wording suggests that TRP specific BFR is for inter-cell mTRP only, while TRP speific BFR is for inter-cell mTRP and intra-cell mTRP operation. Based on RAN1 design, TRP specific BFD resources are only periodic CSI-RS resources. But RAN4 has requirements for both SSB and CSI-RS based BFD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clarification in text that resources configured for TRP specific BFR are be from serving cell alone or serving cell and cell with different PCI. Deleted section on SSB based BFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability of TRP specific link recovery is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.1 8.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Change 1---------------------

## 8.18	TRP specific Link Recovery Procedures

### 8.18.1	Introduction

The UE shall assess the downlink radio link quality of TRPs from a serving cell or TRPs from serving cell and  a cell with different PCI based on the reference signal provided in the two sets q0,0 and q0,1 as specified in TS 38.213 [3] in order to detect beam failure on each TRP in:

-	PCell in SA, NR-DC, or NE-DC operation mode,

-	PSCell in NR-DC and EN-DC operation mode,

-	SCell in SA, NR-DC, NE-DC or EN-DC operation mode.

The RS resource configurations in the two sets q0,0 and q0,1 on PCell , PSCell or SCell are periodic CSI-RS resources. q0,0q0,1UE is not required to perform beam failure detection outside the active DL BWP. UE is not required to meet the requirements in clause 8.18.2 and 8.18.3 if UE does not have configured sets q0,0 and q0,1. UE is not required to perform beam failure detection on a deactivated SCell, and also not required to perform beam failure detection on resources which is implicitly configured for a deactivated SCell. UE is not required to perform beam failure detection on a SCell on which  q1,0 and q1,1 is not configured.

On each RS resource configuration in the two sets q0,0 and q0,1, the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the TRPs from the serving cell or TRPs from serving cell and cell with different PCI .

The threshold Qout_LR is defined as the level at which the downlink radio level link of a given resource configuration on set two sets q0,0 and q0,1 cannot be reliably received and shall correspond to the BLERout = 10% block error rate of a hypothetical PDCCH transmission. For CSI-RS based beam failure detection, Qout_LR_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in Table 8.18.3.1-1.

Upon request the UE shall deliver configuration indexes from the two sets q1,0 and q1,1as specified in TS 38.213 [3] , to higher layers,  and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR, which is indicated by higher layer parameter rsrp-ThresholdSSB. The UE applies the Qin_LR threshold to the L1-RSRP measurement obtained from an SSB. The UE applies the Qin_LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by higher layer parameter powerControlOffsetSS. The RS resource configurations in the two sets q1,0 and q1,1 can be periodic CSI-RS resources or SSBs or both SSB and CSI-RS resources. UE is not required to perform candidate beam detection outside the active DL BWP. UE is not required to perform candidate beam detection on a SCell on which q1,0 and q1,1 is not configured.

### 8.18.2	Requirements for TRP specific SSB based beam failure detection

#### q0,0q0,1

#### q0,0q0,1

#### P=11-TSSBMGRP

#### P=11-TSSBTSMTCperiod

#### P=11-TSSBMGRP - TSSBTSMTCperiod

#### P=Psharing factor1-TSSBMGRP

#### P=11-TSSBTSMTCperiod

#### P=Psharing factor1-TSSBMGRP

#### q0,0q0,1

| #### q0,0q0,1 |  |
| --- | --- |

| #### q0,0q0,1 |  |
| --- | --- |

#### Void