3GPP TSG-RAN WG4 Meeting #104-e	R4-2213943

E-Meeting, 15 Aug. 2022 – 26 Aug. 2022

Title:	Reply LS on SSB measurement for L1-RSRP on inter-cell beam management

Response to:	R1-2205640

Release:	Rel-18

Work Item:	NR_feMIMO-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

#### Name: 				Venkatarao Gonuguntla

#### E-mail Address :	venkatarao.gonuguntla@ericsson.com

Attachments:	None

1. Overall description:

RAN4 thanks RAN1 for the LS on SSB measurement for L1-RSRP on inter-cell beam management. RAN4 discussed the LS and would like to provide a below response.

RAN4 response:

RAN4 defined scheduling availability of UE during L1-RSRP measurement on serving and additional-serving cells in clause 9.3.16 of TS 38.133. UE can receive PDSCH/PDCCH while performing L1-RSRP measurements based on the below scheduling availability conditions specified in RAN4.

#### 9.13.6.1 Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1 There are no scheduling restrictions due to L1-RSRP measurement performed on SSB as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH on serving cell(s) and cell(s) with PCI different from serving cell(s) in FR1. #### 9.13.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement. - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement, where the transmission of PUCCH/PUSCH/SRS and reception of PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI may be on serving cell(s) and cell(s) with PCI different from serving cell(s). When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on cell(s) with PCI different from serving cell(s) where L1-RSRP measurement is performed apply to all serving cells and cell(s) with PCI different from serving cell(s) in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) and cell(s) with PCI different from serving cell(s) configured in other bands than the bands in which the serving cell where L1-RSRP measurement is performed is configured. #### 9.13.6.3 Scheduling availability of UE performing L1-RSRP measurement on FR2 The following scheduling restriction applies due to L1-RSRP measurement. - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement, where where the transmission of PUCCH/PUSCH/SRS and reception of PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI may be on serving cell(s) and cell(s) with PCI different from serving cell(s). When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions on cell(s) with PCI different from serving cell(s) where L1-RSRP measurement is performed apply to all serving cells and cell(s) with PCI different from serving cell(s) in the band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to L1-RSRP measurement performed on FR2 serving cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair. Editor’s note: FFS the joint requirement of inter-cell BM and IBM. If following conditions are met, - UE has been notified about system information update through paging, - The gap between UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots, For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for L1-RSRP measurement; and For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for L1-RSRP measurement. #### 9.13.6.4 Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA There are no scheduling restrictions on FR1 serving cell(s) and cell(s) with PCI different from a serving cell(s) due to L1-RSRP measurement performed on FR2 cell(s) with PCI different from a serving cell(s). There are no scheduling restrictions on FR2 serving cell(s) and cell with PCI different from a serving cell(s) due to L1-RSRP measurement performed on FR1 cell with PCI different from a serving cell(s). Editor’s note: FFS whether the scheduling availability on non-serving cell is needed or not when the UE is performing L1-RSRP measurement on serving cell(s)

2. Actions:

To RAN4:

RAN4 kindly requests RAN1 to consider the above response into their future specification work.

3. Date of Next RAN1 Meetings:

TSG RAN WG4 Meeting #104bis-e                  10 – 19 October 2022                              	e-Meeting

TSG RAN WG4 Meeting #105-e                       14 - 18 November 2022                              	e-Meeting