3GPP TSG RAN WG4 Meeting #104-e	R4-2213958

Electric meeting, 15 August 2022 – 26 August 2022

Agenda Item:	11.9.3

Source: 	Ericsson

Title:	Discussion on FR2 SCell activation delay reduction

Document for:	Discussion

# Introduction

In Rel-18 following WID [1] is agreed as part of Even Further RRM enhancement for NR and MR-DC.

- FR2 SCell activation delay reduction

  - Identify cases where FR2 SCell activation delay can be reduced (e.g., unknown target cell cases), and specify reduced delay requirements for such cases, including but not limited to [RAN4]

    - Study and, if feasible, enhance cell detection for unknown SCell and time/frequency tracking

    - Study and, if feasible, enhance L1-RSRP measurement delay reduction on target SCell

    - Note: Subject to RAN4’s agreement, the technical solutions can be extended to other general RRM requirements if applicable.

  - Specify if needed, reference signal enhancement and/or signaling enhancement for the UE to meet the enhanced delay requirements [RAN4, RAN2]

    - Note: No RAN1 work, i.e. introducing new RS, is expected.

  - Note: the technical solutions can be extended to FR1, when applicable

  - Note: RAN1/RAN2 work can be triggered by RAN4 LS only

- Define RRM requirements for FR1-FR1 NR-DC scenarios [RAN4]

  - RRM requirements include the number of serving carriers, PSCell addition/release delay requirement, PSCell change and conditional PSCell change delay, scheduling availability, and CSSF. Other Rel-15 requirements are not precluded and are subject to WI stage discussion.

  - For R16 and R17 features, RRM requirements for FR1-FR1 NR-DC including HO with PSCell, SCG activation/deactivation and CPAC.

    - Note: no other R16/17 features are considered

  - Note: RAN1/RAN2 work can be triggered by RAN4 LS only

In this contribution, we provide our views on first high level objective, that is FR2 SCell activation delay reduction.

In the following sections we first analyse the objectives of the WID to provide our understanding of the WID and in later sections, we also provide our thoughts on FR2 SCell delay reduction.

# WID interpretation

In Rel-15 and Rel-16 single and multiple SCell activation delay requirements are defined respectively mostly based on the SSB and SMTC periodicity of the SCell to be activated. Since SSB is a cell specific signal, SCell activation delay is relatively higher in Rel-15 and Rel-16. Based on the scenario of operation, SCell activation delay could take anywhere from 3ms to few seconds. In Rel-17, A-TRS based fast SCell activation delay requirements are defined to speed up the SCell activation of a deactivated SCell. Rel-17 fast SCell activation is mainly targeted at improving the SCell activation delay of a known FR1 and FR2 single SCell. Further, Rel-17 also introduced a PUCCH SCell activation delay requirements when a PUCCH SCell is activated alone or PUCCH SCell is activated along with other normal SCells (i.e., SCell without PUCCH).

In the following sections we first analyse the two main objectives of the SCell activation delay reduction.

## First objective

- Identify cases where FR2 SCell activation delay can be reduced (e.g., unknown target cell cases), and specify reduced delay requirements for such cases, including but not limited to [RAN4]

    - Study and, if feasible, enhance cell detection for unknown SCell and time/frequency tracking

    - Study and, if feasible, enhance L1-RSRP measurement delay reduction on target SCell

    - Note: Subject to RAN4’s agreement, the technical solutions can be extended to other general RRM requirements if applicable.

Based on the scenario of the operation, legacy SCell activation delay requirement contains all or some of the following steps.

- Coarse AGC retuning (Tcoarse_AGC)

- Fine AGC retuning (Tfine_AGC)

- Cell search or detection (Tsearch)

- L1-RSRP measurement and report to NW (TL1-RSRP)

- TCI indication to UE (Tbeam_indication)

- CSI activation command delay (Tuncertainity)

- Fine time tracking (Tfine_timing)

- CSI report (TCSI_Reporting)

If we look at the work item objective, only few steps (step 3, 4, 7) mentioned above are mentioned as part of the study. However, we think these steps in the objective are not to limit the study to only the steps 3, 4 and 7. We think RAN4 shall look at all steps to see any delay reduction is possible.

Proposal 1: RAN4 to look at all the steps of the legacy SCell activation delay requirement to study the possible delay reduction.

The last bullet of the first main objective is bit vague, and it may be subjected to different interpretations. It says:

Note: Subject to RAN4’s agreement, the technical solutions can be extended to other general RRM requirements if applicable.

As per above sentence, any potential enhancement introduced by RAN4 shall be applicable to other general RRM requirements. Here, other general RRM may be interpreted differently by different companies. Therefore, we think it is essential to have common understanding among the companies. Our understanding of the above sentence is whatever the enhancement introduced cannot be made applicable to all the general RRM requirements specified in TS 38.133. As per our understanding, what it means is other SCell activation related requirements. That means, the solution introduced may be extended to single SCell activation requirements, multiple SCell activation requirements, PUCCH SCell activation delay requirements, etc. In short, we understand the other general RRM requirements as RAN4 should discuss the applicability of the solution introduced to different sections SCell activation delay requirements.

### Applicability of the potential enhancement to PUCCH SCell

Our understanding is potential enhancement shall be applicable to PUCCH SCell activation case as the downlink activation delay is same as normal SCell activation delay and it is straight forward to apply any improvement introduced for single SCell activation to PUCCH SCell activation case.

Proposal 2: RAN4 to agree that potential enhancement that is going to be introduced shall be applicable to PUCCH SCell activation too.

### Applicability of the potential enhancement to direct SCell activation

Our understanding is potential enhancement shall be applicable to direct SCell activation case as the downlink activation delay of direct SCell activation is same as normal SCell activation delay and it is straight forward to apply any improvement introduced for single SCell activation to direct SCell activation scenario.

Proposal 3: RAN4 to agree that potential enhancement that is going to be introduced shall be applicable to direct SCell activation at addition, direct SCell activation at HO, direct SCell activation at RRC resume.

### Applicability of potential enhancement to multiple SCell activation

We defined multiple SCell activation delay requirements as a series of single SCell activation delay requirements in Rel-16 and Rel-17. Any potential improvement in the single SCell activation delay shall be logical to extend to multiple SCell activation scenarios.

Proposal 4: RAN4 to agree that potential enhancement that is going to be introduced shall be applicable to multiple SCells activation.

## Second objective

  - Specify if needed, reference signal enhancement and/or signaling enhancement for the UE to meet the enhanced delay requirements [RAN4, RAN2]

    - Note: No RAN1 work, i.e., introducing new RS, is expected.

As per the second objective of the WID, there is no RAN1 work is expected w.r.t introducing a new RS to achieve the SCell activation delay reduction. Further, it is expected that RAN4 can look at existing reference signal enhancement and/or signalling enhancement to achieve the SCell activation delay reduction.

A-TRS based fast SCell activation achieved significant SCell delay reduction in Rel-17. However, the usage of A-TRS was limited to single known SCell in FR1 and FR2. We think it is worthwhile to look at possible enhancement A-TRS to use for FR2 and FR1 unknown SCells.

Proposal 5: RAN4 to study A-TRS based fast SCell activation for FR2 and FR1 unknown SCells.

Further in Rel-17, A-TRS based fast SCell activation delay requirements are defined for single SCell due to time constraints. When multiple SCells are activated using the same MAC CE command, and when one or more SCells among the activated SCells have A-TRS based SCell activation configured, A-TRS based SCell activation can positively impact the normal SCell activation (e.g., normal SCell can skip some of the steps involved in the SCell activation). Hence, we think along with enhancement of A-TRS based solution to unknown FR2 and FR1 SCells, it is worth studying multiple SCell activation delay requirements as it can act as an improvement to the other SCell activation delay. At least in some cases, it may so happen that without many enhancements being introduced, SCell activation delay can be improved for normal SCells, when they are activated along with A-TRS based SCell activation.

One such example scenario could be intra-band SCell activation, where the SCells to be activated may belong to same band and if one SCell from the to be activated SCells are configured with A-TRS based fast SCell activation, it can have impact on the other to be SCells activation from the same band.

Proposal 6: RAN4 to study and define multiple SCell activation delay requirements, where at least one SCell is activated using A-TRS.

# FR2 SCell activation delay reduction

In Rel-17, fast SCell activation method is specified to reduce the delay of a known SCell and known cell need not be considered in this WI, unless the specified Rel-18 solution further improves the known SCell activation delay than A-TRS.

Proposal 7: RAN4 to prioritize at least FR2 unknown SCell delay reduction in the initial phase of the WI.

The unknown SCell activation delay for SP and periodic CSI reporting is copied below

If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

We believe following four components are the main sources contributing to higher delay.

- Coarse AGC (TFirstSSB_MAX + 7*TSMTC_MAX)

- Fine AGC (8*TSMTC_MAX)

- Cell search (8*Trs)

- L1-RSRP measurement (TL1-RSRP, measure)

Compared to FR1, these four components contribute to high delay mainly due to RX beam sweeping assumed in each of these 4 steps/components. Currently, RX beam sweeping factor (N1) is assumed as always 8 for all the steps in FR2 SCell activation

One approach to achieve delay reduction is by achieving reduction in Rx beam sweeping factor. We think RAN4 should look at possible solutions to achieve the reduction in scaling factor.

Proposal 8: RAN4 to discuss the solutions or pre-conditions to reduce the scaling factor N1 in FR2 SCell activation.

Main reason RAN4 agreed to define RX beam sweeping for all the four steps during SCell activation is under the assumption that RX beams may change from step to step. In RAN4, currently, we assume that up to 1280ms, RX beams may not change, and same RX beams are assumed to be valid. By assuming RX beam may change only after 1280ms or after some X value, UE can reuse the RX beam sweeping information obtained in one step for next 1280ms or X value.

We consider below scenarios to analyse RX beam sweeping necessity.

Scenario 1: SMTC periodicity is 40ms

Scenario 2: SMTC periodicity is 80ms

Scenario 3: SMTC periodicity is equal to higher than 160ms

For scenario 1, UE RX beam sweeping time is 320ms. That means Coarse AGC takes 320ms and for remaining 960ms of 1280 ms, RX beam sweeping is not needed as the beam can be assumed to be known. That means next three samples do not need RX beam sweeping.

For scenario 2, UE RX beam sweeping time is 640ms. That means Coarse AGC takes 640ms and for remaining 640ms of 1280 ms, RX beam sweeping is not needed as the beam can be assumed to be known. That means next three samples (fine AGC, cell search, L1-RSRP) do not need RX beam sweeping.

For scenario 3, UE RX beam sweeping time is 1280ms. That means Coarse AGC takes 1280ms and, RX beam sweeping is needed for next next three samples (fine AGC, cell search, L1-RSRP) also.

We can reuse the Rel-15 value of 1280ms as RX beam constant time or we discuss some other value in Rel-18.

Proposal 9: RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X=1280ms].

Proposal 10: If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping.

One other approach to reduce the SCell activation delay is reducing the RX beam sweeping scaling factor for some steps based on the information obtained during the previous step. For example, UE performs coarse AGC with RX beam sweeping with N1=8 and based on the information obtained during coarse AGC, UE may be able to use N1=4 for fine AGC and N1=4 for cell search and N1=2 for L1-RSRP. The information UE could use to enable this method can be discussed further in next meetings.

Proposal 11: UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before.Other possible solution to reduce the unknown SCell activation delay is using the A-TRS method for this cell. However, A-TRS method needs beam information at the gNB. If RAN4 can introduce a solution or method of UE sharing a beam information (even though it is not based on accurate L1-RSRP) seen at the UE before L1-RSRP report or as early possible after the SCell activation start, gNB can configure A-TRS and thereby enabling a quicker SCell activation.

Proposal 12: RAN4 to study solutions to enable A-TRS based fast SCell activation for unknown FR2 SCell.

# Summary and Conclusion

In this contribution we have provided our views on the FR2 SCell activation delay reduction and made following proposals:

Proposal 1: RAN4 to look at all the steps of the legacy SCell activation delay requirement to study the possible delay reduction.

Proposal 2: RAN4 to agree that potential enhancement that is going to be introduced shall be applicable to PUCCH SCell activation too.

Proposal 3: RAN4 to agree that potential enhancement that is going to be introduced shall be applicable to direct SCell activation at addition, direct SCell activation at HO, direct SCell activation at RRC resume.

Proposal 4: RAN4 to agree that potential enhancement that is going to be introduced shall be applicable to multiple SCells activation.

Proposal 5: RAN4 to study A-TRS based fast SCell activation for FR2 and FR1 unknown SCells.

Proposal 6: RAN4 to study and define multiple SCell activation delay requirements, where at least one SCell is activated using A-TRS.

Proposal 7: RAN4 to prioritize at least FR2 unknown SCell delay reduction in the initial phase of the WI.

Proposal 8: RAN4 to discuss the solutions or pre-conditions to reduce the scaling factor N1 in FR2 SCell activation.

Proposal 9: RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X=1280ms].

Proposal 10: If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping.

Proposal 11: UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before.

Proposal 12: RAN4 to study solutions to enable A-TRS based fast SCell activation for unknown FR2 SCell.

# References

- RP-220977 “WID on Even Further RRM enhancement for NR and MR-DC”, Apple Oppo