3GPP TSG-RAN WG4 #104-e	R4-2214000

August 15th ‒ 26th, 2022

Electronic Meeting

Agenda item:	10.19.3

Source: 	Qualcomm Incorporated

Title: 	UE RF requirements for the APT 600 MHz band

Document for:	Discussion

# Introduction

The APT 600 MHz band was the subject of a study item with a corresponding technical report published [1] during the Rel-17 timeframe.  During the course of the study item, a preferred band plan according to option B1 was identified as well as feasibility and requirements evaluated.  A work item to specify the band has now been agreed [2].  UE RF requirements are presented for the APT 600 MHz band herein with a draft CR in [8].

# Discussion

## Frequency arrangement

According to an LS [3] from AWG, the preferred frequency plan is according to option B1.  Thus, we have the frequency arrangement as

Table 1: NR operating band (option B1)

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

It is further indicated in [2] that “Option B1 with a single duplexer” should be the basis for the normative work.

## Channel bandwidths

According to [2], the requested channel bandwidths are 5, 10, 15, 20, 25, 30, and 35 MHz.  Thus, although the band is wide enough to support 40 MHz channels, these have not been requested at this time.  Furthermore, it is indicated that the uplink channel bandwidth should be limited to 20 MHz to limit the MSD similar to the case with Band n71.  Thus, the channel bandwidths can be specified as follows

| nXYZ | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Channel and sync raster

Due to the assumption of single duplexer, all channel raster points within the band are available for all channel bandwidths except for those where the channel would extend beyond the edges of the band.  The 100 kHz channel raster resolution is maintained at this frequency range to maximize compatibility with Band n71.

Sync raster compatibility should also be maintained between the APT 600 MHz band and Band n71 to lessen the disruption in case deployments are first enabled with Band n71 and later migrated to APT 600.

Table 2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| nXYZ | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. NOTE 2: The following NREF are allowed for operation in Band n46: see Table 5.4.2.3-2. NOTE 3: The following NREF are allowed for operation in Band n96: see Table 5.4.2.3-3. NOTE 4: The following NREF are allowed for operation in Band n102: see Table 5.4.2.3-4. |  |  |  |

Table 2.3-2: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n71 | 15 kHz | Case A | 1547 – <1> – 1624 |
| nXYZ | 15 kHz | Case A | 1535 – <1> – 1624 |

## Maximum output power

The work item [2] objective states the use of UE power class 3 for this band, therefore, the maximum output power shall be 23 dBm.  The maximum output power tolerance for Band n71 is +2/-2.5 dB and is proposed to be adopted for this band as well.  It had been indicated during the study item and documented in the TR 38.860 [1] that the lower tolerance of 2.5 dB would be needed to accommodate Tx filter insertion towards the band edge.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| nXYZ |  |  |  |  |  |  | 23 | +2/-2.5 |

## Spurious emissions

Additional spurious emissions are specified for Band n71 using NS_35.  According to [3], NS_35 is not expected to be signaled by cell operating in Band nXYZ.  It would be desirable from an ecosystem perspective for a UE to support both bands n71 and nXYZ since they are so similar in frequency range and requirements.  In that case, the UE would be required to support NS_35 but only for Band n71.  However, text in [3] hints that there may also be the possibility that NS_35 requirements also apply in APT countries.  While this possibility cannot be ruled out, it is also indicated in [3] that at the present time no additional requirements beyond the general spurious emissions have been identified.  Thus, it is recommended that only the general NS_01 is specified for Band nXYZ at this time.

UE coexistence requirements should also be specified for Band nXYZ.  Since the motivation as explained in [2] is to enable companion spectrum to Band 28 in the 700 MHz frequency range, it is proposed to list the same UE coexistence bands for Band nXYZ as have been specified for Band n28.

## Reference sensitivity

Due to the small Tx-Rx separation, reference sensitivity is impacted for this band particularly for wider bandwidths.  In fact, the same phenomenon has already been studied for Band n71.  Therefore, it is expected that Band n71 reference sensitivity studies and specifications can be leveraged for Band nXYZ.  The uplink and downlink bandwidths for both bands are the same.  However, due to the wider passband for Band nXYZ (40 MHz vs. 35 MHz), the duplexer insertion loss and isolation may be slightly worse.  On the other hand, the Tx-Rx separation is slightly larger (51 MHz vs. 46 MHz) which will reduce the Tx interference providing for better (lower) reference sensitivity.

### Duplexer performance

The duplexer performance was studied and reported in [1].  It was found that the Rx insertion loss may increase relative to Band n71 by approximately 0.5 dB currently.  However, it was also reported that with design optimization and employing more advanced filter technologies, performance is expected to improve in the future.  Nonetheless, we assume a 0.5 dB higher front-end loss in the analysis compared to Band n71.  Thus, while the assumed FE loss for Band n71 was 4 dB (see [5] and [6] for examples), we assume 4.5 dB for this analysis.  The Rx isolation of 50 dB and Tx isolation of 55 dB is retained in the analysis (TBD to be confirmed by filter simulation).

### Uplink channel placement and RB restriction

For bands with asymmetric uplink and downlink bandwidths, i.e., 35 MHz downlink and 20 MHz uplink, such as Band n71 and Band nXYZ, there was previous discussion on where the uplink channel should be located.  Three options were consider:  best, middle, and worst placement corresponding to the largest Tx-Rx separation, Tx-Rx separation according to band’s default, and smallest Tx-Rx separation.  It was agreed to define the specification for reference sensitivity with middle uplink channel placement.  This is reflected in Note 5 of Table 7.3.2-3 of TS 38.101-1 which reads

Note 5:	For this DL channel bandwidth, the UL configuration of the highest UL channel bandwidth specified in Table 5.3.6-1 and the default Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used.

indicating the default Tx-Rx separation applies.  It is expected that Band nXYZ will use this same convention.  The uplink RB’s within the channel will be placed closest to the downlink band – left justified in the case of Band nXYZ.  An example is illustrated below

where the downlink channel bandwidth is 35 MHz centered at 634.5 MHz and the uplink channel bandwidth is 20 MHz centered at 685.5 MHz – the Tx-Rx separation is 51 MHz using the default for the band.  The downlink is fully allocated while the uplink is limited to 20 RB’s starting at RB0.

### Transmitter noise

This configuration is similar to the one studied for Band n71 with the key difference being the potential difference in duplexer performance and the larger separation between Tx and Rx.  As discovered during the Band n71 study, the Tx interference into Rx is composed of PA regrowth as well as Tx and image IM3.  For Band n71, there was small overlap of IM3 due to Tx image and Tx CIM5 emission [5] for the 35 MHz DL / 20 MHz UL pairing.  For Band nXYZ, with a larger default Tx-Rx spacing of 51 MHz, the impact is less severe but is still present.  Moreover, the PA spectral regrowth noise will also be smaller due to the larger separation.  We note that the Tx-Rx separation is increased by 5 MHz while the Tx UL configuration is 3.6 MHz (20 RBs for 15 kHz SCS) so the IM order needed to reach the receiver are 1 or 2 higher with the corresponding drop in magnitude.

### Reference sensitivity analysis

Following the analysis presented for Band n71 in [5] and [6], reference sensitivity is calculated for Band nXYZ.  As described above, the front-end loss is increased by 0.5 dB to account for the increase in filter insertion loss, the noise terms and overlap correction factors for IM products have been recomputed for the larger Tx-Rx separation.  The same assumption of middle uplink channel placement was used for the cases of asymmetric uplink/downlink channel bandwidths with 20 RB uplink restriction placed at the closest channel edge.  The results are summarized below

For 30 kHz SCS, the reference sensitivity values are derived by bandwidth scaling rather than by direct computation.

## In-band blocking

One of the critical specifications for Band nXYZ is the in-band blocking.  Because the frequency range of the band is adjacent to digital terrestrial television (DTT), it is important to capture an in-band blocking requirement for the UE to ensure satisfactory performance when in the vicinity of a television broadcast.  The spectrum landscape is illustrated in [3] and copied below in Figure 1.

Figure 1.  Spectrum landscape copied from Figure A1 of [3].

Of interest is the Option B1 with its downlink as low as 612 MHz.  The DTT frequency channelization is shown for 6, 7, and 8 MHz rasters where it has been suggested in [3] that the smallest feasible guard band between the DTT channel and the Band nXYZ downlink is 4 MHz.  Therefore, the highest DTT channel practically available is CH37, CH38, and CH36 for countries with 8, 7, and 6 MHz rasterization.  Of these, the CH36 with 6 MHz raster is only provided a 4 MHz guard band while the other rasters enjoy a larger guard band.  Therefore, we focus on specifying an in-band blocking requirement to model the CH36 broadcast signal.  If the CH36 broadcast blocker can be met, then the blockers for 7 and 8 MHz channelization can also be met since they are separated with larger guard bands from Band nXYZ and blocking performance is expected to be monotonic in this frequency range.

A similar situation exists for Band n71 where CH36 blocker is separated by 9 MHz guard band.  For this, a Case 3 in-band blocker was specified for Band n71 as follows

Table 7.6.2-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. |  |  |  |  |  |  |

The blocker is a 5 MHz modulated signal centered at FDL_low  – 12 MHz which is 605 MHz corresponding to CH36.  The power level of the blocker is specified as -15 dBm.  It is possible to extend this blocker requirement to Band nXYZ.  The blocker is still centered at 605 MHz but might be more appropriately specified using absolute frequency rather than relative to FDL_low since the DTT channel is fixed.  In fact, the frequency of the blocker for Band n71 should probably also be specified in absolute frequency at 605 MHz.  The power level of blocker for Band nXYZ needs further consideration.  A blocker level of -15 dBm was derived for Band n71 by assuming a 1 mega-watt DTT transmitter and a deterministic propagation loss profile based on drive test measurements conducted in other frequency bands [4] from DTT broadcast tower to UE receive antenna before applying UE antenna gain.  On the other hand, a statistical system simulation was conducted and reported in [3] for the APT region assuming a 200 kW DTT transmitter with 4 MHz guard band and a statistical propagation loss profile.  The conclusion from the AWG study was a cumulative density of 97% for interference power level at -44 dBm, where the -44 dBm value was obtained from the general case 2 in-band blocking requirement for the UE.  The study includes an assumed 20 dB UE filter rejection and a UE antenna gain of -3 dBi so it can be concluded that the received power at the UE antenna input could be approximately -44 + 20 + 3 = -21 dBm.  This is extremely well aligned with the -15 dBm assumption used by the FCC for Band n71 blocking value when scaled by from 1 MW to 200 kW DTT transmission power leading to a UE interference power of -22 dBm without taking into antenna gain.  By the above reasoning, a -22 dBm interfering signal power at CH36 is appropriate for Band nXYZ.

However, one further aspect must be considered.  The -22 dBm blocker described above accurately reflects the expected received power at the UE’s antenna due to 200 kW CH36 broadcast in APT countries.  Moreover, the value is expected to be manageable by the UE when at least 20 dB rejection is available from the duplexer as conservatively assumed in [3].  In fact, two of the Band nXYZ filter designs reported in [1] indicate 10 dB higher rejection of approximately 30 dB when integrated over the CH36 frequency range from 602 – 608 MHz.  However, this was studied under the context of a case 2 in-band blocker which is nominally specified at -44 dBm.  The case 2 in-band blocker is specified as a 5 MHz modulated signal located with a center frequency offset of 12.5 MHz (FIoffset, case 2) from the channel edge.  However, since the guard band for Band nXYZ has been truncated by 5 MHz, the CH36 interfering signal falls inside of this case 2 frequency offset for the channel located at the edge of the band.  In fact, it falls approximately into the case 1 frequency offset.  The case 1 in-band blocker is specified at -56 dBm, rather than the -44 dBm for case 2.  The more relaxed value of -56 dBm compared to -44 dBm reflects the baseband filtering in the UE at small offset.  When taking this into consideration, the in-band blocker should be relaxed from -22 dBm to -34 dBm (relaxed by the difference between -44 dBm and -56 dBm) for channels at the lower edge of the band where CH36 falls inside the case 1 frequency offset range.  For channels further removed from the lower edge of the band within case 2 frequency offsets, the -22 dBm blocking requirement applies.

The final result for in-band blocking is then as follows

Table 7.6.2-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |
| nXYZ | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 | 605 MHz (NOTE 4) |  |
| NOTE 4: For Band nXYZ, the Case 3 requirement applies at the specified Finterferer with Pinterferer power level modified as follows. The Pinterferer power level shall be modified to -22 dBm for -BWChannel/2 – FIoffset, case 2 ≤ Finterferer ≤ BWChannel/2 + FIoffset, case 2 and to -34 dBm for -BWChannel/2 – FIoffset, case 1 ≤ Finterferer ≤ BWChannel/2 + FIoffset, case 1. |  |  |  |  |  |  |

# Conclusion

The UE RF requirements for the APT 600 MHz band have been introduced in this contribution including frequency arrangement, channel bandwidths, raster, output power, spurious emissions, reference sensitivity, and blocking.  The reference sensitivity analysis has been provided as well as the justification for the blocking requirement.  A draft CR can be found in [8].

# Reference

- TR 38.860 v17.0.0

- RP-221778, “New WID on APT 600 MHz NR band,” Spark NZ, Qualcomm Inc

- RP-221045, FREQUENCY ARRANGEMENTS FOR IMT IN THE BAND 470 – 703 MHZ

- Expanding the Economic and Innovation Opportunities of Spectrum Through Incentive Auctions, GN Docket No. 12-268, Report and Order, FCC 14-50.

- R4-2107339, “35MHz and 45MHz REFSENS,” Qualcomm Incorporated

- R4-2117155, “n71 REFSENS for 25M and 30M BWs,” Qualcomm Incorporated

- R4-2105346, “WF on UE REFSENS for 35 MHz and 45 MHz,” Huawei, HiSilicon

- R4-2114001, “Introduction of APT 600 MHz band,” Qualcomm Incorporated