3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214089

Electronic Meeting, 15 – 26 August 2022

Agenda item:			9.14.9

Source:	Moderator (Qualcomm Inc)

Title:	Email discussion summary for [104-e][111]

Document for:	Information

# Introduction

This discussion pertains to agenda items:

9.14.3	UE RF requirements	[NR_ext_to_71GHz-Core]

9.14.3.1	TX requirements	[NR_ext_to_71GHz-Core]

9.14.3.2	RX requirements	[NR_ext_to_71GHz-Core]

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm Inc | Phil Coan | pcoan@qti.qualcomm.com |
| Murata Manufacturing Co.,Ltd. | Hidefumi Ohira | hidefumi.ohira@murata.com |
| vivo | Shuai Zhou | Shuai.zhou@vivo.com |
| Anritsu Limited | Hassen Chouli | hassen.chouli@anritsu.com |
| Rohde & Schwarz | Niels Petrovic | Niels.petrovic@rohde-schwarz.com |
| Sony | Kun Zhao | kun.1.zhao@sony.com |
| LGE | Markus Pettersson | markus.pettersson@lge.com |
| Nokia | Johannes Hejselbaek | Johannes.hejselbaek@nokia.com |
| HW | Chunying Gu | guchunying@huawei.com |
| AT&T | Ron Borsato | ronald.borsato@att.com |
| Intel | Aida Vera Lopez | aida.l.vera.lopez@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic: Unwanted emissions and signal quality

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposal |
| --- | --- | --- | --- |
| R4-2211628 | 60GHz UE TX | Qualcomm Inc | Proposals: Proposal 10: The DMRS based channel estimate shall utilize CPE-corrected DMRS symbols Proposal 11: The PTRS extraction and correction stage is used as the final refinement of the received signal. Proposal 12: For CP-OFDM, all non-DMRS symbols in a slot must be equipped with PTRS, and frequency density of PTRS tones maximized. Proposal 13a: For DFT-s-OFDM, PTRS is specified with 4 symbols per group, and the groups are configured in a ‘head and tail’ configuration. Proposal 13b: For DFT-s-OFDM, the number of PTRS groups is maximised so the ratio of PUSCH symbols to PTRS symbols stays at 1 or higher. Proposal 14: RAN4 uses the proposed PN mask for development of EVM requirements. Proposal 15: Adopt the EVM power limit tables scaled for bandwidth, min peak EIRP, and for the higher FR2-2 operating frequency. Use n262 400 MHz as the basis for scaling. Using that method, the values to be used are in the table above. Proposal 16: PC1 carrier leakage for n263 as shown in the table: Proposal 17: PC3 carrier leakage for n263 as shown in the table: Proposal 18: Use the PC1 and PC3 inband emissions as in the table above. Observations: Observation : TR example 1 phase noise mask is not suitable for development of the EVM specification. Observation: Ex2 phase noise mask is not suitable for development of EVM spec. Observation : R4-2010176 phase noise mask is not suitable for development of EVM spec. Observation : The proposed PN mask provides good performance for 64 QAM. Observation: 60 GHz UE will tend to operate at power levels higher than the min EVM power limits. |
| R4-2212372 | Discussion on minimum UE EIRP scaling for FR2-2 | Apple | Observation: Due to the short time frame in which the CR had to be crafted during the meeting an oversight was introduced for minimum UE EIRP requirements. The UE EIRP levels for 100MHz CBW in Table 6.4.2.1-3a are 3dB tighter compared to the ones listed in Table 6.4.2.1-3 for FR2-1. It is not logical that a UE in FR2-2 range can perform better than in FR2-1 range as phase noise contribution tends to increase with higher frequencies. If any a relaxation would be required. Proposal: Correct the tentative FR2-1 minimum UE EIRP values for 100MHz by replacing those with the values used for FR2-1 and follow the agreed scaling pattern from CR R4-2211196. The changes have been implemented in the companion CR R4-2212373. |
| R4-2213369 | On remaining RF requirements on n263 | Huawei, HiSilicon | Proposal 3: The min EVM power for 100MHz reuses values from FR2-1 and is not scaled. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### EVM PTRS processing

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 10: The DMRS based channel estimate shall utilize CPE-corrected DMRS symbols

Proposal 11: The PTRS extraction and correction stage is used as the final refinement of the received signal.

Proposal 12: For CP-OFDM, all non-DMRS symbols in a slot must be equipped with PTRS, and frequency density of PTRS tones maximized.

Proposal 13a: For DFT-s-OFDM, PTRS is specified with 4 symbols per group, and the groups are configured in a ‘head and tail’ configuration.

Proposal 13b: For DFT-s-OFDM, the number of PTRS groups is maximised so the ratio of PUSCH symbols to PTRS symbols stays at 1 or higher.

- Recommended WF

  - Agree with all the proposals

Agreement:

Proposal 10: The DMRS based channel estimate shall utilize CPE-corrected DMRS symbols

Proposal 11: The PTRS extraction and correction stage is used as the final refinement of the received signal.

Proposal 12: For CP-OFDM, all non-DMRS symbols in a slot must be equipped with PTRS, and frequency density of PTRS tones maximized.

Proposal 13a: For DFT-s-OFDM, PTRS is specified with 4 symbols per group, and the groups are configured in a ‘head and tail’ configuration.

Proposal 13b: For DFT-s-OFDM, the number of PTRS groups is maximised so the ratio of PUSCH symbols to PTRS symbols stays at 1 or higher.

FFS:

- PTRS configuration during the test will be based on UE declaration.

### EVM compliance power levels

Sub-topic description

Open issues and candidate options before e-meeting:

CCBW = 100 MHz Power class 1 and 2

- Proposals

Proposal 1: EVM compliance levels for FR2-2 CBW=100MHz in PC2 and PC1 are the same as FR2-1

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1.

CCBW = 100 MHz Power class 3

- Proposals

Proposal 1: EVM compliance levels for FR2-2 CBW=100MHz in PC3 is the same as FR2-1

PC3 FR2-2 proposal

| Parameter | Unit | 100 MHz |
| --- | --- | --- |
| UE EIRP | dBm | [ -13] |
| UE EIRP for UL 16 QAM | dBm | [ -10] |
| UE EIRP for UL 64 QAM | dBm | [ -6] |

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1

CCBW >= 400 MHz Power class 3

- Proposals

Proposals in R4-2111628 and R4-2212372 differ by 3 dB

- Recommended WF

  - Discuss between the two proposals

Discussions:

Moderator: wonder where 3dB difference comes from in Apple paper.

Apple: the intention is to correct the values. We start with FR2-1 range.

CCBW >= 400 MHz, Power class 1 and 2

Proposal in R4-2111628

- Recommended WF

  - Agree the proposal for PC1 and PC2 for CBW >= 400 MHz

Discussions:

LGE: These numbers are aligned with ours.

Agreement:

- The numbers in the table of the proposal are agreeable, but further checking is needed.

- Recommended WF

  - Discuss during round 1

### Carrier leakage for PC1 and PC3

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: PC1 carrier leakage for n263 as shown in the table:

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 13.4 dBm | -25 |
| 0.4 dBm ≤ EIRP ≤ 13.4 dBm | -20 |

Proposal 2: PC3 carrier leakage for n263 as shown in the table:

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > -1.9 dBm | -25 |
| -14.9dBm ≤ EIRP ≤ -1.9 dBm | -20 |

- Recommended WF

  - Agree with the proposals

Agreement:

- Agree proposal 1 and proposal 2.

### Inband emissions for PC1,PC2 and PC3

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Use the PC1 and PC3 inband emissions as in the table above. (R4-2211628)

Table 6.4.2.3.2-1: Requirements for in-band emissions for power class 1

| Parameter description | Unit | Limit (NOTE 1) |  |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  |  | Any non-allocated (NOTE 2) |
|  |  |  | Output power for FR2-1 | Output Power for FR2-2 |  |
| IQ Image | dB | -25 | > 27 dBm | > 23.4 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | ≤ 27 dBm | ≤ 23.4 dBm |  |
| Carrier leakage | dBc | -25 | > 17 dBm | > 13.4 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | 4 dBm ≤ Output power ≤ 17 dBm | 0.4 dBm ≤ Output power ≤ 13.4 dBm |  |

Table 6.4.2.3.4-1: Requirements for in-band emissions for power class 3

| Parameter description | Unit | Limit (NOTE 1) |  |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  |  | Any non-allocated (NOTE 2) |
|  |  |  | Output power for FR2-1 | Output Power for FR2-2 |  |
| IQ Image | dB | -25 | > 10 dBm | > 8.1 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | ≤ 10 dBm | ≤ 8.1 dBm |  |
| Carrier leakage | dBc | -25 | > 0 dBm | > -1.9dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm | -14.9 dBm ≤ Output power ≤ -1.9 dBm |  |

Proposal 2 on formatting of PC2 agreed inband emissions (from LGE comment in thread)

- Recommended WF

  - WF #1 - Agree with the proposal 1

  - WF #2  - agree with proposal 2 using modified table title with the “in FR2-1 and FR2-2” removed

Agreement:

  - Agree with the proposal 1

  - Agree with proposal 2 using modified table title with the “in FR2-1 and FR2-2” removed

## Companies views’ collection for 1st round

### Open issues

EVM PTRS processing

| Company | Comments |
| --- | --- |
| QCOM | Agree with the recommended WF. How to capture the PTRS processing in the spec is FFS. |
| Rohde & Schwarz | Thanks Qualcomm for the proposals. Seems fine to us. |
| LGE | We are OK with WF. |
| Anritsu | We agree with the WF |

EVM compliance power levels

| Company | Comments |
| --- | --- |
| QCOM | Agree Option 1 which is scaled from n262, the closest FR2-1 band. Option 2 does not account for the fac that both noise BW and frequency will affect the thermal noise floor and EVM. |
| Murata | We support option 1. We have similar view with Qualcomm. In addition, we can partially agree with option 3 only in 100 MHz CBW, because EIRP value is fixed regardless of CBW up to 400MHz in FR2-1(R4-1710102). In this case Table 6.4.2.1-3a become bellow. |
| LGE | To follow the same approach for PC2 (as for PC3) we propose to add following table 6.4.2.1-3b for PC2. |
| Nokia | We could accept the compromise given by Murata of scaling only =>400MHz CBW for PC3 and the corresponding proposal for PC2 by LGE. |
| Apple | Option 1: Applying the scaling to all power classes is a good improvement for FR2-2. It should be considered that CBW below 400MHz are not specified below FR2-1 minimum UE EIRP levels |
| HW | Option 1 is acceptable for BW >= 400MHz. Option 2 could be followed for 100MHz. |

Carrier leakage for PC1 and PC3

| Company | Comments |
| --- | --- |
| QCOM | Agree with the WF. The method of scaling from minimum peak EIRP is reasonable. |
| Nokia | We are okay with the proposed WF |

Inband emissions for PC1 and PC3

| Company | Comments |
| --- | --- |
| QCOM | Agree with the WF. |
| LGE | - Inband emissions for PC2 already included into TS38.101-2 based on CR in last meeting, but it's proposed that section 6.4.2.3.3 is aligned with proposal made for PC1 and PC3. Proposed modification - Merge Table 6.4.2.3.3-1 and Table 6.4.2.3.3-2 into one Table 6.4.2.3.3-1 as shown below and delete Table 6.4.2.3.3-2 - "Intro" modified: The average of the in-band emission measurement over 10 sub-frames shall not exceed the values specified in Table 6.4.2.3.3-1 and Table 6.4.2.3.3-2 for power class 2. Example: |
| Nokia | We are okay with the proposed WF and can also see the merits of the LGE proposal. |

## Summary for 1st round

## Discussion on 2nd round

In GTW the discussion of whether the EVM test would allow UE to communicate it’s preference indicating whether or not it wouldprefer PTRS to be configured. RAN1/RAN2 have specifically instituted signalling (PTRS-DensityRecommendationUL) to allow the UE to convey its preferred PTRS configuration to accommodate indication.

Option1: EVM test uses PTRS-DensityRecommendationUL

Option 2: EVM test does not use PTRS-DensityRecommendationUL

| Company | Comments |
| --- | --- |
| QCOM | Our preference is option 1, however we think some more study of the IE and how it maps to SCS and BW and mod order is needed. |
| Company B |  |

Recommended WF: TBA

# Topic : MPR and A-MPR

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposal |
| --- | --- | --- | --- |
| R4-2211628 | 60GHz UE TX | Qualcomm Inc | Proposals: Proposal 1: RAN4 adopt the PC1 MPRWT values in the tables and use the same MPRNARROW definition and values as FR2-1. Proposal 2: RAN4 adopt the PC3 MPRWT values in the tables and use the same MPRNARROW definition and values as FR2-1. Proposal 3: No A-MPR requirement needed for the EN 303753 emissions mask. |
| R4-2213220 | On UE Tx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Proposal 3: Agree there is no need for further MPR for FR2-2 as compared to FR2-1 for PC3 UEs also for 800, 1600 and 2000 MHz CBW. Proposal 4: Agree TP#2 below to 38.101-2 removing the square brackets and X/Y values. |
| R4-2213466 | CR on FR2-2 PC3 MPR | LG Electronics Finland |  |
| R4-2213573 | Discussion on MPR values for FR2-2 PC3 | LG Electronics Finland | Proposal 1: Consider the following MPR delta for CBW of 800MHz, 1600MHz and 200MHz - X1 = 1.0, Y1 = 1.0 for 800MHz - X2 = 2.0, Y2 = 2.5 for 1600MHz - X3 = 2.0, Y3 = 2.5 for 2000MHz |
| R4-2213744 | UE Tx requirements for band n263 | Apple | Proposal: Parameterized MPR tables were implemented for FR2-2 with values left for further discussion. It is proposed to implement the proposed values from last meeting X1=[1.0], X2=[2.0], X3=[2.0] dB, Y1=[1.5], Y2=[2.5] and Y3=[2.5] dB. If not agreeable use those values as starting point for further discussion. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### MPR

Sub-topic description

Open issues and candidate options before e-meeting:

Power class 1

- Proposals

Proposal 1: RAN4 adopt the PC1 MPRWT values in the tables and use the same MPRNARROW definition and values as FR2-1. (R4-2211628)

- Table 6.2.2.1-3 MPRWT for power class 1, BWchannel = 100 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 100 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.5 | 0.0 | ≤ 3.5 |
|  | QPSK | ≤ 6.5 | 0.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 7.0 | ≤ 2.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 1.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |

Table 6.2.2.1-4 MPRWT for power class 1, BWchannel >= 400 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 400, 800, 1600, 2000 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6.0 | ≤ 1.0 | ≤ 3.5 |
|  | QPSK | ≤ 6.0 | ≤ 1.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 1.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

- Recommended WF

  - Agree proposal 1. Note the 16QAM and 64QAM values are valid only if PTRS proposal is agreed as the values will change without PTRS.

Discussions:

Huawei: need further checking.

Nokia: there is significant jump of 64QAM over other modulations.

Qualcomm: to Nokia, 64QAM values that Qualcomm provided are not based on phase noise achievable and we base on the simulation. The phase noise mask is even cleaner. PTRS compensation is conducted to improve the MPR and achievable phase noise.

Ericsson: to look at the implementation of values. We looks at one example of proposal 3 of 16QAM DFT. In practice UE may be better. We recognize the difficulty but we need consider the usefulness of these numbers. We provided the comments and encouraged companies to check.

Power class 3 100 MHz

Option 1: Numbers in the “PROP #1” column assuming QCOM PTRS processing is agreed

Option 2: PROP2 column

| Modulation |  | PC3 MPRWT, BWchannel = 100 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 |  | Edge RB allocations |  |
|  |  | PROP #1 | PROP #2 | PROP #1 | PROP #2 |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 0 | ≤ 0.5 | 2 |
|  | QPSK | 0.0 | 0 | ≤ 0.5 | 2 |
|  | 16 QAM | ≤ 3.0 | 3 | ≤ 2.5 | 3.5 |
|  | 64 QAM | ≤ 8.5 | 5 | ≤ 8.5 | 5.5 |
| CP-OFDM | QPSK | ≤ 1.5 | 3.5 | ≤ 1.5 | 4 |
|  | 16 QAM | ≤ 4.0 | 5 | ≤ 4.0 | 5 |
|  | 64 QAM | ≤ 10.0 | 7.5 | ≤ 10.0 | 7.5 |

- Recommended WF

  - Discuss option 1 and option 2

Power class 3 > 100 MHz

Proposal 1 option A: MPR is the same for 400, 800, 1600, and 200 MHz and uses the PROP#1 numbers

Proposal 1 option B: MPR is the same for 400, 800, 1600, and 200 MHz and uses the PROP#2 numbers

| Modulation |  | PC3 MPRWT, BWchannel >= 400 MHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 |  | Edge RB allocations |  |
|  |  | PROP #1 | PROP #2 | PROP #1 | PROP #2 |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 0 | ≤ 1.0 | 3 |
|  | QPSK | 1.0 | 0 | ≤ 1.0 | 3 |
|  | 16 QAM | ≤ 3.5 | 4.5 | ≤ 3.0 | 4.5 |
|  | 64 QAM | ≤ 9.5 | 6.5 | ≤ 9.0 | 6.5 |
| CP-OFDM | QPSK | ≤ 2.0 | 5 | ≤ 2.0 | 5 |
|  | 16 QAM | ≤ 4.0 | 6.5 | ≤ 4.0 | 6.5 |
|  | 64 QAM | ≤ 10.0 | 9 | ≤ 10.0 | 9 |

Proposal 2: Consider the following MPR delta for CBW of 800MHz, 1600MHz and 200MHz

  - X1 = 1.0, Y1 = 1.0 for 800MHz

  - X2 = 2.0, Y2 = 2.5 for 1600MHz

  - X3 = 2.0, Y3 = 2.5 for 2000MHz

Table 6.2.2.3-3 MPRWT for power class 3, BWchannel = 800 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 800 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 4.0 |
|  | QPSK | 0.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 5.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 7.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 |

Table 6.2.2.3-4 MPRWT for power class 3, BWchannel = 1600 and 2000 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 1600 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 5.0 |
|  | QPSK | 0.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 7.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 9.0 |
|  | 64 QAM | ≤ 11.5 | ≤ 11.5 |

Proposal 3: Same as proposal 3 except the 800 MHz table is slightly different.

X1=[1.0], X2=[2.0], X3=[2.0] dB, Y1=[1.5], Y2=[2.5] and Y3=[2.5] dB.

Table 6.2.2.3-3 MPRWT for power class 3, BWchannel = 800 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 800 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 4.0 |
|  | QPSK | 0.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 6.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 8.0 |
|  | 64 QAM | ≤ 10.5 | ≤ 10..5 |

- Recommended WF

  - For PC3 > 100 MHz Discuss between proposal 1, 2, and 3

### A-MPR for EN 303753

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: No A-MPR requirement needed for the EN 303753 emissions mask.

- Recommended WF

  - No A-MPR needed for EN 303753

Agreement:

- No A-MPR requirement needed for the EN 303753 emissions mask.

## Companies views’ collection for 1st round

### Open issues

QPSK and BPSK MPR PC1

| Company | Comments |
| --- | --- |
| QCOM | Agree QPSK and BPSK numbers from Proposal 1 |
| HW | Proposal 5 or 6 could be considered. |

16 QAM and 64QAM MPR PC1

| Company | Comments |
| --- | --- |
| QCOM | Proposal 1 16 and 64 QAM numbers only if our PTRS proposal is agreed |
| HW | Proposal 5 or 6 could be considered. |

QPSK and BPSK MPR PC3

| Company | Comments |
| --- | --- |
| QCOM | Agree QPSK and BPSK numbers from Proposal 2 |
| LGE | Agree QPSK and BPSK numbers according to Proposal 5 (R4-2213573) |
| Nokia | We prefer proposal 4. We however are okay to further discuss proposal 2. In specific we believe the 1dB addition for Inner RB allocations, as compared to FR2-1, for CBW => 400MHz is excessive. Our assumption is that no additional MPR is needed, for the remaining values we are okay. |
| Apple | Proposal 6 for QPSK and BPSK. |
| HW | Proposal 5 or 6 could be considered. |

16 QAM and 64QAM MPR PC3

| Company | Comments |
| --- | --- |
| QCOM | Proposal 2 16 and 64 QAM numbers only if our PTRS proposal is agreed |
| LGE | We are OK with proposed PTRS but support proposal 5 values (R4-2213573) |
| Nokia | We are okay with proposal 2 for 16 QAM but we believe the 3dB addition, as compared to FR2-1, for all CBWs is excessive. Our assumption is that no additional MPR is needed, i.e. proposal 4. |
| Apple | Proposal 6 could be scaled dependent on gain from PTRS |
| HW | Proposal 5 or 6 could be considered. |
| Ericsson | The PASS/FAIL limit may be quite low. For PC3 and 16QAM, DFT-s-OFDM and 400 MHz for example, 7.6 (PC3) – 4.5 (MPR) – T(4.5) – TT = -1.9 dBm – TT. This PASS/FAIL is not too fat from the proposed minimum output power level -13 dBm. Actual performance may be better. Now, MPR = 0 dB for QPSK (MOP = 7.6 dBm) and CA should be available for increased throughput. Put the MPR for 16QAM/64QAM in square brackets for now? |

A-MPR for EN 303753

| Company | Comments |
| --- | --- |
| QCOM | Agree with recommended WF. |
| Nokia | We agree with the proposed WF |
| HW | We agree with the proposed WF. |

## Summary for 1st round

## Discussion on 2nd round

MPR power class 1

In the MPR power class 1 Huawei wanted time to check and Nokia and Ericsson had some comments.

Further company comments or updated understanding on proposal 1 here

| Company | Comments |
| --- | --- |
| QCOM | No other companies brought in any work on this. To Huawei you want time to further check.. To Ericsson and Nokia the 64QAM MPR is largely driven by EVM and phase noise with PTRS. The EVM becomes more limiting in the 64QAM condition with PN mask. We would be OK to compromise and agree proposal 1 with []. |
| Company B |  |

Recommended WF: TBA

MPR Power class 3 100 MHz

Two options are proposed. We did not get a chance to discuss in GTW.

| Company | Comments |
| --- | --- |
| QCOM | The option2 64QAM numbers do not reflect the effect of increased phase noise and the EVM impact. These numbers are some estimate howerver our option 1 was based on circuit sim, PTRS simulation, and simulating all waveforms which is a more comprehensive approach. Option 2 is not acceptable. We prefer option 1. |
| Company B |  |

Recommended WF: TBA

MPR Power class 3 > 100 MHz

We can have further discussion or comment on proposals 1, 2, and 3.

| Company | Comments |
| --- | --- |
| QCOM | Our proposal, proposal 1 option A. As we described above our method was comprehensive. The 64 QAM numbers for prop 1 option B are too small. |
| Company B |  |

Recommended WF: TBA

# Topic : Other TX power related issues

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposal |
| --- | --- | --- | --- |
| R4-2211626 | Draft CR to 38.101-2 on band n263 Tx aspects | Qualcomm Inc |  |
| R4-2211628 | 60GHz UE TX | Qualcomm Inc | Proposals: Proposal 7: PC1 Pmin to be 4 dBm. PC2 and PC3 Pmin to be -13 dBm as shown in the tables |
| R4-2211950 | Multi-band relaxation for FR2-2 handheld UE | Murata Manufacturing Co Ltd. | Observation 1: We could design feasible multi band antenna which covers 28 GHz range and 60 GHz range by simulation. Observation 2: From the simulation result, the degradation value of peak gain is 0.2 dB and that of spherical coverage is 0.3 dB in the 60 GHz range. Proposal 1: Fix the MBR value of band n263 to 1.0 dB. |
| R4-2212119 | Open issues for UE Tx requirements in FR2-2 | Intel Corporation | Note on max TRP Observation 1: The 27 dBm value listed for band n263 in Table 6.2.1.3-2 is a conducted limit that can be verified by max TRP ≤ 27dBm when suitable methods to measure maximum power level at antenna port or ports are not available. Therefore, we should consider whether a clarifying note is needed in the table. Multi-band relaxation Proposal 1: Remove the brackets on the multi-band relaxation factors (∆MBP,n and ∆MBS,n) of band n263 and confirm both values are 1.0 dB. |
| R4-2213369 | On remaining RF requirements on n263 | Huawei, HiSilicon | Proposal 1: The UL gap for Tx power measurement doesn’t apply to FR2-2 in Rel-17. Proposal 2: Reuse FR2-1 requirements for minimum output power and OFF power. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### PC3 max TRP

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Observation 1: The 27 dBm value listed for band n263 in Table 6.2.1.3-2 is a conducted limit that can be verified by max TRP ≤ 27dBm when suitable methods to measure maximum power level at antenna port or ports are not available. Therefore, we should consider whether a clarifying note is needed in the table:

- Recommended WF

  - Discuss in round 1

### UL gap for TX power management

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: The UL gap for Tx power measurement doesn’t apply to FR2-2 in Rel-17.

- Recommended WF

  - Discuss in round 1

Discussions:

Huawei: uplink gap for power measurement is based on FR2-1. We do not think the requirements can be applied for FR2-2.

Apple: We believe that UL gap helps UE handles MPE. I think the basic feature should be equally applicable to FR2-2.

Huawei: we does not object it but we want to check the requirements.

Apple: What do you mean by saying any particular action?

Huawei: the whole discussion of UL gap for requirement is based on the assumption of FR2-1. For example the measurement period and duty cycle. We have not discussed the details for FR2-2.

Apple: FR2-2 is introduce the new band. UL gap is a general feature.

### Multi-band relaxation

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Remove the brackets on the multi-band relaxation factors (∆MBP,n and ∆MBS,n) of band n263 and confirm both values are 1.0 dB.

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1.

### Pmin

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: PC1 Pmin to be 4 dBm. PC2 and PC3 Pmin to be -13 dBm as shown in the tables

- Recommended WF

  - PC1 Pmin to be 4 dBm. PC2 and PC3 Pmin to be -13 dBm

Agreement:

- PC1 Pmin to be 4 dBm. PC2 and PC3 Pmin to be -13 dBm

### TX OFF power

Sub-topic description:Transmit off power

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Reuse FR2-1 requirements for minimum output power and OFF power, specifically remove the [] from this table

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1.

## Companies views’ collection for 1st round

### Open issues

PC3 max TRP

| Company | Comments |
| --- | --- |
| HW | Suggest to remove the 27dBm requirement considering there is no feasible method to measure conducted limit in FR2 so far. |
| Intel | We think including a clarifying note is likely ok |

UL gap for TX power management

| Company | Comments |
| --- | --- |
| QCOM | We do not have a strong opinion on this topic. In general, we don’t think the UL gap is necessary however we interested to hear other companies views on the necessity of the gap. |

Multi-band relaxation

| Company | Comments |
| --- | --- |
| QCOM | We are OK with the WF. |
| Murata | We are OK with the WF. |
| Nokia | We are OK with the WF |
| Apple | Ok with WF |
| HW | We are OK with the WF |
| Intel | We support the recommended WF |

Pmin

| Company | Comments |
| --- | --- |
| QCOM | We agree with the WF |
| HW | We are OK with the WF |

TX OFF power

| Company | Comments |
| --- | --- |
| QCOM | We agree with the WF. |
| Nokia | We agree with the WF |
| HW | We are OK with the WF |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213366 Draft CR for n263 RF Tx requirements | QCOM: Some alignment needed on PC3 EVM numbers as there are multiple proposals . The note on ACLR is a RAN5 issue and should be in our spec. |
|  | Regarding ACLR, maybe an LS to RAN5 is needed to inform them about the RAN4 decision. |
|  | Moderator: Just some additional information, GTW discussion was LS not needed. |
| R4-2212278 | QCOM: A number of companies have pointed out this editorial error and this CR content is also in other CRs. The correction should be implemented. |
| R4-2212373 CR for TS 38.101-2: Correcting oversight with UE EIRP CBW scaling for FR2-2 | QCOM: Our numbers differ. More discussion is needed. |
|  | Company B |
| R4-2211626 Draft CR to 38.101-2 on band n263 Tx aspects | QCOM This is our CR and we think it is the most comprehensive and likely a good candidate to start with as we merge agreements from the meeting. |
|  | LGE: We are OK to use this as starting point. Need to align on some numbers and also align+include PC2 (see our comments in earlier sections) |
|  | Nokia: We are fine to use this as a starting point but would to discuss some values further. |
| R4-2213466 CR on FR2-2 PC3 MPR | QCOM: Our numbers differ. Some discussion needed. |
|  | Company B |

## Summary for 1st round

### Open issues

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2212278 | not pursued however the same proposal is made in other CRs and will be implemented |

## Discussion on 2nd round

PC3 max TRP

Further comments and discussion on proposal 1

| Company | Comments |
| --- | --- |
| Company A |  |

Proposed WF: TBA

UL gap for TX power management

We did not conclude on proposal 1 for the UL gap. Further discussion is welcome

| Company | Comments |
| --- | --- |
| Company A |  |

Proposed WF: TBA

# Topic : CA unwanted emissions and signal quality

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-22xxxxx | Company A | Proposal 1: Observation 1: |

| T-doc number | Title | Company | Proposal |
| --- | --- | --- | --- |
| R4-2211628 | 60GHz UE TX | Qualcomm Inc | Proposals: Proposal 20: For CA carrier leakage use the PC1 and PC3 values in the tables. For PC2 use the same value as in FR2-1 since the min peak EIRP values are nearly the same. Proposal 21: Re-use the FR2-1 CA inband emissions method for PC1 and PC3 CA with the same output power values we are proposing for FR2-2 single carrier. Proposal 22: Re-use the FR2-2 single carrier 99% OBW for CA Proposal 23: Re-use the FR2-1 CA SEM requirements for FR2-2. Proposal 24: Use the FR2-2 single carrier 15 dB ACLR value for CA |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Carrier leakage for power classes 1 and 3 in CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

- Proposal 1: For CA carrier leakage use the PC1 and PC3 values in the tables.

- Proposal 2: For n263 PC2 use the same value as in FR2-1 since the min peak EIRP values are nearly the same.

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 13.4 dBm | -25 |
| 0.4 dBm ≤ EIRP ≤ 13.4 dBm | -20 |

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > -1.9 dBm | -25 |
| -14.9dBm ≤ EIRP ≤ -1.9 dBm | -20 |

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1 and proposal 2.

### Inband emissions for power classes 1 and 3 in CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Re-use the FR2-1 CA inband emissions method for PC1 and PC3 CA with the same output power values we are proposing for FR2-2 single carrier. (R4-2211628)

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1.

### SEM for CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Re-use the FR2-1 CA SEM requirements for FR2-2.

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1.

### ACLR for CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Use the FR2-2 single carrier 15 dB ACLR value for CA

- Recommended WF

  - Agree proposal 1

Discussion:

Huawei: in previous RAN4 agreement, OBW is more stringent than ACLR. Should we send RAN5 to indicate the RAN4 agreement?

Nokia: We agree with moderator. This is RAN5 discussion. To Huawei, we do not believe it is necessary. Huawei can raised it in RAN5.

Agreement:

- Agree proposal 1

- The common understanding in RAN4 is that OBW requirement is more stringent than ACLR for FR2-2.

### OBW for CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Re-use the FR2-2 single carrier 99% OBW for CA

- Recommended WF

  - Agree proposal 1

Agreement:

  - Agree proposal 1

## Companies views’ collection for 1st round

### Open issues

Carrier leakage for power classes 1 and 3 in CA

| Company | Comments |
| --- | --- |
| QCOM | Agree with the proposed WF |
| Nokia | We are OK with the WF |

Inband emissions for power classes 1 and 3 in CA

| Company | Comments |
| --- | --- |
| QCOM | Agree with the proposed WF |
| Nokia | We are OK with the WF |

SEM for CA

| Company | Comments |
| --- | --- |
| XXX | Agree with the proposed WF |
| Nokia | We are OK with the WF |

ACLR for CA

| Company | Comments |
| --- | --- |
| XXX | Agree with the proposed WF |
| Nokia | We are OK with the WF |

OBW for CA

| Company | Comments |
| --- | --- |
| XXX | Agree with the proposed WF |
| Nokia | We are OK with the WF |

# Topic : CA output power, MPR,  and A-MPR

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-22xxxxx | Company A | Proposal 1: Observation 1: |

| T-doc number | Title | Company | Proposal |
| --- | --- | --- | --- |
| R4-2211628 | 60GHz UE TX | Qualcomm Inc | Proposals: Proposal 4: FR2-2 PC1 and PC3 power classes for CA are the same as for FR2-2 single carrier. Note this is the same approach as in FR2-1. Proposal 5: Adopt the CA MPR tables for PC1 and PC3. Proposal 6: No CA A-MPR needed for the EN 303753 emissions mask. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Maximum output power for CA

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: FR2-2 PC1 and PC3 power classes for CA are the same as for FR2-2 single carrier. Note this is the same approach as in FR2-1.

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1.

### MPR for CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Adopt the CA MPR tables for PC1 and PC3. (R4-2211628)

Table TBD Maximum power reduction (MPRWT_C_CA) for FR2-2 UE power class 1

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 7.0 | ≤ 5.0 | ≤ 2.0 | ≤ 2.0 |
| QPSK | ≤ 8.0 | ≤ 6.0 | ≤ 3.0 | ≤ 3.0 |
| 16 QAM | ≤ 8.0 | ≤ 6.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

Table TBD Maximum power reduction (MPRWT_C_CA) for FR2-2 UE power class 3

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 1.0 | ≤ 1.0 | ≤ 1.0 | ≤ 1.0 |
| QPSK | ≤ 2.0 | ≤ 2.0 | ≤ 2.0 | ≤ 2.0 |
| 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

- Recommended WF

  - Agree proposal 1

### A-MPR for CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: No CA A-MPR needed for the EN 303753 emissions mask.

- Recommended WF

  - Agree proposal 1

Agreement:

  - Agree proposal 1

## Companies views’ collection for 1st round

### Open issues

Max output power for CA

| Company | Comments |
| --- | --- |
| QCOM | Agree with the recommended WF |
| Nokia | We are OK with the WF |

MPR for CA

| Company | Comments |
| --- | --- |
| QCOM | Agree with the recommended WF |
| Nokia | We think some of the proposed MPR values are excessive as also discussed under Topic 2. Let’s focus on single carrier MPR first. |
| Apple | The decision on CA MPR depends on Issue 2.2.1 (MPR). If it would be concluded that more power backoff is required for MPR it might also be required for CA MPR. |

A-MPR for CA

| Company | Comments |
| --- | --- |
| QCOM | Agree with the recommended WF |
| Nokia | We are OK with the WF |

## Summary for 1st round

## Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

MPR for CA

Proposal 1 was made in the round1 section above.

| Company | Comments |
| --- | --- |
| QCOM | Proposal 1 is our proposal. We would be ok to agree proposl 1 with values in []. |
| Company B |  |

Recommended WF: TBA

# Topic: Other TX or general Issues

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposal |
| --- | --- | --- | --- |
| R4-2211628 | 60GHz UE TX | Qualcomm Inc | Proposals: Proposal 8: The transient period from FR2-1 is based on the capability of the UE to configure the transmitter and receiver. The same capability will exist in FR2-2. Use the same 5usec for FR2-2. Proposal 9: PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown. Proposal 25: All FR2-2 UEs shall support beamCorrespondenceWithoutUL-BeamSweeping. |
| R4-2212119 | Open issues for UE Tx requirements in FR2-2 | Intel Corporation | ON/ON transient period Observation 2:  Opt. 1: No gNB scheduling optimizations for ON/ON transient period  Using 5 µS ON/ON transient period leads to high throughput reduction due to corruption of the PUSCH data symbols. Up to 50% and 12% throughput loss can be expected for bundling size 2 and 8, respectively.  An improved ON/ON transient period faster than 5 µS is required to support at least full MCS for 16 QAM modulation.  Opt. 2: Optimized gNB scheduling for ON/ON transient period  Using 5 µS ON/ON transient period leads to high throughput loss even with optimized gNB scheduling without corrupted symbols on UE side. Up to 25% and 6% throughput loss can be expected for bundling size 2 and 8, respectively.  An improved ON/ON transient period faster than 5 µS allows better throughput performance with almost 20%, 10% and 5% improvement for scenarios with bundling size 2, 4 and 8, respectively. Proposal 2: Introduce 2 µS improved ON/ON transient period as optional UE capabilities for 480 and 960 kHz SCS. |
| R4-2213220 | On UE Tx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Proposal 2: Agree TP#1 above to 38.101-2 removing the square brackets in Table 5.3.3-1 Proposal 5: Use a UE beam direction switching time of 59 ns. |
| R4-2213232 | SSB side conditions for band n263 | Apple | Proposal 1: RAN4 shall apply the minimum SSB and minimum CSI-RS as provided in Table 1 and Table 2 for band n263. (R4-2213232) |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Minimum guard band (Table 5.3.3-1)

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Agree TP#1 above to 38.101-2 removing the square brackets in Table 5.3.3-1

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A | N/A | N/A | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 | N/A | N/A | N/A |
| 480 | N/A | N/A | N/A | 9680 | 42640 | 85520 | N/A |
| 960 | N/A | N/A | N/A | 9440 | 42400 | 85280 | 147040 |

- Recommended WF

  - Agree proposal 1

Agreement:

  - Agree proposal 1

### PRACH time mask

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown. (R4-2211628)

Table 6.3.3.4-1: PRACH ON power measurement period

| Format | SCS | Measurement period | Note |
| --- | --- | --- | --- |
| A1 | 60 kHz | 0.035677 ms |  |
|  | 120 kHz | 0.017839 ms |  |
|  | 480 kHz | 0.004460 ms |  |
|  | 960 kHz | 0.002230 ms |  |
| A2 | 60 kHz | 0.071354 ms |  |
|  | 120 kHz | 0.035677 ms |  |
|  | 480 kHz | 0.008919 ms |  |
|  | 960 kHz | 0.004460 ms |  |
| A3 | 60 kHz | 0.107031 ms |  |
|  | 120 kHz | 0.053516 ms |  |
|  | 480 kHz | 0.013379 ms |  |
|  | 960 kHz | 0.006690 ms |  |
| B1 | 60 kHz | 0.035091 ms |  |
|  | 120 kHz | 0.0175455 ms |  |
|  | 480 kHz | 0.004386 ms |  |
|  | 960 kHz | 0.002193 ms |  |
| B4 | 60 kHz | 0.207617 ms |  |
|  | 120 kHz | 0.103809 ms |  |
|  | 480 kHz | 0.025952 ms |  |
|  | 960 kHz | 0.012976 ms |  |
| A1/B1 | 60 kHz | 0.035677 ms for front X1 occasion 0.035091 ms for last occasion | X1 = [2,5] |
|  | 120 kHz | 0.017839 ms for front X1occasion 0.017546 ms for last occasion |  |
|  | 480 kHz | 0.004460 ms for front X1 occasion 0.004387 ms for last occasion |  |
|  | 960 kHz | 0.017839 ms for front X1occasion 0.017546 ms for last occasion |  |
| A2/B2 | 60 kHz | 0.071354 ms for front X2 occasion 0.069596 ms for last occasion | X2 = [1,2] |
|  | 120 kHz | 0.035677 ms for front X2 occasion 0.034798 ms for last occasion |  |
|  | 480 kHz | 0.008919 ms for front X2 occasion 0.008700 ms for last occasion |  |
|  | 960 kHz | 0.004460 ms for front X2 occasion 0.004350 ms for last occasion |  |
| A3/B3 | 60 kHz | 0.107031 ms for first occasion 0.104101 ms for second occasion |  |
|  | 120 kHz | 0.053515 ms for first occasion 0.052050 ms for second occasion |  |
|  | 480 kHz | 0.013379 ms for first occasion 0.013013 ms for second occasion |  |
|  | 960 kHz | 0.006689 ms for first occasion 0.006506 ms for second occasion |  |
| C0 | 60 kHz | 0.026758 ms |  |
|  | 120 kHz | 0.013379 ms |  |
|  | 480 kHz | 0.003345 ms |  |
|  | 960 kHz | 0.001672 ms |  |
| C2 | 60 kHz | 0.083333 ms |  |
|  | 120 kHz | 0.0416667 ms |  |
|  | 480 kHz | 0.010417 ms |  |
|  | 960 kHz | 0.005208 ms |  |
| NOTE: For PRACH on PRACH occasion start from begin of 0ms or 0.5 ms boundary, the measurement period will plus 0.032552 μs |  |  |  |

- Recommended WF

  - Agree proposal 1

Discussions:

Huawei: I have no concern on the values. The maximum period is only 2us. The off power mask in RAN5. I do not know if 2us is feasible or not for measurement from test perspective.

Qualcomm: we can have discussion with TE vendor.

### Beam correspondence

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: All FR2-2 UEs shall support beamCorrespondenceWithoutUL-BeamSweeping.

Proposal 2:	RAN4 shall apply the minimum SSB and minimum CSI-RS as provided in Table 1 and Table 2 for band n263. (re: 38.101-2 6.6.4.3.1)

Table 1: Conditions for SSB based L1-RSRP measurements for beam correspondence

| Band | Minimum SSB (dBm/SCSSBB) |
| --- | --- |
| n257 | -96.2 |
| n258 | -96.2 |
| n259 | -90.7 |
| n260 | -91.9 |
| n261 | -96.2 |
| n262 | -88.5 |
| n263 | -88.2 |

Table 2: Conditions for CSI-RS based L1-RSRP measurements for beam correspondence

| Band | Minimum CSI-RS (dBm/SCSSBB) |
| --- | --- |
| n257 | -96.2 |
| n258 | -96.2 |
| n259 | -90.7 |
| n260 | -91.9 |
| n261 | -96.2 |
| n262 | -88.5 |
| n263 | -88.2 |

- Recommended WF

  - Discuss during round 1

Discussions:

Qualcomm: prefer proposal 1.

Huawei: I do think we can agree on proposal 2 without proposal 1. Regarding proposal 1, for FR2-2, people have different design for the antenna the frequency range is different from FR2-1. We can keep the capability optional.

Qualcomm: for table 2, it is said CSI-RS based …

Nokia: we prefer option 1. The UL sweeping is agreed in Rel-15. In Rel-17 it is not needed. Proposal 2 needs further discussions.

OPPO: regarding proposal 1, we slightly prefer not to mandate without beam sweeping. In market no device can support 71. The propagation condition would be different from below 71Ghz. And the antenna is different. We would like to be conservative.

Sony: for proposal 1 we echo Qualcomm and Nokia. From antenna, even if we is moving up to higher frequency, there is no fundamental difference.

### ON/ON transient periods

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Options

Option 1: The transient period from FR2-1 is based on the capability of the UE to configure the transmitter and receiver. The same capability will exist in FR2-2. Use the same 5usec for FR2-2.

Option 2: Introduce 2 µS improved ON/ON transient period as optional UE capabilities for 480 and 960 kHz SCS.

- Recommended WF

  - Discuss during round 1

Discussion:

OPPO: we prefer Option 1. For option 2, where is 2us coming from?

Intel: we had it a few meetings ago. We presented the benefit. If companies want to discuss the exact numbers, we are open to discussion. But we want to have it as optional feature.

AT&T: it was discussed for many meetings. The views are diverse. We agreed the benefit and optional features. We would like to introduce it in Rel-17. We would like to consider it in Rel-18 if needed and if we get the alignment in RAN4 to add it into FR2 enhancement.

Nokia: we support AT&T. We should consider it for Rel-18.

Chair=> align companies’ view if it needs be discussed in Rel-18.

### Beam direction only switching time

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Use a UE beam direction switching time of 59 ns.

- Recommended WF

  - Discuss during round 1

Discussions:

Ericsson: 200us requirement is not feasible. It is longer than CP. I wonder if we could consider the compromised value 100ns at least for 480KHz SCS.

Nokia: from many meetings, we know 200us is longer than CP which does not work.

Qualcomm: the UE architecture of FR2-2 is similar as FR2-1. From feasibility perspective, it does not work. We think 200ns is needed for FR2-2.

Huawei: we agree with Qualcom’s view. 200ns is generated based on practical design. 59ns is not feasible.

WF: No further discussion this meeting. Companies are definitely split on the as they have been over many meetings in the past.

## Companies views’ collection for 1st round

### Open issues

Min guard band

| Company | Comments |
| --- | --- |
| QCOM | Agree the recommended WF |
| Vivo | Agree the recommended WF |
| Nokia | We agree with the proposed WF |
| HW | We are OK with the WF |
| Ericsson | We agree with the recommended WF |

PRACH time mask

| Company | Comments |
| --- | --- |
| QCOM | Agree the recommended WF |
| HW | Don’t agree with the proposal. The measurement period is reduced to 2us. Needs TE vendors to confirm the testability. |

Beam correspondence

| Company | Comments |
| --- | --- |
| QCOM | Proposal 1. non-BC is not needed and a waste of UL resources for beam sweeping. |
| Sony | We support Proposal 1. BC with Uplink beam sweeping was introduced at Rel-15 to accommodate early implementation of mmwave devices, which consumes extra uplink resources. We don’t see the need of it after a few years development of mmwave devices. |
| Apple | Proposal 1 doesn’t contradict Proposal 2. Proposal 2 introduces the side conditions for SSB/CSI-RS for band n263. According to the specification, the beam correspondence requirement is fulfilled if the UE satisfies one of the conditions depending on the UE’s beam correspondence capability IE beamCorrespondenceWithoutUL-BeamSweeping. Not defining side conditions for band n263 will preclude UEs that fulfill the UE beam correspondence conditions as listed in TS 38.101-2 in section 6.6.4.1 For example, if UE supports beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 then the UE shall meet the minimum peak EIRP requirement and spherical coverage requirement using the side conditions for SSB. Therefore, the side conditions should be also defined for FR2-2. |
| HW | We don’t agree with Proposal 1. FR2-2 is a new frequency range, which requires different antenna design. The optional capability of BC with UL Sweeping needs to be kept for FR2-2 to avoid restricting the UE implementation. |
| OPPO | Proposal 2 is ok. For proposal 1, slightly prefer to not mandate beamCorrespondenceWithoutUL-BeamSweeping. The reason is that up to now the UEs on the market are focusing on 28G and 39G, and no device has support as high as 71GHz which has different wireless channel conditions and antenna performance is unknown. Therefore, at this stage maybe we can keep a slightly conservative and give UE time to optimize as Rel-15 did. |

ON/ON transient

| Company | Comments |
| --- | --- |
| QCOM | Option 1. |
| vivo | Option 1. |
| Nokia | Option 2 is our preference but given we are in the 2nd extension of this WI we can accept Option 1. |
| Apple | Option 1. |
| HW | Option 1 |
| AT&T | We support Option 2. Having an optional capability for these higher SCS values is important to include in Rel-17. Company views continue to be split between network providers/operators and UE/chipset vendors. Similar optional UE capabilities have been introduced in the past based on the level of support expressed by network providers/operators. If RAN4 goes with Option 1, it should be clear that ON/ON transient period improvements can be considered in Rel-18 NR RF requirements enhancement for frequency range 2 (FR2) Phase 3 WID as companies have expressed in the past that they were willing to consider in Rel-18. |
| Ericsson | Option 2. |
| OPPO | Option 1, and question for clarification where is 2us coming from? |
| Intel | As its proponent, we support Option 2 to define an optional UE capability for improved transient period. Additionally, we also think we should consider ON/ON transient period improvements in Rel-18 if Option 1 is agreed. To OPPO, the 2µs is from Proposal 2 of our contribution (R4-2212119). |

Beam direction only switching time

| Company | Comments |
| --- | --- |
| QCOM | As we have discussed for many meetings 59 ns is not doable be the UE. We don’t agree with the proposal/ |
| vivo | We share the same view with QC. The value of 200 ns can be assumed for UE. |
| Nokia | As we indeed have discussed for many meetings, we think 200ns is excessive. This large delay would result in system performance degradation. |
| Apple | Cannot agree with this proposal of 59 ns. We support 200 ns definition for beam switching time. |
| HW | Share same view as QC and vivo. 200ns is needed. |
| Ericsson | 200 ns is excessive and longer than the CP for 960 kHz. We propose a compromise in the range 100-120 ns, which would be within the CP for 480 kHz. |
| OPPO | 200ns is preferred. |

## Summary for 1st round

## Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

PRACH time mask

Further comments on proposal 1

| Company | Comments |
| --- | --- |
| QCOM | Proposal 1. RAN5 can conclude whether or not a requirement is testable. |
| Company B |  |

Recommended WF: TBA

Beam correspondence

Further comments on proposals 1 and 2

| Company | Comments |
| --- | --- |
| QCOM | Repeating as above we support proposal 1. Right now we are looking at proposal 2 to make sure we understand it. |
| Company B |  |

Recommended WF: TBA

ON/ON transient

Further discussion on whether this should be a topic in rel-18

| Company | Comments |
| --- | --- |
| QCOM | Our view it is up to proponent to bring proposal to the plenary to decide what is in the scope for rel-18. Upon approval then studies to assess any significant gains. |
| Company B |  |

# Topic: RX issues

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposal |
| --- | --- | --- | --- |
| R4-2213369 | On remaining RF requirements on n263 | Huawei, HiSilicon | Proposal 5: Implement the agreed PC1 REFSENS in the specification. Proposal 6: Specify the uplink configuration for band n263 as in Table 2.6-1. Proposal 7: The ACS of band n263 is 21dB. The measurement bandwidth is the same as minimum output power. Proposal 8: The IBB of band n263 is specified for all supported bandwidths. Proposal 9: The IBB requirement of band n263 is the same as ACS requirement |
| R4-2211629 | 60GHz UE RX | Qualcomm Inc | Proposal 1: Use the same values as in FR2-1. For 1600-2000 MHz FR2-2 EIS relaxation dB value as [1.5] Proposal 2: Use the FR2-1 max input for intra-band contiguous requirement for FR2-2. Proposal 3: Agree ACS the Case1 and Case 2 tables based on 21 dB ACS. Proposal4: For CA Agree ACS the Case1 and Case 2 tables changes based on 21 dB ACS. Proposal 5: Agree the IBB values as shown in the table. Proposal 6: For DL CA IBB extend the FR2-1 requirement and method to n263. Pinterferer for n263 to be 19.5 dBm. |
| R4-2213221 | On UE Rx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Proposal 1: Agree ACS requirements as given in TP#1 and TP#2 Proposal 2: Agree IBB requirements as given in TP#3 and TP#4. |
| R4-2213231 | UE Rx requirements for band n263 | Apple | Proposal 1: Define ACS requirement as provided in Table 2, Table 3, and Table 4 for the remaining CBW for band n263. |
| R4-2213368 | Draft CR for n263 RMC | Huawei, HiSilicon |  |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### REFSENS

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Implement the agreed PC1 REFSENS in the specification. (-79 dBm/400 MHz)

Proposal 2: Specify the uplink configuration for band n263 as in Table 2.6-1. (R4-2213369)

Table 2.6-1

| Operating band | NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n262 | 32 | 64 | 128 | 256 | N.A | N.A | N.A | 120 kHz | TDD |
| n263 | N.A | 64 | N.A | 256 | N.A | N.A | N.A | 120 kHz | TDD |
|  | N.A | N.A | N.A | N.A | 120 | 243 | N.A | 480 kHz | TDD |
|  | N.A | N.A | N.A | N.A | N.A | N.A | 144 | 960 kHz | TDD |

Proposal 3: vivo in thread

The NRB number for uplink configuration for band n263 is not the same with what we agreed for.

The numbers for 400M with 480/960k and 800M/1600M with 960k are missing.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A | N/A | N/A | N/A |
| 120 | 32 | 66 | 132 | 264 | N/A | N/A | N/A |
| 4801 | N/A | N/A | N/A | 66 | [124] | [248] | N/A |
| 9601 | N/A | N/A | N/A | 33 | [62] | [124] | 148 |
| Note 1: This SCS is optional in this release of the specification. |  |  |  |  |  |  |  |

- Recommended WF

  - Agree proposal 1 and proposal 2

Discussions:

Huawei: Proposal 2 comes from Huawei. The uplink configuration is not targeting to provide all the combinations of channel bandwidth and SCS. When the REFSEN test is conducted for one SCS, we do not need to do it for other SCS. We are not sure if UE can support 480KHz all the time. So we should add 800 and 1600MHz for 960KHz to ensure all the bandwidths combination can be tested.

Agreement:

- Agree proposal 1.

### EIS relaxation for intraband contiguous CA

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Use the same values as in FR2-1. For 1600-2000 MHz FR2-2 EIS relaxation dB value as [1.5]

Table 7.3A.2.1-1: EIS Relaxation for CA operation by aggregated channel bandwidth

| Aggregated Channel BW 'BWChannel_CA' (MHz) | (dB) |
| --- | --- |
| BWChannel_CA ≤ 800 | 0 |
| 800 < BWChannel_CA ≤ 1200 | 0.5 |
| 1200 < BWChannel_CA ≤ 1600 | 1.0 |
| 1600 < BWChannel_CA ≤ 2000 | [1.5] |

- Recommended WF

  - Agree proposal 1

Agreement:

- Agree proposal 1.

### In-band blocking

Sub-topic description

Open issues and candidate options before e-meeting:

Proposal 1:  Agree the IBB values as shown in the table (R4-2213221 and identical proposal in R4-2211629)

Table 7.6.2-1: In band blocking requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |  | 800 MHz |  | 1600 MHz |  | 2000 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |  | 800 |  | 1600 |  | 2000 |  |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB |  | N.A. |  | N.A. |  | N.A. |  |
| PInterferer for bands n259, n260, n262 | dBm | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB |  | N.A. |  | N.A. |  | N.A. |  |
| PInterferer for band n263 | dBm | N.A. | REFSENS + 33.5 dB | N.A. | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  | REFSENS + 33.5 dB |  |
| FIoffset | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |  | ≤ -1600 & ≥ 1600 NOTE 5 |  | ≤ -3200 & ≥ 3200 |  | ≤ -4000 & ≥ 4000 |  |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |  | FDL_low + 400 to FDL_high - 400 |  | FDL_low + 800 to FDL_high - 800 |  | FDL_low + 1600 to FDL_high - 1600 |  |

- Recommended WF

  - Agree proposal 1

Discussions:

Huawei: to the format, we have similar changes with different format to introduce the requirements in the separate tables.

Agreement:

- Agree the IBB values as shown in the table in proposal 1

  - FFS whether to use one table or separate tables to capture the requriements.

### Maximum input level for Intra-band contiguous CA

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Use the FR2-1 max input for intra-band contiguous requirement for FR2-2.

- Recommended WF

  - Agree proposal 1

Agreement:

  - Agree proposal 1

### Adjacent channel selectivity

Sub-topic description

Open issues and candidate options before e-meeting:

Sub-topic description

- Proposals

The tables below with:

Option 1: using 21 dB for the ACS for all CCBWs.

Option 2: using 21 dB for CCBW <= 400 MHz and 20 dB for > 400 MHz.

Table 7.5-1: Adjacent channel selectivity

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257, n258, n261 | dB | 23 | 23 | 23 | 23 | N.A. | N.A. | N.A. |
| n259, n260, n262 | dB | 22 | 22 | 22 | 22 | N.A. | N.A. | N.A. |
| n263 | dB | N.A. | 21 | N.A. | 21 | 21 or 20 | 21 or 20 | 21 or 20 |

Table 7.5-2: Adjacent channel selectivity test parameters, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS +35.5 dB | REFSENS +35.5 dB | REFSENS +35.5 dB | N.A. | N.A. | N.A. |
| PInterferer for band n259, n260, n262 | dBm | REFSENS + 34.5 dB | REFSENS +34.5 dB | REFSENS +34.5 dB | REFSENS +34.5 dB | N.A. | N.A. | N.A. |
| PInterferer for band n263 | dBm | N.A. | REFSENS +33.5 dB | N.A. | REFSENS +33.5 dB | REFSENS +33.5 dB or 32.5 | REFSENS +33.5 dB or 32.5 | REFSENS +33.5 dB or 32.5 |
| BWInterferer | MHz | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 | 800 / -800 NOTE 3 | 1600 / -1600 NOTE 3 | 2000 / -2000 NOTE 3 |

Table 7.5-3: Adjacent channel selectivity test parameters, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| Ptxbc for bands n257, n258, n261 | dBm | -46.5 | -46.5 | -46.5 | -46.5 | -N.A. | -N.A. | -N.A. |
| Ptxbc for bands n259, n260, n262 | dBm | -45.5 | -45.5 | -45.5 | -45.5 | -N.A. | -N.A. | -N.A. |
| Ptxbc for band n263 | dBm | -N.A. | -44.5 | -N.A. | -44.5 | -44.5 or -43.5 | -44.5 or -43.5 | -44.5 or -43.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 2 | 100 / -100 NOTE 2 | 200 / -200 NOTE 2 | 400 / -400 NOTE 2 | 800 / -800 NOTE 2 | 1600 / -1600 NOTE 2 | 2000 / -2000 NOTE 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern TDD as described in Annex A and set-up according to Annex C. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 3: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. NOTE 4: Ptxbc is the power in the transmission bandwidth configuration |  |  |  |  |  |  |  |  |

- Recommended WF

  - Discuss between options 1 and 2

Discussions:

Apple: 1dB relaxation is needed.

Agreement:

- Option 2: using 21 dB for CCBW <= 400 MHz and 20 dB for > 400 MHz.

### Adjacent channel selectivity for Intra-band contiguous CA

Sub-topic description

Open issues and candidate options before e-meeting:

- Proposals

Proposal1:  For CA Agree ACS the tables below based on 21 dB ACS. This proposal may need modification based on the single carrier ACS discussion above as 20 dB is being proposed for BW > 400 MHz.

Table 7.5A.1-1: Adjacent channel selectivity for intra-band contiguous CA

| Operating band | Units | Adjacent channel selectivity / CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth class |
| n257, n258, n261 | dB | 23 |
| n259, n260, n262 | dB | 22 |
| n263 | dB | 21 |

Table 7.5A.1-2: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 1

| Rx Parameter | Units | All CA bandwidth Classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| PInterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| PInterferer for band n259, n260, n262 | dBm | Aggregated power + 20.5 |
| PInterferer for band n263 | dBm | Aggregated power + 19.5 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | + BWchannel CA / - BWchannel CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern as described in Annex A and set-up according to Annex C. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interfering signal has the same SCS as that of the closest carrier. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. |  |  |

Table 7.5A.1-3: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 2

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n257, n258, n261 | dBm | - 46.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n259, n260, n262 | dBm | - 45.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n263 | dBm | - 44.5 |
| Pinterferer | dBm | - 25 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | + BWchannel CA / - BWchannel CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interfering signal has the same SCS as that of the closest carrier. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.1-2. |  |  |

- Recommended WF

  - Agree proposal 1. If 20 dB ACS is decided for BW> 400 MHz there would need to be modification.

### CA In-band blocking

Sub-topic description

Open issues and candidate options before e-meeting:

Issue

- Proposals

Proposal 1: Agree IBB requirements as given in TP#3 and TP#4. (R4-2213221)

Table 7.6A.2.1-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| Pinterferer for band n260, n262 | dBm | Aggregated power + 20.5 |
| Pinterferer for band n263 | dBm | Aggregated power + 19.5 |

Proposal 2: For DL CA IBB extend the FR2-1 requirement and method to n263. Pinterferer for n263 to be 19.5 dBm. (R4-2211629)

Table 7.6A.2.1-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for bands n257, n258, n261 | dBm | Aggregated power + 21.5 |
| Pinterferer for bands n260, n262 | dBm | Aggregated power + 20.5 |
| Pinterferer for band n263 | dBm | Aggregated power + 19.5 |

- Recommended WF

  - Discuss during round 1

Agreement:

  - Agree proposal 1

## Companies views’ collection for 1st round

### Open issues

REFSENS

| Company | Comments |
| --- | --- |
| QCOM | We are probably OK with proposal 1 but want to check. Proposal 2 is agreeable. |
| vivo | We have comments for Proposal 2. The NRB number for uplink configuration for band n263 is not the same with what we agreed for. The numbers for 400M with 480/960k and 800M/1600M with 960k are missing. Table 5.3.2-1: Maximum transmission bandwidth configuration NRB SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A; N/A; N/A; N/A. 120; 32; 66; 132; 264; N/A; N/A; N/A. 4801; N/A; N/A; N/A; 66; [124]; [248]; N/A. 9601; N/A; N/A; N/A; 33; [62]; [124]; 148. Note 1: This SCS is optional in this release of the specification.. |
| Nokia | We agree with the corrections from Vivo and propose to include without square brackets. |
| HW | To vivo and Nokia: Thanks for the comment. The intention of this table is not to provide all the combinations of BW and SCS. In FR2-1 only 120kHz SCS is specified as it’s the mandatory SCS, and the REFSENS is not expected to be repeatedly verified for every applicable SCS. In FR2-2, if similar principle applies, the uplink configuration for each bandwidth only needs to be specified for single SCS. For 100MHz and 400MHz, 120kHz is picked as that’s mandatory SCS. For 800~2000MHz, the lower applicable SCS is picked. Considering both 480 and 960kHz are optional, maybe the proposal could be revised to add both SCS for >800MHz. For 100MHz and 400MHz, I still believe 120kHz only should be enough. |

EIS relaxation CA

| Company | Comments |
| --- | --- |
| QCOM | We are OK with the proposed WF. |
| Nokia | We are OK with the WF |
| Intel | The recommended WF is ok |

In-band blocking

| Company | Comments |
| --- | --- |
| QCOM | We want to compare the proposals more. We believe they are aligned and consistent with each other but need to check |
| Nokia | We are fine with the proposal from Qualcomm. It is aligned to our but have correctly added 50 and 200 MHz as N/A which were missed on our proposal. |
| Apple | Similar to our proposal for the ACS definition for CBW > 400 MHz, we think 1 dB relaxation should be considered. |
| HW | In our CR R4-2213367, we have introduced a separate table for FR2-2 with applicable IBB requirements identical with QC and Nokia. We prefer this way as this is aligned with many Tx test cases where separate tables are created for FR2-2, and the specification would look clearer. |
| Xiaomi | The proposal of QC is aligned with our proposal in CR R4-2213203. |

Max input level for CA

| Company | Comments |
| --- | --- |
| QCOM | We agree with the recommended WF |
| Nokia | We are okay with the WF |
| Apple | We need time to check. |
| Intel | Recommended WF is agreeable |

ACS

| Company | Comments |
| --- | --- |
| QCOM | We agree with our proposal 1 and 2. |
| Nokia | We are fine with the proposal from Qualcomm. It is aligned to our but have correctly added 50 and 200 MHz as N/A which were missed on our proposal. |
| Apple | Proposal 3 (R4-2213231). ACS level definition for larger CBW > 400 MHz needs to consider the additional limitation for the digital filter. In FR2-1 the single carrier CBW goes up to 400 MHz, but in FR2-2 the filter needs to cover up to 5 times this CBW. The large channel BW for band n263 can contribute to ADC aliasing. Our proposal is to have 1 dB relaxation (20 dB for ACS), since having less interferer to wanted ratio can help to achieve higher SNDR. In addition, the higher dynamic range to handle the increases interference will contribute to the power consumption of the UE. For these reasons, we think that the 1 dB relaxation for larger CBW is reasonable. |
| HW | In our CR R4-2213367, we have introduced a separate table for FR2-2 with applicable ACS requirements identical with QC and Nokia. We prefer this way as this is aligned with many Tx test cases where separate tables are created for FR2-2, and the specification would look clearer. |
| Xiaomi | The proposal of QC is aligned with our proposal in CR R4-2213203. |

ACS for CA

| Company | Comments |
| --- | --- |
| QCOM | We agree with the recommended WF |
| Nokia | We agree with the WF |
| Apple | RAN4 has to find an agreement for the single carrier requirement first before defining CA. |

IBB for CA

| Company | Comments |
| --- | --- |
| QCOM | We think the proposals are the same but want some more time to check. |
| Nokia | We are fine with the proposal from Qualcomm. It is aligned to our but have correctly added 50 and 200 MHz as N/A which were missed on our proposal. |
| Apple | RAN4 has to find an agreement for the single carrier requirement first before defining CA. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213367 Draft CR for n263 RF Rx requirements | QCOM: Some alignment needed with other RX CRs |
|  | Apple: Before agreeing on this CR, we need to finalise the agreement for the Rx requirements and then capture the agreement in a CR. |
| R4-2213203 draftCR on the UE RX requirement for band n263 | QCOM: We think this is aligned with our CR R4-2211627. Need to double check but this is probably fine. Maybe some editorial tweaks needed. |
|  | Apple: Before agreeing on this CR, we need to finalise the agreement for the Rx requirements and then capture the agreement in a CR. |
|  | HW: For ACS and IBB, may need to decide which format is better. Merged table or separate tables. |
| R4-2211627 Draft CR to 38.101-2 on band n263 Rx aspects | QCOM: Our CR so we are good with it. It may be a good candidate for the starting point to merge in this meeting. |
|  | Apple: Before agreeing on this CR, we need to finalise the agreement for the Rx requirements and then capture the agreement in a CR. |
|  | HW: For ACS and IBB, may need to decide which format is better. Merged table or separate tables. |
| R4-2213368 Draft CR for n263 RMC | QCOM: We need some time to check on the RMCs. |
|  | Anritsu: Thanks Huawei/HiSilicon for that contribution, please find below some comments. 1. Regarding SCS 960kHz, number of DL slots and UL slots are inconsistent in the table. They should be swapped as shown below, we think. 2. The S slot of SCS480/960kHz is 1 DL symbol. But, CORESET is 2 OFDM symbols in each slot. Hence, it is better to change from 1D to 2D (and consequently 13G to 12G), we think. 3. Regarding the payload size, we think the following conditions highlighted in yellow are not correct. It seems the values are calculated with MCS index = 19, not 18? |

## Summary for 1st round

### Open issues

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2213367 Draft CR for n263 RF Rx requirements | not pursued |
| R4-2213203 draftCR on the UE RX requirement for band n263 | not pursued |
| R4-2211627 Draft CR to 38.101-2 on band n263 Rx aspects | to be revised |

## Discussion on 2nd round

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Uplink configuration for REFSENS

Perhaps we can have a further discussion about the REFSENS proposals 2 and 3

| Company | Comments |
| --- | --- |
| Company A |  |

RMC CR discussion

Any additional company comments on the RMC CR .

R4-2213368 Draft CR for n263 RMC

| Company | Comments |
| --- | --- |
| QCOM | We agree with Anrisu comments 1 and 2. Our view comment 3 is not valid. If TDD ULDL config for 480 and 960 SCS is not in A.3.3.1 iw needs to be added. |
| Company B |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| TBD | Draft CR to 38.101-2 on band n263 Tx aspects | TBD | revision of R4-2211626 |
| TBD | Draft CR to 38.101-2 on band n263 Rx aspects | TBD | revision of R4-2211627 |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2211626 | TBD | Draft CR to 38.101-2 on band n263 Tx aspects | Qualcomm Inc | Revised | This will be the basis for draft CR on TX |
| R4-2211628 |  | 60GHz UE TX | Qualcomm Inc | Noted |  |
| R4-2211950 |  | Multi-band relaxation for FR2-2 handheld UE | Murata Manufacturing Co Ltd. | Noted |  |
| R4-2212119 |  | Open issues for UE Tx requirements in FR2-2 | Intel Corporation | Noted |  |
| R4-2212278 |  | Draft CR to 38.101-2 on minimum peak EIRP for PC3 band n263 | Murata Manufacturing Co Ltd. | Not pursued | The content is correct and is covered in other CR |
| R4-2212372 |  | Discussion on minimum UE EIRP scaling for FR2-2 | Apple | Noted |  |
| R4-2212373 |  | CR for TS 38.101-2: Correcting oversight with UE EIRP CBW scaling for FR2-2 | Apple | Not pursued |  |
| R4-2213220 |  | On UE Tx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213232 |  | SSB side conditions for band n263 | Apple | Noted |  |
| R4-2213366 |  | Draft CR for n263 RF Tx requirements | Huawei, HiSilicon | Not pursued |  |
| R4-2213369 |  | On remaining RF requirements on n263 | Huawei, HiSilicon | Noted |  |
| R4-2213466 |  | CR on FR2-2 PC3 MPR | LG Electronics Finland | Not pursued |  |
| R4-2213573 |  | Discussion on MPR values for FR2-2 PC3 | LG Electronics Finland | Noted |  |
| R4-2213744 |  | UE Tx requirements for band n263 | Apple | Not treated |  |
| R4-2211627 | TBD | Draft CR to 38.101-2 on band n263 Rx aspects | Qualcomm Inc | Revised |  |
| R4-2211629 |  | 60GHz UE RX | Qualcomm Inc | Noted | This will be the basis for draft CR on RX |
| R4-2213203 |  | draftCR on the UE RX requirement for band n263 | Xiaomi | Not pursued |  |
| R4-2213221 |  | On UE Rx RF aspects for FR2-2 | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213231 |  | UE Rx requirements for band n263 | Apple | Noted |  |
| R4-2213367 |  | Draft CR for n263 RF Rx requirements | Huawei, HiSilicon | No pursued |  |
| R4-2213368 |  | Draft CR for n263 RMC | Huawei, HiSilicon | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents