3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214259

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.7.3

Source:	Moderator (Samsung)

Title:	Email discussion summary for [104-e][207] NR_HST_FR2-Perf

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train (HST) scenario in FR2 has been approved [RP-202118] (which has been further revised to [RP-210800] with editorial revisions and updates on time schedule).

As big CR [R4-2206016] agreed in RAN4#102-e and further approved in RAN#95-e, the relevant RRM core requirement for FR2 HST scenarios are introduced and enhanced over existing FR2 RRM requirements. Specifically, the enhancements for FR2 HST are introduced on cell re-selection, RRC connection mobility control, gradual timing adjustment, SSB based radio link monitoring and beam failure detection, intra-frequency measurement and L1-RSRP/SINR measurement. The new requirements of one-shot large UL timing adjustment and TCI state switch delay are specified for FR2 HST scenarios. In RAN4#103-e, the work on performance part of RRM requirement has been started with the WF approved [R4-2210609]

In this email thread, the following agenda items will be discussed:

- 9.7.3   RRM performance requirement

List of candidate target of email discussion for 1st round and 2nd round

Based on the discussion papers is submitted under agenda 9.7.3. It is suggested to have the following target of 1st and 2nd round email discussion:

- 1st round: Further discussion on RRM performance requirement: (1) continue discussion on general issues, including applicability etc (2) measurement accuracy (3) TC drafting and discussion.

- 2nd round: Achieve agreement as much as possible, and endorse TC draft CRs if possible.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung | Wang, He (Jackson) | h0809.wang@samsung.com |
| Nokia, Nokia Shanghai Bell | Anthony Lo | Anthony.Lo@nokia.com |
| CATT | Yanze Fu | fuyanze@catt.cn |
| Qualcomm | Chu-Hsiang Huang | chuhsian@qti.qualcomm.com |
| Ericsson | Ming Li | Ming.l.li@ericsson.com |
| ZTE | Chenchen Zhang | Zhang.chenchen@zte.com.cn |
| Huawei | Jing Han | Hw.hanjing@huawei.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# 1 Topic #1: RRM performance requirement for FR2 HST

Main technical topic overview. The structure can be done based on sub-agenda basis.

## 1.1 Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211596 | Qualcomm | Observation 1: UE needs to support both uni- and bi-directional deployment, and larger frequency offset leads to worse measurement performance. Proposal 1: Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset, to neighboring cell measurement tests including idle and connected modes. For connected mode neighboring cell measurement and L1-RSRP tests, since non-DRx is more common for CPE, the tests should be in non-DRx mode. Observation 2: Connection re-establishment and connected mode neighboring cell measurement are both requirement on looking for new cells to connect to. In FR2 HST enhancement, they have identical enhancements: replace the Rx sweeping factor of 8 by 2 and 6. Therefore, connection re-establishment test verifies the same function with the same enhancement with connected mode neighboring cell measurement. Same observation applies to L1-SINR test given that L1-RSRP test is introduced. Proposal 2: Do no introduce Re-establishment and L1-SINR test since no additional functionalities and enhancement to be verified by this test given the agreed test set. Observation 3: Tq increase is a relaxation on UE requirement in which even legacy UE can satisfy the requirement. On the other hand, it brings new challenge for network to handle the increased UE autonomous adjustment step properly and keep track of UL reception timing. Proposal 3: Introduce a base station test to verify if network can function properly with increased UE autonomous adjustment step. Observation 4: One shot large UL timing adjustment requirement is applicable only for cross-RRH TCI state switch. Proposal 4: Since the only applicable scenario of one shot large UL timing adjustment requirement is cross-RRH TCI state switch, this requirement should be verified together with TCI state switch test to avoid duplicated tests. Observation 5: Uni- and bi-directional deployment is irrelevant in this test since no related DL measurement accuracy requirements in this test and therefore the propagation condition is irrelevant. The TCI state switch should be in DRx active mode. Observation 6: Legacy test procedure and requirements can verify that UE correctly utilize the extended TCI state switch to correct the >4CP timing offset to continue receiving the grants and transmit UL signals (ACK/NACK/L1 RSRP report). Proposal 5: Reuse A.7.5.8 test with the following modifications to design MAC-CE based TCI state switch delay test: - Timing offset between SSB 0 and SSB 1 is 2.33us - The requirement of reception on TCI state 1: after receiving MAC-CE command in slot n, UE shall be able to start receiving on TCI state 1 after slot n+ THARQ + 3Nslotsubframe,µ + (Tfirst-SSB + TSSB-proc + Trs + Trs-proc) / NR slot length, as defined in 8.10.3A The rest can follow A.7.5.8. Proposal 6: For set 1 and 2 requirements applicability - Use different sets for connected mode neighboring cell measurement and L1-RSRP measurement to verify both sets in connected mode - Choose between set 1 and 2 for idle mode neighboring cell measurement - No need to decide set 1 or 2 for timing and TCI state switch test since it is irrelevant. Observation 7: The accuracy impact can also be analytically derived by 10*log10(frequency offset/SCS). Note that the frequency offset impact is small since the frequency offset is ~16% of SCS, and the sinc function on frequency domain is flat around 0. Observation 8: L1-SINR measurement is not reliable except in low SINR region, similar behavior as in FR1 HST which capping the applicable range of L1-SINR measurement at SNR < 5dB. Proposal 7: Legacy RSRP measurement accuracy requirement is applicable to FR2 HST. Legacy L1-SINR measurement accuracy requirement is applicable to FR2 HST when SNR < 5dB |
| R4-2211677 | CATT | Proposal 1: We slightly prefer to define two test cases for uni-directional and bi-directional respectively. Proposal 2: Define TC for Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17. Proposal 3: For one shot large UL timing adjustment for FR2 PC6 UE, define new test case. For gradual timing adjustment requirement for FR2 PC6 UE, do not modify A.7.4.1.1. If FR2 PC6 UE is really to test, define new test case similar as A.7.4.1.1 Proposal 4: For MAC-CE based TCI State Switch Delay Requirement, define one new test case to verify it for FR2 power class 6 UE with target TCI state is known for unidirectional scenario. Use A.7.5.8.1 as reference. Proposal 5: For L3 measurement test case, use non-DRX mode. Proposal 6: For L1-RSRP measurement test case, use DRX cycle 40ms. Proposal 7: For L1-SINR, core requirements haven’t been decided. If there is core requirement for L1-SINR, new test case can be defined. |
| R4-2211678 | CATT | Draft CR on test case for cell re-selection in HST FR2 |
| R4-2211679 | CATT | Draft CR on test case for MAC-CE based TCI state switch Delay requirement in HST FR2 |
| R4-2211680 | CATT | Draft CR on test case for L3 measurement in HST FR2 |
| R4-2211681 | CATT | Draft CR on test case for L1-RSRP measurement requirement in HST FR2 |
| R4-2212469 | Samsung | Observation 1: For FR2 HST, the L1-SINR measurement with NZP CSI-RS configuration is relatively more accurate than that with CSI-IM configuration. Observation 2: For L1/L3 measurement accuracy, there is less than 1dB difference between the two propagation conditions (AWGN and AWGN with 19444 Hz frequency offset). Proposal 1: The side conditions of achieving the ideal SINR (-3dB) for both NZP CSI-RS and CSI-IM configurations in Non-HST FR2 can also be adopted in HST FR2, i.e., - For NZP CSI-RS: Side conditions (SNR) for CMR and IMR are both 0 dB - For CSI-IM: Side condition (SNR) for CMR is -3 dB Proposal 2: The FR2 measurement accuracy requirements of L1-RSRP, L1-SINR, SS-RSRP, SS-RSRQ, and SS-SINR defined in Rel-15/16 can be reused for L1/L3 measurement in FR2 HST scenario. |
| R4-2212471 | Samsung | Proposal 1: Considering no prioritization between Scenario-A and B, to further reduce the test effort, the applicability of set1 and set2 is provided as: TC category (if confirmed in Sub-Topic 1); Applicability of set1 and set2 (either set1 with 2RX beams or set2 with 6RX beams, or both). TC for Cell Re-selection Requirement; Set1 only. TC for Re-establishment Delay Requirement; Set2 only (if TC for re-establishment delay confirmed). TC for MAC-CE based TCI State Switch Delay Requirement; Set1 only (for target TCI is unknown, SSB-based TCI switching to a RS with new QCL-TypeD reference). TC for L3 measurement; Set2 only. TC for L1-RSRP/SINR Measurement Requirement; Set1 for L1-RSRP and Set2 for L1-SINR (if TC for L1-SINR measurement confirmed). Proposal 2: For (1) cell reselection TC and (2) re-establishment delay TC (if confirmed), RAN4 adopt: • Option-1: 1 test case defined from unidirectional, i.e., AWGN (serving cell) and AWGN with 9722 Hz frequency offset (neighbour cell) Proposal 3: For (3) MAC-CE based TCI state switching delay TC, RAN4 adopt: • Option-1: 1 test case defined from bi-directional, i.e., AWGN (serving cell) and AWGN with 19444 Hz frequency offset (neighbour cell). Proposal 4: Considering to further reduce testing effort, TC for Re-establishment Delay Requirement can be skipped. Proposal 5: The necessity of TC for gradual timing adjustment for FR2 PC6 UE shall be confirmed, while both Option 1 and Option 2 works for this purpose. Proposal 6: For TC for one shot large UL timing adjustment for FR2 PC6 UE, new test case shall be introduced to verify (1) the condition to apply one shot large UL timing adjustment, and (2) new transmit timing behavior and accuracy for one shot large UL timing adjustment. Proposal 7: For TC for SSB-based L1-SINR measurement, RAN4 reuse Rel-16 L1-SINR test cases, and no need to define new test cases under AWGN with 19444 Hz frequency offset. |
| R4-2212472 | Samsung | Draft CR to 38.133 on test case for MAC-CE based active TCI state switch for FR2 PC6 UE |
| R4-2212977 | Huawei, HiSilicon | Draft CR on test case for Cell Re-selection Requirement for FR2 HST |
| R4-2213340 | Ericsson | Proposal 1: Comparing AWGN and AWNG+Doppler, no accuracy degradation is observed for SS-RSRP, SS-SINR, L1-RSRP and L1-SINR. The legacy accuracy of L1 RSRP, SS-RSRP and SS-RSRQ can be reused for L1 and L3 measurement in FR2 HST. |
| R4-2213341 | Ericsson | Proposal 1: Regarding TC for gradual timing adjustment requirement for FR2 PC6 UE, support Option 2: Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case. Proposal 2: Regarding TC for one shot large UL timing adjustment for FR2 PC6 UE, support Option 2: one shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test. Proposal 3: General rule is adopting set2 in L3 relevant tests and adopting set1 in L1 relevant tests. |
| R4-2213343 | Ericsson | Proposal 1: Tend to support DRX cycle = 40 ms. Proposal 2: Don’t introduce L1-SINR test case. |
| R4-2213344 | Ericsson | Proposal 1: Slightly support DRX cycle = 40 ms. |
| R4-2213345 | Ericsson | Proposal 1: Bi-directional test case shall be defined for cell re-selection requirement just since the coverage is greater than Un-directional, specially for cell mobility. Proposal 2: Bi-directional test case shall be defined for cell re-selection requirement just since the coverage is greater than Un-directional, specially for cell mobility. |
| R4-2213346 | Ericsson | Proposal 1: We have not strong view on unidirectional or bi-directional on Test Cases for MAC-CE based TCI State Switch Delay Requirement, but lean towards unidirectional case slightly, and support DRX cycle = 40 ms. |
| R4-2213385 | Ericsson | Draft CR on test cases on Intra-frequency Measurements for HST FR2 |
| R4-2213883 | ZTE | Draft CR on test case for L1-RSRP measurement for FR2 HST PC6 UE in TS38.133 |
| R4-2213893 | Nokia, Nokia Shanghai Bell | Observation 1: From our simulation results, there is no degradation of SS-RSRP and SS-RSRQ accuracy with 19444 Hz carrier frequency offset as compared to without carrier frequency offset for both unidirectional and bidirectional scenarios. Proposal 1: For Rel-17 FR2 HST unidirectional and bidirectional scenarios, the Rel-15 FR2 SS-RSRP and SS-RSRQ accuracy requirements are reused. Observation 2: For unidirectional deployment scenarios, there is no performance degradation in SS-SINR accuracy. For bi-directional deployment scenarios, the SS-SINR accuracy becomes unbounded as SNR increases. Proposal 2: For Rel-17 FR2 HST unidirectional scenarios, the Rel-15 FR2 SS-SINR accuracy requirements can be reused. However, for Rel-17 FR2 HST bidirectional scenarios, the Rel-15 FR2 SS-SINR accuracy requirements can be limited to SNR ≤ 10 dB. - For L1-SINR measurement accuracy under Rel-17 FR2 HST, simulation results are presented in Tables 11-13 for consideration in RAN4. - For L1-RSRP measurement accuracy under Rel-17 FR2 HST, simulation results are presented in Tables 15-17 for consideration in RAN4. |
| R4-2213895 | Nokia, Nokia Shanghai Bell | - For the MAC-CE based TCI state switch, it is sufficient to perform one test assuming the unidirectional scenarios with 19444 Hz frequency offset. |
| R4-2213896 | Nokia, Nokia Shanghai Bell | - For FR2 HST, specify a new test case for the gradual timing adjustment with the adjustment value equals 10*64*Tc with DRX off. Option-2: one shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test. (The test shall verify the one shot large UL timing adjustment requirement and MAC-CE based TCI switch delay requirement.) For the one shot large UL timing adjustment, - (a) it is sufficient to perform one test assuming the unidirectional scenario; (b) set the one-way differential delay Told-Tnew≥76*64*Tc |
| R4-2213897 | Nokia, Nokia Shanghai Bell | CR to TS 38.133: Test cases for gradual timing adjustment, one shot large UL timing adjustment and MAC-CE based TCI state switch for FR2 NR HST |
| R4-2213894 | Nokia, Nokia Shanghai Bell | A placeholder to collect L1 and L3 simulation results contributed by different companies for alignment. |
| R4-2212470 | Samsung | A placeholder to collect L1 and L3 simulation results contributed by different companies for alignment. |

## 1.2 Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### 1.2.1 Sub-topic 1-1: General Configuration and Applicability

Sub-topic description:

Open issues and candidate options before e-meeting:

[Moderator] Based on the approved WF [R4-2210609], the RRM Test Configuration and Applicability is introduced into RRM Performance Requirements for FR2 HST. And the issue is discussed in below by breaking down into two individual sub-issues, one is for Applicability of Set-1 and Set-2 of enhanced RRM requirements and the other is for Test case definition and applicability. The general sub-topic is for RRM Test Configuration and Applicability" and "the Scope of RRM Performance Requirement" issues discussion. Multiple companies provided their views on the two issues, so they are discussed as follows respectively.

#### Issue 1-1-1: Applicability of Set-1 and Set-2 of enhanced RRM requirements

- [Background] The approved WF on the applicability of Set-1 and Set-2 of enhanced RRM requirements is summarized as follows, the TC category with “TBD” in the column of “applicability of set1 and set2” is needed to be defined

<WF>: - Applicability of Set-1 and Set-2 of enhanced RRM requirements: TC category (if confirmed in Sub-Topic 1); Applicability of set1 and set2 (either set1 with 2RX beams or set2 with 6RX beams, or both). TC for Cell Re-selection Requirement; TBD. TC for Re-establishment Delay Requirement; TBD. TC for Timing Related Requirement; N/A (not differentiated by set1 and set2). TC for MAC-CE based TCI State Switch Delay Requirement; TBD (for target TCI is unknown, SSB-based TCI switching to a RS with new QCL-TypeD reference). TC for L3 measurement; TBD. TC for L1-RSRP/SINR Measurement Requirement; TBD.

- Proposals:

  - Option 1 (Samsung, Qualcomm, Ericsson):  Different sets for different TCs

- Option 1-1 (Samsung):

Use NW RRC configuration highSpeedMeasFlagFR2-r17 = set1 or set2 as TC condition

- Option 1-2 (Ericsson):

Adopting set2 in L3 relevant tests and adopting set1 in L1 relevant tests

The details from Samsung and Qualcomm proposals:

- For MAC-CE based TCI state switching delay TC:

- Option 1-a (Samsung):

- Set 1 only

- Option 1-b (Qualcomm):

- No need to decide

- For Re-establishment Delay Requirement TC :

- Option 1-a (Samsung):

- Set 2 only (if TC for re-establishment delay confirmed)

- For L1-RSRP/SINR measurement Requirement TC:

- Option 1-a (Samsung):

- Set1 for L1-RSRP measurement and Set2 for L1-SINR measurement (if TC for L1-SINR measurement confirmed)

- Option 1-b (Qualcomm):

- Choose a different set than L3 measurement (e.g., set 2)

- For Cell Re-selection Requirement TC:

- Option 1-a (Samsung):

- Set1 only

- Option 1-b (Qualcomm):

- Choose between set 1 and 2

- For L3 measurement TC:

- Option 1-a (Samsung):

- Set2 only

- Option 1-b (Qualcomm):

- Choose a different set than L1-RSRP (e.g., set 1)

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Our preference is similar to Samsung in connected mode, but with L1-RSRP and L3 measurements swapped. L1-RSRP is mainly for beam management, and set 2 has more Rx beams and therefore beam management is more important in set 2 configuration. If we select set 2 for L1-RSRP, set 1 can be covered by L3 measurement. Hence our compromised proposal is: L3 measurement TC: set 1 L1-RSRP measurement TC: set 2 |
| Ericsson | We agree on the view that L1 is more beam oriented. If so, can we use more stringent set-1, because it restrict UE’s total sweeping time? |
| Nokia | Share the view as Ericsson. So, we prefer Option 1-a for L1-RSRP/SINR measurement Requirement TC. |
| CATT | In general, we support option 1-1. The set can be discussed in each test case. |
| OPPO | Support option 1-1. Test case for L1-SINR should be postponed after we agree to define L1-SINR measurement requirements. |
| Samsung | Support option 1, i.e., different set chosen for different TC. TC for Cell Re-selection Requirement; Set1 only. TC for Re-establishment Delay Requirement; Set2 only (if TC for re-establishment delay confirmed). TC for MAC-CE based TCI State Switch Delay Requirement; Set1 only (for target TCI is unknown, SSB-based TCI switching to a RS with new QCL-TypeD reference). TC for L3 measurement; Set2 only. TC for L1-RSRP/SINR Measurement Requirement; Set1 for L1-RSRP and Set2 for L1-SINR (if TC for L1-SINR measurement confirmed). - TC for L3 measurement Set 2 only, the reasons are as follows: In HST scenario, UE moves with a high speed, with small DRX cycle length and SMTC period, the larger number of RX beams, the better the UE measurements involving SS are, which can improve the mobility performance. 5) TC for L1-RSRP/SINR Measurement Requirement We think Set 1 could be very typical case for the near-track deployment scenario, which can be very critical performance to be tested. |
| Huawei | We also have the same view as SS on L3 measurement, to guarantee mobility performance, set 2 is preferred. We are open on L1-RSRP measurement. |

### 1.2.2 Sub-topic 1-2: RRM performance Scope

- [Background] Based on the analysed during RAN4 103-e, the necessity of several TCs is still under discussion.

#### Issue 1-2-1: Necessity of Test Cases for Re-establishment Delay Requirement

- Channel model

- Proposals:

  - Option 1 (Ericsson, Qualcomm):  Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset

  - Option 1 (Samsung):  Apply unidirectional deployment channel model, AWGN with 9722Hz frequency offset

- DRX cycle

- Proposals:

  - Option 1 (Qualcomm):  non-DRX

- TC

- Proposals:

  - Option 1 (Samsung, Qualcomm): TCs for Re-establishment Delay Requirement can be skipped

  - Option 2 (CATT): Need new test case

- Option 1-1 (CATT) :

- Need new TC for Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Option 1. As we explained in our contribution, re-establishment delay requirement is verified functionally by L1-RSRP and L3 measurement, and therefore this TC can be skipped. |
| Ericsson | Channel model: Option 1 DRX cycle: We have not strong views on it, non-DRX is acceptable. TC: We can agree to add test case. |
| Nokia | Channel model: Option 1 TC: Option 1-1. |
| CATT | Channel model: We do not have strong view on set1 or set2. DRX cycle: Non-DRX TC: Tidentify_intra, NR is only part of UE re-establishement delay. We prefer to add the test for full tested re-establishement delay. |
| OPPO | Prefer to skip to test case for UE re-establishment delay. |
| Samsung | TC: Support Option 1 In the last meeting, some company proposed that TC re-establishment requirement is not necessary because similar function is already covered and it can be verified by neighboring cell and L1-RSRP measurement test. We think the proposal is make sense, so to reduce testing effort, we support option 1, but should note the updated TC with the time period for re-establishment for PC6 UE. Channel Model: Discuss TC necessity firstly. No strong preference and can compromise to have Option 1 if TC is confirmed. |
| Huawei | Channel model: Option 1 DRX cycle: Option 1 TC: Option 1. |

#### Issue 1-2-2: Necessity of Test Cases for gradual timing adjustment

- Proposals:

  - Option 1 (Samsung, CATT, Nokia, Nokia Shanghai Bell): Need new test case (similar as A.7.4.1.1)

- Option 1-2 (CATT):

- If FR2 PC6 UE is really to test, define new test case similar as A.7.4.1.1

- Option 1-3 (Nokia, Nokia Shanghai Bell) :

- A new TC for the gradual timing adjustment with the adjustment value equals 10*64*Tc with DRX off

  - Option 2 (Samsung, Ericsson): Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case

  - Option 3 (CATT, QC): No new test case needed

  - Option 4 (QC): introduce a base station test to ensure proper network operation

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Option 4 will be ideal to ensure system performance, as we explained in our contribution: Tq increase is a relaxation on UE requirement in which even legacy UE implementation w/o any enhancement can satisfy the requirement. On the other hand, it brings new challenge for network to handle the increased UE autonomous adjustment step properly and keep track of UL reception timing. Note that LTE HST R16 has similar requirement change on Tq but there is not test introduced to UE. Note that LTE R16 HST has a similar enhancement, but the TC is not introduced for UE. |
| Ericsson | Our intent isn’t that gradual timing adjustment for PC6 UE shall be added into A.7.4.1.1. We support Option 3. |
| Nokia | Support Option 1. We provide a text proposal in a draft CR for the gradual timing adjustment. In response to QC’s comments about Option 4, the network can deal the enhanced gradual timing adjustment. |
| CATT | Support option 1-2 or option 3. Do not support option 2 to change current TC in A.7.4.1.1. |
| OPPO | Support option 3 and 1-2. |
| Samsung | Support option 1. We think we can reuse existing test cases of A.7.4.1.1, or just give the necessary changes to avoid setup the new test cases To option 3, considering the new requirement provided for PC6 UE, the TC should not be skipped. To QC’s comment for introducing gNB TC, currently no similar cases for gNB, so we are not quite sure there is testability issue or not. |
| Huawei | Fine with option 3. |
| QC | We checked the test condition in A.7.4.1, test 1 has DL timing adjustment 8*64Tc > the new requirement 4.5*64Tc. Therefore, legacy test configuration is suffice to test FR2 HST requirement, and no spec change is needed. |

GTW on 17th Aug, Chair Notes:

Discussions:

Qualcomm: support option 2. Our understanding is option 2 and 3 are the same.

Ericsson: support comment from Qualcomm and Samsung. Regarding option 4, it does not belong to test case.

Nokia: we have comment for option2. How can Option 2 work? For HST, there is new requirement.

Qualcomm: we have all the test. The original test A 7.4.1.1 does not inclue PC6. But not applicability needs be added.

Agreement:

- Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case

#### Issue 1-2-3: Necessity of TC for one shot large UL timing adjustment for FR2 PC6 UE

- Proposals on necessity of TC:

  - Option 1 (Samsung, CATT, Nokia): New test defined in clause A.7.4.1, similar as A.7.4.1.1

- Option 1-1 (Samsung):

- New TC shall be introduced to verify (1) the condition to apply one shot large UL timing adjustment, and (2) new transmit timing behavior and accuracy for one shot large UL timing adjustment

  - Option 2: (Qualcomm, Ericsson, Nokia, Nokia Shanghai Bell): One shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test

- Proposals of others:

  - Proposal (Nokia): For the one shot large UL timing adjustment,

- (a) It is sufficient to perform one test assuming the unidirectional scenario;

- (b) Set the one-way differential delay Told-Tnew≥76*64*Tc

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Since one shot large UL timing adjustment always come with TCI state switch, we don’t see why option 2 can’t cover option 1 from testing functionality perspective. |
| Ericsson | Support Option 2 on ‘Proposals on necessity of TC’ On ‘Proposals of others’, Agree on Proposal by Nokia, may we know how to derive 76 in equation? |
| Nokia | Support Options 1 and 2. We provided a text proposal in a draft CR for a test case for one shot large UL timing adjustment in combination with MAC-CE based TCI switch delay. In response to Ericsson’s comments, 76*64*Tc = 2.47 us is derived using our simulation results. |
| CATT | We can accept option 2. |
| OPPO | Support option 2, and we agree with Nokia that unidirectional scenario should be used. |
| Samsung | Still prefer Option 1. The condition to apply the one shot large timing adjustment is dependent not only on NW signaling but also on the detected absolute difference between DL timing from new and old TCI states. For a complete conformance test of one shot large timing adjustment procedure, we need to consider different timing difference which trigger or not trigger one shot large timing adjustment to reflect the core requirement. We see the difficulty to combine this together with MAC-CE based beam switch TC. |

#### Issue 1-2-4: Necessity of TC for L1-SINR Measurement

- Proposals on necessity of TC:

  - Option 1 (Samsung, Ericsson): Reuse the Rel-16 test case for SSB-based L1-SINR measurement.

  - Option 3 (CATT, QC): Wait core requirements decision to define a new test case if needed

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Pending core disucssion |
| Ericsson | We can wait for core requirement. |
| Nokia | Pending the outcome in summary [206]. |
| CATT | Support option 3 to wait. |
| OPPO | Support option 3. |
| Samsung | Option 3 is okay, i.e., Discuss core requirement firstly |
| Huawei | Option 3. |

### 1.2.3 Sub-topic 1-3: RRM TC Configuration

[Moderator] Based on the approved WF, the TC configuration has not yet concluded in some requirements, including cell re-selection requirement, re-establishment delay requirement, timing Related Requirement, MAC-CE based TCI state switch delay requirement, L3 measurement requirement, L1-RSRP/SINR measurement requirement. And the discussion of TC configuration is also included in the corresponding issues

#### Issue 1-3-1: Test Cases for L1-RSRP Measurement

- Proposals:

  - Option 1 (Qualcomm): non-DRx mode

  - Option 2 (CATT, Ericsson): 40ms

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Non-DRx mode is more common for CPE and should be tested for connect mode TCs. |
| Ericsson | We suggest adopting DRX, either in L1 or L3. |
| Nokia | Support Option 2. |
| CATT | We agree that non-DRX mode is more common for CPE. But we suggest not all test cases are used non-DRX mode. That is why we use 40ms in L1-RSRP measurement and non-DRX in L3 measurement to involve DRX mode to test. |
| OPPO | Support option 1. |
| Samsung | Support Option 1. The measurement requirements are highly related with DRX cycle length, if non-DRX is considered in FR2 HST scenario, the benefit can be envisioned. So we support prefer to prioritize non DRX case |

#### Issue 1-3-2: Test Cases for Cell Re-selection Requirement

- Proposals:

  - Option 1 (Samsung):  Apply unidirectional deployment channel model, AWGN with 9722Hz frequency offset

  - Option 2 (Qualcomm, Ericsson):  Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset

  - Option 3 (CATT): Apply both uni-directional and bi-directional to define two TCs respectively

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | We support option 2, but can compromise to option 1. Option 3 introduces additional unnecessary TCs and we oppose it. |
| Ericsson | We support Option 2, but Option 3 are acceptable. |
| Nokia | Option 2 or Option 3 is fine. |
| CATT | We add two tests in proposed CR for both uni-directional and bi-directional for test coverage. We don’t see any technical reason to choose set1 or set2. If only pick one, we are slightly prefer bi-directional because uni-directional is chosen to be used in multiple TCs in connected state. |
| OPPO | We prefer to choose one scenario to define test case, either option 1 or option 2 is acceptable for us. |
| Samsung | Even though we apply the bi-directional deployment channel model, the test setup of AoA configuration is still Setup-1, i.e., 1AoA test setup in R15 test case for cell re-selection. So we see no strong reason to consider bi-directional scenario, but prefer to consider unidirectional test case only. Option 3 is not necessary. |
| Huawei | Option 2 with bi-directional. |

#### Issue 1-3-3: Test Cases for MAC-CE based TCI State Switch Delay Requirement

- Channel model

- Proposals:

  - Option 1 (Samsung, Nokia, Nokia Shanghai Bell): Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset

  - Option 2 (Qualcomm): Uni-directional and bi-directional deployment is not relevant as long as the timing difference is set properly as proposed: Timing offset between SSB 0 and SSB 1 is 2.33us

  - Option 3 (CATT, Ericsson): Apply unidirectional deployment channel model, AWGN with 9722Hz frequency offset

- DRX cycle

- Proposals:

  - Option 1 (Qualcomm, Ericsson): 40 ms

- Timing offset

- Proposals:

  - Option 1 (Qualcomm): Reuse A.7.5.8 test but need modify the Timing offset between SSB 0 and SSB 1, and the requirement of reception on TCI state 1

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Comment to channel model option 1: according to the models agreed in demod, no timing jump is observed in bi-directional model. RRM test should align with demod test. We also support non-DRx. |
| Ericsson | Regarding Channel mode, as Qualcomm mentioned, timing offset dominates in TCI-state switch. Our intent is to use exact number of timing offset calculated based on unidirectional deployment. But we’re open to discuss other possibilities. |
| Nokia | If one shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay, then it is recommended to use the same scenario, i.e., the unidirectional scenario. Otherwise, the MAC-CE based TCI switch delay can use a different scenario. |
| CATT | Channel model: prefer to unidirectional scenario. DRX: 40ms is ok. |
| OPPO | Channel model: uni-directional with timing offset between SSB 0 and SSB 1 is 2.33us DRX: 40ms is fine. |
| Samsung | Option 1 for channel model. As the value of frequency offset increases, the measurement performance deteriorates, so we think only considering the worst case (bi-directional with 19444Hz frequency offset ) can be enough but non-DRX instead. But if that is two AoA-based test case, seems we don’t need the timing offset to better reflect practical bi-directional deployment. |

#### Issue 1-3-4: Test Cases for L3 Measurement Requirement

- DRX cycle

- Proposals:

  - Option 1 (CATT): non-DRX mode

  - Option 2 (Ericsson): 40 ms

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | Support option 1. Non-DRx mode is more common for CPE and should be tested for connect mode TCs. |
| Ericsson | We prefer to reserve DRX case. |
| Nokia | Support Option 2. |
| CATT | We agree that non-DRX mode is more common for CPE. But we suggest not all test cases are used non-DRX mode. That is why we use 40ms in L1-RSRP measurement and non-DRX in L3 measurement to involve DRX mode to test. |
| OPPO | Prefer option 1, and can compromise to option 2. |
| Samsung | Support Option 1 as the same as Issue 1-3-1. |
| Huawei | To balance, if Issue 1-3-1 use non-DRX, this case can use 40ms. We also fine to use non-DRX for both cases. |

### 1.2.4 Sub-topic 1-4: Measurement accuracy for FR2 HST UE

#### Issue 1-4-1: L1-measurement simulation result and alignment

- Proposals:

  - Option 1 (Samsung, Ericsson): The legacy accuracy of L1-RSRP, LI-SINR in Rel-16 can be reused for L1 measurement in FR2 HST.

  - Option 2 (Qualcomm): The legacy accuracy of L1-RSRP can be reused, but Legacy L1-SINR need satisfy SNR < 5dB if reused

  - Option 3 (Nokia, Nokia Shanghai Bell): L1 measurement simulation is provided

<LI-SINR measurement accuracy> Table 11: Absolute L1-SINR measurement accuracy for AWGN with 19444 Hz frequency offset (frequency tracking disabled) CMR; IMR; 5th percentile (dB); 50th percentile (dB); 95th percentile (dB). CSI-RS; N/A; -1.47; 0.13; 1.55. SSB; NZP-IMR; -1.74; -0.13; 1.38. CSI-IM; -1.71; -0.1; 1.3. CSI-RS; NZP-IMR; -1.53; -0.01; 1.32. CSI-IM; -1.32; -0.02; 1.14. Table 12: Absolute L1-SINR measurement accuracy for AWGN with 19444 Hz frequency offset (frequency tracking enabled for bidirectional scenarios) CMR; IMR; 5th percentile (dB); 50th percentile (dB); 95th percentile (dB). CSI-RS; N/A; -0.82; 0.64; 1.94. SSB; NZP-IMR; -1.06; 0.53; 2.07. CSI-IM; -1.07; 0.47; 1.98. CSI-RS; NZP-IMR; -0.88; 0.53; 1.91. CSI-IM; -0.77; 0.48; 1.61. Table 13: Absolute L1-SINR measurement accuracy for AWGN with 0 Hz frequency offset CMR; IMR; 5th percentile (dB); 50th percentile (dB); 95th percentile (dB). CSI-RS; N/A; -0.41; 0.37; 1.29. SSB; NZP-IMR; -1.04; 0.45; 1.75. CSI-IM; -1.1; 0.44; 1.78. CSI-RS; NZP-IMR; -0.81; 0.57; 1.88. CSI-IM; -0.7; 0.58; 1.71.

< L1-RSRP measurement accuracy > Table 15: Absolute L1-RSRP measurement accuracy for AWGN with 19444 Hz frequency offset (frequency tracking disabled) CMR; IMR; SINR; 5th percentile (dB); 50th percentile (dB); 95th percentile (dB). CSI-RS; N/A; -3 dB; -1.17; 0.14; 1.24. SSB; N/A; -3 dB; -1.54; 0.04; 1.44. Table 16: Absolute L1-RSRP measurement accuracy for AWGN with 19444 Hz frequency offset (frequency tracking enabled) CMR; IMR; SINR; 5th percentile (dB); 50th percentile (dB); 95th percentile (dB). CSI-RS; N/A; -3 dB; -0.7; 0.48; 1.53. SSB; N/A; -3 dB; -0.98; 0.42; 1.73. Table 17: Absolute L1-RSRP measurement accuracy for AWGN with 0 Hz frequency offset CMR; IMR; SINR; 5th percentile (dB); 50th percentile (dB); 95th percentile (dB). CSI-RS; N/A; -3 dB; -0.63; 0.54; 1.55. SSB; N/A; -3 dB; -1.11; 0.43; 1.64.

- Recommended WF:

  - Initial discussion provided in the 1st round discussion, on confirm/disconfirm the necessity of this test case.

| Company | Comments |
| --- | --- |
| QC | We support option 2. Comment to option 1: When there is a frequency offset of 5 to 10% of SCS, there is roughly 5-10% of power from the adjacent carrier leaked into the current carrier as interference. And therefore, the estimated SINR can’t exceed 10*log10(95/5) = 9.5dB or 10*log10(95/5)=12.8dB, and the closer the true SINR (without ICI) to this limit, the accuracy gets worse, as we shown in our paper. The test points in the simulations are too low, and we can’t observe the SINR upper bound as agreed in FR1 HST SS-SINR measurements. |
| Ericsson | We agree to reuse L1-SINR (option1) and the upper boundary(option2) is needed if L1-SINR in HST FR2 is agreed. |
| Nokia | In Option 3, our L1 simulation results are provided for the purpose of comparison and alignment. L1-RSRP Our simulation results show that the legacy accuracy requirement of L1-RSRP can be reused. L1-SINR In 38.133, three types of L1-SINR accuracy requirements are specified, which are summarized as follows: CMR; IMR; L1-SINR accuracy requirements. CSI-RS; N/A; Type 1: CSI-RS based CMR and no dedicated IMR configured. SSB; NZP-IMR; Type 2: SSB based CMR and dedicated IMR configured. CSI-IM. CSI-RS; NZP-IMR; Type 3: CSI-RS based CMR and dedicated IMR configured. CSI-IM. Referring to the above table, dedicated CMR and IMR resources are used to compute L1-SINR for Type 2 and Type 3. For Type 1, L1-SINR is computed using CMR only. As such, the impact of ICI caused by carrier offset on L1-SINR for Types 2 and 3 is minimal as compared with Type 1. We propose - The legacy L1-SINR accuracy requirements for Types 2 and 3 can be reused. - An upper bound SNR = FFS is defined for L1-SINR (type 1: CSI-RS based CMR and no dedicated IMR configured) |
| Samsung | Support option 1. Although large frequency offset will bring the ICI and degrades the SINR, from our simulation results, for L1-RSRP/L1-SINR measurement, we do not see any performance loss in 19444 Hz (the worst case). In HST FR2 scenario, the distance of two RRHs is large, and in DPS, the two RRH transmitting simultaneously is impossible, so the interference is low even if under high SNR. So the legacy accuracy of L1-RSRP, LI-SINR in Rel-16 can be reused for L1 measurement in FR2 HST. |

#### Issue 1-4-2: L3-measurement simulation result and alignment

Moderator’s Note: Whether SS-SINR can be reused or not is not mentioned in paper R4-2213340

- Proposals:

  - Option 1 (Samsung, Ericsson): All the legacy accuracy in Rel-15/Rel-16 including SS-RSRP, SS-RSRQ, SS-SINR can be reused

  - Option 2 (Nokia, Nokia Shanghai Bell): All the legacy accuracy except SS-SINR can be reused

The details from Nokia, Nokia Shanghai Bell’s option:

- Option 2 (Nokia, Nokia Shanghai Bell): Whether SS-SINR can be reused or not depend on deployment scenarios

- For Rel-17 FR2 HST unidirectional scenarios, the Rel-15 FR2 SS-SINR accuracy requirements can be reused.

- For Rel-17 FR2 HST bidirectional scenarios, the Rel-15 FR2 SS-SINR accuracy requirements can be limited to SNR ≤ 10 dB

- Recommended WF

  - Initial discussion provided in the 1st round discussion, and companies’ view are collected.

| Company | Comments |
| --- | --- |
| QC | Option 1 should be revised as legacy “FR1 HST” requirement. Comment to option 2: Bidirectional upper bound seems too aggressive given that the bound for FR1 HST is 5dB, we suggest to follow FR1 HST. For uni-directional, we should have similar bound with larger upper bound, given that the same ICI effect contributing to the SINR measurement error exists in uni-directional model, the only difference is that uni-directional has lower Doppler and the effect is smaller. We suggest to have 5dB for bi-directional, and 8dB for uni-directional given that the Doppler is half and 3dB difference reflect the Doppler decrease We can support option 1 or 2 if the suggestions are taken for revision. |
| Ericsson | We did not simulate upper boundary, but we believe an upper boundary is needed in order to adopt SS-SINR requirement in HST FR2 scenario. The only question is the number10dB, shall it be checked? |
| Nokia | The 10 dB for the bidirectional scenario is derived from our simulation results. Figure 1: Delta SS-SINR vs SNR for bidirectional deployment scenarios As observed in the figure, the SS-SINR accuracy is still in the within the requirement at SNR = 10 dB for SCS = 240 kHz for bidirectional scenarios. For unidirectional scenario, the Doppler shift of neighbouring cell can be assumed to be the same as the serving cell, resulting in residual Doppler which is not significant compared with bidirectional. As such, it is sufficient to define the SNR upper bound based on the bidirectional scenario. We can go forward with an SNR upper bound = [8dB]. |
| Samsung | Support option 1, based upon our simulation results. |

## 1.3 Companies views’ collection for 1st round

### 1.3.1 Open issues

Note: Companies’ view collection under each issue.

### 1.3.2 CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

##### Issue 1-3-2-1a: Necessity of Test Cases for Cell Re-selection Requirement

| CR/TP number | Comments collection |
| --- | --- |
| R4-2211678 (CATT) | Draft CR on test case for cell re-selection in HST FR2 |
| R4-2212977 (Huawei) | Test case for Cell Re-selection Requirement for FR2 HST |
|  | QC: pending open issue discussion. Could you also share the derivation of delay requirement? |
|  | Samsung: Same comment as QC |

##### Issue 1-3-2-1b: Necessity of Test Cases for MAC-CE based TCI State Switch Delay Requirement

| CR/TP number | Comments collection |
| --- | --- |
| R4-2211679（CATT） | Draft CR on test case for MAC-CE based TCI state switch Delay requirement in HST FR2 |
| R4-2213897 (Nokia, Nokia Shanghai Bell) | CR to TS 38.133: Test cases for gradual timing adjustment, one shot large UL timing adjustment and MAC-CE based TCI state switch for FR2 NR HST (Only the discussion of MAC-CE based TCI state switch for FR2 NR HST) |
| R4-2212472 (Samsung) | Draft CR to 38.133 on test case for MAC-CE based active TCI state switch for FR2 PC6 UE |
|  | QC: pending open issue discussion |
|  | Samsung: agree with QC, we need to discuss issues firstly. |

##### Issue 1-3-2-1c: Necessity of Test Cases for L3 Measurement Requirement

| CR/TP number | Comments collection |
| --- | --- |
| R4-2211680（CATT） | Draft CR on test case for L3 measurement in HST FR2 |
| R4-2213385 (Ericsson) | Test cases on Intra-frequency Measurements for HST FR2 |
|  | QC: pending open issue discussion. Could you also share the derivation of delay requirement? |
|  | CATT: It seems that the lower bounds are not considered. The values are different from that in R4-2211680 QC: There is a type in this note: Note 1: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Also, there are two test cases included, while we only agreed to introduce one. Keep only one after 1-3-4 concluded. |

##### Issue 1-3-2-1d: Necessity of Test Cases for L1-RSRP/L1-SINR Measurement

| CR/TP number | Comments collection |
| --- | --- |
| R4-2211681（CATT） | Draft CR on test case for L1-RSRP measurement requirement in HST FR2 |
| R4-2213883 (ZTE) | Draft CR on test case for L1-RSRP measurement for FR2 HST PC6 UE in TS38.133 A.7.6.3 |
|  | QC: pending open issue discussion. Could you also share the derivation of delay requirement? |
|  | CATT: we need to decide how many tests cases for L1-RSRP firstly. |
|  | To QC: We determine the delay requirement based on the approved scaling factor reduction. Of course, we can further check the exact value. |

##### Issue 1-3-2-2: Test Cases for Timing Related Requirement

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213897（Nokia, Nokia Shanghai Bell） | CR to TS 38.133: Test cases for gradual timing adjustment, one shot large UL timing adjustment and MAC-CE based TCI state switch for FR2 NR HST (Only the discussion of gradual timing adjustment, one shot large UL timing adjustment for FR2 NR HST) |
|  | QC: pending open issue discussion. |
|  | Samsung: discuss relevant issues firstly. |

## 1.4 Summary for 1st round

### 1.4.1 Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | #### Issue 1-1-1: Applicability of Set-1 and Set-2 of enhanced RRM requirements During 1st round email discussion, different sets (i.e., either set1 or set2) configured for different TCs are the common understanding, which can be adopted as tentative agreement below. For detailed configuration (set1 or set2) for different TCs: TC; Applicability of Set1 and Set2: Summarized Preferences from Companies. TC for Cell Re-selection Requirement; - Option 1: Set1 only - Supported by QC, Samsung. TC for Re-establishment Delay Requirement; - Option 1: Set2 only (if TC for re-establishment delay confirmed) - Supported by QC, Samsung. TC for MAC-CE based TCI State Switch Delay Requirement; - Option 1: Set1 only - Supported by QC, Samsung. TC for L3 measurement; - Option 1: Set1 only - Supported by QC - Option 2: Set2 only - Supported by Samsung, Huawei. TC for L1-RSRP/SINR Measurement Requirement; For L1-RSRP measurement: - Option 1: Set1 only - Supported by Samsung, Ericsson, Nokia - Option 2: Set2 only - Supported by QC FFS Set1 or Set2 for L1-SINR: - Dependent on the decision to introduce TC or not.. Tentative agreements: - Applicability of Set-1 and Set-2 of enhanced RRM requirements: - General principle: - Use NW RRC configuration highSpeedMeasFlagFR2-r17 = set1 or set2 as TC condition. No need to test both set1 and set2 for one TC. - Detailed applicability for relevant tests: - TC for Cell Re-selection Requirement - Set1 only - TC for Re-establishment Delay Requirement - Set2 only (if TC for re-establishment delay confirmed) - TC for MAC-CE based TCI State Switch Delay Requirement - Set1 only - TC for L3 measurement - Option 1: Set1 only - Option 2: Set2 only - TC for L1-RSRP Measurement Requirement - Option 1: Set1 only - Option 2: Set2 only - TC for L1-SINR Measurement Requirement - FFS Set1 or Set2 for L1-SINR: Dependent on the decision to introduce TC or not. Recommendations for 2nd round: - Discussion on whether or not the above tentative agreement can be accepted. |
| Sub-Topic 1-2 | #### Issue 1-2-1: Necessity of Test Cases for Re-establishment Delay Requirement During 1st round email discussion, the necessity of TC for re-establishment delay requirement is of most importance, and serves as the premise for other points to be discussed: Necessity of TC for re-establishment delay requirement: - Option 1: TCs for Re-establishment Delay Requirement can be skipped - Supported by Samsung, Qualcomm, Huawei, OPPO - Option 2: New TC for Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17 - Supported by CATT, Ericsson, Nokia Though depending on whether or not TC for re-establishment delay requirement is introduced, seems the following test configuration can be adopted: - Test configuration of TC for reestablishment delay requirement (if necessity of TC is confirmed): - Channel model: - Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset - DRX cycle: - Non-DRX Tentative agreements: - Necessity of TC for re-establishment delay requirement: - TCs for Re-establishment Delay Requirement can be skipped Recommendations for 2nd round: - Confirm the above tentative agreement (which is based on the majority view (4 vs. 3)). |
|  | #### Issue 1-2-2: Necessity of Test Cases for gradual timing adjustment During GTW on 17th Aug, the following agreement is achieved: Agreement: - Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case Recommendations for 2nd round: - Focus on TC draft CR revision directly. |
|  | #### Issue 1-2-3: Necessity of TC for one shot large UL timing adjustment for FR2 PC6 UE Based on 1st round discussion, the necessity of introducing performance requirement (either new test or integrated into existing MAC-CE based TCI switch delay test) is confirmed as the common understanding: - Option 1: New test defined in clause A.7.4.1, similar as A.7.4.1.1 - Supported by Samsung, CATT, Nokia - Option 2: One shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test - Supported by Qualcomm, Ericsson, Nokia, CATT, OPPO Considering more companies support Option 2, and it is the common understanding that both Option 1 and Option 2 can serve the test purpose of verifying the one shot large timing adjustment procedure and accuracy purpose. Tentative agreements: - Necessity of TC for one shot large UL timing adjustment for FR2 PC6 UE - One shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test Recommendations for 2nd round: - Confirm the above tentative agreement. - Discussion on dCR revision by integrating the test purpose for one shot large UL timing adjustment into TC of MAC-CE based TCI switch delay for PC6 UE. |
|  | #### Issue 1-2-4: Necessity of TC for L1-SINR Measurement Since the necessity of TC for L1-SINR measurement in performance part is dependent on the on-going discussion for core requirement, it is the common understanding that we can wait core requirements decision to define a new test case if needed. Conclusion: - Necessity of TC for L1-SINR Measurement: - Wait core requirements decision to define a new test case if needed. Recommendations for 2nd round: - Discuss only if core requirement on L1-SINR measurement is specified. |
| Sub-Topic 1-3 | #### Issue 1-3-1: Test Cases for L1-RSRP Measurement During 1st round discussion, companies are still having different preference on below two options for DRX configuration: - Option 1: non-DRX mode - Supported by Qualcomm, OPPO, Samsung - Option 2: DRX mode, DRX cycle = 40ms - Supported by CATT, Ericsson, Nokia. Tentative Agreement: - Test Cases for L1-RSRP Measurement and L1-SINR Measurement (if introduced) - DRX mode, DRX cycle = 40ms Recommendations for 2nd round: - FFS the above tentative agreement - Suggest to consider tentative agreements in Issue 1-3-1 and Issue 1-3-4 together. - Note: tentative agreement is based on DRX for L1 measurement, and non-DRX for L3-measurement. |
|  | #### Issue 1-3-2: Test Cases for Cell Re-selection Requirement During 1st round discussion, companies are still having different preference on below three options for uni- and bi-directional deployment scenarios to be used, which impacts the channel model for the tests. - Option 1: Apply unidirectional deployment channel model, AWGN with 9722Hz frequency offset - Supported by Samsung, OPPO - Option 2: Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset - Supported by QC, Ericsson, Nokia, CATT, OPPO, Huawei - Option 3: Apply both uni-directional and bi-directional to define two TCs respectively - Supported by CATT. Based on the majority view, Option 2 is adopted as tentative agreement. Tentative Agreement: - Test Cases for Cell Re-selection Requirement: - Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset Recommendations for 2nd round: - Confirm the above tentative agreement. |
|  | #### Issue 1-3-3: Test Cases for MAC-CE based TCI State Switch Delay Requirement The following aspects related to TC for MAC-CE based TCI State switch delay requirement are discussed: Channel model and timing offset: - Option 1: Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset - Supported by Samsung, Nokia - Option 2a: Uni-directional with the timing difference is set properly as proposed: Timing offset between SSB 0 and SSB 1 is 2.33us - Supported by QC, Ericsson, OPPO, Nokia (if timing requirement is also tested) - Option 3: Apply unidirectional deployment channel model, AWGN with 9722Hz frequency offset - Supported by CATT, Ericsson Based on 1st round discussion, Option 2a seems the majority view. DRX cycle: - Option 1: 40ms - No opposition on 40ms as DRX cycle. Tentative Agreement: - Test Cases for MAC-CE based TCI State Switch Delay Requirement - Channel model and timing offset: - Uni-directional with the timing difference is set properly as proposed: Timing offset between SSB 0 and SSB 1 is 2.33us - DRX cycle: - 40ms. Recommendations for 2nd round: - Confirm the above tentative agreement. |
|  | #### Issue 1-3-4: Test Cases for L3 Measurement Requirement During 1st round discussion, companies are still having different preference on below two options for DRX configuration: - Option 1: non-DRX mode - Supported by CATT (if L1 use DRX), Qualcomm, OPPO, Samsung, Huawei - Option 2: DRX mode, DRX cycle = 40ms - Supported by Ericsson, Nokia, Huawei (if L1 use non-DRX). Tentative Agreement: - Test Cases for L3 Measurement - Non-DRX mode - Note: tentative agreement is based on DRX for L1 measurement, and non-DRX for L3-measurement. Recommendations for 2nd round: - FFS the above tentative agreement - Suggest to consider tentative agreements in Issue 1-3-1 and Issue 1-3-4 together. - Note: tentative agreement is based on DRX for L1 measurement, and non-DRX for L3-measurement. |
| Sub-Topic 1-4 | #### Issue 1-4-1: L1-measurement simulation result and alignment Based on the available simulation results, the side condition and measurement accuracy are discussed. It should be noted that there is a spreadsheet for L1/L3 simulation results summary circulated over email reflector by Nokia to have the results be aligned and compared. Tentative Agreement: - L1-RSRP measurement: - The legacy accuracy of FR2 L1-RSRP measurement can be reused for L1-RSRP measurement in FR2 HST. - L1-SINR measurement: - For the below three categories of L1-SINR measurement (introduced in Rel-16 eMIMO WI): CMR; IMR; L1-SINR accuracy requirements. CSI-RS; N/A; Type 1: CSI-RS based CMR and no dedicated IMR configured. SSB; NZP-IMR; Type 2: SSB based CMR and dedicated IMR configured. CSI-IM. CSI-RS; NZP-IMR; Type 3: CSI-RS based CMR and dedicated IMR configured. CSI-IM. - For Type-2 and Type-3 L1-SINR measurement: - The legacy accuracy of FR2 L1-SINR measurement can be reused for L1-SINR measurement in FR2 HST. - For Type-1 L1-SINR measurement: - The legacy accuracy of FR2 L1-SINR measurement can be reused for L1-SINR measurement in FR2 HST; - An upper bound SNR = FFS is defined. Recommendations for 2nd round: - Review and check the L1 measurement simulation result alignment - Discussion on the above tentative agreement. |
|  | #### Issue 1-4-2: L3-measurement simulation result and alignment Based on the available simulation results, the side condition and measurement accuracy are discussed. It should be noted that there is a spreadsheet for L1/L3 simulation results summary circulated over email reflector by Nokia to have the results be aligned and compared. Tentative Agreement: - For SS-RSRP and SS-RSRQ measurement: - The legacy accuracy requirement for Rel-15 FR2 SS-RSRP and SS-RSRQ measurement can be reused. - For SS-SINR measurement: - FFS whether or not Rel-15 FR2 SS-SINR measurement can be reused, by further study: - Impact of uni- and bi-directional deployment scenario; - SNR upper bound. Recommendations for 2nd round: - Review and check the L3 measurement simulation result alignment - Discussion on the above tentative agreement. |

### 1.4.2 CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2211678 | Draft CR on test case for cell re-selection in HST FR2 | CATT | Merged to the revision of R4-2212977 |
| R4-2212977 | Test case for Cell Re-selection Requirement for FR2 HST | Huawei | Revised to address open issues. |
| R4-2211679 | Draft CR on test case for MAC-CE based TCI state switch Delay requirement in HST FR2 | CATT | Merged to the revision of R4-2212472 |
| R4-2213897 | CR to TS 38.133: Test cases for gradual timing adjustment, one shot large UL timing adjustment and MAC-CE based TCI state switch for FR2 NR HST | Nokia | Only focus on gradual timing adjustment part |
| R4-2212472 | Draft CR to 38.133 on test case for MAC-CE based active TCI state switch for FR2 PC6 UE | Samsung | Revised to address open issues, and potentially capture test for one-shot timing adjustment requirement |
| R4-2211680 | Draft CR on test case for L3 measurement in HST FR2 | CATT | Merged to the revision of R4-2213385 |
| R4-2213385 | Test cases on Intra-frequency Measurements for HST FR2 | Ericsson | Revised to address open issues. |
| R4-2211681 | Draft CR on test case for L1-RSRP measurement requirement in HST FR2 | CATT | Merged to the revision of R4-2213883 |
| R4-2213883 | Test cases on Intra-frequency Measurements for HST FR2 | ZTE | Revised to address open issues. |

## 1.5 Discussion on 2nd round (if applicable)

### 1.5.1 Open issues

#### Issue 1-1-1: Applicability of Set-1 and Set-2 of enhanced RRM requirements

[Background] During 1st round email discussion, different sets (i.e., either set1 or set2) configured for different TCs are the common understanding, which can be adopted as tentative agreement below.

For detailed configuration (set1 or set2) for different TCs:

| TC | Applicability of Set1 and Set2: Summarized Preferences from Companies |
| --- | --- |
| TC for Cell Re-selection Requirement | - Option 1: Set1 only - Supported by QC, Samsung |
| TC for Re-establishment Delay Requirement | - Option 1: Set2 only (if TC for re-establishment delay confirmed) - Supported by QC, Samsung |
| TC for MAC-CE based TCI State Switch Delay Requirement | - Option 1: Set1 only - Supported by QC, Samsung |
| TC for L3 measurement | - Option 1: Set1 only - Supported by QC - Option 2: Set2 only - Supported by Samsung, Huawei |
| TC for L1-RSRP/SINR Measurement Requirement | For L1-RSRP measurement: - Option 1: Set1 only - Supported by Samsung, Ericsson, Nokia - Option 2: Set2 only - Supported by QC FFS Set1 or Set2 for L1-SINR: - Dependent on the decision to introduce TC or not. |

Tentative Agreement:

- Applicability of Set-1 and Set-2 of enhanced RRM requirements:

  - General principle:

    - Use NW RRC configuration highSpeedMeasFlagFR2-r17 = set1 or set2 as TC condition. No need to test both set1 and set2 for one TC.

  - Detailed applicability for relevant tests:

    - TC for Cell Re-selection Requirement

      - Set1 only

    - TC for Re-establishment Delay Requirement

      - Set2 only (if TC for re-establishment delay confirmed)

    - TC for MAC-CE based TCI State Switch Delay Requirement

      - Set1 only

    - TC for L3 measurement

      - Option 1: Set1 only

      - Option 2: Set2 only

    - TC for L1-RSRP and L1-SINR (if introduced) Measurement Requirement

      - Option 1: Set1 only

      - Option 2: Set2 only

    - TC for L1-SINR Measurement Requirement

      - FFS Set1 or Set2 for L1-SINR: Dependent on the decision to introduce TC or not.

Recommendations for 2nd round:

- Discussion on whether or not the above tentative agreement can be accepted.

- Furthermore, can we further narrow down the configuration for L1/L3 measurement by using:

  - TC for L3 measurement:

    - Option 2: Set2 only

  - TC for L1 measurement (for L1-RSRP and L1-SINR if introduced):

    - Option 1: Set1 only

| Company | Comments |
| --- | --- |
| QC | Our preference is set 1 for L3 and set 2 for L1, but we can compromise to the recommended WF. |
| Ericsson | Rrecommended WF is fine. |
| Huawei | Rrecommended WF is fine. |

#### Issue 1-2-1: Necessity of Test Cases for Re-establishment Delay Requirement

[Background] During 1st round email discussion, the necessity of TC for re-establishment delay requirement is of most importance, and serves as the premise for other points to be discussed:

Necessity of TC for re-establishment delay requirement:

- Option 1: TCs for Re-establishment Delay Requirement can be skipped

  - Supported by Samsung, Qualcomm, Huawei, OPPO

- Option 2: New TC for Intra-frequency RRC Re-establishment in FR2 for PC6 UE configured with highSpeedMeasFlagFR2-r17

  - Supported by CATT, Ericsson, Nokia

Though depending on whether or not TC for re-establishment delay requirement is introduced, seems the following test configuration can be adopted:

- Test configuration of TC for reestablishment delay requirement (if necessity of TC is confirmed):

  - Channel model:

    - Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset

  - DRX cycle:

    - Non-DRX

Tentative Agreement:

- Necessity of TC for re-establishment delay requirement:

  - TCs for Re-establishment Delay Requirement can be skipped

Recommendations for 2nd round:

- Confirm the above tentative agreement (which is based on the majority view (4 vs. 3)).

| Company | Comments |
| --- | --- |
| QC | Support the agreement. |
| Ericsson | We support Option2 but can compromise to recommendations. |
| Huawei | Support the agreement. |
| CATT | Can compromise to tentative agreement to move forward. |

#### Issue 1-2-2: Necessity of Test Cases for gradual timing adjustment

During GTW on 17th Aug, the following agreement is achieved:

Agreement:

- Applying PC6 UE new requirement into A.7.4.1.1, and no need to define new test case

#### Issue 1-2-3: Necessity of TC for one shot large UL timing adjustment for FR2 PC6 UE

[Background] Based on 1st round discussion, the necessity of introducing performance requirement (either new test or integrated into existing MAC-CE based TCI switch delay test) is confirmed as the common understanding:

- Option 1: New test defined in clause A.7.4.1, similar as A.7.4.1.1

  - Supported by Samsung, CATT, Nokia

- Option 2: One shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test

  - Supported by Qualcomm, Ericsson, Nokia, CATT, OPPO

Considering more companies support Option 2, and it is the common understanding that both Option 1 and Option 2 can serve the test purpose of verifying the one shot large timing adjustment procedure and accuracy purpose.

Tentative Agreements:

- Necessity of TC for one shot large UL timing adjustment for FR2 PC6 UE

  - One shot large UL timing adjustment test is combined into MAC-CE based TCI switch delay test

Recommendations for 2nd round:

- Confirm the above tentative agreement.

- Discussion on dCR revision by integrating the test purpose for one shot large UL timing adjustment into TC of MAC-CE based TCI switch delay for PC6 UE.

| Company | Comments |
| --- | --- |
| QC | Support the agreement. |
| Ericsson | Support the agreement. |
| Huawei | Support the agreement. |
| CATT | Can agree with tentative agreements. |

#### Issue 1-2-4: Necessity of TC for L1-SINR Measurement

[Background] Since the necessity of TC for L1-SINR measurement in performance part is dependent on the on-going discussion for core requirement, it is the common understanding that we can wait core requirements decision to define a new test case if needed.

Conclusion:

- Necessity of TC for L1-SINR Measurement:

  - Wait core requirements decision to define a new test case if needed.

Recommendations for 2nd round:

- Discuss only if core requirement on L1-SINR measurement is specified.

- Seems the above conclusion is acceptable, if not, pls. give comments below

| Company | Comments |
| --- | --- |
| QC | We have the same understanding as the conclusion. |
| Ericsson | Agree on the conclusion in recommendations |
| Huawei | Fine with the conclusion |
| CATT | Ok with the conclusion. Due to the lack of time, there is no decision in core part, can it be handled in maintenance phase? |
| Samsung | The core part has been completed, so anyway core part-relevant discussion should be handled as maintenance issues. |

#### Issue 1-3-1: Test Cases for L1-RSRP Measurement

[Background] During 1st round discussion, companies are still having different preference on below two options for DRX configuration:

  - Option 1: non-DRX mode

    - Supported by Qualcomm, OPPO, Samsung

  - Option 2: DRX mode, DRX cycle = 40ms

    - Supported by CATT, Ericsson, Nokia.

Tentative Agreement:

- Test Cases for L1-RSRP Measurement and L1-SINR Measurement (if introduced)

  - DRX mode, DRX cycle = 40ms

Recommendations for 2nd round:

- FFS the above tentative agreement

  - Suggest to consider tentative agreements in Issue 1-3-1 and Issue 1-3-4 together.

  - Note: tentative agreement is based on DRX for L1 measurement, and non-DRX for L3-measurement.

| Company | Comments |
| --- | --- |
| QC | Based on the first round comments, companies seem to agree that non-DRx mode is more common for PC6 UEs. But to make progress, if companies insist to have one of L1 and L3 test as DRx, we can compromise to the recommended agreement to move forward. |
| Ericsson | We support recommendations. |
| CATT | Support tentative agreement. |

#### Issue 1-3-2: Test Cases for Cell Re-selection Requirement

[Background] During 1st round discussion, companies are still having different preference on below three options for uni- and bi-directional deployment scenarios to be used, which impacts the channel model for the tests.

  - Option 1: Apply unidirectional deployment channel model, AWGN with 9722Hz frequency offset

    - Supported by Samsung, OPPO

  - Option 2: Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset

    - Supported by QC, Ericsson, Nokia, CATT, OPPO, Huawei

  - Option 3: Apply both uni-directional and bi-directional to define two TCs respectively

    - Supported by CATT.

Based on the majority view, Option 2 is adopted as tentative agreement.

Tentative Agreement:

- Test Cases for Cell Re-selection Requirement:

  - Apply AWGN with 19444Hz frequency offset

Recommendations for 2nd round:

- Confirm the above tentative agreement.

| Company | Comments |
| --- | --- |
| QC | Support the agreement. |
| Ericsson | Support the agreement. |
| Huawei | Fine with AWGN with 19444Hz frequency offset. In test case the propagation is set as “AWGN with 19444hz offset”, without mentioning bi-directional deployment |
| CATT | Can compromise to have bi-directional deployment only. |
| Samsung | Revision by removing “bi-directional deployment channel model” as commented by Huawei. |

#### Issue 1-3-3: Test Cases for MAC-CE based TCI State Switch Delay Requirement

The following aspects related to TC for MAC-CE based TCI State switch delay requirement are discussed:

Channel model and timing offset:

- Option 1: Apply bi-directional deployment channel model, AWGN with 19444Hz frequency offset

  - Supported by Samsung, Nokia

- Option 2a: Uni-directional with the timing difference is set properly as proposed: Timing offset between SSB 0 and SSB 1 is 2.33us

  - Supported by QC, Ericsson, OPPO, Nokia (if timing requirement is also tested)

- Option 3: Apply unidirectional deployment channel model, AWGN with 9722Hz frequency offset

  - Supported by CATT, Ericsson

Based on 1st round discussion, Option 2a seems the majority view.

DRX cycle:

- Option 1: 40ms

  - No opposition on 40ms as DRX cycle.

Tentative Agreement:

- Test Cases for MAC-CE based TCI State Switch Delay Requirement

  - Channel model and timing offset:

    - Uni-directional with the timing difference is set properly as proposed: Timing offset between SSB 0 and SSB 1 is 2.33us

  - DRX cycle:

    - non-DRX.

Recommendations for 2nd round:

- Confirm the above tentative agreement.

| Company | Comments |
| --- | --- |
| QC | If we have L1 measurement in DRx mode, can we have TCI state switch in non-DRx mode? Note that legacy test is in non-DRx mode, and if we have DRx, the TCI state switch delay has to consider DRx, including the SSB/TRS reception and the DL reception after TCI state switch as requirement. |
| Ericsson | We support recommendations but can consider QC’s suggestions. |
| CATT | Can compromise to uni-directional |
| Nokia | In our contribution, the timing difference (i.e., timing offset between SSB0 and SSB1) is 76*64*Tc, which is derived from our simulation results. We can accept QC’s request to have non-DRx if the timing difference is set to 76*64*Tc. |
| Samsung | As most companies agree non-DRX can be used here instead, considering L1-measurement is drafted based on DRX mode, the proposed agreement is revised. For the timing difference, we assume the timing difference derived based on deployment scenario assumption, i.e., 2.33us = 700 meter/c, is much larger the CP/4. It is okay for us. |

#### Issue 1-3-4: Test Cases for L3 Measurement Requirement

[Background] During 1st round discussion, companies are still having different preference on below two options for DRX configuration:

  - Option 1: non-DRX mode

    - Supported by CATT (if L1 use DRX), Qualcomm, OPPO, Samsung, Huawei

  - Option 2: DRX mode, DRX cycle = 40ms

    - Supported by Ericsson, Nokia, Huawei (if L1 use non-DRX).

Tentative Agreement:

- Test Cases for L3 Measurement

  - Non-DRX mode

- Note: tentative agreement is based on DRX for L1 measurement, and non-DRX for L3-measurement.

Recommendations for 2nd round:

- FFS the above tentative agreement

  - Suggest to consider tentative agreements in Issue 1-3-1 and Issue 1-3-4 together.

  - Note: tentative agreement is based on DRX for L1 measurement, and non-DRX for L3-measurement.

| Company | Comments |
| --- | --- |
| QC | Same comment as 1-3-1 |
| Ericsson | Support recommendations. |
| CATT | Support tentative agreement. |

#### Issue 1-4-1: L1-measurement simulation result and alignment

Based on the available simulation results, the side condition and measurement accuracy are discussed.

It should be noted that there is a spreadsheet for L1/L3 simulation results summary circulated over email reflector by Nokia to have the results be aligned and compared.

Tentative Agreement:

- L1-RSRP measurement:

  - The legacy accuracy of FR2 L1-RSRP measurement can be reused for L1-RSRP measurement in FR2 HST.

- L1-SINR measurement:

  - For the below three categories of L1-SINR measurement (introduced in Rel-16 eMIMO WI):

| CMR | IMR | L1-SINR accuracy requirements |
| --- | --- | --- |
| CSI-RS | N/A | Type 1: CSI-RS based CMR and no dedicated IMR configured |
| SSB | NZP-IMR | Type 2: SSB based CMR and dedicated IMR configured |
|  | CSI-IM |  |
| CSI-RS | NZP-IMR | Type 3: CSI-RS based CMR and dedicated IMR configured |
|  | CSI-IM |  |

    - For Type-2 and Type-3 L1-SINR measurement:

      - The legacy accuracy of FR2 L1-SINR measurement can be reused for L1-SINR measurement in FR2 HST.

        - An upper bound SNR = FFS is defined.

          - FFS FR1 HST side condition can be reused or not

    - For Type-1 L1-SINR measurement:

      - The legacy accuracy of FR2 L1-SINR measurement can be reused for L1-SINR measurement in FR2 HST;

        - An upper bound SNR = FFS is defined.

          - FFS FR1 HST side condition can be reused or not

Recommendations for 2nd round:

- Review and check the L1 measurement simulation result alignment

- Discussion on the above tentative agreement.

| Company | Comments |
| --- | --- |
| QC | There is an agreement reached in FR1 HST, we suggest to reuse it, as the measurement channel model is similar and this accuracy requirement upper bound is due to ICI, which is a function of frequency_offset/center_frequency, not center frequency, band or FR. |
| Ericsson | Similar view to QC, FR1 results can be used here PROVIDED L1-SINR is used in HST FR2, even we concern the validity of L1-SINR in HST FR2 scenario. |
| Nokia | The expression provided by QC is not in line with the one in R4-2211596. The center frequency in the denominator should be SCS in the above expression? From R4-2211596, the expression is frequency offset/SCS Using this expression, legacy requirements for Type 2 and Type 3 L1-SINR accuracy can be reused. This is in line with our preliminary simulation results. Suggest further discussions taking moderator’s recommendations. |
| Samsung | To response QC’s comment, the FFS bullet is added to study FR1 HST side condition can be reused or not. |

#### Issue 1-4-2: L3-measurement simulation result and alignment

Based on the available simulation results, the side condition and measurement accuracy are discussed.

It should be noted that there is a spreadsheet for L1/L3 simulation results summary circulated over email reflector by Nokia to have the results be aligned and compared.

Tentative Agreement:

- For SS-RSRP and SS-RSRQ measurement:

  - The legacy accuracy requirement for Rel-15 FR2 SS-RSRP and SS-RSRQ measurement can be reused.

- For SS-SINR measurement:

  - FFS whether or not Rel-15 FR2 SS-SINR measurement can be reused, by further study:

    - Impact of uni- and bi-directional deployment scenario;

    - SNR upper bound.

      - FFS FR1 HST side condition can be reused or not

Recommendations for 2nd round:

- Review and check the L3 measurement simulation result alignment

- Discussion on the above tentative agreement.

| Company | Comments |
| --- | --- |
| QC | Similar to the previous issue, we suggest to reuse FR1 HST conclusion. |
| Ericsson | Support recommendations. |
| Nokia | Support moderator’s recommendations. |
| Samsung | To response QC’s comment, the FFS bullet is added to study FR1 HST side condition can be reused or not. |

### 1.5.2 CRs/TPs comments collection

[Moderator] The companies responsible for relevant CRs will trigger email discussion with the revised draft CRs. Views and comments will be provided directly under each sub-email threads for CRs.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on RRM performance requirement for FR2 HST | Samsung |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2211678 |  | Draft CR on test case for cell re-selection in HST FR2 | CATT | Merged | Merged to the revision of R4-2212977 |
| R4-2212977 |  | Test case for Cell Re-selection Requirement for FR2 HST | Huawei | Revised |  |
| R4-2211679 |  | Draft CR on test case for MAC-CE based TCI state switch Delay requirement in HST FR2 | CATT | Merged | Merged to the revision of R4-2212472 |
| R4-2213897 |  | CR to TS 38.133: Test cases for gradual timing adjustment, one shot large UL timing adjustment and MAC-CE based TCI state switch for FR2 NR HST | Nokia | Revised | Only focus on gradual timing adjustment part |
| R4-2212472 |  | Draft CR to 38.133 on test case for MAC-CE based active TCI state switch for FR2 PC6 UE | Samsung | Revised | Revised to address open issues, and potentially capture test for one-shot timing adjustment requirement |
| R4-2211680 |  | Draft CR on test case for L3 measurement in HST FR2 | CATT | Merged | Merged to the revision of R4-2213385 |
| R4-2213385 |  | Test cases on Intra-frequency Measurements for HST FR2 | Ericsson | Revised |  |
| R4-2211681 |  | Draft CR on test case for L1-RSRP measurement requirement in HST FR2 | CATT | Merged | Merged to the revision of R4-2213883 |
| R4-2213883 |  | Test cases on Intra-frequency Measurements for HST FR2 | ZTE | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2214470 |  | WF on RRM performance requirement for FR2 HST | Samsung | Agreeable | Further revised based on GTW agreement |
| R4-2214964 (revision of R4-2212472) |  | Draft CR to 38.133 on test case for MAC-CE based active TCI state switch for FR2 PC6 UE | Samsung | Agreeable | Further revised based on GTW agreement |
| R4-2214987 (revision of R4-2212977) |  | Test case for Cell Re-selection Requirement for FR2 HST | Huawei, HiSilicon | Agreeable |  |
| R4-2215050 (revision of R4-2213385) |  | draft CR on Test Cases for L3 Measurement Requirement for HST FR2 | Ericsson | Agreeable |  |
| R4-2215096 (revision of R4-2213883) |  | Draft CR on test case for L1-RSRP measurement for FR2 HST PC6 UE in TS38.133 A.7.6.3 | ZTE Corporation | Agreeable |  |
| R4-2215098 (revision of R4-2213897) |  | CR to TS 38.133: Test cases for gradual timing adjustment, one shot large UL timing adjustment and MAC-CE based TCI state switch for FR2 NR HST | Nokia, Nokia Shanghai Bell | Return To | Still different views on whether/how the introduced changes follows the existing GTW agreement, based on latest revision: R4-2215097 REV OF R4-2213897 CR to 38.133 - FR2 HST RRM grad tst_v1.docx |
| R4-2213894 |  | L1 and L3 simulation results alignment | Nokia, Nokia Shanghai Bell, Samsung | Noted | Based on: R4-2213894 (104e) L3nL1 sim v02_Nok_Samsung.xlsx The large measurement error from one company on SINR-related simulation can be further checked after this meeting. |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents