3GPP TSG-RAN WG4 Meeting # 103-e										R4-2214293

Electronic Meeting, May 09 – May 20, 2022

Agenda item:			9.5.1, 9.5.2 and 9.5.3

Source:	Moderator (CMCC)

Title:	Email discussion summary for [104-e][304] NR_Repeater_RFMaintenance

Document for:	Information

# Introduction

RAN#90e approved a new “New WID on NR Repeaters” with RAN4 as the responsible WG, which includes development of FR1 FDD specifications as well as TDD specifications for FR1 and FR2. The scope of this email discussion focuses on all RF maintenance requirements, the same as the agenda 9.5.1, 9.5.2, 9.5.3 for current meeting.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: discuss the open issues, strive to finish all the open issues and collect comment for draft CRs

- 2nd round: strive to approve all draft CRs.

# Topic #1:	General requirement maintenance

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212309 | CMCC | Draft CR for 38.106: add requirements applicability for LA 1-C repeater |
| R4-2213712 | ZTE Corporation | Proposal 1: to define the repeater in band n104 since the existing requirement defined in TS 38.106 is not applicable for it. |
| R4-2213713 | ZTE Corporation | Draft CR to TS38.106: the introduction of band n104 |

## Open issues summary

Agenda 9.5.1

    - Sub-topic 1-1 supported bands for NR repeater

Issue 1-1: supported bands for NR repeater

- Proposals

  - Option 1: to define the repeater in band n104 since the existing requirement defined in TS 38.106 is not applicable for it. (ZTE)

- Recommended WF

  - Option 1.

    - Sub-topic 1-2 LA requirements applicability

Issue 1-2: LA requirements applicability for downlink

- Proposals

  - Option 1: all RF requirements for LA 1-C also applies for repeater with declared output power less than LA rated maximum power. And emphasize above agreements in the spec as in 2212309 (CMCC)

- Recommended WF

  - Option 1.

GTW discussion on August 19th

Issue 1-2: LA requirements applicability for downlink

- Proposals

  - Option 1: all RF requirements for LA 1-C also applies for repeater with declared output power less than LA rated maximum power. And emphasize above agreements in the spec as in 2212309 (CMCC)

- Discussion:

  - NEC: We think Note is not needed.

  - ZTE: We support the proposal from CMCC.

  - CMCC: We prefer to keep the note to ensure the requirements applicable for Femoto cell Repeater with lower power.

  - Ericsson: We think the note still helpful.

  - Nokia: We are fine to change the repeater specification and shall we make clear understanding the changes to repeater specification has no linkage to BS TS 38.104 specification.

- Agreement: update the repeater spec as below

“Local Area repeaters are characterised by requirements derived from Pico Cell scenarios with a repeater to UE minimum distance along the ground equal to 2 m and/or from Femto Cell scenarios.

Note: The requirements in this specification for LA 1-C repeaters apply to repeaters with declared output power less than or equal to the LA rated output power limits as in table 6.2.1-1.”

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| CMCC | Option 1 is OK |
| Nokia, Nokia Shanghai Bell | OK with option 1. |
| ZTE | Support it |
| Ericsson | OK with option 1 |
| NEC | Ok with option 1. |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| CMCC | Option 1 is preferred. We will deploy repeater with less output power compared with LA repeater, to make current 1-C repeater requirements also apply for such lower power repeater, we suggest to approve option 1. The detailed CR is 2212309. |
| Nokia, Nokia Shanghai Bell | OK with option 1. |
| ZTE | OK with option 1. |
| Ericsson | We agree with the principle of clarifying that repeaters intended home and other low power deployment scenarios and with lower power than LA can be based on the LA requirements. We propose a slight change to the note to mention the deployment scenarios, to make the point that it is not just lower power that is covered but also the home deployment scenario. Note: The requirements in this specification for LA 1-C repeaters also apply to 1-C repeaters that are intended for home or similar deployment scenarios and have declared output power less than the LA rated output power limits as in table 6.2.1-1 |
| CMCC | Thank Ericsson for the update. Changed version is OK for us. |
| NEC | We do not feel comfortable to emphasize it in the spec as in 2212309. It is not a special case that rated output power is less than the maximum power allowed for the class. Why do we need a note for repeater type 1-C, Local area. It would give a wrong impression that it does not apply to Wide Area repeater, for example. |
| CMCC2 | To NEC. The purpose is to include home class or equivalent low power repeater into the spec. as analyzed into R4-2212311, it seems all the RF requirements for LA BS also apply for home BS. Considering most repeater requirements also inherited from BS spec, so we guess 1-C LA repeater requirements also applies for home BS station. If all other requirements are the same, there is no need to explicitly define a new class, e.g. home class into the spec. that’s the reason why we want to reuse LA to cover home class or equivalent low power class. After updating the spec as proposed by Ericsson with emphasize “home or similar deployment scenario”, there is no confusing again. |
| NEC | To CMCC. We guess we understand your intention. However, we are not comfortable with Ericsson text because it seems conflicting with the definition of the Local Area repeaters. How about modifying the definition of LA repeaters as below instead of adding a note? Local Area repeaters are characterised by requirements derived from Pico Cell scenarios with a repeater to UE minimum distance along the ground equal to 2 m and/or Femto Cell scenarios. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212309 all RF requirements for LA 1-C also applies for repeater with declared output power less than LA rated maximum power | Company B: Nokia: Text of note should be modified, as now it is not in the same format as in the whole spec: "Note: The requirements in this specification for LA repeater type 1-C r apply to repeater type 1-C with declared output power less than LA rated output power limits as in table 6.2.1-1." Ericsson: See comment to 1-2 above; we propose an update to the note NEC: See comment for sub topic 1-2. |
| R4-2213713 introduce the licensed band 6425-7125MHz into the repeater spec | CMCC: this CR is OK for us NEC: Ok for us. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | It seems all companies support option 1. Tentative agreement: define the repeater in band n104 since the existing requirement defined in TS 38.106 is not applicable for it. |
| Sub-topic #1-2 | After the discussion on Friday GTW, we have following agreements. Corresponding CR will be revised based on following agreements. - Agreement: update the repeater spec as below “Local Area repeaters are characterised by requirements derived from Pico Cell scenarios with a repeater to UE minimum distance along the ground equal to 2 m and/or from Femto Cell scenarios. Note: The requirements in this specification for LA 1-C repeaters apply to repeaters with declared output power less than or equal to the LA rated output power limits as in table 6.2.1-1.” |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

- Topic #2:	Conductive RF core requirement maintenance

  - Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212260 | NEC | Proposal 1 Nominal channel bandwidth is defined as min(100 MHz, BWpassband) for repeater type 1-C. If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest channel bandwidth for the operating band which is narrower than BWpassband. Proposal 2 Nominal channel bandwidth is defined as min(400 MHz, BWpassband) for repeater type 2-O. If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest channel bandwidth for the operating band which is narrower than BWpassband. Proposal 3 ACLR limit tables shall be updated considering the modified nominal channel bandwidth. Proposal 4 Symbol BWNominal shall be introduce for nominal channel bandwidth. |
| R4-2212261 | NEC | CR to 38.106: NR repeater ACLR requirements |
| R4-2212262 | NEC | CR to 38.106: NR repeater ACRR requirements |
| R4-2212263 | NEC | CR to 38.106: NR repeater receiver spurious emissions requirements |
| R4-2212264 | NEC | CR to 38.106: Removal of unlicensed bands for NR repeaters |
| R4-2212308 | CMCC | Draft CR for 38.106: delete bracket and add declaration of location requirement for1-C LA repeater |
| R4-2212631 | Ericsson | CR to 38.106: Correction of LA ACLR requirements |
| R4-2213980 | Nokia, Nokia Shanghai Bell | CR to TS 38.106 with updates and corrections for conductive part |

  - Open issues summary

Agenda 9.5.2

    - Sub-topic 2-1 normal channel bandwidth

Issue 2-1: normal channel bandwidth

Background information: Nominal channel bandwidth is defined as min(100 MHz, BWpassband) for repeater type 1-C. and min(400 MHz, BWpassband) for repeater type 2-O

- Proposals

  - Option 1: If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest channel bandwidth for the operating band which is narrower than BWpassband. (NEC)

- Recommended WF

  - Option 1.

  - Note: if option 1 is approved, ACLR requirements should be updated accordingly as in R4-2212261. ACRR requirements should be updated accordingly as in R4-2212262.

  - Companies views’ collection for 1st round

    - Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| CMCC | Option 1 is OK for us to facilitate testing. |
| Nokia, Nokia Shanghai Bell | OK with option 1. |
| Ericsson | OK |

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212261 Update ACLR due to update of normal CBW definition | CMCC: this CR is OK for us Nokia: OK with CR ZTE: it’ okay for us. Company B: |
| R4-2212262 Update ACRR due to update of normal CBW definition | CMCC: this CR is OK for us Nokia: OK with CR ZTE: this note might be not correct, narrow than pass band BW/2? WHY If min{100 MHz, passband BW} is not defined for BS channel bandwidth for the operating band, {the widest channel bandwidth for the operating band which is narrower than passband BW}/2 shall be used NEC: To ZTE, if min{100 MHz, passband BW} is not defined for BS channel bandwidth we should use {the widest channel bandwidth for the operating band which is narrower than passband BW}as nominal channel bandwidth to get BWconfig for filter bandwidth. Channel offset of the assumed adjacent channel from the passband edge should be the half of it. Could you explain your concern in more detail? Company B: |
| R4-2212263 receiver spurious emissions | CMCC: this CR is OK for us, deleting 12.75-26GHz frequency range is reasonable since 5th harmonic of certain bands (e.g. n104) will be larger than 26GHz and we doesn’t consider unlicensed band. ZTE: for licensed 6GHz band , we still need this NOTE and please keep 12.75-26GHz frequency Company B: |
| R4-2212264 Remove all unlicensed band related description | CMCC: this CR is OK for us Nokia: OK with CR ZTE: Okay for it. Company B: |
| R4-2212308 LA co-location requirements are not mandatory and delete square bracket | CMCC: this CR is OK for us. Nokia: General ok, but CR should use term "for repeaters type 1-C" like is already used in the whole spec, otherwise this is some new term. ZTE: okay for us. NEC: Removing square brackets is ok. But concern on adding “The co-location xxx requirements are not mandatory for LA 1-C repeaters”. We think co-location requirements are not mandatory for any repeater class or for any repeater type. They are applied only when co-located. Is it proposed that co-location requirements may not be applied to co-located repeaters?? CMCC2: to NEC, for LA, we try to also cover home scenario as proposed by Ericsson. For home class, there is no possibility for co-located requirements. that’s why we want to emphasize such co-location requirement is not applicable for 1-C. NEC: Just say “co-location requirements are not mandatory for LA 1-C repeaters” sounds like “there are no co-location requirements for LA 1-C repeaters”. Conditions when requirements are applied/not applied shall be mentioned, at least. Company B: |
| R4-2212631 Correction of LA ACLR requirements | CMCC: Only when UL output power is less than 12dBm/20MHz, absolute ACLR applies. We assume no repeater’s output power is such small. So no UL LA absolute ACLR requirement is considered. We agree to add table 6.5.2.2-3a for non-contiguous relative ACLR for LA UL. There is a typo as below in this CR. For repeater type 1-C, for DL (all repeater classes), and for UL for WA class, either the ACLR (CACLR) absolute minimum requirements in table 6.5.2.2-2, 6.5.2.2-5 or else the relevant the ACLR (CACLR) limits in table 6.5.2.2-1, 6.5.2.2-3 or 6.5.2.2-4, whichever is less stringent, shall apply for each antenna connector. For UL for LA class, the ACLR (CACLR) limits in table 6.5.2.2-1a, 6.5.2.2-3a or 6.5.2.2-4a shall apply. |
| R4-2213980 introduce correction in general clauses and conducted part of specification. | CMCC: this CR is OK for us NEC: Concern on the reference clause. Reference numbers shall be kept and voided. Company B: |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 | All companies support option 1. Tentative agreements: If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest channel bandwidth for the operating band which is narrower than BWpassband. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

  - Discussion on 2nd round (if applicable)

- Topic #3:	Radiated RF core requirement maintenance

  - Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212310 | CMCC | Draft CR for 38.106: delete bracket for radiated related requirements |
| R4-2212632 | Ericsson | CR to 38.106: Correction of LA ACLR requirements |
| R4-2213981 | Nokia, Nokia Shanghai Bell | CR to TS 38.106 with updates and corrections for radiated part |

  - Open issues summary

Agenda 9.5.3.

    - Sub-topic 3-1 FR2 LA UL absolute ACLR/CACLR

Issue 3-1: whether to define FR2 LA UL absolute ACLR/CACLR limit.

- Proposals

  - Option 1: no.

  - Option 2: yes, the same as LA DL, i.e. -20dBm/MHz

- Recommended WF

  - TBD.

  - Companies views’ collection for 1st round

    - Open issues

Sub topic 3-1

| Company | Comments |
| --- | --- |
| CMCC | It seems there is no need for such requirements. absolute ACLR applies only when UL output power is less than -4dBm/MHz=16dBm/100MHz, we are afraid such output power is so small that no repeater will use this value. |
| Ericsson | CMCC have a good explanation and it is OK for us to keep with no absolute limit for LA. |

.

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212310 delete bracket for radiated related requirements | CMCC: in last meeting, it is approved that if no further comment is received, we could remove the square bracket for scaling factor X in TRP output power limits. If no further comments after 1st round discussion, we will delete square bracket. Nokia: OK with CR ZTE: okay with CR NEC: Ok with CR. |
| R4-2212632 Correction of LA ACLR requirements | CMCC: this CR is OK for us Nokia: OK with CR ZTE: OK with CR |
| R4-2213981 | CMCC: this CR is OK for us ZTE: okay for it. |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #3-1 | All companies support option 1. Tentative agreements: No absolute ACLR/CACLR limit for FR2 LA UL. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

  - Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2212309 |  | Draft CR for 38.106: add requirements applicability for LA 1-C repeater | CMCC | revised |  |
| R4-2213712 |  | Further discussion on the supported bands for NR repeater | ZTE Corporation | noted |  |
| R4-2213713 |  | Draft CR to TS38.106: the introduction of band n104 | ZTE Corporation | Agreeable |  |
| R4-2212260 |  | Discussion onNR repeater ACLR requireents | NEC | noted |  |
| R4-2212261 |  | CR to 38.106: NR repeater ACLR requirements | NEC | Agreeable |  |
| R4-2212262 |  | CR to 38.106: NR repeater ACRR requirements | NEC | Wait for more discussion in 2nd round. | It seems ZTE has some comment on this CR but NEC doesn’t get the point of ZTE’s comment. |
| R4-2212263 |  | CR to 38.106: NR repeater receiver spurious emissions requirements | NEC | revised | 12.75-26GHz should be kept for n104 |
| R4-2212264 |  | CR to 38.106: Removal of unlicensed bands for NR repeaters | NEC | Agreeable |  |
| R4-2212308 |  | Draft CR for 38.106: delete bracket and add declaration of location requirement for1-C LA repeater | CMCC | revised | Refining the wording of “co-location is not mandatory for LA” Updating non-contiguous related description in 4.6 Add inside passband OBUE requirements |
| R4-2212631 |  | CR to 38.106: Correction of LA ACLR requirements | Ericsson | revised | A typo should be updated. |
| R4-2213980 |  | CR to TS 38.106 with updates and corrections for conductive part | Nokia, Nokia Shanghai Bell | revised | if we delete some references, remaining reference should be renumbered. otherwise, such reference number should be kept and component voided. |
| R4-2212310 |  | Draft CR for 38.106: delete bracket for radiated related requirements | CMCC | Agreeable |  |
| R4-2212632 |  | CR to 38.106: Correction of LA ACLR requirements | Ericsson | Agreeable |  |
| R4-2213981 |  | CR to TS 38.106 with updates and corrections for radiated part | Nokia, Nokia Shanghai Bell | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2212309 | R4-2214767 | Draft CR for 38.106: add requirements applicability for LA 1-C repeater | CMCC | Endorsement |  |
| R4-2213712 |  | Further discussion on the supported bands for NR repeater | ZTE Corporation | Noted |  |
| R4-2213713 |  | Draft CR to TS38.106: the introduction of band n104 | ZTE Corporation | Endorsement |  |
| R4-2212260 |  | Discussion onNR repeater ACLR requireents | NEC | Noted |  |
| R4-2212261 |  | CR to 38.106: NR repeater ACLR requirements | NEC | Endorsement |  |
| R4-2212262 |  | CR to 38.106: NR repeater ACRR requirements | NEC | Noted |  |
| R4-2212263 | R4-2214529 | CR to 38.106: NR repeater receiver spurious emissions requirements | NEC | Endorsement |  |
| R4-2212264 |  | CR to 38.106: Removal of unlicensed bands for NR repeaters | NEC | Endorsement |  |
| R4-2212308 | R4-2214766 | Draft CR for 38.106: delete bracket and add declaration of location requirement for1-C LA repeater | CMCC | Endorsement |  |
| R4-2212631 | R4-2214532 | CR to 38.106: Correction of LA ACLR requirements | Ericsson | Endorsement |  |
| R4-2213980 | R4-2214555 | CR to TS 38.106 with updates and corrections for conductive part | Nokia, Nokia Shanghai Bell | Endorsement |  |
| R4-2212310 |  | Draft CR for 38.106: delete bracket for radiated related requirements | CMCC | Endorsement |  |
| R4-2212632 |  | CR to 38.106: Correction of LA ACLR requirements | Ericsson | Endorsement |  |
| R4-2213981 |  | CR to TS 38.106 with updates and corrections for radiated part | Nokia, Nokia Shanghai Bell | Endorsement |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Bartlomiej Golebiowski | Bartlomiej.golebiowski@nokia.com |
| NEC | Tetsu Ikeda | tetsu.ikeda@nec.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)