3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214316

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.17.4

Source:	Moderator (Samsung)

Title:	Email discussion summary for [104-e][327] NR_FeMIMO_Demod

Document for:	Information

# Introduction

Rel-17 NR FeMIMO WI is a RAN1 leading WI with below major enhancement in RAN1 area

- Enhancement on multi-beam operation

- Enhancement on multi-TRP

- Enhancement on SRS

- Enhancement on CSI reporting

In the last meeting, the scope of performance requirement of NR FeMIMO was under discussion and the related agreement was summarized as following table

| Items |  | BS demodulation | UE demodulation | CSI |
| --- | --- | --- | --- | --- |
| Enhancement on multi-beam operation |  | NO | NO | NO |
| Enhancements on multi-TRP | Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH | NO | M-TRP PDCCH repetition | NO |
|  | Enhancements on Multi-TRP inter-cell operation | NO | No | NO |
|  | Enhancements on beam management for multi-TRP | NO | NO | NO |
|  | Enhancements on HST-SFN deployment | NO | PDSCH for SFN scheme A FFS for SFN scheme B | NO |
| Enhancement on SRS |  | NO | NO | NO |
| Enhancement on CSI reporting | M-TRP | NO | NO | CSI for M-TRP |
|  | FDD reciprocity | NO | NO | FFS PMI for enhanced Type II port selection codebook |

Based on the RAN1 feature and work plan of NR FeMIMO, the scope of this email discussion mainly focuses to identify the test scope of performance requirements of NR FeMIMO, identify the potential impact of the UE demodulation requirements and CSI requirements. Meanwhile, the initial simulation assumption also should be discussed to facilitate the test case setup for requirements

In practical, the scope of this email discussion is indicated as follows agenda:

- UE Demodulation and CSI requirements (9.17.4)

- Demodulation requirement (9.17.4.1)

- Enhancement on HST-SFN scenario (9.17.4.1.1)

- Enhancement on Multi-TRP (9.17.4.1.2)

- CSI requirement (9.17.4.2)

- CSI reporting for multi-TRP (9.17.4.2.1)

- Rel-17 eType II port selection codebook (9.17.4.2.2)

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Finalize the test scope for remaining issue of UE demodulation and CSI parts test cases

- 2nd round: Finalize the test setup and simulation assumption for UE demodulation and CSI parts test cases

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung (Moderator) | Yunchuan Yang | yc0301.yang@samsung.com |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| MediaTek | Hannu Vesala | hannu.vesala@mediatek.com |
| Nokia, Nokia Shanghai Bell | Karsten Petersen | Karsten.petersen@nokia-bell-labs.com |
| Anritsu | Osamu Yamashita | Osamu.Yamashita@anritsu.com |
| Qualcomm | Jahidur Rahman | rahman@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: Demodulation requirement for Multi-TRP enhancement

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211872 | Apple | Proposal 1: Define multi-TRP PDCCH requirements with balanced SNR method. Proposal 2: Define PDCCH demod requirements with mTRP enhancement with AL2. |
| R4-2212112 | Nokia, NSB | Proposal 1: Introduce PDCCH requirements for multi-TRP repetition transmission assuming UE soft-combining Observation 1: Simulation conditions must be aligned for deciding on the on the aggregation level for the test set-up for PDCCH requirements for multi-TRP repetition transmission. Observation 2: The main purpose of the PDCCH test set-up is to verify the UE processing performance during multi-TRP repetition transmission while TRP blocking in general is more related to system performance. Proposal 2: The SNR simulation settings for each TRP should be balanced with no blocking considered. Observation 3: An aggregation level of AL=4 is expected to provide 2-4 dB improvement over AL=2, however the SNR operation condition might be satisfied with sufficient margin for AL=2 depending on the simulation results. Proposal 3: Decide on the aggregation level for the test set-up for PDCCH requirements for multi-TRP repetition transmission after company alignment during RAN4#104e. |
| R4-2212448 | MTK | Proposal 1: We support Option 2 to define PDCCH requirement for multi-TRP repetition transmission scheme without UE soft-combing assumption. Proposal 2: We support Option 1 for SNR setting for each TRP that corresponds balanced SNR. Proposal 3: We prefer Option 1 for aggregation level that corresponds value 2. |
| R4-2212551 | Ericsson | Observation 1: There is a big performance gap between applying soft-combining or not. Proposal 1: Introduce one PDCCH requirement for Multi-TRP repetition transmission schemes with assuming UE doing soft-combining Observation 2: The SNR for achieving 1% BLER with aggregation level AL=4 is too low to define requirement. Proposal 2: Consider AL = 2 to define performance requirement Proposal 3: Consider balance SNR for each TRP |
| R4-2212559 | Qualcomm | Proposal 1: Option 2 (Introduce PDCCH requirements for multi-TRP repetition transmission without UE soft-combining) Proposal 2: Option 1 (Select AL assuming operating SNR > -4 dB) Proposal 3: Option 1 (Choose AL=2) |
| R4-2212560 | Qualcomm | simulation results for multi-TRP PDCCH repetition scheme |
| R4-2213840 | Huawei, HiSilicon | Proposal 1: Define PDCCH requirement with UE soft-combining for multi-TRP repetition transmission schemes Proposal 2: Select AL2 for PDCCH with FDM repetition scheme. Proposal 3: Do not consider TRP blocking for multi-TRP PDCCH enhancement |

## Open issues summary

Last RAN4 meeting agreements in the WF R4-2210670

List of open issues

- Sub-topic 1-1: Test Setup for PDCCH requirement for Enhancement on Multi-TRP

  - Issue 1-1-1: Receiver Assumption for PDCCH requirement for multi-TRP repetition transmission

  - Issue 1-1-2: Aggregation Level

  - Issue 1-1-3: Operating SNR condition for selection AL

  - Issue 1-1-4: SNR setting for each TRP

### Sub-topic 1-1: Test Setup for PDCCH requirement for Enhancement on Multi-TRP

Issue 1-1-1: Receiver Assumption for PDCCH requirement for multi-TRP repetition transmission

- Observations

  - Observation 1 (Ericsson): There is a big performance gap between applying soft-combining or not.

- Proposals

  - Option 1 (Ericsson, Huawei, Nokia): With UE soft-combining

  - Option 2 (Qualcomm, MTK): Without UE soft-combining

- Recommended WF

  - Encourage comments if any

  - Define PDCCH requirement for multi-TRP repetition transmission with additional margin. Soft-combining is up to UE implementation, FFS on the value of additional margin, pending on the alignment results?

  - Encourage companies further align the simulation results during RAN4#104-E meeting

| Duplex | Antenna configuration | Aggregation level |  |  |  |  | SNR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Huawei |  | MTK (without) | MTK (with soft-combining) |  | MTK (without)-3dB | MTK (with soft-combining)-3dB | Apple | Qualcomm | Qualcomm-3dB | Ericsson |
| FDD | 2x2 | 2 | -1.71 |  | 4.03 | 2.41 |  | 1.03 | -0.59 | -1.4 | 3.56 | 0.56 |  |
| FDD | 2x4 | 2 | -5.78 |  | 0.07 | -1.50 |  | -2.93 | -4.5 | -5.2 | -0.5 | -3.5 |  |
| TDD | 2x2 | 2 | -1.37 |  | 4.15 | 2.57 |  | 1.15 | -0.43 | -1.2 | 4.45 | 1.45 |  |
| TDD | 2x4 | 2 | -5.73 |  | 0.11 | -1.40 |  | -2.89 | -4.4 | -4.8 | 0.22 | -2.87 |  |

- GTW agreement

Issue 1-1-2: Aggregation Level

- Observations

  - Observation 1 (Ericsson): The SNR for achieving 1% BLER with aggregation level AL=4 is too low to define requirement.

  - Observation 2 (Nokia):

    - Simulation conditions must be aligned for deciding on the on the aggregation level for the test set-up for PDCCH requirements for multi-TRP repetition transmission.

    - An aggregation level of AL=4 is expected to provide 2-4 dB improvement over AL=2, however the SNR operation condition might be satisfied with sufficient margin for AL=2 depending on the simulation results.

- Proposals

  - Option 1 (Apple, Qualcomm, Ericsson, Huawei, MTK, [Nokia]): AL=2

  - Option 2 (Nokia): Decide on the aggregation level for the test set-up for PDCCH requirements for multi-TRP repetition transmission after company alignment during RAN4#104e

- Recommended WF

  - AL =2

Issue 1-1-3: Operating SNR condition for selection AL

- Proposals

  - Option 1(Qualcomm): SNR > [-4] dB

- Recommended WF

  - Apply SNR > [-4] dB for operating SNR condition for selection AL

Issue 1-1-4: SNR setting for each TRP

- Proposals

  - Option 1 (Apple, Nokia, MTK, Huawei, Ericsson): The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP, without considering TRP blocking for multi-TRP PDCCH enhancement

- Recommended WF

  - Do not consider TRP blocking for PDCCH requirement for Multi-TRP enhancement. The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP

- GTW agreement

  - Do not consider TRP blocking for PDCCH requirement for Multi-TRP enhancement. The SNRs for TRP #1 and TRP #2 are assumed to be balanced for the transmitted signal from each TRP

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1 Issue 1-1-2 Issue 1-1-3 Issue 1-1-4 |
| Apple | Issue 1-1-1 The span is results is large. We are fine with defining requirements with additional margin to accommodate possible UE implementations. Issue 1-1-2 We are fine with the recommended WF. Issue 1-1-3 We are fine with the recommended WF. Issue 1-1-4 We are fine with the recommended WF. We would like to double check the necessity and assumption of 1/sqrt(2) scaling on each TRP. We didn’t have any such assumption for mTRP PDSCH demod requirements. |
| Ericsson | Issue 1-1-1 Pleases find our simulation results (AL=2): Duplex; Antenna configuration; Aggregation level; SNR (dB). Ericsson(with soft-combining); Ericsson(without soft-combining). FDD; 2x2; 2; -3.8; -2.4. Given the big performance difference among different companies’ results, we are fine with the recommended WF to consider additional margin and leave soft-combining to UE implementation. Issue 1-1-2 Fine with the recommended WF. Issue 1-1-3 Fine with the recommended WF. Issue 1-1-4 Fine with the recommended WF. Further check on the assumption of scaling factor 1/sqrt(2). |
| Huawei | Issue 1-1-1: Receiver Assumption for PDCCH requirement for multi-TRP repetition transmission For the receiver assumption, since we have UE capability to report 2 or 3 of the BD number, we can define two SNRs in one case that the requirements with soft-combining assumption are applied if UE report 3 BDs and the requirements with non-soft-combining assumption are applied if UE report 2 BDs. Otherwise the requirements will be too relaxed for UE with soft-combining if we define one SNR requirement with additional margin, since there is very large gain (about 3dB) from soft-combining. For the simulation results alignment, currently there is still large span about the simulation results even for the same receiver assumption. It is needed for further alignment. We suggest that interesting companies can provide both soft-combining and non-soft-combining results for alignment. Issue 1-1-2: Aggregation Level We are OK with the recommended WF. Issue 1-1-3: Operating SNR condition for selection AL We are OK with the recommended WF. Issue 1-1-4: SNR setting for each TRP Considering that the resource for different TRP are not overlapped, the scaling factor of 1/sqrt(2) should not be considered. |
| Samsung | Issue 1-1-1: Receiver Assumption for PDCCH requirement for multi-TRP repetition transmission We think option 1 is more reasonable as with soft-combining requirement could cover more scenarios than without soft-combining case. Extra margin can be considered to accommodate different UE implementation. Based on the collected results, large span observed better further effort to align the assumption and results required. Issue 1-1-2: Aggregation Level We support the recommended WF. Issue 1-1-3: Operating SNR condition for selection AL We support the recommended WF. Issue 1-1-4: SNR setting for each TRP We support the recommended WF. |
| MediaTek | Issue 1-1-1 There seems to be large span in simulation results. Therefore we would like other companies to clarify if they have included additional 1/sqrt(2) scaling on each TRP in simulation assumptions. We have assumed this additional scaling in our results. We support Option 2 to define requirements without UE soft-combining assumption to make requirements applicable for all UE implementations. Our simulations show approximately 1.5dB performance difference with and without soft-combining. Issue 1-1-2 We are fine with the recommended WF. Issue 1-1-3 We are fine with the recommended WF. Issue 1-1-4 We are fine with the recommended WF. We would also like to check need of 1/sqrt(2) scaling on each TRP. |
| Nokia, Nokia Shanghai Bell | Issue 1-1-1 We support that the PDCCH requirement for multi-TRP repetition transmission should be defined assuming UE soft combining. Any additional requirement margin (as proposed in the WF) should be based on simulation result alignment assuming soft-combining. We don’t see soft-combining as an advanced receiver scheme, but rather the baseline of receivers. Issue 1-1-2 Based on the provided simulation results among companies we agree to the recommended WF (AL=2) Issue 1-1-3 We agree to the recommended WF (Operating SNR > [-4] dB) Issue 1-1-4 We agree with the intention of the WF to not include blocking. However, we agree with companies that the assumption of 1/sqrt(2) is not immediately evident, and need to do further checking on this part. More input from proponents of this factor would be appreciated. |
| Qualcomm | Issue 1-1-1 We support option 2. We note that soft combining is an optional feature based on the UE capability, e.g., for BD factor 3. We think that RAN4 test cases should not be designed with an optional feature. Additionally, RAN1 did not make any explicit agreement on how BD factors would map to certain decoding assumptions, and we don’t see a necessity for this in RAN4. Issue 1-1-2 We are okay with the recommended WF. Issue 1-1-3 We are okay with the recommended WF. Issue 1-1-4 We are okay with the recommended WF. However, we agree that further checking is needed in regard to TRP scaling and align across companies. Also, we would like to clarify that our results include the TRP scaling. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: Receiver Assumption for PDCCH requirement for multi-TRP repetition transmission - Proposals - Option 1 (Ericsson, Huawei, Nokia, Samsung): With UE soft-combining - Option 2 (Qualcomm, MTK): Without UE soft-combining - Option 3 (Samsung, Apple, Ericsson): left to UE implementation, additional margin can be considered GTW agreement - Specify requirements without specific receiver assumption, additional margin can be considered for UE supporting BD =2 - FFS whether specific receiver assumption shall be considered for UE supporting BD =3 - Option 1: Specify requirements without specific receiver assumption, additional margin can be considered - Option 2: Assuming soft-combining Candidate options: - FFS whether specific receiver assumption shall be considered for UE supporting BD =3 - Option 1: Specify requirements without specific receiver assumption, additional margin can be considered - Option 2: Assuming soft-combining Recommendations for 2nd round: - Simulation results summary among companies with non-considering scaling factor - Results from three companies are well aligned within 1.5db gap for soft-combining - Around 2.8dB performance gap between the best result (with soft-combing) and worst results (without soft-combining), additional [0.8] dB margin can be considered - Companies are encouraged to provide the updated results in the 2nd round - Based on simulation results, encourage companies can check whether option 1 can be agreeable to move forward Duplex; Tx/RX; HW （w）; MTK (w0); MTK (w); MTK (w0)-3dB; MTK (w)-3dB; Apple (w); QC (wo); QC(wo-3dB); Gap (w); Gap (w0); Gap(w wo. FDD; 2x2; -1.71; 4.03; 2.41; 1.03; -0.59; -1.4; 3.56; 0.56; 1.12; 0.47; 2.74. FDD; 2x4; -5.78; 0.07; -1.50; -2.93; -4.5; -5.2; -0.5; -3.5; 1.28; 0.57; 2.85. TDD; 2x2; -1.37; 4.15; 2.57; 1.15; -0.43; -1.2; 4.45; 1.45; 0.94; 0.30; 2.82. TDD; 2x4; -5.73; 0.11; -1.40; -2.89; -4.4; -4.8; 0.22; -2.87; 1.33; 0.02; 2.86. Issue 1-1-2: Aggregation Level - Proposals - Option 1 (Apple, Qualcomm, Ericsson, Huawei, MTK, [Nokia]): AL=2 - Option 2 (Nokia): Decide on the aggregation level for the test set-up for PDCCH requirements for multi-TRP repetition transmission after company alignment during RAN4#104e Tentative agreements: - AL=2 Issue 1-1-3: Operating SNR condition for selection AL - Proposals - Option 1(Qualcomm): SNR > [-4] dB Tentative agreements: - Apply SNR > -4 dB for operating SNR condition for selection AL Issue 1-1-4: SNR setting for each TRP - Proposals - Option 1 (Apple, Nokia, MTK, Huawei, Ericsson): The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP, without considering TRP blocking for multi-TRP PDCCH enhancement Tentative agreements: - The SNRs for TRP #1 and TRP #2 are assumed to be balanced for the transmitted signal from each TRP, without considering TRP blocking for multi-TRP PDCCH enhancement GTW agreement - No need to consider scaling factor for PDCCH Multi-TRP test case with FDM scheme. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Issue 1-1-1: Receiver Assumption for PDCCH requirement for multi-TRP repetition transmission

- Proposals

  - Option 1 (Ericsson, Huawei, Nokia, Samsung): With UE soft-combining

  - Option 2 (Qualcomm, MTK): Without UE soft-combining

  - Option 3 (Samsung, Apple, Ericsson): left to UE implementation, additional margin can be considered

GTW agreement

- Specify requirements without specific receiver assumption, additional margin can be considered for UE supporting BD =2

- FFS whether specific receiver assumption shall be considered for UE supporting BD =3

  - Option 1: Specify requirements without specific receiver assumption, additional margin can be considered

- Option 2: Assuming soft-combining

Candidate options:

- FFS whether specific receiver assumption shall be considered for UE supporting BD =3

  - Option 1: Specify requirements without specific receiver assumption, additional margin can be considered

  - Option 2: Assuming soft-combining

Recommendations for 2nd round:

- Simulation results summary among companies with non-considering scaling factor

- Results from three companies are well aligned within 1.5db gap for soft-combining

- Around 2.8dB performance gap between the best result (with soft-combing) and worst results (without soft-combining), additional [0.8] dB margin can be considered

- Companies are encouraged to provide the updated results in the 2nd round

- Based on simulation results, encourage companies can check whether option 1 can be agreeable to move forward

| Duplex | Tx/RX | HW （w） | MTK (w0) | MTK (w) | MTK (w0)-3dB | MTK (w)-3dB | Apple (w) | QC (wo) | QC(wo-3dB) | Gap (w) | Gap (w0) | Gap(w wo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 2x2 | -1.71 | 4.03 | 2.41 | 1.03 | -0.59 | -1.4 | 3.56 | 0.56 | 1.12 | 0.47 | 2.74 |
| FDD | 2x4 | -5.78 | 0.07 | -1.50 | -2.93 | -4.5 | -5.2 | -0.5 | -3.5 | 1.28 | 0.57 | 2.85 |
| TDD | 2x2 | -1.37 | 4.15 | 2.57 | 1.15 | -0.43 | -1.2 | 4.45 | 1.45 | 0.94 | 0.30 | 2.82 |
| TDD | 2x4 | -5.73 | 0.11 | -1.40 | -2.89 | -4.4 | -4.8 | 0.22 | -2.87 | 1.33 | 0.02 | 2.86 |

GTW agreement

- Specify requirements without specific receiver assumption, additional margin will be included:

  - Additional margin

    - Option 1: [0.8] dB

    - Option 2: [1] dB

    - Other options not precluded

| Company | Comments |
| --- | --- |
| Apple | We are fine with the recommended WF and to use 0.8 dB as additional margin as a starting point. |
| Huawei | As per the shared results from companies, the results with soft-combing from most companies are well aligned, so it is feasible to define the PDCCH requirements with the soft-combing assumption. |
| Ericsson | We support to assume ‘with soft-combining’ for UE reports BD = 3. We think it is reasonable to consider soft-combining when UE is reporting BD = 3, which should indicate the assumption of UE decoding each PDCCH candidate individually, and also decodes the combined candidate. If such UE is not capable for doing the soft-combining, what is the meaning of it reports BD = 3? Besides that, one thing we would like to clarify is that the Rel-17 PDCCH repetition is an optional feature subject to UE capability. What UE will do when it declares its support is not and will never be defined in RAN1. It is RAN4’s responsibility to specify the assumptions and performance requirement. If there are performance gap then companies are encouraged to align the simulation assumption, e.g., receiver assumption to better help align the results in order to further define the requirement. We don’t think it is a solid argument that since ‘soft-combining’ is an optional UE capability, it can’t be considered just because we are defining the minimum requirement. The ‘soft-combining’ is just one of the possible UE receiver implementations, which is not even present in Rel-17 RAN1 spec. It is only an assumption, and whether we should assume the same receiver assumption when defining requirement is up to whether there is a big performance gap. Now we are discussing to define demodulation requirement and there is indeed performance gap among companies’ results. In this case, either we align the receiver assumption among companies, or we add additional margin to make up for it. One compromised way forward would be no assumption for whatever UE (UE reports 2BDs and UE reports 3BDs, no differentiation) and apply some additional margins. After looking into the simulation results summary table, we found that there are only two companies share their results for assuming ‘without soft-combining’. To better realize the gap between with and without soft-combining among companies results, we encourage companies to submit their results for with and without soft-combining assumption and then decide the additional margin. |
| MediaTek | Recommended WF looks fine to us as a starting point. As Ericsson suggests it would be good if more companies could provide simulation results without soft-combining assumption. Then we would have complete picture of performance. We need to have enough margin for UEs not assuming soft-combining for final requirement. I checked results in R4-2214404 xls and Qualcomm numbers do not match table in this email. Could someone confirm which numbers are correct? |
| Qualcomm | We support option 1. We would like to mention that soft-combining is not a mandatory feature for BD factor 3. Therefore, we don’t think RAN4 should mandate such an assumption for the UE implementation. We also think that recommended WF should reflect the FFS item and additional margin, so we recommend to move everything else out of the recommended WF. Since the spread is quite large, we propose to add 1dB margin. To MTK: Thanks for your comment. We also noticed that same--our PDCCH results do not seem to be captured correctly in R4-2214404 xls. However, the results in the email thread are correct, which are captured in R4-2214212. To Moderator: We have updated R4-2214404 xls to include our results as captured in R4-2214212. |
| Samsung | Thanks MTK and QC, Sorry I had made a mistake when copying QC’s PDCCH results, and I noticed QC also add results for HST cases. Thanks a lot! About the gap between with and without soft-combing “Around 2.8dB performance gap between the best result (with soft-combing) and worst results (without soft-combining)”, we’d better to use the gap between the same company. Current 2.8dB seems come from HW (with) and MTK (without), we think this is a little bit inappropriate as different companies may have different SINR levels. We are open to define two sets of requirements, separately for BD=2 and BD=3. But considering the number of test cases, we prefer option 1. As from the simulation results of MTK, it seems the SINR gap between with and without soft-combing is a fix value (about 1.6dB). |
| Nokia, Nokia Shanghai bell | As Ericsson points out the Rel-17 PDCCH repetition is an optional feature subject to UE capability. We believe it is feasible to assume UE soft combining for defining the PDCCH requirements for multi-TRP repetition transmission (option 2). However, ultimately it does not seem to be needed to decide on the combining scheme, as we can account for the resulting increase in simulation result variability, by adding [0.8] or [1.0] dB on the average SNR result as an additional margin. Finally, we are quite reluctant to capture an implied direct link between “numBD-twoPDCCH-r17” and the soft-combining capabilities of a device in the WF or the specification. |

# Topic #2: Demodulation requirement for Enhancement on HST-SFN scenario

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211936 | CMCC | Proposal 1: for scheme A, it is proposed to define PDSCH requirements with 870Hz doppler shift and MCS17. Proposal 2: for scheme B, it is proposed to define PDSCH requirements, and applicability rule is introduced. Proposal 3: for pre-compensation modeling, it is proposed to consider frequency error between two RRHs. |
| R4-2211937 | CMCC | Simulation Result for HST-SFN scheme A |
| R4-2212395 | NTT DoCoMo | Proposal 1: As for TDD requirements of Rel-17 HST SFN scheme A, define MCS17 rank 2. Proposal 2: As for FDD requirements of Rel-17 HST SFN scheme A, define 972Hz + MCS13 rank2. |
| R4-2212447 | MTK | Proposal 1: We support Option 2 not to introduce the PDSCH requirement with HST-SFN scheme B. Observation 1: UE cannot achieve relative 70% throughput with Rank2 MCS17 and maximum doppler 972Hz. Observation 2: UE has problems to achieve relative throughput of higher than 90% with Rank2 MCS17. Proposal 2: We support Option 1 870Hz for maximum doppler shift. Proposal 3: We prefer MCS 13 and Rank 2. Proposal 4: We support UE capability proposal in Issue 2-2-3. Observation 3: TRS slots have higher BLER due to higher code rate. Observation 4: The last code block has higher BLER compared to other code blocks. |
| R4-2212547 | Qualcomm | Proposal 1: Option 2 (Not to introduce the PDSCH requirement with HST-SFN B) Proposal 2: (15KHz SCS) Option 1: 870 Hz |
| R4-2212549 | Ericsson | Observation 1: Configuring MCS13 can reach 70% max Tput at a much lower SNR in comparison to the performance of MCS17. Observation 2: A much higher SNR (around 19dB with 972Hz) is needed to reach 70% maximum throughput for MCS17 compared to MCS13. After the impairment margin, possibly the requirement will be over 20dB. Observation 3: No clear performance difference between configuring 870Hz and 972Hz with MCS13. Approximately 2dB gap with MCS17. Proposal 1: Propose to select either combination 2: MCS13 + 972Hz for HST-SFN scheme A 15kHz SCS scenario. Observation 4: The UE processing is quite different compared to HST scheme A, B and HST single tap. Observation 5: The performance of HST-SFN scheme B is quite different in comparison to both HST-SFN scheme A and HST Single Tap. Observation6: The SNR for achieving 70% maximum throughput for HST-SFN scheme B is around 11.6dB. Proposal 2: Define PDSCH requirements for HST-SFN scheme B. Proposal 3: Assume perfect modeling of TRP pre-compensation. |
| R4-2212558 | Qualcomm | Simulation result for HST-SFN scheme A |
| R4-2212705 | Samsung | Observation1: for FDD, compared with MCS13, MCS17 has larger SNR difference between maximum Doppler 870Hz and 972Hz. Observation2: for FDD MCS17, the SNR points difference at 70% of peak rate is 2.6~3dB between maximum Doppler 870Hz and 972Hz. Proposal 1: MCS17 and rank2 should be used for PDSCH requirement setup for HST-SFN scheme A with single carrier. Proposal 2: Maximum Doppler shift for PDSCH requirement setup for HST-SFN scheme A with single carrier - FDD, 870Hz - TDD, 1667Hz Proposal 3: SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme A with single carrier Case Number; Duplex; MCS level; Maximum Doppler(Hz); Antenna Configuration; SNR(dB). 1; FDD; 17; 870; 2x2; 13.1. 2; FDD; 17; 870; 2x4; 10.0. 3; TDD; 17; 1667; 2x2; 11.3. 4; TDD; 17; 1667; 2x4; 8.3. Observation3: the SNR point of HST-SFN scheme B with perfect modeling of TRP pre-compensation, is 1.1~2.8dB worse than that of HST single tap channel modeling. Proposal 4: Introduce PDSCH requirements for HST-SFN scenario with scheme B. Proposal 5: For PDSCH requirements for HST-SFN scheme B, both perfect modelling of TRP pre-compensation and delta Doppler shift 0.1ppm*fc modelling are acceptable. Proposal 6: SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme B with single carrier Case Number; Duplex; MCS level; Antenna Configuration; Max Doppler shift; others; SNR(dB). 2; FDD; 17; 2x2; 870Hz; 2 taps and 0Hz delta between 2 taps; 12.8. 3; FDD; 17; 2x2; 870Hz; 2 taps and 0.1ppm delta between 2 taps; 13.6. 5; FDD; 17; 2x4; 870Hz; 2 taps and 0Hz delta between 2 taps; 9.9. 6; FDD; 17; 2x4; 870Hz; 2 taps and 0.1ppm delta between 2 taps; 10.6. 8; TDD; 17; 2x2; 1667Hz; 2 taps and 0Hz delta between 2 taps; 13.8. 9; TDD; 17; 2x2; 1667Hz; 2 taps and 0.1ppm delta between 2 taps; 14.6. 11; TDD; 17; 2x4; 1667Hz; 2 taps and 0Hz delta between 2 taps; 11.2. 12; TDD; 17; 2x4; 1667Hz; 2 taps and 0.1ppm delta between 2 taps; 11.9. |
| R4-2213839 | Huawei, HiSilicon | Proposal 1: Select maximum Doppler 870Hz for 15kHz SCS for HST-SFN scheme A. Proposal 2: Select MCS 17 and rank 2 for HST SFN scheme A Proposal 3: Define PDSCH performance requirements for Scheme B for HST scenario. Proposal 4: If UE supporting both HST SFN scheme A and B and supporting both 15kHz SCS and 30kHz SCS, then UE shall only pass scheme A with 15kHz and scheme B with 30kHz requirements. Do not define any other applicability rule between scheme A and scheme B. Proposal 5: Explicitly consider the TRP pre-compensation model for scheme B requirement definition. - The frequency shift FD,k (Hz) from kth RRH is given by: FD,k=fc×real-v×y-xky-xk×C for k*Ds≤a<k*Ds+Ds and FD,k+1=FD,k+F∆,k+1 for k*Ds≤a<k*Ds+Ds where F∆,k=+0.1 ppm*fc , k mod 2=0-0.1 ppm*fc , k mod 2=1 |
| R4-2213841 | Huawei, HiSilicon | Observation 1: Maximum Doppler 972Hz for 15kHz SCS cannot achieve the maximum throughput for HST SFN scheme A. |

## Open issues summary

Last RAN4 meeting agreements in the WF R4-2207208

List of open issues

- Sub-topic 2-1 Test Scope

  - Issue 2-1-1: Whether to define PDSCH requirement with HST-SFN scheme B

  - Issue 2-1-2: Modelling of TRP pre-compensation for HST-SFN scheme B

- Sub-topic 2-2 Test setup for PDSCH requirement for SFN scheme A with Single Carrier

  - Issue 2-2-1: Maximum Doppler Shift

  - Issue 2-2-2: MCS & Rank

  - Issue 2-2-3: UE capability

### Sub-topic 2-1: Test Scope

Issue 2-1-1: Whether to define PDSCH requirement with HST-SFN scheme B

- Observations

  - Observation 1(Ericsson):

    - The UE processing is quite different compared to HST scheme A, B and HST single tap.

    - The performance of HST-SFN scheme B is quite different in comparison to both HST-SFN scheme A and HST Single Tap.

    - The SNR for achieving 70% maximum throughput for HST-SFN scheme B is around 11.6dB.

- Proposals

  - Option 1 (Samsung, CMCC, Ericsson, Huawei): Introduce the PDSCH requirement with HST-SFN B,

    - Option 1a (CMCC):  with test applicability rule

    - Option 1b(Huawei): If UE supporting both HST SFN scheme A and B and supporting both 15kHz SCS and 30kHz SCS, then UE shall only pass scheme A with 15kHz and scheme B with 30kHz requirements. Do not define any other applicability rule between scheme A and scheme B.

  - Option 2 (Qualcomm, MTK):  Not to introduce the PDSCH requirement with HST-SFN B

- Recommended WF

  - Further discuss the difference compared other transmission schemes from UE processing, deployment scenario and the receiver performance aspect and RAN4 will make decision on RAN4#104-e meeting whether HST SFN scheme B test cases will be specified or not

Issue 2-1-2: Modelling of TRP pre-compensation

- Observations

  - Observation 1(Samsung):

    - the SNR point of HST-SFN scheme B with perfect modeling of TRP pre-compensation, is 1.1~2.8dB worse than that of HST single tap channel modeling

- Proposals

  - Option 1 (Samsung, Ericsson): Assume perfect modeling of TRP pre-compensation

  - Option 2 (Huawei, Samsung, CMCC): using the following method to modelling the Doppler shift during the test

    - The frequency shift FD,k (Hz) from kth RRH is given by:

FD,k=fc×real-v×y-xky-xk×C for k*Ds≤a<k*Ds+Ds

and

FD,k+1=FD,k+F∆,k+1 for k*Ds≤a<k*Ds+Ds

where

F∆,k=+0.1 ppm*fc ,  k mod 2=0-0.1 ppm*fc ,  k mod 2=1

- Recommended WF

  - Encourage comments if any

### Sub-topic 2-2: Test Setup for PDSCH requirement for SFN scheme A with Single Carrier

Issue 2-2-1: Maximum Doppler Shift.

- Observations

  - Observation 1 (Huawei): Maximum Doppler 972Hz for 15kHz SCS cannot achieve the maximum throughput for HST SFN scheme A. Maximum Doppler 972Hz for 15kHz SCS cannot achieve the maximum throughput for HST SFN scheme A.

  - Observation 2 (Samsung):

    - for FDD, compared with MCS13, MCS17 has larger SNR difference between maximum Doppler 870Hz and 972Hz.

    - for FDD MCS17, the SNR points difference at 70% of peak rate is 2.6~3dB between maximum Doppler 870Hz and 972Hz.

    - The performance of HST-SFN scheme B is around 1.2~1.4dB worse than that of HST single tap

  - Observation 3 (Ericsson)

    - A much higher SNR (around 19dB with 972Hz) is needed to reach 70% maximum throughput for MCS17 compared to MCS13. After the impairment margin, possibly the requirement will be over 20dB.

    - No clear performance difference between configuring 870Hz and 972Hz with MCS13. Approximately 2dB gap with MCS17.

  - Observation 4 (MTK)

    - UE cannot achieve relative 70% throughput with Rank2 MCS17 and maximum doppler 972Hz.

- Proposals

  - Option 1(CMCC, Qualcomm, Samsung, Huawei, MTK): 870Hz for 15 KHz SCS

  - Option 2 (NTT DoCoMO): 972Hz for 15 KHz SCS

  - Option 3 (Ericsson): MCS 13+ 972Hz

- Recommended WF

  - Agreement in the last meeting

    - RAN4 will make final decision about the value of Maximum Doppler shift in RAN4#104-e meeting based on the simulation results, e.g. compare the required SNR to achieve 70% of peak rate with MCS17 (or MCS13) with rank 2.

      - Option 1: 870Hz (baseline)

      - Option 2: 972Hz

      - Encourage interested companies provide the simulation results for both Options in RAN4#104-e meeting

  - Based on majority views, encourage companies can check whether 870Hz can be agreeable?

- GTW agreement

  - Define PDSCH requirement with HST SFN scheme A with 870Hz Doppler shift for 15KHz SCS

Issue 2-2-2: MCS & Rank

- Observations

  - Observation1 (Ericsson): Configuring MCS13 can reach 70% max Tput at a much lower SNR in comparison to the performance of MCS17.

  - Observation 2 (MTK):

    - UE has problems to achieve relative throughput of higher than 90% with Rank2 MCS17.

    - TRS slots have higher BLER due to higher code rate.

    - The last code block has higher BLER compared to other code blocks.

- Proposals

  - Option 1 (MTK): MCS 13 with Rank 2

  - Option 2 (NTT DoCoMo)

    - TDD:  MCS 17 with Rank 2

    - FDD:  MCS 13 with Rank 2

  - Option 3 (CMCC, Samsung, Huawei): MCS 17 with Rank 2

- Recommended WF

  - Agreement in the last meeting

    - MCS17 rank 2 as the baseline.

    - RAN4 may consider MCS13 rank 2 according to the simulation results

  - Based on majority views, encourage companies can check whether MCS17 with rank 2 can be agreeable?

  - Simulation result summary

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Samsung | CMCC | Ericsson | Huawei | MTK | QC |
| 1 | FDD | 13 | 870 | 2x2 | 9.5 | 8.8 | 8.5 |  | 8.5 |  |
| 2 | FDD | 13 | 972 | 2x2 | 10.3 | 8.9 | 8.5 |  | 10.0 |  |
| 3 | FDD | 13 | 870 | 2x4 | 6.1 | 6.2 |  |  | 6.4 |  |
| 4 | FDD | 13 | 972 | 2x4 | 6.9 | 6.4 |  |  | 8.0 |  |
| 5 | FDD | 17 | 870 | 2x2 | 13.1 | 13.2 | 17.0 | 14.86 | 17.1 |  |
| 6 | FDD | 17 | 972 | 2x2 | 16.1 | 13.8 | 19.0 | N.A | N/A |  |
| 7 | FDD | 17 | 870 | 2x4 | 10.0 | 10.4 |  | 12.35 | 13.3 |  |
| 8 | FDD | 17 | 972 | 2x4 | 12.6 | 10.5 |  | N/A | N/A |  |
| 9 | TDD | 13 | 1667 | 2x2 | 9.4 |  |  |  | 11.0 |  |
| 10 | TDD | 13 | 1667 | 2x4 | 6.3 |  |  |  | 8.6 |  |
| 11 | TDD | 17 | 1667 | 2x2 | 11.3 |  |  | 14.80 | 19.3 |  |
| 12 | TDD | 17 | 1667 | 2x4 | 8.3 |  |  | 12.32 | 15.0 |  |

Issue 2-2-3: UE capability

- Proposals

  - Option 1 (MTK): The PDSCH demodulation requirements for HST-SFN Scheme A should be applicable for UE capable of “23-6-1 SFN scheme A (scheme 1) for PDSCH and PDCCH”

- Recommended WF

  - Agreement in the last meeting

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1 Issue 2-1-2 |
| Apple | Issue 2-1-1 We still support not to introduce requirements for HST Scheme B. HST Scheme B is based on Doppler compensation at the gNB. If the gNB compensates the Doppler reasonably well for 2nd TRP, it is common understanding that better performance will be observed compared to HST Scheme A. We should introduce gNB requirements for Doppler compensation in HST Scheme B rather than UE demod requirements. |
| CMCC | Issue 2-1-1 Option 1. Scheme B is different from existing Rel-16 HST-SFN joint transmision scheme, HST single tap or scheme A. With this approach of pre-compensation at gNB, even the doppler frequency shift experienced by UE is not so large, but it is noted that there is still residual doppler shift. Since UE still need to handle multi-path with doppler shift, it is better to guarantee UE demodulation performance. As for the applicability rule, it is a compromise way to slove companies’ concern on the number of tests. And for the detail of the applicability rule, we are open to discussion. Issue 2-1-2 Slightly prefer option 2. In our understanding, with doppler pre-compensation at gNB, there is still residual doppler shift, it is too ideal to assume frequency error is zero between two RRHs. It is better that the modeling could reflect the practical scenario. |
| Ericsson | Issue 2-1-1 We support option 1. The pre-compensation of scheme B assumed by RAN1 is only for Doppler shift from the 2nd RRH but the gNB does not compensate the time difference between two RRHs. Therefore, we would like to verify the UE demodulation performance under such circumstance. Meanwhile, scheme A and scheme B are two different UE features and capabilities. The performance of UE declaring its support on scheme B should be verified. Issue 2-1-2 We prefer option 1. But, ok to consider option 2 as well. |
| Huawei | Issue 2-1-1: Whether to define PDSCH requirement with HST-SFN scheme B Option 1 for the following reasaons. 1. Scheme A and scheme B are different UE features defined from RAN1, there is necessity to define requirements for both of them to ensure the test coverage since UE may only support only one of them. 2. For delay, scheme B have two taps with two corresponding TCI for tracking at the same time SFN or single-tap scheme that has only one TCI state. 3. For Doppler, scheme A and scheme B have different propagation conditions that require different UE processing algorithm. For scheme B, UE still needs to estimate the Doppler pre-compensation error between two TRP for channel estimation. Therefore, we prefer to introduce the PDSCH requirement with HST-SFN scheme B. For the applicability rule, to reduce the test effort and to ensure the test coverage at the same time, we propose to reuse the same method as Rel-17 HST WI to test different UE feature with different SCS, i.e. Option 1b. We don’t think other applicability rule should be defined since there are different UE feature and different UE processing for scheme A and scheme B as the reason above. Issue 2-1-2: Modelling of TRP pre-compensation for HST-SFN scheme B We prefer Option 1. The modelling frequency error between different TRP is not to verify TRP performance, but to verify proper UE implementation under such case that is more consistent with the real scenario. Also the modelled frequency error value is reasonable based on following calculation that there is still 2fe1-2fe2 frequency error for the tap from different TRP at UE receiver side that is twice of frequency error that RAN4 considered for Rel-16 normal multi-TRP scenario. In addition, we suggest to change the issue title to “Channel model for HST-SFN scheme B”. TRP#1; TRP#2. 1st; BS DL Tx freq; fc+fe1; fc+fe2. UE DL Rx freq; fc+fe1+fd1; fc+fe2+fd2. UE UL Tx freq; fc+fe1+fd1; -. BS UL Rx freq; fc+fe1+2fd1; fc+fe1+fd1+fd2. BS UL Rx freq offset; 2fd1; fd1+fd2+fe1-fe2. pre-comp freq; -; fd1-fd2-fe1+fe2. 2nd; BS DL Tx freq; fc+fe1; fc+fd1-fd2-fe1+2fe2. UE DL Rx freq; fc+fe1+fd1; fc+fd1-fe1+2fe2. |
| Samsung | Issue 2-1-1: Whether to define PDSCH requirement with HST-SFN scheme B We support option 1. As scheme A is UE based solutions for HST-SFN deployment and scheme B is NW based solution for HST-SFN deployment, for 2 TRPs scenario, UE should handle two paths with two different Doppler shifts for scheme A, while UE should handle two paths with the same Doppler shifts for scheme B. UE has obviously different processing for these two schemes. In addition, test applicable rules between Scheme A and Scheme B can be considered to minimize test effort. Issue 2-1-2: Modelling of TRP pre-compensation From UE processing aspect, there is no difference for both options. We are ok with both options. But from the simplicity of TE aspect, it is more convenience to use option 1. |
| MediaTek | Issue 2-1-1 We still support Option 2 not to introduce requirements for HST-SFN Scheme B. Scheme B has lower doppler compared to Scheme A and testing Scheme A can be considered enough. Apple has good point that if something would be tested for Scheme B that should be doppler compensation in gNB. |
| Qualcomm | Issue 2-1-1 We support option 2. With Scheme B, PDSCH will be Doppler pre-compensated from the network side, therefore it is not expected to be impacting the UE demod processing in a meaningful way. Even in the presence of residual Doppler (after Doppler pre-compensation), UE should be able to deal with it as the residual Doppler is expected to be very small. Similar to Apple/MTK, we also think that perhaps gNB verification of Doppler pre-compensation is more relevant here rather than defining UE requirements for this feature. |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| XXX | Issue 2-2-1 Issue 2-2-2 Issue 2-2-3 |
| Apple | Issue 2-2-1 Based on simulation results from companies, we see that max TP is nor reached with Max Doppler of 972 Hz. We support to use max Doppler of 870Hz for HST Scheme A requirements in FDD. Issue 2-2-2 The agreement was to decide MCS based on simulation results and based on simulation results we observe that max TP is not reached with MCS17 with rank 2. We don’t think it is feasible to define requirements with MCS 17 for rank 2 for this reason. We think MCS 13 is a more reasonable. |
| CMCC | Issue 2-2-1 Option 1. This issue can be considered together with issue 2-2-2, we support 870Hz doppler shift with MCS17 rank 2. According to our simulation results, the maximum throughput can be achieved with 870Hz doppler shift and MCS17. While for the case with 972Hz doppler shift and MCS17, there is performance degradation and the maximum throughput cannot be achieved. Based on above observation, it is proposed to consider 870Hz doppler shift and MCS17. Issue 2-2-2 Option 3. MCS17 with rank 2 is agreed in previous meeting, and according to our simulation results, the maximum throughput can be achieved with 870Hz doppler shift and MCS17. |
| Docomo | Issue 2-2-1 Our preference is Option 3. Based on all simulation results in the contribution, MCS13 + 870Hz can reach the peak throughput. So, this combination is feasible. However, since MCS13 + 870Hz is the same as Rel-16 HST-SFN, we think it would be better to avoid this combination in order to reflect the improvements made by Rel-17 HST-SFN scheme A in the test parameters. In addition, in the contributions of some companies, there are simulation results that can reach the peak throughput even in the case of 972Hz + MCS13 and 870Hz + MCS17. Therefore, we think that these combinations are still considerable. From Japanese operator's point of view, Maximum Doppler shift is considered more important than MCS because this value affects the frequencies that can be operated in commercial NW in 500 km/h. Issue 2-2-2 For TDD, we support MCS 17 with Rank2. For FDD, our first priority is MCS 13 with Rank2. If RAN4 defines 870Hz, we support MCS 17 with Rank2. |
| Ericsson | Issue 2-2-1 and 2-2-2 We share similar view as DOCOMO. It would be better to select higher Doppler shift than Rel-16 HST-SFN to show the improvements brought by scheme A. We prefer to discuss the Doppler shift and MCS together. In Rel-17 HST-SFN scheme A, UE only needs to assume one TRP per TRS. Therefore, theoretically, it is possible to assume the maximum Doppler shift up to 1650Hz for SCS=15kHz, which is same assumption as Rel-16 HST single tap scenario. We also think setting fmax=972Hz is important to guarantee the UE performance with 500km/h in FDD low band (band n1 2.1GHz) as requested by operators. From simulation point of view, from our simulation results, configuring MCS13 and 972Hz can also reach the max throughput, and it can achieve 70% max throughput at a reasonable SNR point (). Based on the above, we support option 3. |
| Huawei | Issue 2-2-1: Maximum Doppler Shift. We prefer Option 1. It is not feasible to consider 972Hz Doppler for 15kHz SCS since it exceeds the maximum UE TRS tracking capability for 15kHz and cause the “Doppler wrap” issue so that UE cannot achieve the maximum throughput. Issue 2-2-2: MCS & Rank We prefer Option 3. As per our evaluation, MCS 17 with rank 2 is feasible for HST SFN scheme A. |
| Samsung | Issue 2-2-1: Maximum Doppler Shift. We support the recommended WF. 870Hz for 15kHz. Issue 2-2-2: MCS & Rank We support the recommended WF. MCS17 with rank 2. |
| MediaTek | Issue 2-2-1 We support Option 1 for maximum doppler shift of 870Hz for 15kHz SCS. We can see performance issues in many simulation results with maximum doppler shift of 972Hz. Lower doppler is more robust option for performance requirement. Issue 2-2-2 We support Option 1 for MCS13 with Rank2. We can see performance issues in some simulation results with higher MCS17 and therefore we would suggest selecting more robust and still adequate MCS13 for final requirements. In previous meeting MCS17 was considered as a baseline but the final decision was left open for more complete simulation results. |
| Qualcomm | Issue 2-2-1 With Scheme A, UE will make use of the estimated Dopplers from two TRSs tracked by two TCI states. We think that the assumption of single-tap model, for which a higher Doppler may be assumed is not applicable here. Hence, we support a lower Doppler, i.e., option 1 (870 Hz). Issue 2-2-2 We support option 1. Many companies can’t achieve peak throughput with MCS17. Furthermore, Rel-15 HST-SFN test cases were defined with MCS13. Therefore, we would like to go with MCS13 here as well. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: Whether to define PDSCH requirement with HST-SFN scheme B Candidate options - Proposals - Option 1 (Samsung, CMCC, Ericsson, Huawei): Introduce the PDSCH requirement with HST-SFN B, - Option 1a (CMCC): with test applicability rule - Option 1b(Huawei): If UE supporting both HST SFN scheme A and B and supporting both 15kHz SCS and 30kHz SCS, then UE shall only pass scheme A with 15kHz and scheme B with 30kHz requirements. Do not define any other applicability rule between scheme A and scheme B. - Option 2 (Qualcomm, MTK, Apple): Not to introduce the PDSCH requirement with HST-SFN B Recommendations for 2nd round: - Scheme A and scheme B are different UE feature with UE capability, different baseband process, channel model, TCI configuration for them. Considering the test coverage for UE only supported scheme B, encourage companies can check whether the following compromise option can be agreeable? - Introduce PDSCH requirement with HST-SFN scheme B with test applicability rule - If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B Issue 2-1-2: Modelling of TRP pre-compensation - Proposals - Option 1 (Samsung, Ericsson): Assume perfect modeling of TRP pre-compensation - Option 2 (Huawei, Samsung, CMCC): using the following method to modelling the Doppler shift during the test - The frequency shift FD,k (Hz) from kth RRH is given by: FD,k=fc×real-v×y-xky-xk×C for k*Ds≤a<k*Ds+Ds and FD,k+1=FD,k+F∆,k+1 for k*Ds≤a<k*Ds+Ds where F∆,k=+0.1 ppm*fc , k mod 2=0-0.1 ppm*fc , k mod 2=1 Recommendations for 2nd round: - Minor performance different with option 1 and option 2. From UE baseband processing perspective, there is no different with two options. To simplify the TE complexity, encourage companies to check whether option 1 can be agreeable? Case Number; Duplex; MCS level; Antenna Configuration; Max Doppler shift; others; SNR(dB). 2; FDD; 17; 2x2; 870Hz; 2 taps and 0Hz delta between 2 taps; 12.8. 3; FDD; 17; 2x2; 870Hz; 2 taps and 0.1ppm delta between 2 taps; 13.6. 5; FDD; 17; 2x4; 870Hz; 2 taps and 0Hz delta between 2 taps; 9.9. 6; FDD; 17; 2x4; 870Hz; 2 taps and 0.1ppm delta between 2 taps; 10.6. 8; TDD; 17; 2x2; 1667Hz; 2 taps and 0Hz delta between 2 taps; 13.8. 9; TDD; 17; 2x2; 1667Hz; 2 taps and 0.1ppm delta between 2 taps; 14.6. 11; TDD; 17; 2x4; 1667Hz; 2 taps and 0Hz delta between 2 taps; 11.2. 12; TDD; 17; 2x4; 1667Hz; 2 taps and 0.1ppm delta between 2 taps; 11.9. |
| Sub-topic#2-2 | Issue 2-2-1: Maximum Doppler Shift. GTW agreement - Define PDSCH requirement with HST SFN scheme A with 870Hz Doppler shift for 15KHz SCS Issue 2-2-2: MCS & Rank Candidate options - Proposals - Option 1 (MTK, Apple, Qualcomm): MCS 13 with Rank 2 - Option 2 (NTT DoCoMo) - TDD: MCS 17 with Rank 2 - FDD: MCS 13 with Rank 2 - Option 3 (CMCC, Samsung, Huawei, NTT DoCoMo (if Doppler is 870Hz)): MCS 17 with Rank 2 Recommendations for 2nd round: - Agreement in the last meeting - MCS17 rank 2 as the baseline. - RAN4 may consider MCS13 rank 2 according to the simulation results - Simulation results summary - Both MCS 13 and MCS 17 are feasible under Doppler with 870Hz - Results with MCS 13 are aligned well among companies within 2dB gap - Results with MCS 17 are aligned well among three companies within 2 dB gap - Companies are encouraged to further check the simulation results during 2nd round discussion - Based on majority views, encourage companies can check whether MCS17 with rank 2 can be agreeable to move forward, considering the performance improvement compared with Rel-16 HST SFN Case Number; Duplex; MCS level; Maximum Doppler(Hz); Antenna Configuration; SNR(dB). Samsung; CMCC; Ericsson; Huawei; MTK; QC. 1; FDD; 13; 870; 2x2; 9.5; 8.8; 8.5; 8.5; 7.98. 2; FDD; 13; 972; 2x2; 10.3; 8.9; 8.5; 10.0. 3; FDD; 13; 870; 2x4; 6.1; 6.2; 6.4; 6.27. 4; FDD; 13; 972; 2x4; 6.9; 6.4; 8.0. 5; FDD; 17; 870; 2x2; 13.1; 13.2; 17.0; 14.86; 17.1. 6; FDD; 17; 972; 2x2; 16.1; 13.8; 19.0; N.A; N/A. 7; FDD; 17; 870; 2x4; 10.0; 10.4; 12.35; 13.3. 8; FDD; 17; 972; 2x4; 12.6; 10.5; N/A; N/A. 9; TDD; 13; 1667; 2x2; 9.4; 11.0; 9.75. 10; TDD; 13; 1667; 2x4; 6.3; 8.6; 7.32. 11; TDD; 17; 1667; 2x2; 11.3; 14.80; 19.3. 12; TDD; 17; 1667; 2x4; 8.3; 12.32; 15.0. Issue 2-2-3: UE capability - Proposals - Option 1 (MTK): The PDSCH demodulation requirements for HST-SFN Scheme A should be applicable for UE capable of “23-6-1 SFN scheme A (scheme 1) for PDSCH and PDCCH” Tentative agreement - Agreement in the last meeting: The PDSCH demodulation requirements for HST-SFN Scheme A should be applicable for UE capable of “23-6-1 SFN scheme A (scheme 1) for PDSCH and PDCCH” |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Issue 2-1-1: Whether to define PDSCH requirement with HST-SFN scheme B

- Proposals

  - Option 1 (Samsung, CMCC, Ericsson, Huawei): Introduce the PDSCH requirement with HST-SFN B,

    - Option 1a (CMCC):  with test applicability rule

    - Option 1b(Huawei): If UE supporting both HST SFN scheme A and B and supporting both 15kHz SCS and 30kHz SCS, then UE shall only pass scheme A with 15kHz and scheme B with 30kHz requirements. Do not define any other applicability rule between scheme A and scheme B.

  - Option 2 (Qualcomm, MTK, Apple):  Not to introduce the PDSCH requirement with HST-SFN B

- Recommended WF

  - Scheme A and scheme B are different UE feature with UE capability, different baseband process, channel model, TCI configuration for them. Considering the test coverage for UE only supported scheme B, encourage companies can check whether the following compromise option can be agreeable?

    - Introduce PDSCH requirement with HST-SFN scheme B with test applicability rule

      - If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B

- GTW agreement

  - Introduce PDSCH requirement with HST-SFN scheme B with test applicability rule

    - [If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B]

| Company | Comments |
| --- | --- |
| Apple | We still dont see the necessity to introduce UE demod requirement for Scheme B as it mainly depends on gNB Doppler pre-compensation. |
| Huawei | We think it is not valid reason for blocking defining schemeB requirement since there are different UE feature with UE capability, different baseband process, channel model, TCI configuration for schemeA and schemeB. For the test applicability rule, we think UE processing of scheme B is not exactly the subset of schemeA even scheme A processing is more complexity than scheme B. It is more proper to consider test different scheme with different SCS, just as Rel-17 HST CA requirements did. If UE supporting both HST SFN scheme A and B and supporting both 15kHz SCS and 30kHz SCS, then UE shall only pass scheme A with 15kHz and scheme B with 30kHz requirements. With this method, the total number of test case is not increased comparing to Option 1a but more test coverage is ensured. |
| CMCC | We support to introduce the PDSCH requirement with HST-SFN B. And with applicability rule, the test coverage is guaranteed but the test burden is not increased. |
| Ericsson | We still think the required UE processing for scheme A and scheme B are different. However, if having the test applicability rule can be a compromised way to define requirement for scheme B we are also fine with it. |
| MediaTek | We agree with Apple and still don’t see a need for requirement for Scheme B. |
| Qualcomm | We think that Scheme B is simplified implementation of Scheme A. Therefore, we still think it’s not essential to introduce requirements for Scheme B. |
| Samsung | We prefer option 1a. As if UE support both HST-SFN scheme A and B, and UE could pass the test of HST SFN scheme A, there will be no need to test scheme B (the performance of scheme B should be ensured). |
| Docomo | As for the test applicability rule, if RAN4 introduce PDSCH requirement for scheme B, our preference is Option 1a (if UE support both HST-SFN scheme A and B, and UE could pass the test of HST SFN scheme A, there will be no need to test scheme B). |

Issue 2-1-2: Modelling of TRP pre-compensation

- Proposals

  - Option 1 (Samsung, Ericsson): Assume perfect modeling of TRP pre-compensation

  - Option 2 (Huawei, Samsung, CMCC, Ericsson): using the following method to modelling the Doppler shift during the test

    - The frequency shift FD,k (Hz) from kth RRH is given by:

FD,k=fc×real-v×y-xky-xk×C for k*Ds≤a<k*Ds+Ds

and

FD,k+1=FD,k+F∆,k+1 for k*Ds≤a<k*Ds+Ds

where

F∆,k=+0.1 ppm*fc ,  k mod 2=0-0.1 ppm*fc ,  k mod 2=1

- Recommended WF

  - Minor performance different with option 1 and option 2. From UE baseband processing perspective, there is no different with two options. To simplify the TE complexity, encourage companies to check whether option 1 can be agreeable?

| Case Number | Duplex | MCS level | Antenna Configuration | Max Doppler shift | others | SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | FDD | 17 | 2x2 | 870Hz | 2 taps and 0Hz delta between 2 taps | 12.8 |
| 3 | FDD | 17 | 2x2 | 870Hz | 2 taps and 0.1ppm delta between 2 taps | 13.6 |
| 5 | FDD | 17 | 2x4 | 870Hz | 2 taps and 0Hz delta between 2 taps | 9.9 |
| 6 | FDD | 17 | 2x4 | 870Hz | 2 taps and 0.1ppm delta between 2 taps | 10.6 |
| 8 | TDD | 17 | 2x2 | 1667Hz | 2 taps and 0Hz delta between 2 taps | 13.8 |
| 9 | TDD | 17 | 2x2 | 1667Hz | 2 taps and 0.1ppm delta between 2 taps | 14.6 |
| 11 | TDD | 17 | 2x4 | 1667Hz | 2 taps and 0Hz delta between 2 taps | 11.2 |
| 12 | TDD | 17 | 2x4 | 1667Hz | 2 taps and 0.1ppm delta between 2 taps | 11.9 |

- GTW agreement

  - Assume perfect modeling of TRP pre-compensation

| Company | Comments |
| --- | --- |
| Huawei | We still prefer Option 2. The modelling frequency error between different TRP is not to verify TRP performance, but to verify proper UE implementation under such case that is more consistent with the real scenario. Also the modelled frequency error value is reasonable based on following calculation that there is still frequency error for the tap from different TRP at UE receiver side that is twice of frequency error that RAN4 considered for Rel-16 normal multi-TRP scenario. |
| Rohde & Schwarz | We prefer Option 1. We would like to further check on the implementation feasibility for Option 2 for the next meeting. |
| Huawei | From our understanding, since the purpose of the test is to test proper UE implementation, TE does not really perform frequency pre-compensation during the test for scheme B. Instead, only channel model is modified comparing to other HST schemes, i.e. different taps are added different frequency offset. It is fine for us to further check the implementation feasibility by TE vendor. |
| CMCC | Our preference is option 2, however, if the performance difference is minor, we are also OK with option 1. |
| Ericsson | We support option 1. |
| Samsung | We could accept option 1. As there has no UE process difference between option 1 and option 2, the only requirement difference between these two options is the SNR value, for option 1 we need to define a smaller SNR value compared with option 2. From our simulation result, the SNR gap between these two options is 0.8dB for 2x2 setting, and 0.7dB for 2x4 setting. |
| Docomo | If RAN4 introduce PDSCH requirement for scheme B, we slightly prefer Option 1. |

Issue 2-2-2: MCS & Rank

- Proposals

  - Option 1 (MTK, Apple, Qualcomm): MCS 13 with Rank 2

  - Option 2 (NTT DoCoMo)

    - TDD:  MCS 17 with Rank 2

    - FDD:  MCS 13 with Rank 2

  - Option 3 (CMCC, Samsung, Huawei, NTT DoCoMo (if Doppler is 870Hz)): MCS 17 with Rank 2

- Recommended WF

  - Agreement in the last meeting

    - MCS17 rank 2 as the baseline.

    - RAN4 may consider MCS13 rank 2 according to the simulation results

  - Simulation results summary

    - Both MCS 13 and MCS 17 are feasible under Doppler with 870Hz

    - Results with MCS 13 are aligned well among companies within 2dB gap

    - Results with MCS 17 are aligned well among three companies within 2 dB gap, meanwhile, the target SNR with 70% is not very high at least for 3 companies results

  - Companies are encouraged to further check the simulation results during 2nd round discussion

    - Based on majority views, encourage companies can check whether MCS17 with rank 2 can be agreeable to move forward, considering the performance improvement compared with Rel-16 HST SFN

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Samsung | CMCC | Ericsson | Huawei | MTK | QC |
| 1 | FDD | 13 | 870 | 2x2 | 9.5 | 8.8 | 8.5 |  | 8.5 | 7.98 |
| 2 | FDD | 13 | 972 | 2x2 | 10.3 | 8.9 | 8.5 |  | 10.0 |  |
| 3 | FDD | 13 | 870 | 2x4 | 6.1 | 6.2 |  |  | 6.4 | 6.27 |
| 4 | FDD | 13 | 972 | 2x4 | 6.9 | 6.4 |  |  | 8.0 |  |
| 5 | FDD | 17 | 870 | 2x2 | 13.1 | 13.2 | 17.0 | 14.86 | 17.1 |  |
| 6 | FDD | 17 | 972 | 2x2 | 16.1 | 13.8 | 19.0 | N.A | N/A |  |
| 7 | FDD | 17 | 870 | 2x4 | 10.0 | 10.4 |  | 12.35 | 13.3 |  |
| 8 | FDD | 17 | 972 | 2x4 | 12.6 | 10.5 |  | N/A | N/A |  |
| 9 | TDD | 13 | 1667 | 2x2 | 9.4 |  |  |  | 11.0 | 9.75 |
| 10 | TDD | 13 | 1667 | 2x4 | 6.3 |  |  |  | 8.6 | 7.32 |
| 11 | TDD | 17 | 1667 | 2x2 | 11.3 |  |  | 14.80 | 19.3 |  |
| 12 | TDD | 17 | 1667 | 2x4 | 8.3 |  |  | 12.32 | 15.0 |  |

- GTW agreement

  - Further discuss and decide in next Ran4 meeting with following options

    - MCS 13 with Rank 2

    - MCS 17 with Rank 2

    - It’s encouraged companies can provide results for both options under FDD and TDD modes.

    - RAN4 will decide based on the alignment outcome among companies’ results and whether peak TP can be achieved or not.

| Company | Comments |
| --- | --- |
| Apple | Based on our evaluation, for HST Scheme A max TP cannot be achieved for either FDD or TDD with MCS17 rank 2. The TP is capped at ≤ 90% Max TP and it is not a good test setup to define requirements for MCS that doesn't achieve max TP, even though test requirement is at 70% max TP. Based on performance results we strongly support to define requirements with MCS13 Rank 2 for HST Scheme A and MCS 17 with rank 2 is not feasible. Also, we don't think that its a valid argument and reason that Rel-17 HST scheme A should have better performance than Rel-16 HST SFN. RAN1 didn’t introduce HST Scheme A as an improvement over HST-SFN, but rather as another multi-TRP transmission scheme applicable to HST scenario. It was proposed earlier in RAN4 to evaluate performance compared to HST-SFN, but it wasn’t agreed by most companies that its a different transmission scheme and channel model and hence not comparable. |
| Huawei | We are OK to consider MCS17 with rank 2. Based on our evaluation, MCS17 with rank 2 is feasible for scheme A. |
| CMCC | We support MCS17 rank 2 for 870Hz doppler shift. |
| Ericsson | Based on the simulation results, we see MCS17 is feasible. We support MCS17 for both FDD and TDD |
| MediaTek | We support MCS13 based on existing simulation results. It can be observed that with MCS13 all results are well aligned and requirement can be easily defined. On the other hand MCS17 results have large span and it may be difficult to find reasonable requirements that would be acceptable to all companies. As Apple mentions, we also did observe TP gap around 90% with MCS17 indicating bad test configuration. MCS13 works fine in all companies results. |
| Qualcomm | We support defining requirements with MCS13. Given that many companies can’t achieve peak throughput with MCS17, we don’t think requirements should be defined with MCS17. We further note that MCS17 was also on the table during Rel-16 HST-SFN requirement definition. However, it was not agreed as peak throughput could not be achieved with MCS17. Also, we don’t think it is straightforward to compare Rel-16 HST-SFN scheme to Rel-17 Scheme A. We note that channel model itself is different between these two schemes. Therefore, we think that it is not a valid comparison to begin with. To Moderator: We have updated R4-2214404 xls to include our results as captured in R4-2214214. |
| Samsung | We prefer option 3. But if other companies have difficult to apply MCS17, we could also accept MCS13. |
| MediaTek | We updated simulation results with HST-SFN Scheme A TDD MCS13 results of MTK that were missing. |
| Docomo | We support MCS17 for both FDD and TDD. In our understand, based on simulation results in the contribution, the simulation results of MCS17 + 870Hz and 1667Hz reaching the peak rate are majority. So, we think that MCS17 with RANK2 is feasible |

# Topic #3: CSI reporting requirment for multi-TRP

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212544 | Qualcomm | Proposal 1: Consider beam-steering model into the multi-TRP PMI reporting test set-up. |
| R4-2212550 | Ericsson | Observation 1: the throughput gain for 4Tx is 1.43 at 70% max Tput, 1.42 at 70% max Tput and 1.35 at 90% max Tput Observation 2: the throughput gain for 8Tx is 2.86 at 70% max Tput, 2.61 at 70% max Tput and 2.36 at 90% max Tput Observation 3: 8Tx has obviously better performance over that of 4Tx on 70%/80%/90% max performance Proposal 1: 8 ports for each TRP |
| R4-2212706 | Samsung | Proposal 1: Introduce multi-TRP PMI test cases with number of CSI ports per TRP: 8 ports Proposal 2: Introduce multi-TRP PMI test cases with minimum requirement as [1.6] |
| R4-2213837 | Huawei, HiSilicon | Proposal 1: Select 4 ports per TRP for CSI reporting requirements for mTRP. |

## Open issues summary

Last RAN4 meeting agreeements in the WF R4-2210669

List of open issues

- Sub-topic 3-1: Test setup for CSI reporting requirement for m-TRP transmission

  - Issue 3-1-1: Number of CSI-RS Port

  - Issue 3-1-2: Beamforming Model

  - Issue 3-1-3: Gamma value

### Sub-topic 3-1: Test Setup of CSI reporting requirement for m-TRP transmission

Issue 3-1-1: Number of CSI-RS port

- Observations

  - Observation 1 (Ericsson)

    - the throughput gain for 4Tx is 1.43 at 70% max Tput, 1.42 at 70% max Tput and 1.35 at 90% max Tput

    - the throughput gain for 8Tx is 2.86 at 70% max Tput, 2.61 at 70% max Tput and 2.36 at 90% max Tput

    - 8Tx has obviously better performance over that of 4Tx on 70%/80%/90% max performance

- Proposals

  - Option 1 (Ericsson, Samsung): 8 ports per TRP

  - Option 2 (Huawei): 4 ports per TRP

- Recommended WF

  - Agreement in the last meeting

    - MIMO correlation: XP high

    - 8 ports per TRP as baseline

    - Test Metric: TP ratio between following PMI and Random PMI

    - Test point: 90%

    - Encourage interesting companies to provide simulation results for both 4 and 8 ports in RAN4#104-e meeting, and make the final decision in RAN4#104-e meeting

  - Simulation results summary

|  | Samsung |  | Huawei |  | Ericsson |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR point(dB)/TP ratio (following Type I PMI /random Type I PMI) | XP High 4x2 (90%) | XP High 8x2 (90%) | XP High 4x2 (90%) | XP High 8x2 (90%) | XP High 4x2 (90%) | XP High 8x2 (90%) |
| FDD 2RX | 11.0 dB/1.51 | 8.7 dB/2.6 | 1.31 | 2.06 | 12.7dB/1.35 | 9.3 dB/2.36 |
| FDD 4RX | 8.6 dB/1.56 | 5.1 dB/3.27 | 1.61 | 2.39 |  |  |
| TDD 2RX | 10.9 dB/1.55 | 7.6 dB/2.96 | 1.44 | 2.06 |  |  |
| TDD 4RX | 8.1 dB/1.63 | 4.9 dB/3.38 | 1.70 | 2.38 |  |  |

  - Based on companies result, 8 Tx port can achieve large performance gain compared with 4Tx. Encourage companies can check whether number of CSI-RS port as 8 can be agreeable?

Issue 3-1-2: Beamforming Model

- Proposals

  - Option 1(Qualcomm): Consider beam-steering model into the multi-TRP PMI reporting test set-up.

- Recommended WF

  - Option 1?

Issue 3-1-3: Gamma value

- Proposals

  - Option 1 (Samsung): [1.6]

- Recommended WF

  - Encourage comments if any

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1 Issue 3-1-2 Issue 3-1-3 |
| Ericsson | Issue 3-1-1 Fine with the recommended WF. Issue 3-1-2 Fine with option1. Issue 3-2-3 Further discuss based on the simulation results from more companies. |
| Huawei | Issue 3-1-1: Number of CSI-RS port We prefer Option 2. From our understanding, considering that we are defining the minimum requirements, more test coverage should be considered firstly to ensure UE such that support only 4 ports can also perform the test. Also, we think it is feasible to define cases with lower gain since in Rel-15 RI requirements, some cases are even defined with TP ratio of 0.9. Issue 3-1-2: Beamforming Model We are OK with Option 1. The beam steering matrix for different TRxP can be independently generated, applying the same method as single TRP PMI case. Issue 3-1-3: Gamma value If the 4-port case is introduced, we prefer to use the gamma value 1.15. If the 8-port case is introduced, we are OK to use the gamma value 1.6. |
| Samsung | Issue 3-1-1: Number of CSI-RS port We prefer 8 ports. Issue 3-1-2: Beamforming Model We are ok with option 1. Issue 3-1-3: Gamma value We suggest set gamma value as 1.6 for 8 ports case. |
| MediaTek | Issue 3-1-1 We are fine with recommended WF. |
| Nokia, Nokia Shanghai Bell | Issue 3-1-1 We agree to the WF recommendation (number of CSI-RS ports = 8) We still don’t like that more CSI-RS ports are configured than needed, to achieve the same performance, but we can compromise to achieve a common test setup. Issue 3-1-3 We can agree to use Gamma=[1.6] as baseline and to discuss further based on simulation results |
| Qualcomm | Issue 3-1-1 We support option 1. It has already been pointed out the throughput gain is minimal with 4 ports per TRP. Issue 3-1-2 We support the recommended WF. We note that the existing single TRP PMI reporting tests have beam steering so that it could be tested whether UE is able to properly report PMI as the channel direction changes over time. Without considering beam-steering model into the m-TRP test setup, we will not be able to test whether UE can adapt to changing channel directions. Therefore, the performance requirement would be incomplete. We encourage companies to bring results considering beam steering in the next meeting. Issue 3-1-3 We think that gamma value should be decided based on the simulation results, including beam steering. This can be decided after reviewing results in the next meeting. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Issue 3-1-1: Number of CSI-RS port Candidate options: - Observations - Observation 1 (Ericsson) - the throughput gain for 4Tx is 1.43 at 70% max Tput, 1.42 at 70% max Tput and 1.35 at 90% max Tput - the throughput gain for 8Tx is 2.86 at 70% max Tput, 2.61 at 70% max Tput and 2.36 at 90% max Tput - 8Tx has obviously better performance over that of 4Tx on 70%/80%/90% max performance - Proposals - Option 1 (Ericsson, Samsung, Nokia, Qualcomm, MTK): 8 ports per TRP - Option 2 (Huawei): 4 ports per TRP Recommendations for 2nd round: - Agreement in the last meeting - MIMO correlation: XP high - 8 ports per TRP as baseline - Test Metric: TP ratio between following PMI and Random PMI - Test point: 90% - Encourage interesting companies to provide simulation results for both 4 and 8 ports in RAN4#104-e meeting, and make the final decision in RAN4#104-e meeting - Simulation results summary - Based on companies result, 8 Tx port can achieve large performance gain compared with 4Tx. Based on the majority view - Encourage companies can check whether number of CSI-RS port as 8 can be agreeable? Samsung; Huawei; Ericsson. SNR point(dB)/TP ratio (following Type I PMI /random Type I PMI); XP High 4x2 (90%); XP High 8x2 (90%); XP High 4x2 (90%); XP High 8x2 (90%); XP High 4x2 (90%); XP High 8x2 (90%). FDD 2RX; 11.0 dB/1.51; 8.7 dB/2.6; 1.31; 2.06; 12.7dB/1.35; 9.3 dB/2.36. FDD 4RX; 8.6 dB/1.56; 5.1 dB/3.27; 1.61; 2.39. TDD 2RX; 10.9 dB/1.55; 7.6 dB/2.96; 1.44; 2.06. TDD 4RX; 8.1 dB/1.63; 4.9 dB/3.38; 1.70; 2.38. Issue 3-1-2: Beamforming Model Candidate options: - Proposals - Option 1(Qualcomm, Samsung, Huawei): Consider beam-steering model into the multi-TRP PMI reporting test set-up. Tentative agreement: - Consider beam-steering model into the multi-TRP PMI reporting test set-up. Issue 3-1-3: Gamma value Candidate options: - Proposals - Option 1 (Samsung, Huawei, Nokia): [1.6] for 8 ports - Option 2 (Huawei): [1.15] for 4 ports Recommendations for 2nd round: - Option1 as baseline, further discussion gamma value based on simulation results - Companies are encouraged to provide the updated results during 2nd round if possible |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Issue 3-1-1: Number of CSI-RS port

- Observations

  - Observation 1 (Ericsson)

    - the throughput gain for 4Tx is 1.43 at 70% max Tput, 1.42 at 70% max Tput and 1.35 at 90% max Tput

    - the throughput gain for 8Tx is 2.86 at 70% max Tput, 2.61 at 70% max Tput and 2.36 at 90% max Tput

    - 8Tx has obviously better performance over that of 4Tx on 70%/80%/90% max performance

- Proposals

  - Option 1 (Ericsson, Samsung, Nokia, Qualcomm, MTK): 8 ports per TRP

  - Option 2 (Huawei): 4 ports per TRP

- Recommended WF

  - Agreement in the last meeting

    - MIMO correlation: XP high

    - 8 ports per TRP as baseline

    - Test Metric: TP ratio between following PMI and Random PMI

    - Test point: 90%

    - Encourage interesting companies to provide simulation results for both 4 and 8 ports in RAN4#104-e meeting, and make the final decision in RAN4#104-e meeting

  - Simulation results summary

    - Based on companies result, 8 Tx port can achieve large performance gain compared with 4Tx. Based on the majority view

    - Encourage companies can check whether number of CSI-RS port as 8 can be agreeable?

|  | Samsung |  | Huawei |  | Ericsson |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNR point(dB)/TP ratio (following Type I PMI /random Type I PMI) | XP High 4x2 (90%) | XP High 8x2 (90%) | XP High 4x2 (90%) | XP High 8x2 (90%) | XP High 4x2 (90%) | XP High 8x2 (90%) |
| FDD 2RX | 11.0 dB/1.51 | 8.7 dB/2.6 | 1.31 | 2.06 | 12.7dB/1.35 | 9.3 dB/2.36 |
| FDD 4RX | 8.6 dB/1.56 | 5.1 dB/3.27 | 1.61 | 2.39 |  |  |
| TDD 2RX | 10.9 dB/1.55 | 7.6 dB/2.96 | 1.44 | 2.06 |  |  |
| TDD 4RX | 8.1 dB/1.63 | 4.9 dB/3.38 | 1.70 | 2.38 |  |  |

- GTW agreement

  - 8 ports per TRP

| Company | Comments |
| --- | --- |
| Huawei | We still think 4Tx port case is needed from the test coverage point of view. However, to move forward, we are also OK to only consider 8Tx case. |
| Qualcomm | Given that we have pending simulations assumptions, such as beam-steering, we don’t think requirement should be defined without this as requirements could change with updated simulation assumptions. Also, we think that it is not quite feasible to provide results during the meeting period. Therefore, in our view we should discuss this in the next meeting in conjunction with updated simulation assumptions |
| Samsung | We support option 1. As 4 port has limited performance gain here, if we use 4 port, then it is hard to detect if UE PMI selection function worked or not. Although there has 0.9 for RI, it is not suitable to use such kind of low gamma value for PMI requirement. About the beam-steering, currently we think each TRP use independent modeling by default. |
| Qualcomm2 | For beam-steering, we propose to use step sizes specified in Table B.2.3.2.3A-1 with independent start value for each TRP. |

Issue 3-1-3: Gamma value

- Proposals

  - Option 1 (Samsung, Huawei, Nokia): [1.6] for 8 ports

  - Option 2 (Huawei): [1.15] for 4 ports

- Recommended WF

  - Option1 as baseline, further discussion gamma value based on simulation results

  - Companies are encouraged to provide the updated results during 2nd round if possible

| Company | Comments |
| --- | --- |
| Ericsson | We are fine with option 1. We haven’t propose option 2 please moderator help and doubt check. Thanks |
| Samsung | We support the recommended WF. |

# Topic #4: PMI reporting requirement for Rel-17 enhanced Type II PS codebook

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211539 | Anritsu | Observation 1: It is challenging for the test equipment to update a random PMI in slot-by-slot basis while maintaining its integrity of each parameter. Observation 2: To achieve the random PMI from a TE implementation viewpoint, a presumable method is to limit parameters to randomize only with the similar parameters between Type I single panel and FeType II for the sake of simplifying the implementation. Observation 3: In a case we apply the method with observation 2, since i18 (i.e. strongest coefficient) isn’t randomized, there would be a case that a position of the strongest beam does not change. Observation 4: Even with the hardcoding method by the test equipment to randomize PMI, there are remaining issues to be studied yet. Proposal 1: Support Option 3 for test metric: Following FeType II CB PMI over Type I single panel random PMI. |
| R4-2212113 | Nokia, NSB | Observation 1: There is a need to specify how to beamform the CSI-RS for Rel-17 TypeII-PS codebook performance requirements. Observation 2: Beamforming of CSI-RS should fit the spatial and frequency domain properties of the radio multipath fading channel. Observation 3: TDL “low” channel models do not have a realization independent spatial component. Observation 4: The TPUT performance gain of Rel-17 eTypeII PS, and the feedback compression from measuring the SD precoder in UL, can only be achieved using a spatially dependent channel model. Observation 5: Rel-13 LTE Class B K=1 MIMO fading channel, and the fading channel/steering matrix approach from TS 36.101, B2.3.A4 and B2.3B.4, will not differ significantly from each other performance wise when used in combination with TDL. There is no technical basis to prefer one over the other out of these three. Observation 6: As stated in TS 38.901, TDL channel model should not be used for MIMO evaluations. Observation 7: In spatially dependent codebook performance testing, a spatial channel model should be used. We suggest using a link level channel model such as CDL with angular translation and scaling, for both CSI and data. Observation 8: In one implementation, the FD precoding of CSI-RS ports can be taken into account by scaling the SD precoding vectors with the average path gain of the TDL model in each subband. Observation 9: We outlined a method to specify a robust and common approach for implementation and derivation of hard-codable CSI-RS port precoders from TDL/CDL channel models in TEs. Observation 10: All three test metric proposals are usable in our opinion, option1 (follow PMI over random PMI) might be the easiest/lowest complexity to implement in TEs. Proposal 1: RAN4 to include performance requirements for Rel-17 eTypeII PS codebooks in the Rel-18 timeframe, and to start from the test setups contributed to, and discussed in, RAN4#104. |
| R4-2212546 | Qualcomm | Proposal 1: Do not define performance requirement for Rel-17 FeTye II PS codebook |
| R4-2212552 | Ericsson | Observation 1: Power scaling method has low complexity and makes TEs from different vendors comparable Observation 2: Option 2 and 3 are easier for testing, more feedback on the feasibility of option 2 and 3 from TE vendors are necessary Proposal 1: Power scaling method with fixed or no Wf can be considered to do the initial evaluation Proposal 2: Consider option 2: transparent to UE under test operation Proposal 3: Consider option 1: follow PMI with random PMI if the requirement is introduced Observation 3: Power scaling method with fixed or no Wf has its own limitation. More practical way is needed to better reflect the actual processing for both BS and UE side so that to guarantee the overall performance Proposal 4: Postpone the requirement for Rel-17 Type II PS CB to the at least next release to find better test setup with low complexity and better reflection of actual overall performance. |
| R4-2212707 | Samsung | Proposal 1: for wide beam W1 modeling, option 1a (MIMO fading channel as Rel-13 LTE Class B K=1 PMI test case) should be used. Proposal 2: for frequency selective precoding Wf modeling, option 1 (explicitly derived from chosen TDL model) should be used if TE support. Proposal 3: support option 1 (following PMI with random PMI) as Test metric. |
| R4-2213838 | Huawei, HiSilicon | - Define PMI reporting requirement for Rel-17 FeTypeII port selection codebook based on evaluation on the performance gain over eTypeII codebook. Considering the timeline of this WI, the basic feature M=1 should be prioritized and therefore the Wf is assumed with all-one vector if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. Each angle-delay pair in the collection represent a tap with the fixed delay associated with one CSI-RS port so TE can keep CSI-RS precoding unchanged during the test. Select all CSI-RS ports with no non-zero coefficients (i.e. α=1, β=1, configuration 4 in TS 38.214 Table 5.2.2.2.7-1) if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. Consider “following FeType II CB PMI over Type 1 single panel random PMI” as the test metric if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. Only consider SU-MIMO setup if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. Select 16 ports, rank2 and MCS20 if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. For other parameters such as CSI-RS resource Type, reportConfigType, reuse from Rel-16 eTypeII PMI cases. |

## Open issues summary

Last RAN4 meeting agreeements in the WF R4-2210669

List of open issues

- Sub-topic 4-1: Test Scope

  - Issue 4-1-1: Whether to define PMI requirement for Rel-17 FeTye II PS codebook

- Sub-topic 4-1: Test setup of PMI reporting

  - Issue 4-2-1: BF modelling

  - Issue 4-2-2 Test Metric

  - Issue 4-2-3: CSI-RS precoder

  - Issue 4-2-4: Test Scenario

  - Issue 4-2-5: Test Parameters

### Sub-topic 4-1: Test Scope

Issue 4-1-1: Whether to define PMI requirement for Rel-17 FeTye II PS codebook

- Proposals

  - Option 1(Nokia): RAN4 to include performance requirements for Rel-17 eTypeII PS codebooks in the Rel-18 timeframe, and to start from the test setups contributed to, and discussed in, RAN4#104.

  - Option 2 (Ericsson): Postpone the requirement for Rel-17 Type II PS CB to the at least next release to find better test setup with low complexity and better reflection of actual overall performance.

  - Option 3 (Huawei): Define PMI reporting requirement for Rel-17 FeTypeII port selection codebook based on evaluation on the performance gain over eTypeII codebook.

  - Option 4 (Qualcomm): No

- Recommended WF

  - RAN4 will not introduce requirement of Rel-17 Type II PS CB in Rel-17 Timeframe

  - Further discuss the requirement in Rel-18 Timeframe. Encourage companies to further discuss the feasible test setup.

- GTW agreement

### Sub-topic 4-2: Test setup of PMI reporting

Issue 4-2-1: BF modelling

- Observations

  - Observation 1 (Ericsson):

    - Power scaling method has low complexity and makes TEs from different vendors comparable

    - Option 2 and 3 are easier for testing, more feedback on the feasibility of option 2 and 3 from TE vendors are necessary

    - Power scaling method with fixed or no Wf has its own limitation. More practical way is needed to better reflect the actual processing for both BS and UE side so that to guarantee the overall performance

  - Observation 2 (Nokia)

    - There is a need to specify how to beamform the CSI-RS for Rel-17 TypeII-PS codebook performance requirements

    - Beamforming of CSI-RS should fit the spatial and frequency domain properties of the radio multipath fading channel.

    - TDL “low” channel models do not have a realization independent spatial component.

    - The TPUT performance gain of Rel-17 eTypeII PS, and the feedback compression from measuring the SD precoder in UL, can only be achieved using a spatially dependent channel model.

    - Rel-13 LTE Class B K=1 MIMO fading channel, and the fading channel/steering matrix approach from TS 36.101, B2.3.A4 and B2.3B.4, will not differ significantly from each other performance wise when used in combination with TDL. There is no technical basis to prefer one over the other out of these three.

    - As stated in TS 38.901, TDL channel model should not be used for MIMO evaluations.

    - In spatially dependent codebook performance testing, a spatial channel model should be used. We suggest using a link level channel model such as CDL with angular translation and scaling, for both CSI and data.

    - In one implementation, the FD precoding of CSI-RS ports can be taken into account by scaling the SD precoding vectors with the average path gain of the TDL model in each subband.

    - We outlined a method to specify a robust and common approach for implementation and derivation of hard-codable CSI-RS port precoders from TDL/CDL channel models in TEs

- Proposals

  - Option 1(Samsung):

    - for wide beam W1 modeling, option 1a (MIMO fading channel as Rel-13 LTE Class B K=1 PMI test case) should be used.

    - for frequency selective precoding Wf  modeling, option 1 (explicitly derived from chosen TDL model) should be used if TE support.

  - Option 2(Ericsson):

    - Power scaling method with fixed or no Wf can be considered to do the initial evaluation

    - Consider option 2: transparent to UE under test operation

  - Option 3(Huawei): Considering the timeline of this WI, the basic feature M=1 should be prioritized and therefore the Wf is assumed with all-one vector if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined.

- Recommended WF

  - Encourage comments if any

Issue 4-2-2 Test Metric

- Observations

  - Observation 1 (Anritsu)

    - It is challenging for the test equipment to update a random PMI in slot-by-slot basis while maintaining its integrity of each parameter.

    - To achieve the random PMI from a TE implementation viewpoint, a presumable method is to limit parameters to randomize only with the similar parameters between Type I single panel and FeType II for the sake of simplifying the implementation.

    - In a case we apply the method with observation 2, since i18 (i.e. strongest coefficient) isn’t randomized, there would be a case that a position of the strongest beam does not change.

    - Even with the hardcoding method by the test equipment to randomize PMI, there are remaining issues to be studied yet.

  - Observation 2 (Nokia)

    - All three test metric proposals are usable in our opinion, option1 (follow PMI over random PMI) might be the easiest/lowest complexity to implement in TEs.

- Proposals

  - Option 1 (Ericsson, Samsung, Nokia): follow PMI with random PMI if the requirement is introduced

  - Option 2 (Anritsu, Huawei): Following FeType II CB PMI over Type I single panel random PMI.

- Recommended WF

  - Encourage comments if any

Issue 4-2-3: CSI-RS precoder

- Proposals

  - Option 1 (Huawei):

    - Each angle-delay pair in the collection represent a tap with the fixed delay associated with one CSI-RS port so TE can keep CSI-RS precoding unchanged during the test.

    - Select all CSI-RS ports with no non-zero coefficients (i.e. α=1, β=1, configuration 4 in TS 38.214 Table 5.2.2.2.7-1) if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined.

- Recommended WF

  - Encourage comments if any

Issue 4-2-4: Test Scenario

- Proposals

  - Option 1 (Huawei): Only consider SU-MIMO setup if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined

- Recommended WF

  - Encourage comments if any

Issue 4-2-5: Test Parameters

- Proposals

  - Option 1 (Huawei): Select 12 ports, rank2 and MCS20 if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. For other parameters such as CSI-RS resource Type, reportConfigType, reuse from Rel-16 eTypeII PMI cases

- Recommended WF

  - Encourage comments if any

## Companies views’ collection for 1st round

### Open issues

Sub topic 4-1

| Company | Comments |
| --- | --- |
| XXX | Issue 4-1-1 |
| Apple | Issue 4-1-1 We prefer not to introduce PMI requirements for Rel-17 FeType II PS codebook, given this is the last meeting for this WI and there are several open issues related to test set up. Also, the feature is intended to work with proper CSI-RS beamforming at gNB. Making some assumptions on the beamforming at BS is not a proper way to verify UE support of this feature. |
| Ericsson | Issue 4-1-1 The objective circumstance is that we neither don’t have such tests and requirements for previous PS CB as a reference, nor detailed description for the corresponding BF modeling in the specification. Besides, one TE vendor has given negative feedback on some proposed assumptions. So then we think it might not be ready from both spec. and TE vendors point of views for having this test and requirement within current release. Thus, based on the above and considering we are running out of time for this WI, we propose to consider option 2 and we are fine with the first bullet of the recommended WF. For the second bullet, we think it is better to let the RAN plenary to make the decision. |
| Huawei | We are OK to not discuss Type II PS CB in Rel-17 Timeframe and further discuss the requirement in Rel-18 timeframe by Rel-17 maintenance. |
| Samsung | We support the recommended WF. We also have one open issue for further discussion about this requirement in Rel-18 timeframe: how to handle the Rel-15 and Rel-16 port selection codebook test requirement if we want to introduce the requirement of Rel-17 port selection codebook. |
| MediaTek | Issue 4-1-1 We are fine with recommended WF. We think RAN4 run out of time in Rel-17 scope. For future actions we agree with Ericsson that RAN plenary should make decision for it. |
| Nokia, Nokia Shanghai Bell | Issue 4-1-1 The recommended way forward is acceptable for us, however we would like to continue the technical detailed discussions in the summary to prepare for the future of advanced codebook testing. The discussion and any outcomes reached should be captured in a (potentially informative) WF to serve as a starting point for future RAN4 work. |
| Anritsu | Issue 4-1-1 We are fine with the recommended WF. |
| Qualcomm | We prefer not defining requirements for FeTypeII PS CB. We note that the implementation of the BS beamforming is not standardized for FeTypeII PS CB. Furthermore, any restriction on the BS beamforming will not guarantee optimal performance for UE, meaning that the UE performance cannot be guaranteed with such a CB. Therefore, we think that introducing performance requirement for FeTypeII PS CB would be difficult. |

Sub topic 4-2

| Company | Comments |
| --- | --- |
| XXX | Issue 4-2-1 Issue 4-2-2 Issue 4-2-3 Issue 4-2-4 Issue 4-2-5 |
| Samsung | Issue 4-2-1 BF modelling As Rel-17 TypeII PS CB performance depends on both gNB and UE, if the modeling of BF on gNB or TE side is too simplicity, the performance of UE measured CB will be impacted. Issue 4-2-2 Test Metric If TE could support random PMI of Rel-17 TypeII PS CB, we want to support follow PMI with random PMI of Rel-17 TypeII PS CB (option 1). If TE cannot support random PMI of Rel-17 TypeII PS codebook, we could comprise to using Type I single panel random PMI performance as the baseline (option 2). Issue 4-2-4: Test Scenario We support option 1, Only consider SU-MIMO setup if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. Issue 4-2-5 We think it’s too early to define the port number. |
| Nokia, Nokia Shanghai Bell | Issue 4-2-1 Initially we would like to introduce a spatial channel model such as CDL model in the setup. Then both Spatial Domain and Frequency Domain CSI beamforming should be derived from the CDL model spatial and frequency properties. If TE vendors or RAN4 cannot support CDL model and if TDL model must be selected, we agree with Huawei and others, that the Wf should be derived from the selected TDL model. We can also agree to option 1a (MIMO fading channel as Rel-13 LTE Class B K=1 PMI test case) should be used. Issue 4-2-2 We recognize the complexity issues raised by Anritsu. As such we would be willing to compromise to follow PMI over random PMI with type I PMI as reference, i.e., option 2. Issue 4-2-3 We fully agree with the analysis brought by Huawei. Each CSI-Port maps to one beam, which is defined in the spatial domain via the angle and in the frequency domain via the delay profile (of that specific beam angle, where angle is a bit of misleading term as a single beam might have multiple lobes). This CSI-RS precoding can be kept constant during the test, where the spatial/angle part can come from power scaling in the eigendirections (or directly the eigendirections) of the chosen channel models, and delay part from channel model. The UE will then select the strongest CSI-RS ports/gNB beams, in the port selection step of W1 (where the coefficients can either be 1 or 0). Once the channel model is agreed, the CSI-RS precoder should be selected to follow the spatial and frequency domain properties of that model. Issue 4-2-4 We can agree to only consider SU-MIMO if requirements are defined in rel-17 RAN4 timeline, however, should the item be moved to rel-18 we suggest reconsidering MU-MIMO requirements as well since this is the main use case for TypeII codebooks. Issue 4-2-5 The proposed test parameters seem reasonable from a general perspective; however, we should come back to such details only after the test scenario, csi-precoder, test metric and BF modelling are agreed. |
| Anritsu | Issue 4-2-2 We would like to thank Samsung and Nokia for the big compromise on the test metric issue. Though we are currently facing a difficulty in achieving the test metric of the follow PMI over the random PMI, for the sake of the future study to seek for the random PMI processing, it would be very much appreciated if the specific way of randomization could be provided. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | Issue 4-1-1: Whether to define PMI requirement for Rel-17 FeTye II PS codebook GTW agreements: - RAN4 will not introduce requirement of Rel-17 FeType II PS CB in Rel-17 Timeframe - RAN4 can further discuss detailed parameters in RAN4#104 for reference of future work. Recommendations for 2nd round: - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work. |
| Sub-topic#4-2 | Issue 4-2-1: BF modelling Candidate options: - Proposals - Option 1(Samsung, Nokia): - for wide beam W1 modeling, option 1a (MIMO fading channel as Rel-13 LTE Class B K=1 PMI test case) should be used. - for frequency selective precoding Wf modeling, option 1 (explicitly derived from chosen TDL model) should be used if TE support. - Option 2(Ericsson): - Power scaling method with fixed or no Wf can be considered to do the initial evaluation - Consider option 2: transparent to UE under test operation - Option 3(Huawei, Nokia): Considering the timeline of this WI, the basic feature M=1 should be prioritized and therefore the Wf is assumed with all-one vector if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. Recommendations for 2nd round: - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work. Issue 4-2-2 Test Metric Candidate options: - Proposals - Option 1 (Ericsson, Samsung, Nokia): follow PMI with random PMI if the requirement is introduced - Option 2 (Anritsu, Huawei, Samsung(compromise), Nokia (compromise)): Following FeType II CB PMI over Type I single panel random PMI. Recommendations for 2nd round: - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work. Issue 4-2-3: CSI-RS precoder Candidate options: - Proposals - Option 1 (Huawei, Nokia): - Each angle-delay pair in the collection represent a tap with the fixed delay associated with one CSI-RS port so TE can keep CSI-RS precoding unchanged during the test. - Select all CSI-RS ports with no non-zero coefficients (i.e. α=1, β=1, configuration 4 in TS 38.214 Table 5.2.2.2.7-1) if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. Recommendations for 2nd round: - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work. Issue 4-2-4: Test Scenario Candidate options: - Proposals - Option 1 (Huawei): Only consider SU-MIMO setup if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined - Option 2 (Nokia): both SU-MIMO and MU-MIMO in Rel-18 timeframe if introduced Recommendations for 2nd round: - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work. Issue 4-2-5: Test Parameters Candidate options: - Proposals - Option 1 (Huawei): Select 12 ports, rank2 and MCS20 if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. For other parameters such as CSI-RS resource Type, reportConfigType, reuse from Rel-16 eTypeII PMI cases - Other options are not precluded Recommendations for 2nd round: - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Issue 4-2-1: BF modelling

- Proposals

  - Option 1(Samsung, Nokia):

    - for wide beam W1 modeling, option 1a (MIMO fading channel as Rel-13 LTE Class B K=1 PMI test case) should be used.

    - for frequency selective precoding Wf  modeling, option 1 (explicitly derived from chosen TDL model) should be used if TE support.

  - Option 2(Ericsson):

    - Power scaling method with fixed or no Wf can be considered to do the initial evaluation

    - Consider option 2: transparent to UE under test operation

  - Option 3(Huawei, Nokia): Considering the timeline of this WI, the basic feature M=1 should be prioritized and therefore the Wf is assumed with all-one vector if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined.

- Recommended WF

  - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work.

| Company | Comments |
| --- | --- |
| Company |  |

Issue 4-2-2 Test Metric

- Proposals

  - Option 1 (Ericsson, Samsung, Nokia): follow PMI with random PMI if the requirement is introduced

  - Option 2 (Anritsu, Huawei, Samsung(compromise), Nokia (compromise)): Following FeType II CB PMI over Type I single panel random PMI.

- Recommended WF

  - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work.

| Company | Comments |
| --- | --- |
| Anritsu | Our stance is still supporting option 2. But as commented during the 1st round, if the group still see a benefit to introduce option 1 (follow PMI with random PMI), it would be very much appreciated if the specific way of randomization could be provided. |

Issue 4-2-3: CSI-RS precoder

- Proposals

  - Option 1 (Huawei, Nokia):

    - Each angle-delay pair in the collection represent a tap with the fixed delay associated with one CSI-RS port so TE can keep CSI-RS precoding unchanged during the test.

    - Select all CSI-RS ports with no non-zero coefficients (i.e. α=1, β=1, configuration 4 in TS 38.214 Table 5.2.2.2.7-1) if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined.

- Recommended WF

  - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work.

| Company | Comments |
| --- | --- |
| Company |  |

Issue 4-2-4: Test Scenario

- Proposals

  - Option 1 (Huawei): Only consider SU-MIMO setup if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined

  - Option 2 (Nokia): both SU-MIMO and MU-MIMO in Rel-18 timeframe if introduced

- Recommended WF

  - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work.

| Company | Comments |
| --- | --- |
| Company |  |

Issue 4-2-5: Test Parameters

- Proposals

  - Option 1 (Huawei): Select 12 ports, rank2 and MCS20 if Rel-17 FeTypeII PS codebook PMI reporting requirements are defined. For other parameters such as CSI-RS resource Type, reportConfigType, reuse from Rel-16 eTypeII PMI cases

  - Other options are not precluded

- Recommended WF

  - Encourage companies to further discuss the feasible test setup. Capture the discussion and any outcomes in a (potentially informative) WF to serve as a starting point for future RAN4 work.

| Company | Comments |
| --- | --- |
| Company |  |

# Topic #5: CR work split

Based on current progress,  consideirng the timeline of FeMIMO WI performance part , targeting to complete in this meting, Companies are encouraged to provide the draft CR based on the offline discusion about CR work split work.

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.101-4 |  |  |
|  | Big CR | Samsung |
| 5.1.1 | Applicability of requirements | Nokia |
| 5.3.2 5.3.3 | Minimum requirements for PDCCH with inter-slot repetition (including 2Rx/4Rx for FDD and TDD) | Ericsson |
| 5.2.2 5.2.3 | Minimum requirements for PDSCH HST-SFN scheme A (including 2Rx/4Rx for FDD and TDD) | Apple |
| 5.2.2 5.2.3 | Minimum requirements for PDSCH HST-SFN scheme B (if introduced, including 2Rx/4Rx for FDD and TDD) | Huawei |
| 6.3.2 6.3.3 | [Single PMI with [x] port TypeI-SinglePanel Codebook for multi-TRP, including 2Rx/4Rx for FDD and TDD] | Samsung |
| A.3.3 | Reference measurement channels for PDCCH performance if necessary | Qualcomm |
| B.3 | High speed Train Scenarios for HST SFN scheme A | Apple |
| B.3 | High speed Train Scenarios for HST SFN scheme B (if introduced) | Huawei |

## Companies views’ collection for 1st round

### Open issues

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on UE demodulation and CSI requirement for FeMIMO | Samsung |  |
|  | Draft CR on applicability of requirements | Nokia | 38.101-4, 17.5.0 , draft CR, catB |
|  | Draft CR on PDCCH requirement for enhancement on multi-TRP | Ericsson | 38.101-4, 17.5.0 , draft CR, catB |
|  | Draft CR on PDSCH requirement for HST-SFN scheme A | Apple | Including channel model, 38.101-4, 17.5.0 , draft CR, catB |
|  | Draft CR on PDSCH requirement for HST-SFN scheme B (if agreed) | Huawei | Including channel model, 38.101-4, 17.5.0 , draft CR, catB |
|  | Draft CR on PMI requirement for multi-TRP | Samsung | 38.101-4, 17.5.0 , draft CR, catB |
|  | Draft CR on Reference measurement channels for PDCCH performance | Qualcomm | 38.101-4, 17.5.0 , draft CR, catB |
|  | Simulation results summary for FeMIMO demodulation and CSI requirement | Samsung |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2214397 |  | WF on UE demodulation and CSI requirements for FeMIMO | Samsung | Agreeable | capturing the agreements from GTW |
| R4-2214398 |  | Draft CR on applicability of requirements | Nokia | Not available |  |
| R4-2214399 |  | Draft CR on PDCCH requirement for enhancement on multi-TRP | Ericsson | Postponed |  |
| R4-2214400 |  | Draft CR on PDSCH requirement for HST-SFN scheme A | apple | Postponed |  |
| R4-2214401 |  | Draft CR on PDSCH requirement for HST-SFN scheme B (if agreed) | Huawei | Postponed |  |
| R4-2214402 |  | Draft CR on PMI requirement for multi-TRP | Samsung | Postponed |  |
| R4-2214403 |  | Draft CR on Reference measurement channels for PDCCH performance | Qualcomm | Postponed |  |
| R4-2214404 |  | Simulation results summary for FeMIMO demodulation and CSI requirement | Samsung | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents