3GPP TSG-RAN WG4 Meeting #104-e	R4-2214382

Electronic Meeting, 15 ‒ 26 Aug, 2022

Title: 	WF on SAN RF requirement for IoT over NTN

Agenda Item:	     12.5.2, 12.5.3

Source: 	ZTE

Document for:	Approval

Sub-topic 3-1 General principle for SAN RF requirement for IoT over NTN

Agreements:

  - Proposal 1: consider the NTN NR terminology captured in TS 38.108, as the baseline for the NTN IoT work and TS 36.108skeleton, where possible.

  - Proposal 2: reuse the already defined set of NR NTN SAN classes (i.e. GEO, LEO600, LEO1200) for NTN IoT WI.

  - Proposal 3: For NTN IoT, consider SAN type 1-H and SAN type 1-O as the baseline.

Sub-topic 3-2 Conducted RF requirements for NTN SAN

Agreements:

For SAN type 1-O, it could be further discussed once SAN type 1-H is complete.

| Requirement | Requirement clause of 36.108 | Updates to support SA NB-IoT | Updates to support SA Cat-M1 |
| --- | --- | --- | --- |
| Satellite Access Network output power | 6.2 | reuse from 38.108; declaration based | reuse from 38.108; declaration based |
| Output power dynamics | 6.3 | N/A NOTE: “NB-IoT RB power dynamic range for in-band or guard band operation”: this requirement is not applicable to SA | Add 1.4MHz channel BW for total power dynamic range, remove other channel BW |
| Transmit ON/OFF power | NA | not applicable in NTN scenario, as only for TDD | N/A |
| Frequency error | 6.5.1 | reuse from 36.104 | reuse |
| Modulation quality | 6.5.2 | Reuse from 36.104; Set of considered modulations scheme may be revisited | reuse |
| Time alignment error | NA | not applicable in NTN scenario | N/A |
| DL RS power | Not exist | DL RS power was E-UTRA specific. Motivation and the requirement value to be discussed next meeting. | DL RS power was E-UTRA specific. Motivation and the requirement value to be discussed next meeting. |
| Occupied bandwidth | 6.6.2 | reuse from 38.108 | reuse from 38.108 |
| ACLR | 6.6.3 | Depend on co-existing discussion/simulation | Depend on co-existing discussion/simulation |
| Operating band unwanted emissions | 6.6.4 | Depend on co-existing discussion/simulation | Depend on co-existing discussion/simulation |
| Transmitter spurious emissions | 6.6.5 | Follow ITU-R recommendation SM.329; consider 4 kHz measurement bandwidth as per ITU recommendation SM.1541-6 Protection of the own SAN RX to be defined as well | Follow ITU-R recommendation SM.329; consider 4 kHz measurement bandwidth as per ITU recommendation SM.1541-6 Protection of the own SAN RX to be defined as well |
| Transmitter intermodulation | NA | Follow the same with NTN SAN , i.e. not applicable in NTN scenario | Follow the same with NTN SAN |
| Reference sensitivity level | 7.2 | Add NB-IoT channel BW and REFSENS - Reuse the same FRC and SNR for standalone NB-IoT - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Reuse the implementation margin as 2dB; | Add 1.4MHz channel BW and REFRENS, remove other channel BW - Reuse the same FRC and SNR for 1.4MHz LTE BW - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Reuse the implementation margin as 2dB; |
| Dynamic range | 7.3 | - Add NB-IoT channel BW - For GEO, to follow the same agreement without requirement defined. - For LEO, - Reuse the same FRC and SNR for standalone NB-IoT - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - FFS for IoT Level for IoT over NTN - Ruse the implementation margin as 2.5dB; | Add 1.4MHz channel BW For GEO, to follow the same agreement without requirement defined. For LEO, - Reuse the same FRC A2-1 and SNR for 1.4MHz LTE BW - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Reuse IoT 15dB for NR over NTN - Ruse the implementation margin as 2.5dB; |
| ACS | 7.4.1 | Depend on co-existing simulation | Depend on co-existing simulation |
| In-band blocking | NA | Follow the same agreement for NR over NTN SAN, i.e. not applicable in NTN scenario | Follow the same agreement for NR over NTN SAN i.e. not applicable in NTN scenario |
| Out-of-band blocking | 7.5 | Reuse from TS38.108 Follow worst case scenarios identified in 38.863 for NR NTN, as baseline. | Reuse from TS38.108 Follow worst case scenarios identified in 38.863 for NR NTN, as baseline. |
| Receiver spurious emissions | 7.6 | Reuse the requirement from 38.108 | Reuse the requirement from 38.108 |
| Receiver intermodulation | NA | Follow the same agreement for NR over NTN SAN , i.e. not applicable in NTN scenario | Follow the same agreement for NR over NTN SAN i.e. not applicable in NTN scenario |
| In-channel selectivity | 7.8 | Not needed [Not needed as no requirement for NB-IoT SA in TS36.104.] | - For GEO and LEO, to reuse ICS level as 12 dB for GEO, 24dB for LEO used for NR over NTN in Rel-17. - Reuse the same FRC A1-4 and SNR for 1.4MHz LTE BW; - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Reuse the implementation margin as 2dB; |