3GPP TSG-RAN4 Meeting # 104e	R4-2214402

Electronic Meeting, 15 Aug - 26 Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PMI requirement for multi-TRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO_Demod |  |  |  |  |  | Date: |  |  | 2022-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For FeMIMO performance, RAN4 has agreed to introduce PMI measurement requirement for multi-TRP |  |  |  |  |  |  |  |  |
| Summary of change: |  | For FeMIMO performance, RAN4 has agreed to introduce Single PMI with 8TX TypeI-SinglePanel PMI measurement requirement for multi-TRP, including 2Rx/4Rx for both FDD and TDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for FeMIMO performance will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3.2.1.X	Single PMI with 8TX TypeI-SinglePanel Codebook for Single-DCI basedm-TRP transmission scheme

For the parameters specified in Table 6.3.2.1.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.X-2.

Table 6.3.2.1.X-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 20 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |  | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | FDD |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 2) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #0 | Resource #1 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (6) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #0}, with K1=1 CMR group #2: {NZP CSI-RS resource #1}, with K2=1 CMR paring: {NZP CSI-RS resource #0, NZP CSI-RS resource #1} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | Codebook Type |  |  |  | typeI-SinglePanel |  |  |
|  |  | Codebook Mode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000010 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 8 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.1-6.2 |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.2.1.X-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.6] |

6.3.2.2.X	Single PMI with 8TX TypeI-SinglePanel Codebook for Single-DCI based scheme

For the parameters specified in Table 6.3.2.2.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.X-2.

Table 6.3.2.2.X-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 40 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |  | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | TDD |  |  |
| TDD DL-UL configurations |  |  |  |  |  | FR1.30-1 as specified in Annex A |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 2) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #0 | Resource #1 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (6) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #0}, with K1=1 CMR group #2: {NZP CSI-RS resource #1}, with K2=1 CMR paring: {NZP CSI-RS resource #0, NZP CSI-RS resource #1} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | Codebook Type |  |  |  | typeI-SinglePanel |  |  |
|  |  | Codebook Mode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000010 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 6.5 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.2-8.2 TDD |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.2.2.X-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.6] |

6.3.3.1.X	Single PMI with 8TX TypeI-SinglePanel Codebook for Single-DCI based scheme

For the parameters specified in Table 6.3.3.1.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.X-2.

Table 6.3.3.1.X-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 20 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |  | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | FDD |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 4 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 2) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #0 | Resource #1 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (6) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #0}, with K1=1 CMR group #2: {NZP CSI-RS resource #1}, with K2=1 CMR paring: {NZP CSI-RS resource #0, NZP CSI-RS resource #1} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | Codebook Type |  |  |  | typeI-SinglePanel |  |  |
|  |  | Codebook Mode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000010 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 8 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.1-6.2 |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.3.1.X-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.6] |

6.3.3.2.X	Single PMI with 8TX TypeI-SinglePanel Codebook for Single-DCI based scheme

For the parameters specified in Table 6.3.3.2.X-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.2.X-2.

Table 6.3.3.2.X-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 40 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |  | 20 for CSI-RS resources 5 and 6 21 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | TDD |  |  |
| TDD DL-UL configurations |  |  |  |  |  | FR1.30-1 as specified in Annex A |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 4 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 2) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #0 | Resource #1 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (6) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #0}, with K1=1 CMR group #2: {NZP CSI-RS resource #1}, with K2=1 CMR paring: {NZP CSI-RS resource #0, NZP CSI-RS resource #1} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | Codebook Type |  |  |  | typeI-SinglePanel |  |  |
|  |  | Codebook Mode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000010 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 6.5 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.2-8.2 TDD |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.3.2.X-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.6] |