3GPP TSG-RAN WG4 Meeting # 104-e 												R4-2214485

Electronic Meeting, 15th – 26th August, 2022

Agenda item:			9.18.3, 9.18.3.1 and 9.18.4

Source:	Ericsson

Title:	WF on RedCap RRM requirements

Document for:	Approval

# Introduction

This way forward contains all agreements from RAN4#104-e in email thread: [104-e][223] NR_redcap_RRM_1

# Agreements from discussions on ‘General’

## Small data transmission for RedCap

SDT FR2 requirements

Reuse the FR2 requirements for SDT for legacy NR devices defined in clause 5.5.3 for RedCap UE in clause 5.2B.2.1, where:

    - X1 in FR2 TA validation rules is set to max{480ms, 8*SMTC periodicity}.

    - Z1 is set to 640 ms.

SDT for RedCap with eDRX

FFS: TA validation requirements for RedCap CG-SDT is defined for UE configured with eDRX in RRC_INACTIVE state by reusing the principles from DRX requirements introduced in R17 SDT WI.

Note: eDRX in RRC_INACTIVE state does not have PTW.

# Agreements from discussions on ‘Mobility requirements’

## Sub-topic 2-1 Handover

Issue 2-1-1: Requirements for HO to a RedCap specific BWP with NCD-SSB (no CD-SSB) (Scenario 1)

RAN4 to reuse legacy HO requirements for handover directly to RedCap specific BWP with NCD-SSB only without measurement except Tsearch relaxation from 1 Rx reception.

# Agreements from discussions on ‘Timing requirements’

Timing requirements when SSB is not in the active BWP

No consensus reached to add the conditions to existing requirements.

# Agreements from discussions on ‘Signaling characteristics’

Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17

No consensus reached.

# Agreements from discussions on ‘Measurement procedure’

### Use of NCD-SSB for CONNECTED mode measurements

Whether to define requirements to handle measurement type transition between NCD-SSB and CD-SSB,

When BWP-specific servingCellMO is configured, RAN4 to define requirements for

- Transition from CD-SSB to NCD-SSB and vice versa for RLM/BFD/CBD measurements

- Transition from CD-SSB to NCD-SSB and vice versa for intra-frequency measurement.

- Transition from intra-frequency (with/without MGs) to inter-frequency (with/without MGs) and vice-versa

Follows the Pre-MG transition requirements defined in section 9.1.7 of TS 38.133, i.e. UE can restart the measurement after BWP switching.

Serving cell threshold associated SSB

FFS: The serving cell thresholds of s-MeasureConfig for Connected mode should be checked based on reference SSB measurement.

# Agreements from reply LS

### Sub-topic 6-2: Cell-specific RSRP offset

Applicability of cell-specific RSRP offset

Agreements are captured in the LS in R4-2214484.

### CSSF, gap related issues

Issue 5-2-1: CSSF assumptions for intra/inter-frequency measurement with MG

If intra-frequency measurement is with MG, CSSFoutside_gap,i = Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG.

When SMTC occasions of inter-frequency measurement object are partially overlapped by the measurement gap are measured outside of MG, RedCap UEs should perform inter-frequency MOs outside MG. If UE supports this inter-frequency without gap, the flag of [inter-frequency_config_R16] is configured by network.

Issue 5-2-2: Whether to support for per-FR gap

If a RedCap UE support both FR1 and FR2, whether RedCap UE can support per-FR gap(e.g., independentGapConfigdf) depends on UE capability.

Define the requirements only considering per-UE gap in Rel-17.

# Agreements from discussions on RRM performance part

### Sub-topic 7-3 Testing of SDT

Issue 7-3-1: SDT test case

For 1 Rx. RedCap UEs in FR1, consider the test case with offsetRSRP if test case associated with sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB is introduced in Rel-17 SDT test.

For 1 Rx. RedCap UEs in FR1, consider the test case with offsetRSRPChange,CG-SDT under different RSRP change set-up if TA-validation for CG-SDT is introduced in Rel-17 SDT test.

Note: Following bullet is related to demodulation requirements and shall not be discussed under RRM thread.

Reuse DL/UL pattern for the UE demodulation/CSI requirements as a baseline for SDT RRM test of RedCap UEs

# Other notes from RRM performance part

Moderator’s note: Moderator sees some companies introduced the test case related to NCD-SSB. However, there is not any agreement in the group for NCD-SSB test case.

Issue 1: Whether to define test case for NCD-SSB?

Candidate options:

- Option 1: Yes

- Option 2: No

Issue 2: Which test case(s) should be used for NCD-SSB test in Handover if issue 1 is yes?

| Test Index | Test |
| --- | --- |
| 1 | Intra-frequency handover from FR1 to FR1; known target cell for UE (1Rx, 2Rx) |
| 2 | Intra-frequency handover from FR1 to FR1; unknown target cell for UE (1Rx, 2Rx) |
| 3 | Inter-frequency handover from FR1 to FR1; unknown target cell for UE (1Rx, 2Rx) |

Issue 3: Which test case(s) should be used for NCD-SSB test in intra-frequency measurement if issue 1 is yes?

| Test Index | Test |
| --- | --- |
| 1 | SA event triggered reporting tests without gap under non-DRX (1Rx, 2Rx) |
| 2 | SA event triggered reporting tests with per-UE gaps under non-DRX (1Rx, 2Rx) |
| 3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading (1Rx, 2Rx) |
| 4 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (1Rx, 2Rx) |

Issue 4: What’s the NCD-SSB configuration can be introduced if issue 1 is yes?

Candidate options:

- Option 1: NCD-SSB periodicity 80ms, NCD-SSB offset 5ms

- Option 2: NCD-SSB periodicity 40ms, NCD-SSB offset 20ms

- Option 3: Others