3GPP TSG-RAN WG4 Meeting #104-e	R4-2214554

# Electronic meeting, August 15 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR | 0009 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.176-2 with introduction of timing error between IAB-DU and IAB-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Perf |  |  |  |  |  | Date: |  |  | 10-08-2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is CR to TS 38.176-2 Rel-17 that introduces test description for new introdcued in Rel-17 IAB core specification TS 38.174 timing error between IAB-DU and IAB-MT. New clause is added to clause 6 Transmitted signal quality and accroding work split done in WF R4-2210643. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new clause 6.6.5 for Timing error between IAB-DU and IAB-MT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel- 17 IAB conformance test specification will not include test for timing error between IAB-DU and IAB-MT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 6.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.176-1 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of R4-2213979. |  |  |  |  |  |  |  |  |

### 6.6.5	Timing error between IAB-DU and IAB-MT

#### 6.6.5.1	Definition and applicability

This requirement shall apply to IAB-DU DL and IAB-MT UL simultaneous transmission.

The timing error between IAB-DU and IAB-MT is specified for a specific set of simultaneous signals/transmitter  configuration/transmission mode.

#### 6.6.5.2	Minimum requirement

The minimum requirement for IAB type 1-O is in TS 38.174 [2], clause 6.5.4.

The minimum requirement for IAB type 2-O is in TS 38.174 [2], clause 9.6.4.

#### 6.6.5.3	Test purpose

To verify that the OTA timing error between IAB-DU and IAB-MT simultaneous transmission is within the limit specified by the minimum requirement.

#### 6.6.5.4	Method of test

##### 6.6.5.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

IAB-DU RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see clause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see clause 4.9.1.

Directions to be tested: OTA coverage range reference direction (D.35).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met considering both polarisations. If the measurement antenna does not support dual polarization, time alignment error shall be measured under the condition that measurement antenna is aligned between the IAB-DU polarisations such that it receives half the power from each polarisation.

##### 6.6.5.4.2	Procedure

1)	Place the IAB-DU and IAB-MT at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the IAB-DU and IAB-MT with the test system.

3)	Orient the positioner (and IAB-DU and IAB-MT) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beamforming settings of the IAB-DU and IAB-MT according to the direction of the testing.

5)	Set the IAB type 1-O to transmit IAB-DU-FR1-TM1.1 or IAB-MT-FR1-TM1.1. using the configuration with the minimum number of cells and reference signals.

Set the IAB type 2-O to transmit IAB-DU-FR2-TM 1.1 or IAB-MT-FR1-TM1.1 using the configuration with the minimum number of cells and reference signals.

For an IAB-DU declared to be capable of single carrier operation only, set the IAB-DU to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model at manufacturer's declared rated output power, Prated,c,TRP.

For IAB type 1-O declared to be capable of multi-carrier operation, set the IAB-DU and IAB-MT to transmit according to the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test model on all carriers configured.

For IAB type 2-O declared to be capable of multi-carrier operation, set the IAB-DU and IAB-MT to transmit according to the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test model on all carriers configured.

6)	Measure the timing error between the DM-RS symbols on the IAB-DU and IAB-MT beams. Note that the possible difference in DM-RS symbol position and slot number shall be compensated for in the measured timing error.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 6.6.5.5	Test Requirement

##### 6.6.5.5.1	IAB type 1-O

The timing error between IAB-DU and IAB-MT shall not exceed minimum requirement plus measurement uncertainty defined in Table 4.1.2.2-1.

##### 6.6.5.5.2	IAB type 2-O

The timing error between IAB-DU and IAB-MT shall not exceed minimum requirement plus measurement uncertainty defined in Table 4.1.2.2-2.