3GPP TSG-RAN4 Meeting #104-e	R4-2214750

# Online, , 15th  - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDSCH demod requirements in ICI-FDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 introduced PDSCH demod requirements in ICI. Some of the parameters and requirements are TBD. Updates are needed to complete the requirements definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addressed the TBDs in PDSCH requiremetns in ICI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for PDSCH demod in ICI is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.15 5.2.3.1.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Change 1---------------------

##### 5.2.2.1.15	Minimum requirements for PDSCH with inter-cell interference

The performance requirements are specified in Table 5.2.2.1.15-3, with the addition of test parameters in Table 5.2.2.1.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.15-1.

Table 5.2.2.1.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance under 2 receive antenna conditions, when transmission from the serving cell is interfered by 1 or 2 interfering cells. | 1-1, 1-2 |

Table 5.2.2.1.15-2: Test parameters

| Parameter |  |  |  | Unit |  | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 |  | Cell 2 | Cell 3 |  |
|  |  |  |  |  |  | Enabled |  | Enabled | Enabled for Test 1-1 Disabled for Test 1-2 |  |
| Duplex mode |  |  |  |  |  | FDD |  |  |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |  |  |
| Physical cell ID |  |  |  |  |  | 0 |  | 1 | 2 |  |
| Transmission rank |  |  |  |  |  | 1 |  | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-1 N/A for Test 1-2 |  |
| Time offset to Cell1 |  |  |  | us |  | N/A |  | 3 | -1 |  |
| Frequency offset to Cell 1 |  |  |  | Hz |  | N/A |  | 300 | -100 |  |
| Interference Model |  |  |  |  |  | N/A |  | As specified in B.6.2 |  |  |
| INR (Note 2) |  |  |  | dB |  | N/A |  | 7.77 for Test 1-1 7.58 for Test 1-2 | 2.29 for Test 1-1 N/A for Test 1-2 |  |
| SSB configuration |  | SSB position in burst |  |  |  | First SSB in Slot #0 |  | 1st SSB in Slot#0 for Test 1-1 2nd SSB in Slot #0 for Test 1-2 | 1st SSB in Slot#0 for Test 1-1 N/A for Test 1-2 |  |
|  |  | SSB periodicity |  | ms |  | 20 |  | 20 | 20 |  |
| PDSCH configuration |  | Mapping type |  |  |  | Type A |  |  |  |  |
|  |  | k0 |  |  |  | 0 |  |  |  |  |
|  |  | Starting symbol (S) |  |  |  | 2 |  |  |  |  |
|  |  | Length (L) |  |  |  | 12 |  |  |  |  |
|  |  | PDSCH aggregation factor |  |  |  | 1 |  |  |  |  |
|  |  | PRB bundling type |  |  |  | Static |  |  |  |  |
|  |  | PRB bundling size |  |  |  | 2 |  |  |  |  |
|  |  | Resource allocation type |  |  |  | Type 0 |  |  |  |  |
|  |  | RBG size |  |  |  | Config2 |  |  |  |  |
|  |  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |  |  |  |
|  |  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |  |  |  |
| PDSCH DMRS configuration |  | DMRS Type |  |  |  | Type 1 |  |  |  |  |
|  |  | Number of additional DMRS |  |  |  | 1 |  |  |  |  |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |  |  |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |  |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |  |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 |  |  |  |  |  |  |  |  |  |  |

Table 5.2.2.1.15-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [R.PDSCH.1-2.1 FDD] | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | [15.4] |
| 1-2 | [R.PDSCH.1-2.1 FDD] | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [12.6] |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

<Unchanged sections skipped>

-----------------Change 2---------------------

##### 5.2.3.1.15	Minimum requirements for PDSCH with inter-cell interference

The performance requirements are specified in Table 5.2.3.1.15-3, with the addition of test parameters in Table 5.2.3.1.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.15-1.

Table 5.2.3.1.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance in 4 receive antenna conditions, when the transmission from the serving cell is interfered by 1 or 2 interfering cells. | 1-1, 1-2 |

Table 5.2.3.1.15-2: Test parameters

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
|  |  |  | Enabled | Enabled | Enabled for Test 1-1 Disabled for Test 1-2 |  |
| Duplex mode |  |  | FDD |  |  |  |
| Active DL BWP index |  |  | 1 |  |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |  |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-1 N/A for Test 1-2 |  |
| Time offset to Cell 1 |  | us | N/A | 3 | -1 |  |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |  |
| Interference Model |  |  | N/A | As specified in B.6.2 |  |  |
| INR (Note 2) |  | dB | N/A | 7.77 for Test 1-1 7.58 for Test 1-2 | 2.29 for Test 1-1 N/A for Test 1-2 |  |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | 1st SSB in Slot#0 for Test 1-1 2nd SSB in Slot #0 for Test 1-2 | 1st SSB in Slot#0 for Test 1-1 N/A for Test 1-2 |  |
|  | SSB periodicity | ms | 20 | 20 | 20 |  |
| PDSCH configuration | Mapping type |  | Type A |  |  |  |
|  | k0 |  | 0 |  |  |  |
|  | Starting symbol (S) |  | 2 |  |  |  |
|  | Length (L) |  | 12 |  |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |  |
|  | PRB bundling type |  | Static |  |  |  |
|  | PRB bundling size |  | 2 |  |  |  |
|  | Resource allocation type |  | Type 0 |  |  |  |
|  | RBG size |  | Config2 |  |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |  |
|  | Number of additional DMRS |  | 1 |  |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 |  |  |  |  |  |  |

Table 5.2.3.1.15-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [R.PDSCH.1-2.1 FDD] | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x4, ULA Low | 70 | [9.9] |
| 1-2 | [R.PDSCH.1-2.1 FDD] | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | [7.5] |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |