# 3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214965

# Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TC for joint unified TCI state switching in FR2 NR SA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to agreed WF (R4-2210617) in the last meeting, the test case for joint unified TCI state switching in FR2 NR SA shall be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new test case for joint unified TCI state switching. The settings are provided below. - Joint TCI state switch on serving cell in FR2 NR SA - MAC CE based switch - The target TCI state is known but not in the active TCI list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2212518 |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.7.5.Y.1	MAC-CE based active joint TCI state switching

##### A.7.5.Y.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state

A.7.5.Y.1.1.1	Test Purpose and Environment

The purpose of this test is to verify both active downlink and uplinke TCI state switch delay requirement defined in clause 8.15 and 8.16, respectively, by using joint TCI state of unified TCI state switch framework. In this test, the target TCI state is not in the active TCI state list for PDSCH/PDCCH, and UE is capable of beam correspondence without the need for UL beam sweeping, i.e. beamCorrespondenceWithoutUL-BeamSweeping is set to 1. Supported test configuration is shown in Table A.7.5.Y.1.1.1-1.

The test scenario comprises of one NR PCell (Cell 1) as given in Table A.7.5.Y.1.1.1-2. Cell-specific parameters of NR PCell are specified in Table A.7.5.Y.1.1.2-1 below. The OTA related test parameters for FR2 are shown in Table A.7.5.Y.1.1.2-2.

Table A.7.5.Y.1.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.Y.1.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 160 | Periodic L1-RSRP reporting configured |
| Number of RS for L1-RSRP reporting |  | 2 | Two source RSs in TCI state 0 and 1. |
| T1 | s | 0.2 |  |
| T2 | s | 2 |  |

A.7.5.Y.1.1.2	Test parameters

Before the test starts,

- UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

- PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending.

- UE is provided with dl-OrJoint-TCIStateList-r17 and UE’s higher layer signalling unifiedTCI-StateType-r17 in IE MIMOParam-r17 is set to joint;

- UE is configured with 2 different joint TCI states for PCell, TCI state 0 (QCL’d to SSB0) and TCI state 1 (QCL’d to SSB1), and the TCI state 1 is not in the active TCI state list for PDSCH/PDCCH.

- UE is indicated TCI state 0 as the active PDCCH TCI state

- Index of CSI-RS#0 is configured for UE as pathlossReferenceRS-Id-r17 which is indicated in dl-OrJoint-TCIStateList-r17 of TCI state 0. CSI-RS#0 is QCLed typeD with SSB0. CSI-RS#0 as pathloss RS is maintained by UE.

Editor note’s: FFS whether the CSi-RS #0 is “maintained” or “not configured”.

The test consists of two time periods, T1 and T2. During T1 only source RS in TCI state 0 is transmitted. At the beginning of T2, source RS in TCI state 1 start transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms after the slot in which UE provides L1-RSRP report with results for both source RSs in TCI state 0 and 1, UE receives a MAC-CE command indicating a switch to TCI state 1. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies the following UE behavior for joint TCI state switch:

- UE shall be able to receive and transmit with TCI state 0 until slot slot n + THARQ + 3Nslotsubframe,µ, and

- receive and transmit with TCI state 1 from slot n+THARQ + 3Nslotsubframe,µ + (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length

Index of CSI-RS#1 is configured for UE as pathlossReferenceRS-Id-r17 which is indicated in dl-OrJoint-TCIStateList-r17 of TCI state 1. CSI-RS#1 is QCLed typeD with SSB1. UE maintains CSI-RS#1 as pathloss RS before the test starts.

Table A.7.5.Y.1.1.2-1: NR Cell specific test parameters for TCI state switch

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3. 2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP. 5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| PL-RS Configuration (CSI-RS#1) |  | Resource #4 in TRS.2.2 TDD |
| TCI State 0 |  | DLorJoint TCI.State.2 |
| TCI State 1 |  | DLorJoint TCI.State.3 |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.Y.1.1.2-2: OTA related test parameters for TCI state switch

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Source RS in TCI state 0 |  | Source RS in TCI state 1 |  |  |
|  |  | T1 | T2 | T1 |  | T2 |
| Angle of arrival configuration |  | Setup 3 according to clause A.3.15.3 |  |  |  |  |
|  |  | AoA1 |  |  | AoA2 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |
| Ês | dBm/SCS | -80.6 | -80.6 | -Infinity |  | -80.6 |
| SS B_RP Note 2 | dBm/ SCS | -80.6 | -80.6 | -Infinity |  | -80.6 |
| BB Note 7 | dB | 8.3 | 8.3 | -Infinity |  | 8.3 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.0 | -56.0 | - Infinity |  | -56.0 |
| Note 1: Void Note 2: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

A.7.5.Y.1.1.3	Test Requirements

The test verifies that UE can be scheduled by PCell on TCI state 0 and TCI state 1.

During T2, UE shall send L1-RSRP report with results for source RSs in both TCI state 0 and 1.

After receiving MAC-CE command in slot n, UE shall:

-	be able to receive and transmit with TCI state 0 until  slot n + THARQ + 3Nslotsubframe,µ

-	be able to start receiving and transmitting with TCI state 1 after slot n + THARQ + 3Nslotsubframe,µ + (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length

The rate of correct events observed during repeated tests shall be at least [90]%.

Editor’ note: FFS whether the rate of correct event should be added.

<End of Change 1>