3GPP TSG-RAN4 Meeting #104-e	R4-2215149

# Online, , 15th  - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for test case for FeMIMO - Unified TCI state switching for EN-DC in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has agreed to define test cases to verify the active DL TCI state switch delay requirement for unified TCI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced test cases for active DL TCI state switch for EN-DC in FR2 for cell with different PCI from serving cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Active DL TCI state switch for EN-DC in FR2 for cell with different PCI from serving cell will not be tested and verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Change 1---------------------

### A.5.5.X	Unified TCI state switching delay

#### A.5.5.X.3	MAC-CE based active DL TCI state switch to cell with additional PCI

##### A.5.5.X.3.1	E-UTRAN – NR PSCell FR2 DL TCI state switch for a known TCI state

###### A.5.5.X.3.1.1	Test Purpose and Environment

The purpose of this test is to verify the active DL TCI state switch delay requirement for unified TCI defined in clause 8.15.3. Supported test configurations are shown in Table A.5.5.X.3.1.1-1.

Table A.5.5.X.3.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

A.5.5.X.3.1.2	Test Parameters

The test scenario comprises of one E-UTRA PCell (Cell 1), one NR PSCell (Cell 2), and one NR cell with additional PCI different from from serving cell (Cell 3) configured for intercell L1-RSRP measurement and report as given in Table A.5.5.X.3.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell and cell with additional PCI are specified in Table A.5.5.X.3.1.1-3 below. The OTA related test parameters for FR2 is shown in Table A.5.5.X.3.1.1-4.

PDCCHs indicating new transmissions shall be sent continuously on PSCell (Cell 2) and cell with additional PCI (Cell 3) to ensure that the UE would have ACK/NACK transmission.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC), Cell 3 (Cell with additional PCI) in radio channel 3.

- UE is provided with dl-OrJoint-TCIStateList-r17 and UE’s higher layer signalling unifiedTCI-StateType-r17 in IE MIMOParam-r17 is set to seperate;

-    UE is configured with L1-RSRP measurements on cell with additional PCI (Cell 3)

-	UE is configured with 2 different TCI states for PSCell, PDCCH TCI state 0 (QCL’d to TRS resource set 1, TCI state of which is QCLed to SSB0 of Cell2) and TCI state 1 (QCL’d to TRS resource set 3, TCI state of which is QCLed to SSB1 of Cell3), in Cell 2 before starting the test.

-	UE is indicated in TCI state 0 as the active PDCCH TCI state

The test consists of two time periods, T1 and T2. Figure A.5.5.X.3.1.1-1 and Figure A.5.5.X.3.1.1-2 show the Time multiplexed (allocation in Frequency is symbolic) downlink transmissions from each Angle of Arrival. During T1 only SSB to which PDCCH-TCI-state0 is QCL’d is transmitted. At the beginning of T2, the SSB corresponding to TCI state 1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms of UE providing L1-RSRP report with results for both SSB0 of cell 2 and SSB1 of cell 1, UE receives a MAC-CE command indicating a switch to TCI state 1. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies that UE can be scheduled on PSCell on TCI state 0 till n+ THARQ +3 ms. The test equipment also verifies the TCI state switch time in to cell with additional PCI by scheduling the UE on TCI state 1 after n+ THARQ +3 ms + (Tfirst-SSB + TSSB-proc) .

Table A.5.5.X.3.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| Cell with addiitonal PCI |  | Cell 3 | Cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | 0.2 |  |
| T2 | s | 2 |  |

Table A.5.5.X.3.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter | Unit | Cell 2 | Cell 3 |
| --- | --- | --- | --- |
| Frequency Range |  | FR2 | FR2 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 | 100 MHz: NRB,c = 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3. 2 TDD | SR.3. 2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD | CCR.3.1 TDD |
| OCNG Patterns |  | OP. 5 | OP. 5 |
| SSB Configuration |  | SSB.1 FR2 | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 | SMTC.1 |
| TCI State 0 |  | DLorJoint TCI.State.0 | N/A |
| TCI State 1 |  | N/A | DLorJoint TCI.State.1 |
| TRS Configuration |  | TRS.2.1 TDD | TRS.2.2 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Propagation Condition |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.5.5.X.3.1.1-4: OTA related test parameters for TCI state switch

| Parameter | Unit | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |
|  |  | T1 | T2 | T1 | T2 |
| Angle of arrival |  | Setup 3 according to clause A.3.15.3 |  |  |  |
| configuration |  | AoA1 |  | AoA2 |  |
| Ês | dBm/SCS | -80.6 | -80.6 | -Infinity | -80.6 |
| SS B_RP Note 2 | dBm/ SCS | -80.6 | -80.6 | - Infinity | -80.6 |
| BB Note 7 | dB | 8.3 | 8.3 | -Infinity | 8.3 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.0 | -56.0 | - Infinity | -50.0 |
| Note 1: Void Note 2: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |

Figure A.5.5.X.3.1.1-1: Time multiplexed downlink transmissions during T1

Figure A.5.5.X.3.1.1-2: Time multiplexed downlink transmissions during T2

###### A.5.5.X.3.1.2	Test Requirements

During T2, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive on TCI state 0 till   n+ THARQ +3Nslotsubframe,µ

-	be able to start receiving on TCI state 1 after n+ THARQ +(5 ms + Tfirst-SSB) / NR slot length

The rate of correct events observed during repeated tests shall be at least 90%.

Editor’ note: FFS whether the rate of correct event should be added.