3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211508

# Electronic Meeting, August 15 – August 26, 2022

3GPP TSG RAN WG1 #109-e			R1-2205502

e-Meeting, May 9th – 20th, 2022

Title:	LS on UL Tx switching across 3 or 4 bands in Rel-18

Response to:

Release:	Rel-18

Work Items:	NR_MC_enh-Core

Source:	RAN WG1

To:	RAN WG4

CC:

Contact Person:

#### Name:	Hiroki Harada

E-mail Address:	hiroki.harada@docomo-lab.com

1. Overall Description:

According to the approved WID on Multi-carrier enhancements for NR, RAN1 has started the discussion on UL Tx switching across 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs at RAN1#109-e meeting.

Based on the contributions submitted to the RAN1#109-e meeting, RAN1 made following observations based on evaluation results.

RAN1 observation

- Four contributions [1], [2], [3] and [4] from three companies show their evaluation results on UL Tx switching across 3 or 4 bands at RAN1#109-e meeting.

  - All evaluation results show the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 2 bands, assuming TDD bands with different TDD UL/DL configurations are included in 4 bands.

    - Evaluation results in [1] show the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 3 bands.

    - Evaluation results in [2] show that the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 2 bands depends on achievable switching period, and the longer switching period for UL Tx switching across 4 bands compared with UL Tx switching across 2 bands leads to reduction of the performance gain. Other evaluation results did not consider the impact of longer switching period for UL Tx switching across 4 bands compared with UL Tx switching across 2 bands.

    - Evaluation results in [4] observe that the gain highly depends on the scheduling mechanism.

    - The range of performance gains shown in four contributions varies depending on the simulation assumptions.

In addition, RAN1 is under discussion to identify possible mechanisms for dynamic Tx carrier switching across the 3 or 4 configured bands and following agreement was made.

Agreement

- Companies are encouraged to investigate pros and cons of following possible mechanisms for dynamic Tx carrier switching across the configured bands, and RAN1 strives for the down-selection at RAN1#110

  - Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via UL grant and/or RRC configuration for UL transmission

  - Alt.2: NW indicates 2 bands out of the configured bands (3 or 4 bands) via DCI or MAC-CE, and dynamic Tx carrier switching between indicated bands is same as Rel-17

  - Alt.3: One anchor band is selected among configured bands (3 or 4 bands), and dynamic Tx carrier switching can be performed only from the anchor band to a non-anchor band and from a non-anchor band to the anchor band

  - Note: Other mechanisms are not precluded

Based on the above observations and agreement, RAN1 also agreed to send this LS to respectfully ask RAN4’s feedback on the potential increase of switching period and UE’s complexity in case of UL Tx switching across 3 or 4 bands in comparison to 2 bands. RAN4’s prompt feedback will be appreciated and would be helpful for potential down-selection at RAN1#110.

Additionally, RAN1 would like to also ask RAN4’s feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands.

- “When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period.”

2. Actions:

To RAN WG4

ACTION:

- RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on potential increase of switching period and UE’s complexity in case of UL Tx switching across 3 or 4 bands in comparison to 2 bands.

- RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands.

  - “When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period.”

3. References:

[1]	R1-2203136	Discussion on multi-carrier UL Tx switching	Huawei, HiSilicon

[2]	R1-2204724	On multi-carrier UL Tx switching scheme	MediaTek Inc.

[3]	R1-2204909	Simulation evaluations of single user experience data rate for multi-carrier UL Tx switching	Huawei, HiSilicon

[4]	R1-2205131	Discussion on Multi-carrier UL Tx switching scheme	ZTE

4. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #110			22nd August – 26th August 2022		Toulouse, FR.