3GPP TSG-RAN WG4 #104-e			    R4-2215346

e-Meeting, October 10th – 19th, 2022

Agenda item:       6.17.2.2.1

Source: 	Qualcomm Incorporated

Title: 	SBFD feasibility and impact on RF requirements: BS aspects

Document for:	Discussion

# Introduction

In RAN4#104-e, work on the new Study Item regarding the evolution of NR duplex operation was started [1]. In this paper, we continue the discussion on the feasibility and impact of SBFD deployments on gNB RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator gNB CLI.

# Self-interference mechanisms and mitigation techniques

In RAN4#104-e, the following agreements on feasibility and self-interference modelling were reached [1]:

Topic 1: RAN4 feasibility study and RF requirement impact for SBFD operation Criteria on gNB UL receiver sensitivity degradation due to self-interference: - Taking 1dB sensitivity degradation due to self-interference of DL transmission as starting point for system level evaluation and feasibility study - Other values lower than 1dB e.g., 0.1dB/0.8dB not precluded pending on the feasibility study - Final values used in co-existence evaluation shall be aligned with feasibility analysis conclusion. Topic 2: Self-interference modelling for gNB capable of SBFD operation according to RAN1 LS - Agreement on granularity in frequency domain and question on frequency flat model possibility - Proposal: RSI can be modelled as (almost) frequency flat at least could be scaled to subband level with FFS on below aspects - FFS on guardband assumption between subband for SBFD - FFS on necessity/feasibility on RB level scaling - Agreement on RSI dependency on Blocking and AGC - Proposal 1: The in-band blocking is suggested to applied as starting point to ensure the receiver of UL sub-band is not blocked due to DL sub-band transmission - Besides blocking, LNA and dynamic range can be FFS for receiver side - Proposal 2: AGC may be applied to adjust the receiver gain to avoid ADC saturation if spatial isolation and analog IC, if found feasible, don’t provide enough reduction to self-interference. This may result in cost of an impact on sensitivity and potentially reduced coverage. However, it seems infeasible to model this in SLS. - Note: above proposal will not preclude other study regarding this issue. The size of any impact of coverage associated with AGC should in particular be elaborated. - Agreement on dependency on gNB antenna and beam related - Proposal 1: gNB antenna architecture has impact on RSI model as to achieve high spatial isolation, separate antenna panels between TX and RX chain are requested - Proposal 2: TX/RX beam pair can further contribute to RSI pending on implementation. - Proposal 3: the RSI will have dependency at least on the listed factors in RAN1 LS, but further details will need to be studied in RAN4.

To enable proper reception of the uplink signal at the SBFD gNB receiver, gNB should mitigate the direct self-interference ‘leakage’ and any significant clutter reflections. The amount of residual self-interference into the UL subband depends on the following factors:

- gNB Tx power of the DL signal

- gNB self-interference mitigation capability

The self-interference could be mitigated by different techniques such as spatial isolation, analog subband filter, analog interference cancellation, beamforming and digital interference cancellation.  It is expected that the residual self-interference might cause the noise floor of the SBFD gNB to rise as a result of the residual self-interference. RAN4 discussed in RAN4#104-e [2] how to capture this UL receiver degradation within the adjacent channel coexistence framework. Majority of the companies supported to model this increase as {N=noise floor+1dB}, while some companies wanted to consider lower values than 1 dB. The latter will impose additional burdens on the design complexity and cost which might yield the feature infeasible. Also, if the analysis showed that the 1 dB margin cannot be met, it is safe to say that other scenarios/ assumptions will need to be revisited, while 0.1- or 0.5-dB values will overestimate the impact.  In the remaining of this section, we continue the discussion on the feasibility of different self-interference mitigation stages.

Proposal 1: RAN4 to agree on the metric {N=noise floor+1dB} as a criterion on gNB UL receiver sensitivity degradation due to self-interference to balance between meaningful analysis of SBFD impact and practical complexity.

## Antenna techniques and spatial isolation

For SBFD deployments, gNB antenna configurations should be based on two panels configuration with split of the antenna elements for simultaneous downlink transmission and uplink reception. on the other hand, for legacy TDD deployments, gNB antenna configuration is based on single panel for downlink transmission or uplink reception. With the split panel architecture, the gNB can enable larger spatial isolation is an essential component to mitigate self-interference. In addition, the physical separation between the two panels could be used to add electro-magnetic spatial duplexer that enhances the spatial isolation between the panels. RF measurements for the Tx-Rx spatial fields, including near-field Tx and Rx gains, has been conducted and results are shown in detail in [3], where it has been shown that more than 80 dB of spatial isolation could be achieved using two separate panels with spatial duplexer.

Proposal 2: RAN4 to agree on the value range of 80-90 dB of spatial isolation via be based on two panels configuration with split of the antenna elements for simultaneous downlink transmission and uplink reception.

## Beam nulling and clutter mitigation

In FR1, the DL precoder and UL combiner weights could be optimized to provide some beamform nulling for the clutter and/or self-interference, with additional degrees of freedom for massive MIMO deployments. Beamforming nulling is an efficient technique for clutter mitigation. In FR2, spatially isolated and narrow Tx and Rx beams could be selected to provide extra ‘beam’ isolation, which is an additional factor to be combined with the antenna isolation.

For clutter mitigation, proper selection Tx and Rx beam pair can be used to alleviate the clutter impact, which results from the transmitted signal travelling through the wireless medium and impinging on different scatterers and reflectors. Based on measurements conducted in [4], additional self-interference cancellation utilizing beam nulling in the value range of 5-10 dB was achieved for massive MIMO deployment.

Proposal 4: RAN4 to agree on the value range of 5-10 dB for beam nulling and clutter mitigation for FR1 and FR2 SBFD deployments.

## Digital residual self-interference cancellation

The nonlinearities introduced within the gNB front’s end due to non-ideal components of the Tx chain will lead to residual non-linear self-interference that cannot be fully captured in the RF or analog domain due to the associated high complexity, high sensitivity of the canceler and the system’s stability. In this regard, leveraging adaptive filtering and non-linear modeling of the residual self-interference to accurately model and cancel the residual self-interference is performed to provide additional mitigation in the digital domain and enable higher MCS. A non-linear interference cancellation measurement have been reported in [3] and it was shown that 10-15 dB of additional mitigation can be achieved in the digital domain.

Proposal 5: RAN4 to agree on the value range of 10-15 dB for residual self-interference cancellation in the digital domain for FR1 and FR2 SBFD deployments.

## Aggregate self-interference mitigation capability

Based on the above self-interference mitigation budgets, we can drive a value range for the RSI αSI  at the gNB for SBFD deployments. Table 1 presents the different self-interference mitigation components as well as the aggregate value for the gNB.

Table 1 Aggregate self-interference mitigation budget

|  | Mitigation capability for FR1 (FR2) |
| --- | --- |
| Ant. isolation | 80 dB (80-90 dB) |
| Freq. isolation | 45 dB (28 dB) |
| Tx/Rx beam nulling or beam isolation | 5~10 dB (5~10 dB) |
| Digital IC | 10~15 dB (10 dB) |
| RSI αSI | 140~150 dB (120-140 dB) |

Proposal 6: RAN4 to agree on the value range of the aggregate self-interference mitigation RSI αSI  of 140-150 dB (120-140 dB) for FR1 (FR2).

# Co-channel CLI aspects

In RAN4#104-e, the following agreements on co-channel CLI modelling were reached [1]:

Agreement on feasibility and how to model co-site inter-sector gNB-gNB CLI modelling: similar modelling as for self-interference (RSI) can be applied but may with different parameters especially on antenna isolation - FFS on possibility to apply digital IC for this case On feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity: Agreement: - Proposal: Same Transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation. - TX leakage candidate: gNB ACLR - Receiver impairment candidate: gNB ACS - RAN4 will further study the possibility of improved performance/requirements compared to existing referred requirements list above.

## 3.1 Inter-site inter-sector inter-gNB

For inter-site inter-sector inter-gNB CLI, RAN4 agreed to adopt gNB ACLR and ICS requirements (i.e., Adjacent channel interference ratio per subband) as a starting point. In order to address the inter-gNB CLI, it would be desirable to choose Tx/Rx beams that minimize that inter-gNB CLI. In Rel-16 RIM framework [4], there is no support for beam-based interference detection. Consequently, Inter-gNB coordination can help reduce impact of inter-gNB CLI in SBFD.

# Adjacent-channel CLI aspects

In RAN4#104-e, the following agreements on adjacent channel CLI modelling were reached [1]:

Agreement on feasibility and how to model co-site gNB-gNB CLI modelling: - Proposal : as no path loss model applicable this modelling could be different compared with inter-site gNB-gNB CLI modelling with below alternatives: - Alternative 1: ACLR and ACS based with potential other solution from SBFD capable gNB to cancel co-cite adjacent channel interference(i.e. ACLR from the SBFD gNB towards the victim or ACS impact from the aggressor towards the SBFD gNB) - A non-SBFD aggressor or victim in the adjacent channel should be assumed to have ACLR or ACS according to the RAN4 specifications - Note: RAN4 will further study the possibility of improved performance/requirements compared to existing referred requirements list above. - Alternative 2: similar modelling as for self-interference(RSI) can be applied but may with different parameters especially on antenna isolation and required overall isolation if both gNBs with SBFD capability - And digital IC is not feasible if gNBs belong to different operators for this case Agreement on feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity: - Proposal: to agree with gNB ACLR based model on TX and gNB ACS requirements based model on RX - Path loss should be addressed due to distance between gNBs. - FFS on separate calculation from ACLR and ACS perspective to address potential different antenna gain for wanted signal and unwanted signal (e.g.different antenna modelling for wanted signal and unwanted signal). - Note: RAN4 will further study the possibility of improved performance/requirements compared to existing referred requirements list above.

## Co-site inter-sector inter-gNB

RAN4 considers for co-located gNBs that there is a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class. Considering the proposed ACLR and ACS values, Figure 2 presents the gNb receiver’s desense as a function of the inter-gNB isolation for co-located channel adjacent deployments considering UMa with TRP = 44 dBm, noise figure of 5 dB and 64 chains employed at the gNB. It is clear that the current 30 dB inter-gNB isolation will lead to more than 40 and 60 dB desense given the current ACLR and ACS requirements.  It can also be observed that the inter-gNB isolation is in the ballpark of 80 dB in order to meet the 1 dB desense level for FR1. For FR2, advanced antenna techniques and configurations will be necessary to provide the required inter-gNB isolation. Based on such large requirement for the inter-gNB isolation, it is realistic that for co-site-inter-sector inter-gNB similar modelling as for self-interference (RSI) should be considered.

Figure 2 gNB Rx desense as a function of inter-gNB isolation for co-located adjacent deployments

Proposal 8: For co-located gNBs, the current RAN4 30 dB isolation is not sufficient to address the inter-gNB CLI. For current ACLR and ACS RAN4 requirements, inter-gNB isolation in the ballpark of 80 dB is required for feasible SBFD deployments.

Proposal 9: For co-site inter-sector inter-gNB adjacent channel CLI, similar modelling as for self-interference (RSI) should be considered.

## Inter-site inter-sector inter-gNB

When a gNB is transmitting downlink while an adjacent one is receiving uplink, inter-gNB CLI depends on three main factors 1) inter-gNB channel, 2) the transmitting gNB unwanted emissions (i.e., leakage on the adjacent subband), and 3) the receiving gNB selectivity. In order to determine the impact of CLI on the RF requirements, adequate modelling of the CLI is required. Inter-gNB inter-subband CLI depends on the two following components:

- Inter-gNB channel (i.e., inter-gNB coupling loss, cross antenna gain, etc.)

- Inter-subband leakage

- gNB receiver inter-subband selectivity

The inter-subband leakage is typically generated due to transmitter non-linearities, which may leak to receiver, and cause an increase in the receiver’s thermal noise floor. ACLR and IBE at the UE side requirements are defined to account for the adjacent channel leakage and in-band co-channel emission. While at gNB side, BS ACLR is defined for whole channel transmission and no BS IBE is defined for subband transmission.

Although gNB ACLR requirements provides a decent measure of unwanted emissions on the adjacent channel, it does not account for the sub-band granularity. we can assume that the ACLR is flat across the sub-bands, which has also been shown true as reported in the lab measurement in [4]. Accordingly, 45 and 28 dB ACLR values can be used for FR1 and FR2-1, respectively (see Table 6.6.3.2-1 and Table 9.7.3.3-1, in 38.104). Based on this, it is reasonable for RAN4 to consider the ACLR and ACS (i.e., ACIR) when studying the inter-site inter-sector inter-gNB CLI in the adjacent channel operation.

# Conclusion

Throughout this contribution, we provided our views on CLI modelling and self-interference mitigation in SBFD. Our proposals can be summarized as follows:

Proposal 1: RAN4 to agree on the metric {N=noise floor+1dB} as a criterion on gNB UL receiver sensitivity degradation due to self-interference to balance between meaningful analysis of SBFD impact and practical complexity.

Proposal 2: RAN4 to agree on the value range of 80-90 dB of spatial isolation based on two panels configuration with split of the antenna elements for downlink transmission and uplink reception.

Proposal 4: RAN4 to agree on the value range of 5-10 dB for beam nulling and clutter mitigation for FR1 and FR2 SBFD deployments.

Proposal 5: RAN4 to agree on the value range of 10-15 dB for residual self-interference cancellation in the digital domain for FR1 and FR2 SBFD deployments.

Proposal 6: RAN4 to agree on the value range of the aggregate self-interference mitigation RSI αSI  of 140-150 dB (120-140 dB) for FR1 (FR2).

Proposal 7: For co-located gNBs, the current RAN4 30 dB isolation is not sufficient to address the inter-gNB CLI. For current ACLR and ACS RAN4 requirements, inter-gNB isolation in the ballpark of 80 dB is required for feasible SBFD deployments.

Proposal 8: For co-site inter-sector inter-gNB adjacent channel CLI, similar modelling as for self-interference (RSI) should be considered.

# References

- R4-2214377, “WF on feasibility study from RF perspective”, Samsung RAN4#104-e.

- R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, Samsung RAN4#104-e.

- R1-2205031, “Feasibility and techniques for Subband non-overlapping full duplex”, Qualcomm RAN1#109-e