3GPP TSG-RAN WG4 Meeting # 104b-e	R4-2215355

# Electronic meeting, 10th-19th, Aug., 2022

Agenda Item:	6.9.3

Source:	Intel Corporation

Title:	Discussion on RRM requirements about FR1-FR1 NR-DC

Document for:	Discussion

# Introduction

In RAN96 , the WID about FR1-FR1 NR-DC is agreed, which is shown below and WF is derived[1].

- Define RRM requirements for FR1-FR1 NR-DC scenarios [RAN4] - RRM requirements include the number of serving carriers, PSCell addition/release delay requirement, PSCell change and conditional PSCell change delay, scheduling availability, and CSSF. Other Rel-15 requirements are not precluded and are subject to WI stage discussion. - For R16 and R17 features, RRM requirements for FR1-FR1 NR-DC including HO with PSCell, SCG activation/deactivation and CPAC. - Note: no other R16/17 features are considered

# RRM requirements

Number of serving carriers

Issue 3-2-1: Number of serving carriers Agreement: - RRM requirements for supported number of serving carriers for NR-DC need be updated for FR1-FR1 NR-DC according to RF specification. - FFS on exact number of serving carriers for NR-DC. - Option 1: For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell. - Option 2: Exact number of serving carriers will be updated until the late phase of Rel-18 when RF spec is in more stable.

The number of serving carriers need to be updated for FR1-FR1 NR-DC, however, the exact number would depend on RF specification. At current stage, we prefer option2, i.e. Exact number of serving carriers will be updated until the late phase of Rel-18 when RF spec is in more stable.

Proposal 1: Exact number of serving carriers will be updated until the late phase of Rel-18 when RF spec is in more stable.

Issue 3-2-4: Interruption requirement for PSCell addition/release for FR1+FR1 NR-DC - FFS - Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC): For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused. - Option 2(Nokia, Ericsson, vivo): NR-DC interruption requirements need to be updated to support FR1-FR1 NR-DC. - Option 2a(vivo): When defining the interruption requirements, for the number of SCells for FR1-FR1 NR-DC, up to 3 SCell(s) in FR1 of PCell and up to 3 SCell(s) in FR1 of PSCell are configured, deconfigured, activated or deactivated.

For interruption length for PSCell additoin for FR1+FR1 NR-DC, the legacy requirement for NR PSCell addition can be reused.

For requirement applicability part, in current spec of 8.2.4.1, it specified that:

This clause contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are configured, deconfigured, activated or deactivated or,

Here, 1 Scell  in FR1 and 7 Scells in FR2 are defined for FR1+FR2 NR DC. It needs to be updated for FR1+FR1 NR DC. We are fine to update the SCell number in FR1 in FR1+FR1 NR DC case according to RF spec.

Proposal 2: For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused. For requirement applicability part, SCell number will be updated in FR1 according to RF Spec.

Scheduling availability of UE performing RLM/BFD/CBD/L1-RSRP in case of FR1-FR1 NR-DC

For FR1-FR1 NR-DC, it’s similar with inter-band FR1 CA. there are no scheduling restrictions on FR1 serving cell(s) due to RLM/BFD/CBD/L1-RSRP performed on FR1 PSCell in 38.133.

- When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands.

Therefore, there is also no scheduling restriction for FR1-FR1 NR-DC for RLM/BFD/CBD/L1-RSRP.

For current spec, there are separate chapter for FR1-FR2 inter-band CA and NR-DC, where only FR1+FR2 NR-DC is considered. We think that FR1+FR1 DC can also be clarified in this chapter. One example of newly added clarification for RLM is as below, the other modification for BFD/CBD/L1-RSRP can be similar.

#### 8.1.7.4 Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC There are no scheduling restrictions on FR1 serving cell(s) due to radio link monitoring performed on FR2 serving PCell and/or PSCell. There are no scheduling restrictions on FR2 serving cell(s) due to radio link monitoring performed on FR1 serving PCell and/or PSCell. [ There are no scheduling restrictions on FR1 serving cell(s) due to radio link monitoring performed on FR1 serving PCell and/or PSCell.].

We also notice that in chapter 9.5.6.4, the scheduling availability didn’t consider NR-DC scenario, we also need to add the NR-DC into the chapter title.

Proposal 3:  Scheduling restrictions for UE performing RLM/BFD/CBD/L1-RSRP in case of FR1-FR1 NR-DC needs to be clarified in the specification chapter 8.1.7.4,  8.6.7.4, 8.5.8.4 and 9.5.6.4.

CSSF for FR1+FR1 NR-DC

Issue 3-2-11: CSSF outside gap for FR1+FR1 NR-DC Agreement: - For CSSF, add the scenario of FR1 PCell + FR1 PSCell in CSSFoutside_gap,i. - FFS on option 1a and 2. - Option 1a(Apple, Xiaomi, OPPO, vivo, MTK, Ericsson, HW): CSSF outside gap for FR1+FR1 NR-DC is: Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR1 PSCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap. FR1 + FR1 NR-DC (FR1 PCell and FR1 PScell); 1+NPCC_CSIRS; 2×( NSCC_SSB +Y+2xNSCC_CSIRS); 2x(1+ NPSCC_CSIRS) Note 2; N/A; N/A; 2x(NSCC_SSB +Y+2x NSCC_CSIRS ).

In FR1+FR2 NR DC, 2 searchers are assumed that one searcher is dedicated for PCC and half of the other searcher is dedicated to PSCC, while the remaining half of the searcher is shared among FR1 SCCs and FR2 SCC. Based on similar rule, we suggest to have the same scaling factor for FR1+FR1 NR-DC without gap .

Since one search is assumed for PCC/PSCC, therefore, CSSFoutside_gap,i for FR1 PSCC remains no change, which is still 2x(1+ NPSCC_CSIRS). Therefore, we are fine with option 1a.

Proposal  4: Support option 1a for CSSF outside gap for FR1+FR1 NR-DC.

For CSSF inside measurement, for FR1+FR2 NR DC, how to classify the PCell SCC and PSCell SCC into group A and group B is dependent on whether interfrequency/interRAT/PRS measurement exists or not.

When interfrequency/interRAT/PRS measurement exists, FR1 and FR2 intrafrequency measurement objects belong to group A. It applies to both FR1+FR1 NR DC and FR1+FR2 NR DC. No modification is needed.

However, when the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero, FR1 intrafrequency and FR2 intrafrequency will be classifed into group A and B respectively. It only apply for FR1+FR2 NR DC case, which are as follows:

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero and the UE is configured with per UE gaps: FR1 intrafrequency measurement objects belong to group A FR2 intrafrequency measurement objects belong to group B MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j equals 0. MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j equals 0. Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For FR1+ FR1 NR DC, we need to modify the grouping rule.

There are two options:

Option 1: add additional bullet for FR1+FR1 NR DC:

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero and the UE is configured with per UE gaps in case of FR1-FR1 NR-DC: - intrafrequency measurement objects of FR1 PCell belong to group A - intrafrequency measurement objects of FR1 PSCell belong to group B

Option 2: update the condition which can apply for both FR1+FR2 and FR1+FR1:

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero and the UE is configured with per UE gaps: - intrafrequency measurement objects of PCell belong to group A - intrafrequency measurement objects of PSCell belong to group B

We prepare option 2.

Proposal 5: for CSSF within gap, update the grouping rule:

- If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero and the UE is configured with per UE gaps:

- intrafrequency measurement objects of PCell belong to group A

- intrafrequency measurement objects of PSCell belong to group B

HO with PSCell

The existing RRM requirement for HO with PSCell only cover the case for HO from FR1-FR2 NR-DC to FR1-FR2 NR-DC. For HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC, the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused.

For FR1-FR2 NR-DC to FR1-FR1 NR-DC, the controversial part is whether Tprocessing is 40ms or 20ms. According to Tprocessing defined in section 8.11, Tprocessing = 40 ms when source and target cells are in different FRs. Therefore, we prefer Option 2.

Proposal 6: For HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC, the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused.

Proposal 7: For HO with PSCell for FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

# Conclusion

In this contribution, we provide our views regarding FR1+FR1 NR-DC:

Proposal 1: Exact number of serving carriers will be updated until the late phase of Rel-18 when RF spec is in more stable.

Proposal 2:For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused. For requirement applicability part, SCell number will be updated in FR1 according to RF Spec.

Proposal 3:  Scheduling restrictions for UE performing radio link monitoring in case of FR1-FR1 NR-DC needs to be clarified in the specification chapter 8.1.7.4,  8.6.7.4, 8.5.8.4 and 9.5.6.4.

Proposal 4: Support option 1a for CSSF outside gap for FR1+FR1 NR-DC.

Proposal 5: For CSSF within gap, update the grouping rule:

- If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero and the UE is configured with per UE gaps:

- intrafrequency measurement objects of PCell belong to group A

- intrafrequency measurement objects of PSCell belong to group B

Proposal 6: For HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC, the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused.

Proposal 7: For HO with PSCell for FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

# Reference

[1] R4-2214345, WF on R18 eFeRRM, Apple