3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215365

# Electronic Meeting, October 10 – October 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2587 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 1Rx. margin for RedCap UEs configured with relaxed measurement criterion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the proper configuration of cell-specific 1 Rx. THLDs for relaxed measurement criteria evaluation in RRC IDLE/INACTIVE, UE need to apply single [1.0] dB offset for all the related THLDs configured by higher layer signaling. The sign (negative or postive) of 1 Rx. offset is to be specfied per each THLDs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce 1 Rx. configuring margin for RedCap UEs UEs configured with relaxed measurement criterion in RRC IDLE/INACTIVE as agreed in LS to RAN2 R4-2214484. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE with 1 Rx. may not be able to configure proper THLDs for relaxed measurment criteria. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2B.2.9.1, 4.2B.2.10.1, 4.2B.2.11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

### 4.2B.2.9 	Measurements of intra-frequency NR cells for UE configured with relaxed
measurement criterion for RedCap

4.2B.2.9.1 		Introduction
This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera:
    - Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in [1],
    - Relaxed measurement criterion for a stationary UE not at cell edge defined in clause 5.2.4.9.4 in [1],
    - Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in [1]
       respectively

For RedCap UE with 1 Rx, [1] dB offset is applied to the thresholds used for lowMobilityEvaluation, cellEdgeEvaluation, stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary in [2]. [The offset is subtracted from s-SearchDeltaP [2] and s-SearchDeltaP-Stationary [2] while it is added to other RSRP and RSRQ thresholds].

Editor’s Note: FFS: Sign of the offset to each threshold can be added based on the agreement.

### <End of Change 1>

### <Start of Change 2>

### 4.2B.2.10 	Measurements of inter-frequency NR cells for UE configured with relaxed
measurement criterion for RedCap

4.2B.2.10.1 		Introduction
This clause contains the requirements for measurements on inter-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera:
    - Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in [1],
    - Relaxed measurement criterion for a stationary UE not at cell edge defined in clause 5.2.4.9.4 in [1],
    - Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in [1]
       respectively

For RedCap UE with 1 Rx, [1] dB offset is applied to the thresholds used for lowMobilityEvaluation, cellEdgeEvaluation, stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary in [2]. [This offset is subtracted from s-SearchDeltaP [2] and s-SearchDeltaP-Stationary [2] while it is added to other RSRP and RSRQ thresholds].

Editor’s Note: FFS: Sign of the offset to each threshold can be added based on the agreement.

### <End of Change 2>

### <Start of Change 3>

### 4.2B.2.11 	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed
measurement criterion for RedCap

4.2B.2.11.1 		Introduction
This clause contains the requirements for measurements on inter-RAT E-UTRAN cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera:
    - Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in [1],
    - Relaxed measurement criterion for a stationary UE not at cell edge defined in clause 5.2.4.9.4 in [1],
    - Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in [1]
       respectively

For RedCap UE with 1 Rx, [1] dB offset is applied to the thresholds used for lowMobilityEvaluation, cellEdgeEvaluation, stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary in [2]. [The offset is subtracted from s-SearchDeltaP [2] and s-SearchDeltaP-Stationary [2] while it is added to other RSRP and RSRQ thresholds].

Editor’s Note: FFS: Sign of the offset to each threshold can be added based on the agreement.

### <End of Change 3>