3GPP TSG-RAN4 Meeting #104-bis-e	R4-2215382

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.6.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Lower MSD signalling and the effects of the introduction

Document for:	Approval

# Introduction

RAN4#104-e approved WF of [1], where several way forwards related to signaling aspects are captured.

- Necessity of signaling improved lower MSD

- Clarification of the expected behaviour from NW for the potential lower MSD capability

- How lower MSD capability is indicated which corresponds to “How to indicate the MSD is improved for a band combination”

- How to obtain the threshold(s) of improved MSD

This contribution shares our views on each of the above listed aspects.

# Necessity of signaling improved lower MSD

This section discusses the necessity of signaling improved lower MSD (the below top way forward) by  sharing potential ways to utilize the feature in terms of NW (the below bottom way forward). It should be noted that the actual ways on how to utilize the feature can be different from NW to NW as captured in the below way forward and also may not be the same as described in this paper

Though there have been several proposals as lower MSD feature thus far, they would be roughly split into two categories.

- Static approach:

  - A UE reports a lower MSD capability to indicate the maximum MSD that the UE has under the same test conditions specified in RAN4 specifications. NW can obtain this information when the UE camp on a cell.

- Dynamic approach:

  - A UE reports occurring MSD depending on request by NW and/or autonomous decision by the UE

It should be noted that the issue originally raised was that band combinations with specified MSD, e.g., 30 dB would render the band combinations useless, and it becomes a disincentive for operators to deploy the band combinations even if some of the UEs can have even less MSD than the specified MSD. Consequently, this leads to a lose-lose situation between UE vendors and operators/infra-vendors.

Observation 1: The originally raised issue was the current situation leads to a lose-lose situation between UE vendors and operators/infra-vendors because huge MSD in the specifications would become a disincentive for operators to deploy some band combinations even if some UEs can have even less MSD than the specified MSD.

We also would like to point out that there is fundamental difference between the static and dynamic approaches. The static approach may help NW do admission control, e.g., the NW may be able to avoid a situation that NW doesn’t configure a UE with certain CA whose specified MSD is huge, e.g., 30 dB at all as discussed in [3] if such a NW exists. On the other hand, the dynamic approach focuses on optimization on a situation where MSD is occurring.

Observation 2: Static approach focuses on providing NW with complementary information to assist the NW to carry out a proper decision making on carrier-assignment in a way that the UE is not suffering from MSD if assigned

Observation 3: Dynamic approach focuses on optimizing NW scheduling on UEs experiencing MSD

## Static approach

Roughly there may be two approaches, e.g., DL centric approach and UL centric approach to make maximum use of the reported lower MSD capability. We discuss each of the perspectives assuming CA_n3-n78 configuration with consideration of UL harmonic interference (to fall into n78) and IMD2(to fall into n3 DL) as an example to make the discussion simpler. It should be noted that there are other aspects not mentioned in the below.

### DL centric approach

For common to DL CA and UL CA, as mentioned in Section 2, the static approach can help NW do admission control and the NW may be able to avoid a situation that NW doesn’t configure a UE with certain CA whose specified MSD is huge, e.g., 30 dB at all if there are NW that has not deployed band combinations with huge MSD in the specifications at all.

With respect to DL CA, a NW would check n78 DL carrier channel condition before configuration and the NW may have a threshold to configure a UE with n78 DL carrier if such a NW exists. In this case, lower MSD capability would be utilized in a way that if the NW has a threshold, e.g., RSRP in event A4 is X dBm for n78 configuration for a UE without lower MSD capability meaning that the UE may have the specified MSD, the threshold can be optimized according to lower MSD capability. For example, if there are UEs with lower MSD capability, e.g., with 10 dB, 20 dB and 30 dB less MSD than the specified MSD, the threshold becomes “X – 10” dBm, “X – 20” dBm and “X – 30” dBm, respectively. Regarding UL CA, the NW also may have a threshold on n3 DL carrier to be impacted by IMD2 to configure n78 UL like DL CA. It should be noted that in addition to RSRP, other metrics may be used for deciding carrier assignment/activation, such as RSRQ, RS-SINR, CQI, Pathloss etc. The MSD-derived offsets on those metrics may also be applied for such decisions.

Observation 4: Static DL centric approach can address the original issue as well as help NW set more appropriate threshold for CA configuration not to cause MSD in a similar way written in TS38.331 for DL and UL CA.

### UL centric approach

Regarding DL CA, a NW would check DL channel condition for n78 carrier to be configured before the configuration in any case. Additionally, the NW may take n3 UL power into account, e.g., if the power is lower than a threshold set by the NW, the NW may configure a UE with an additional n78 DL carrier. It means that NW needs to store relation between UL power, DL metric and expected MSD in advance. It leads to complications in setting the threshold since if MSD may happen depending on UL power and a metric to estimate DL channel condition. Given that in any case, UL centric approach uses a metric to estimate DL channel condition, this approach can be considered that it is established on top of DL centric approach. For example, a situation may be that n78 DL signal level is not high enough compared to the DL threshold while n3 UL power is less than maximum output power. Hence, MSD may not happen. Another example may be that n78 DL signal threshold can be adjusted according to the UL power of n3 (or perhaps, vice versa). UL centric approach, however, requires power information in a timely manner. In addition, it requires more accurate UL and DL power levels than DL centric.  For example, if UL power has a measurement deviation for upper side and DL power has it for lower side, it may lead an wrong decision making. . Though some measures like PHR, pathloss information may be used for NW to obtain n3 UL power, they may not provide directly the output power of n3. Finally, finer optimization would require finer granularity and precision of the reported power and DL signal metric. Since the signal levels for UL and DL would fluctuate to some extent, if MSD occurs or not may change based on relation between UL power and DL signal metric very often. Hence, the decision of CA configuration may go back and forth while if the NW takes conservative threshold to avoid it, the benefit of UL centric may be lost.

With respect to UL CA, the NW may take into account the respective powers of two bands as well as ratio of the order of the respective bands, e.g., 1:1 for IMD2(f2-f1) etc., to estimate the extent of the respective bands’ power on MSD on top of n3 DL carrier channel condition estimation as well as the aspects elaborated in UL centric approach for DL CA. Moreover, this approach becomes useful under the assumption that most of the UEs do not transmit signal at maximum output power while it may not be always the case. Finally, this would require even more complex implementation than DL centric approach. This is because a NW unit controlling n3 may estimate n3 UL power via PHR, pathloss etc., apart from accuracy and granularity of frequency of the report while the NW for n3 needs to know UL power of n78 via a NW unit controlling n78. Hence, more tight coordination between n3 and n78 units are required. Finally, it’s also noted that in order to efficiently utilize the UL information, there may be an impact on other WGs like RAN1/2 to optimize UL report scheme.

Observation 5: With respect to static UL centric approach, it would be built up on DL centric approach. The UL centric approach may enable NW to set a threshold in a more flexible way than DL centric approach while the additional gain is uncertain. Specifically, it’s expected that the lower UE’s MSD is, the lower the benefit may become while the UEs’ MSD is not lower enough and channel conditions are not stable, it may not work. Lastly, applying UL centric approach to UL CA requires even more complicated specifications and implementations than DL centric approach while the benefit is uncertain as mentioned before.

## Dynamic approach

As one of the examples, there was a proposal for a UE to dynamically report MSD occurring to a NW. This requires standardization work not only in RAN4 but also in RAN1/2 as well as even more complicated implementation than any of the static approaches mentioned in sub-section 2.1. Hence, it must be out of scope of this WI from the beginning. In addition, a benefit to inform the NW of MSD occurring may be very limited due to following reasons. Since MSD is occurring on a victim DL carrier in any case, the NW may indirectly presume the situation via several DL and/or UL existing reporting metric as well as link failure. For DL CA, if n78 carrier is affected by MSD due to UL harmonics, the NW may just de-configure the UE with DL CA or de-activate n78 carrier. Measures that NW can take may not change with or without dynamic MSD reporting that much given that the rest of the measures that NW can take may be change MCS for UL and/or DL to lower ones while the expected effect is uncertain, and it may not always lead to avoid MSD completely. Of course, if the UL power of the aggressor n3 is at maximum, by scheduling more RBs to n3, the PSD for n3 UL becomes lower and MSD per frequency may be lower while as the price of it, n3 UL may be disconnected due to lack of sufficient UL PSD. For UL CA, if n3 DL carrier quality degradation and/or link failure can be seen right after n78 UL configuration, the NW may somehow presume the root cause as IMD2 due to UL CA. Hence, similar to DL CA, even if the NW knows that n3 DL carrier degradation via dynamic MSD report for UL CA, what the NW can do may be limited. On the other hand, the effort to establish dynamic MSD reporting in terms of standardization as well as implementation is even more significant than any of the static approaches. Moreover, more signaling overhead is expected.

Observation 6: For dynamic MSD reporting as an example, since even if NW receives an MSD value that a UE is experiencing, measures that the NW can take is limited and the effect of it is uncertain while the effort to introduce it would be significant for standardization and implementation. Finally, it would require specification changes on other WGs. Hence, it’s out of scope of this WI from the beginning.

# MSD report indication

This section discusses the following rest of the way forwards related to signalling.

## Threshold(s)

In our understanding, the threshold here means that if a specified MSD for an MSD type for a band combination is e.g., 30 dB and if a UE has e.g., 29 dB MSD, i.e., less than the 30 dB by just 1 dB (note that this is an extreme case), if it is  worth indicating this capability or not. Ultimately, the exact threshold is challenging to propose since it may depend on a way to signalling, band combinations, MSD types or the amount of specified MSD, the number of available bits to indicate lower MSD capability, etc. For example, if a signaling method allowed a sophisticated MSD control, finer granularity would be better. Or even if a signaling method to be adopted doesn’t provide a sophisticated MSD control, still it’s not harmful to report except for signalling overhead perspective. Perhaps, some may think that the meaning of 1 dB difference is different in case of the specified MSD is 30 dB and in case of the specified MSD is 5 dB. Hence, as the way forward mentioned, this aspect is discussed together with a following way forward in section 3.3.

- Whether a single/unique MSD value or MSD threshold(s) for a band combination to be considered

  - If MSD threshold(s), whether a single MSD threshold value or could be multiple intervals? Exact absolute threshold(s) or relative threshold(s)? Different or same threshold(s) for different interference type?

## How lower MSD capability is indicated

This section addresses following way forwards.

- Whether the lower MSD capability is a per BC capability

  - how to handle a band combination with different MSD types

  - how to handle the same BC with different victim bands suffered the same MSD type and order

- Whether lower MSD capability for different interference sources could be reported separately

- Whether lower MSD capability means all MSD types for a band combination have been improved

### Per BC, per MSD type, order and/or per victim band

In order to discuss whether the lower MSD capability is a per BC capability or not, Table 3-1 is provided as below.

Table 3-1: PC 3 MSDs contained in CA_n1-n3-78

| NR-CA | UL | UL | DL | MSD dB | Source of MSD |
| --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | n1 | n3 | n1 | 23 | IMD2 |
| CA_n1-n3 | n1 | N/A | n3 | 3@5MHz, … | Cross band isolation |
| CA_n1-n78 | n1 | N/A | n78 | 8@5MHz | IMD4 |
| CA_ n3-n78 | n3 | N/A | n78 | 23.9@10 MHz,… | Harmonics |
| CA_ n3-n78 | n3 | n78 | n3 | 26@ 5 MHz | IMD2 |
| CA_ n3-n78 | n3 | n78 | n3 | 8@ 5 MHz | IMD4 |
| CA_ n3-n78 | n78 | N/A | n3 | 8.1@ 5 MHz | Harmonic mixing |
| CA_n1-n3-n78 | n1 | n3 | n78 | 28.4@ 10 MHz | IMD2 |
| CA_n1-n3-n78 | n1 | n3 | n78 | 11.2@ 10 MHz | IMD4 |
| CA_n1-n3-n78 | n1 | n78 | n3 | 27.9@ 5 MHz | IMD2 |

### Per BC vs per MSD type and/or order per BC

Though it depends on definition of BC, if CA_n1-n3-n78 is considered, there are several MSDs whose level is more than 20 dB. If only one lower MSD capability per BC is allowed to report, it is too rough and brings the unfortunate situation to lower MSD capability due to following reasons.

- The amount of improvement of the respective MSDs is different, e.g., MSD due to IMD2 for CA_n3-n78 is 15 dB, but MSD due to UL harmonics is 5 dB, the UE needs to report 15 dB. The NW receiving that information has no idea on what to do since it’s not clear if 15 dB is for IMD2 and/or UL harmonics.

  - Even if an accompanied MSD type is reported with 15 dB, NW loses a way to utilize 5 dB MSD for UL harmonics, since the NW cannot know that the MSD for UL harmonics is even better than 23.9 dB.

- Measures that a NW takes would be different according to the amount of MSD, MSD types to types as well as victim band etc.

Hence, a per MSD type and/or order per BC approach provides UE vendors/chipset vendors with more flexibility in terms of UE design. As observed in our companion paper of [4], there are several ways to improve MSD due to the same MSD type (and order). There is a RF component that improves a specific MSD type and/or order while there is a RF component that improves several MSD types and/or order simultaneously. For example, in order to improve MSD due to 2nd harmonic, a UE vendor may use a PA with better H2 suppression. In this case, it may improve MSD due to only the 2nd harmonic, but it may not improve MSD due to other MSD types and/or orders. And if lower MSD capability per MSD type and/or order per BC is selected, UE can report lower MSD capability specific to a certain MSD type and/or order while lower MSD capability per MSD type and/or order per BC would require more signaling overhead than per BC.

Observation 7: Per BC makes signaling simpler than per MSD type and/or order per BC while the former doesn’t provide sufficient information for a NW to make maximum use of it if more than one MSD can be seen under the NW.

### per victim band per MSD type and/or order per BC

In case lower MSD is reported per MSD type and order, signaling may need clarification on affected victim DL bands or frequency component of the order. As can be seen in Table 3-2 where MSD due to IMD for CA_n28-n74 is captured, there are two IMD4 per pair of UL bands per BC. Hence, it is not clear which MSD due to IMD4 is improved even if lower MSD capability is reported per MSD type and order per BC.

Table 3-2: MSD due to IMD for dual UL CA_n28-n74

Observation 8: There is a case that the same BC has two different MSD due to the same MSD type and order, e.g., dual UL CA_n28-n74 has two MSD due to IMD4 where each MSD has a different victim band, i.e., n28 DL or n74 DL. In this case, per MSD type and order per BC cannot differentiate them without additional identification, e.g., victim DL band or frequency component of the order.

### per victim band per MSD type and/or order per BC per PC

If a single low MSD capability is applicable to both PC2 and PC3 or not was discussed in [2]. MSD minimum requirements for different power classes can be significantly different depending on MSD type and the order. For instance, MSD due to IMD4 for PC3 is 8.3 dB while MSD for PC2 is 18.6 dB for dual UL CA_n5-n77 and the difference is more than 10 dB.  Obtaining this difference must be very important. Suppose a UE’s MSD is 10 dB for PC2 and 0 dB for PC3 a given victim band per MSD type and order per BC. Since a NW which doesn’t allow a UE to transmit more than 23 dBm due to regulation, the NW should not refer to the value of 10 dB. But if the NW obtains 0 dB for PC3, then, a measure for 0 dB for PC3 that the NW takes may be different from that for 10 dB for PC2.

Observation 9: MSD minimum requirements for the same MSD type and/or order can be very different from PC to PC, e.g., in some cases, the difference is more than 10 dB, e.g., IMD4 for dual UL CA_n5-n77.

## Lower MSD capability and granularity

This sub-section addresses a following way forward together with threshold aspect discussed in section 3.1.

- Whether delta MSD compared to the minimum requirements or directly improved MSD values to be reported

- Whether a single/unique MSD value or MSD threshold(s) for a band combination to be considered

  - If MSD threshold(s), whether a single MSD threshold value or could be multiple intervals? Exact absolute threshold(s) or relative threshold(s)? Different or same threshold(s) for different interference type?

There was a proposal to define so-called delta between a minimum requirement and a requirement that lower MSD capable UE can meet. It means that if a minimum requirement is 30 dB and reported delta is 10 dB, then, UE’s MSD is lower than 20 dB (30 - 10). Another way would be UE directly indicates an improved MSD value (it’s called “direct approach” in this contribution for convenience).  It means if a UE reports 10 dB, the UE’s MSD is at least lower than 10 dB.

delta approach (delta compared to the min requirements) vs direct approach (directly improved MSD value)

Provided that the same MSD type and order may have different minimum requirements depending on PC and/or UL/DL pair conditions etc., delta approach may have an advantage over direct approach. For example, there may be a case that minimum requirement for PC2 is more stringent than that for PC3 by 10 dB. If a minimum requirement for PC2 is 20 dB and a UE reports a lower MSD capability of 10 dB for PC2 with direct approach, it’s not clear MSD performance for PC3. On the other hand, if delta approach is used and if the spec has a discipline that delta equally applies to both MSD for all the power classes for at least the same MSD type and order per BC, network can consider that MSD for PC2 is lower than that for PC3 by 10 dB which means MSD for PC3 is 0 dB for this example. Though the technical feasibility needs to be further discussed.

Otherwise, there would not be significant difference between them. The importance, however, is that the number of available bits to convey the amount of MSD or the amount of the improved MSD is finite. As discussed later on in granularity discussion, for example, if only two bits are available and maximum MSD specified was e.g., 36 dB in the specifications and the 36 dB is e.g., equally split, the options that UE can have are, 0 dB, 9 dB, 18 dB and 27 dB and in this case, the granularity is 9 dB. Applying this granularity to other cases whose specified MSD is lower, e.g., 18 dB, then, 27 dB is meaningless as an option for this case. Moreover, it loses possible granularity. Hence, whichever option is taken, the granularity must be determined by the number of available bits and the specified MSD per victim band per MSD type/order per BC. More specifically, if the specified MSD is 24 dB for another MSD, then, 0 dB, 6 dB, 12 dB and 18 dB must be used to fully utilize available bits.

Observation 10: If RAN4 has a discipline that the same amount of delta per MSD type and order per BC is applicable regardless of power classes, delta approach may have advantage over direct approach in case the feasibility is confirmed. Otherwise, there may not be significant difference in terms of pros and cons.

Granularity of a lower MSD capability

For the sake of convenience, we call the proposed delta approach in [5] “fixed” delta approach in this paper. There was, however, a raised concern on the fixed delta approach in [6] that “Furthermore, if the “low” MSD requirement is defined as 5 dB, wouldn’t it unfair for UEs with 5.5dB MSD to be excluded from using the combination?”. To make the discussion more concrete, we assume a BC whose MSD due to IMD2 is 35 dB and that due to IMD5 is 11 dB.

IMD2 case: specified MSD is 35 dB

Assume that a UE is allowed to report n * delta, where n is non-negative integer and n = 4 with two bits. MSD range could be defined as shown in Figure 1.

Figure 1: Fixed delta with 2 bits and uneven granularity problem

One of the issues with this fixed delta approach would create a situation that even if a UE can meet MSD of 6 dB, the UE can only report 2*delta of 20 dB in an example shown in Figure 1. Thus, from a network perspective, the performance looks MSD of 15 dB and it is far beyond from the real performance of 6 dB since the difference is 9 dB. The fixed delta approach brings uneven granularity and the impact on unfairness depends on the relation between fixed delta value, a minimum requirement and the number of bits allowed for lower MSD capability.

IMD5 case: specified MSD is 11 dB

The other issue would be that if a UE’s MSD due to IMD5 is 2 dB, the UE cannot indicate the delta at all if the granularity of the delta is 10 dB since the MSD is larger than 1 (11 - 10) dB as shown in Figure 2. NW may consider the MSD is 11 dB and it’s worse than actual MSD by 9 (11 - 2) dB.

Figure 2: An issue that granularity of delta becomes more rough

The issue can be easily mitigated by increasing the granularity, for example, by decreasing delta from 10 dB to 5 dB or even smaller values. However, considering that one UE supports multiple BCs and each of them may have multiple MSD values due to different MSD types and orders such as different IMD orders like IMD2, 3 or 5 etc., finer resolution increases signalling overhead as the price for paying for it,

Observation 11: Reporting a fixed amount of improved MSD value(s) as reference to the minimum requirement value per BC (fixed delta approach) may generate uneven granularity within an MSD as well as MSD across different MSD type and order for the same BC.

In addition, our MSD analysis in [7] observed that larger MSD improvement becomes, i.e., as MSD approaches to MSD = 0 dB, even higher RF components performance improvement it requires as shown in Figure 3 which is an extract of Figure 7 from [4]. It’s noted that Figure 3 is an example of harmonic mixing.

Figure 3: Figure 7 [7]: MSD relation b/w PCB isolation and Mixer spur rejection for 3rd harmonic mixing

Hence, if a fixed delta approach is adopted, efforts to improve MSD close to zero may be underappreciated.

Observation 12: Larger MSD improvement becomes, even higher RF components performance improvement it requires [7] and if a fixed delta approach was adopted, efforts to improve MSD close to zero would be underappreciated.

Given that NW can know minimum requirement values and the number of available bits is limited, e.g., two bits (00, 01, 10, 11), perhaps, it may be a way to divide the minimum requirement values into four MSD regions in an equidistant manner. More specifically, 35 for IMD2 is divided into four regions and it generates 0-8.75, 8.75-17.5, 17.5-26.25 and 26.25-35 dB as shown in Figure 4. Note that if MSD = 0 dB, 11 is reported.

Figure 4: comparison b/w fixed delta and delta in an equidistant way for IMD2

In Figure 4, a benefit of the delta in an equidistant way is that NW can consider UE’s MSD as 8.75 dB (actual value is 6 dB) while with the fixed delta approach, NW considers that the UE’s MSD is 15 dB. Figure 5 shows a comparison in terms of IMD5. Since the minimum requirement for IMD5 is smaller than that for IMD2, delta in an equidistant way can use even finer granularity than fixed delta so that the difference between actual MSD and reported MSD can become smaller.

Figure 5: comparison b/w fixed delta and delta in an equidistant way for IMD5

Observation 13: If delta is defined as the minimum requirement/2n (equidistant delta within each of the minimum requirements) where n is the number of bits, the maximum difference between actual MSD and reported MSD can become smaller than that by fixed delta.

Or perhaps, considering that more efforts, i.e., cost for RF components performance improvement, in order to achieve MSD = 0 dB or at around it, are needed as shown in Figure 3, it may be more suitable to place finer granularity into a region that MSD is closer to 0 dB and coarser granularity into a region that MSD is closer the specified MSD. More specifically, 35 dB for IMD2 is divided into four regions by two bits in a following way.

1st delta: 35 dB / 21 = 17.5 dB à MSD range is 8.75 - 17.5 dB (00)

2nd delta: 1st delta + 35 dB / 22 = 17.5 + 8.75 = 26.25 dB  à MSD range is 4.375 - 8.75 dB (01)

3rd delta: 2nd delta + 35 dB / 23 = 26.25 + 4.375 = 30.625 dB à MSD range is 0 - 4.375 dB (10)

MSD=0 dB (11) are as shown in Figure 6.

Figure 6: comparison b/w fixed delta and finer delta as MSD approaches to 0 dB for IMD2

Figure 7: comparison b/w fixed delta and finer delta as MSD approaches to 0 dB for IMD5

As can be seen in Figure 6 and Figure 7, “finer delta as MSD approaches to 0 dB” can provide more finer resolutions as MSD approaches to 0 dB. The drawback is the resolutions become coarser if MSD is large.

Observation 14: If delta is defined as delta = minimum requirement/M where 1 < M <= M=2n, n is the number of bits, the maximum difference between actual MSD and reported MSD can become even smaller than that by equidistant delta when MSD is small while the difference can become even larger than that by fixed delta when MSD is large.

Table 3-3: Comparison table for the three approaches to indicate MSD ranges.

|  | Approaches |  |  |
| --- | --- | --- | --- |
|  | Approach 1 (Fixed delta) | Approach 2 (Equidistant delta according to minimum requirement) | Approach 3 (Finer delta as MSD approaches to 0 dB) |
| Granularity | Bad and Worst Suitability changes according to minimum requirement. May not be able to indicate MSD improvement at all when MSD is close to 0 dB. If minimum requirement is similar or smaller than the delta, the granularity can be the worst. | Good The difference b/w actual MSD and reported MSD can be minimized in a more fair fashion than the other two approaches. | Best and Worst Granularity close to MSD=0 dB is even finer than approach 2 and Granularity close to minimum requirement can be even coarse than approach 1 when minimum requirement is large. |
| NW impact | NW needs to store minimum requirements and calculate MSD range from the reported bits. |  |  |
| Others | Simpler than the others | Well balanced | Can provide finer resolution when actual MSD is close to 0 dB where more efforts per MSD improvement are needed |

When it comes to the threshold(s), as discussed above, an appropriate threshold for each MSD type and/or indication method must be different. In addition, possible range to be reported as capability is limited by the number of available bits.  Hence, we may not need to dig into the unique threshold or at least it may be better to discuss it (if necessary) after seeing the whole picture.

Observation 15: Provided that whichever approach is selected, NW needs to store MSD information, fixed delta approach cannot have advantage compared to the other approaches.

Observation 16: An appropriate threshold for each MSD type and/or indication method must be different. In addition, possible range to be reported as capability is limited by the number of available bits.  Hence, the unique threshold may not be needed to be defined or at least it may be better to discuss it (if necessary) after seeing the whole picture.

# Signalling overhead

As elaborated in [8], higher order BCs (parent BCs) inherit lower MSD capabilities via fallback BCs. Hence, if a UE reports lower MSD capabilities e.g., per victim band per MSD types/order per BC per PC through all the BCs, it is quite redundant. In order to concretize this, assume that UE supports

-	Parent or highest order BCs: CA_n1-n3-n5-n78, CA_n1-n3-n7-n78, CA_n1-n3-n28-n78 and CA_n1-n3-n8-n78

-	For simplicity, the UE has lower MSD capability only CA_n1-n3 for a certain type of MSD

Then, what UE signals is this:

-	Supported BCs: CA_n1-n3-n5-n78, CA_n1-n3-n7-n78, CA_n1-n3-n28-n78 and CA_n1-n3-n8-n78

-	Lower MSD BCs: CA_n1-n3

This means that UE also supports the lower MSD on all parent BCs where CA_n1-n3 is a component, e.g. as illustrated in Figure 8 below (NOTE: to keep the figure simple, this doesn't show all the possible fallback relations). The green color illustrates the part where the lower MSD applies.

Figure 8: Relation between parent BCs and fallback BCs in terms of lower MSD capability

In summary, if lower MSD capability is reported per minimum BC unit defined in Table 4, signaling overhead specifically, for lower MSD capability per MSD type and/or order would be mitigated. Note that if RAN2 reflects this aspect or not depends on RAN2 decision if lower MSD capability is specified.

Observation 17: Lower MSD capability for all the BCs supported by a UE may not be necessary to be reported. It would be sufficient to report the capabilities for only fallback BCs categorized as minimum BC unit that is the smallest component of each MSD type/order. NW can assume that all the supported higher order BCs by the UE inherit the reported MSD performance per fallback BCs in minimum BC unit levels.

Observation 18: Minimum BC unit per MSD type is summarized in Table 4.

Table 4: Minimum BC unit to report MSD

| MSD Type | Minimum BC unit |  |  |
| --- | --- | --- | --- |
|  | 1UL/2DL | 2UL/2DL | 2UL/3DL |
| UL Harmonic | X |  |  |
| Harmonic mixing | X |  |  |
| Cross band | X |  |  |
| IMD |  | X | X1 |
| NOTE 1: Only MSD impacting on the DL whose UL is not configured with is reported. |  |  |  |

# Conclusion

This document has made the following observations:

For necessity of signaling improved lower MSD,

Observation 1: The originally raised issue was the current situation leads to a lose-lose situation between UE vendors and operators/infra-vendors because huge MSD in the specifications would become a disincentive for operators to deploy some band combinations even if some UEs can have even less MSD than the specified MSD.

Observation 2: Static approach focuses on providing NW with complementary information to assist the NW to carry out a proper decision making on carrier-assignment in a way that the UE is not suffering from MSD if assigned

Observation 3: Dynamic approach focuses on optimizing NW scheduling on UEs experiencing MSD

Observation 4: Static DL centric approach can address the original issue as well as help NW set more appropriate threshold for CA configuration not to cause MSD in a similar way written in TS38.331 for DL and UL CA.

Observation 5: With respect to static UL centric approach, it would be built up on DL centric approach. The UL centric approach may enable NW to set a threshold in a more flexible way than DL centric approach while the additional gain is uncertain. Specifically, it’s expected that the lower UE’s MSD is, the lower the benefit may become while the UEs’ MSD is not lower enough and channel conditions are not stable, it may not work. Lastly, applying UL centric approach to UL CA requires even more complicated specifications and implementations than DL centric approach while the benefit is uncertain as mentioned before.

Observation 6: For dynamic MSD reporting as an example, since even if NW receives an MSD value that a UE is experiencing, measures that the NW can take is limited and the effect of it is uncertain while the effort to introduce it would be significant for standardization and implementation. Finally, it would require specification changes on other WGs. Hence, it’s out of scope of this WI from the beginning.

For MSD report indication,

Observation 7: Per BC makes signaling simpler than per MSD type and/or order per BC while the former doesn’t provide sufficient information for a NW to make maximum use of it if more than one MSD can be seen under the NW.

Observation 8: There is a case that the same BC has two different MSD due to the same MSD type and order, e.g., dual UL CA_n28-n74 has two MSD due to IMD4 where each MSD has a different victim band, i.e., n28 DL or n74 DL. In this case, per MSD type and order per BC cannot differentiate them without additional identification, e.g., victim DL band or frequency component of the order.

Observation 9: MSD minimum requirements for the same MSD type and/or order can be very different from PC to PC, e.g., in some cases, the difference is more than 10 dB, e.g., IMD4 for dual UL CA_n5-n77.

Observation 10: If RAN4 has a discipline that the same amount of delta per MSD type and order per BC is applicable regardless of power classes, delta approach may have advantage over direct approach in case the feasibility is confirmed. Otherwise, there may not be significant difference in terms of pros and cons.

Observation 11: Reporting a fixed amount of improved MSD value(s) as reference to the minimum requirement value per BC (fixed delta approach) may generate uneven granularity within an MSD as well as MSD across different MSD type and order for the same BC.

Observation 12: Larger MSD improvement becomes, even higher RF components performance improvement it requires [7] and if a fixed delta approach was adopted, efforts to improve MSD close to zero would be underappreciated.

Observation 13: If delta is defined as the minimum requirement/2n (equidistant delta within each of the minimum requirements) where n is the number of bits, the maximum difference between actual MSD and reported MSD can become smaller than that by fixed delta.

Observation 14: If delta is defined as delta = minimum requirement/M where 1 < M <= M=2n, n is the number of bits, the maximum difference between actual MSD and reported MSD can become even smaller than that by equidistant delta when MSD is small while the difference can become even larger than that by fixed delta when MSD is large.

Observation 15: Provided that whichever approach is selected, NW needs to store MSD information, fixed delta approach cannot have advantage compared to the other approaches.

Observation 16: An appropriate threshold for each MSD type and/or indication method must be different. In addition, possible range to be reported as capability is limited by the number of available bits.  Hence, the unique threshold may not be needed to be defined or at least it may be better to discuss it (if necessary) after seeing the whole picture.

For signaling overhead,

Observation 17: Lower MSD capability for all the BCs supported by a UE may not be necessary to be reported. It would be sufficient to report the capabilities for only fallback BCs categorized as minimum BC unit that is the smallest component of each MSD type/order. NW can assume that all the supported higher order BCs by the UE inherit the reported MSD performance per fallback BCs in minimum BC unit levels.

Observation 18: Minimum BC unit per MSD type is summarized in Table 4.

Table 4: Minimum BC unit to report MSD

| MSD Type | Minimum BC unit |  |  |
| --- | --- | --- | --- |
|  | 1UL/2DL | 2UL/2DL | 2UL/3DL |
| UL Harmonic | X |  |  |
| Harmonic mixing | X |  |  |
| Cross band | X |  |  |
| IMD |  | X | X1 |
| NOTE 1: Only MSD impacting on the DL whose UL is not configured with is reported. |  |  |  |

And proposes the followings accordingly:

Proposal 1: With consideration of observations 1 - 6, RAN4 should agree that it is essential to introduce lower MSD capability in order to help NW do better admission control and/or set even more appropriate threshold to decide configuration of a carrier at least via static DL centric approach.

Proposal 2: With consideration of observations 7 – 10, in order to make maximum use of the reported value, MSD must be able to be reported per victim band per MSD type/order per BC.

Proposal 3: Handling of different PC should be further discussed together with MSD feasibility study and MSD indication method.

Proposal 4: With consideration of observations 11 – 15, the approach 1(fixed delta approach) shouldn’t be adopted and either the approach 2 (observation 13) or the approach 3 (observation 14) should be selected.

Proposal 5: With consideration of observations 16, not to discuss the unique threshold or at least it may be better to discuss it (if necessary) after seeing the whole picture.

Proposal 6: With consideration of observations 17 - 18, share the information on relation between higher order BCs and fallback BCs in terms of lower MSD capability with RAN2 if lower MSD capability is specified.

# Reference

- R4-2214452, WF on study for lower MSD, Huawei, HiSilicon, TSG-RAN WG4 #104-e

- R4-2016440, Improving PC2 MSD for EN-DC and UL CA, Qualcomm Incorporated, TSG-RAN WG4 #97-e

- R4-2212009, “Network benefit of Lower MSD capability”, Samsung, TSG-RAN WG4 #104-e

- R4-22xxxxx, “xxxx”, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #104-bis-e

- R4-2119438, Proposals on capability-based MSD for CA and DC combinations, Qualcomm Incorporated, TSG-RAN WG4 #101-e

- R4-2114700, “Views on defining “low MSD” for CA and DC”, Apple, TSG-RAN WG4 #100-e

- R4-2212093, “Possible approaches to improve MSD”, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #104-e

- R4-2212092, “Essential considerations of lower MSD signaling”, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #104-e