3GPP TSG RAN WG4 Meeting #104-bis-e                   R4- 2215424

Electronic Meeting, October 10th - October 19th, 2022

Agenda item:			6.20.2

Source:	CATT

Title:	Discussion of improvement on FR2 SCell/SCG setup/resume

Document for:	Discussion

- Introduction

In the last RAN4 meeting, WF [1] on further NR Mobility Enhancements RRM requirements was approved. In this contribution, some issues on improvement on FR2 SCell/SCG setup/resume are further discussed.

- Discussion

For FR2 in NR with large capacity and high date rate, it is the key deployment scenario in 5G. However, the mobility performance is more challenging for FR2 deployment due to the decreased cell size. UE will perform more frequently measurement and more cell re-selection or handover in cells when UE moves. In this WI, FR2 RRM mobility for SCell/SCG setup/resume delay should be studied.

  - Clarification and potential direction

Issue 2-1-2: Potential direction for further study: measurement enhancement when UE is about to enter connected mode

At the RAN4 # 104-e meeting, companies discussed the measurement enhancement when UE is about to enter connected mode based on WID [2], and held different views. This issue incorporates the contents of issue 2-1-1, and directly discussed in detail the specific time nodes when the measurement enhancement performs. The candidate options are duplicated as follows [1]:

<Way forward >: - Option 1(CMCC): UE perform enhanced measurement closely before RRC connection setup/resume. - For MT originating call, the measurement is performed during the period between paging reception and UE send RRCResumeRequest/ RRCSetupRequest. - For MO originating call, the measurement is performed after upper layers request establishment of an RRC connection and before UE send RRCResumeRequest/ RRCSetupRequest. - Option 2 (Apple, Ericsson, Nokia): UE perform enhanced measurement during RRC connection setup/resume. - Option 2a: perform FR2 cell measurements during connection setup (i.e. paging, RRC connection establishment, RRC connection resume) - Option 2b (MTK, vivo, xiaomi, HW): after receiving paging for MT originating call - Option 2c (MTK, vivo, xiaomi, HW): after first RACH preamble transmission, i.e. Msg1

The issue is specifically described in WID [2] as:

- The UE initiates and performs improved measurements when it requests RRC connection setup/resume.

According to our understanding, from the perspective of wording, we prefer option 2, that is, UE perform enhanced measurement during RRC connection setup/resume, which is more consistent with the description in WID.

As discussed in the last meeting, we need to pay attention to specific time on the detailed level. In option 1, the time period for performing enhanced measurement is strictly limited, and the available measurement time may be too short [3]. We believe that more time can be obtained for performing enhanced measurement. At least the earliest measurement time point should be defined, so we prefer option 2b and option 2c. For MT originating calls, the UE perform enhanced measurement cannot be earlier than paging reception, and for MO calls, the UE perform enhanced measurement cannot be earlier than first RACH preamble transmission. To our understanding, UE could continue measurements on one or more carriers during the connection setup and potentially during a period of time while in connected mode. Whether and how to define the end time point of measurement enhancement need further discussion.

Observation 1: Option 1 strictly limits the time period for performing enhanced measurement, and the available measurement time may be too short, may causing the UE to be insufficient to perform enhanced measurement.

Proposal 1: It is suggested to at least define the earliest enhanced measurement time point.

- For MT calls, the UE perform enhanced measurement cannot be earlier than paging reception.

- For MO calls, the UE perform enhanced measurement cannot be earlier than first RACH preamble transmission.

Proposal 2: UE could continue measurements on one or more carriers during the connection setup and potentially during a period of time while in connected mode.

- Whether and how to define the end time point of measurement enhancement need further discussion.

Issue 2-1-3: Potential direction for further study: enhancement on R16 EMR, i.e. measurement enhancement in idle/inactive mode

At the RAN4 # 104-e meeting, companies disagreed on whether to further enhance Rel-16 EMR. The WF is repeated as follows [1]:

<Way forward >: - Option 1 (Apple, xiaomi, MTK, HW, vivo): further enhancement on Rel-16 EMR (measurement during green part) is out of scope - Option 2 (Ericsson, Nokia, LGE): further enhancement on Rel-16 EMR (measurement during green part) is in the scope

As mentioned in WID [2]:

- The UE initiates and performs improved measurements when it requests RRC connection setup/resume.

- After acquiring those improved measurements, the UE subsequently reports those measurements to the network to support SCell/SCG setup.

According to the above figure, we can see that the objective of the enhanced measurement in Rel-18 and Rel-16 EMR discuss different scopes. In our understanding, ‘enhancement on Rel-16 EMR’ should focus on the further tighter Rel-16 EMR measurement delay, and the time of enhancement is before idle/active mode and UE initiating access.

For the enhanced measurement (if feasible), it at least applies to scenarios that the EMR measurement results are unavailable or invalid. When EMR measurement results are valid, NW could use EMR results, so it is unnecessary to introduce new measurements, and when EMR measurement results are unavailable, we do not need to further tighten Rel-16 EMR measurement delay.

The biggest argument about option 1 and option 2 is to attribute the possible behavior of UE to ‘the enhanced measurement’ or ‘enhancement on Rel-16 EMR’ when using EMR measurement results during RRC connection. In our opinion, this behavior should belong to the discussion of ‘the enhanced measurement’ rather than ‘enhancement on Rel-16 EMR’. As mentioned above, it is not necessary to enhance EMR measurement when EMR is invalid.

Observation 2: The key to the discussion is to attribute the possible behavior of UE to ‘the enhanced measurement’ or‘enhancement on Rel-16 EMR’ when using EMR measurement results during RRC connection.

Proposal 3: The possible behavior that UE uses EMR measurement results during RRC connection should belong to the discussion of‘the enhanced measurement’ rather than ‘enhancement on Rel-16 EMR’.

Proposal 4: For the enhanced measurement (if feasible), it at least applies to scenarios that the EMR measurement results are unavailable or invalid.

- When EMR measurement results are valid, NW could use EMR results, so it is unnecessary to introduce new measurements.

- When EMR measurement results are unavailable, do not need to further tighten Rel-16 EMR measurement delay.

## 2.2 Assumptions for feasibility study

Issue 2-2-4: Assumption for feasibility study: Reduce the scaling factor of Rx beam sweeping

In the RAN4#104-e meeting, companies had different views on assumption for feasibility study: reduce the scaling factor of Rx beam sweeping. The options are duplicated as below [1]:

<Way forward >: - Option 1(Ericsson, Nokia): Reduce the scaling factor of Rx beam sweeping - Option 1a (CMCC): use Rx beam sweeping factor of R17 HST FR2 or R17 positioning - For FR2 HST, the value of scaling factor is 2 or 6 pending on the different deployment. - In Rel-17 positioning WI, the candidate Rx beam sweep numbers for reduced Rx beam sweeping factor (<8) UE capability are {1, 2, 4, 6}. - Option 1b (xiaomi, Ericsson, vivo, QC): FFS use prior information on the UE Rx beam to reduce the scaling factor of Rx beam sweeping - Option 2 (Apple, MTK, HW): Not to reduce the scaling factor of Rx beam sweeping - Option 3 (Apple, Ericsson): Further discussion

In FR2, the cell size is smaller than that in FR1. When UE moves fast, UE will change the serving cell frequently. RAN2 introduced early measurements procedures in Rel-16. It allowed faster CA/DC setup when UE change from idle state to connected state. Early measurement report is sent when UE enters connected mode from idle/inactive state. In current measurement report, RSRP/RSRQ and SSB index are stored for idle mode measurement. In current RAN4 requirement for FR2 EMR measurement delay, RX beam sweeping factor is used. It caused long delay with DRX cycle and N = 8. It caused more than hundreds of seconds. If the delay is too long, it degrades the performance for early measurements. It is suggested to reduce RX beam sweeping scaling factor number [4].

In the last meeting, companies further discussed the possible methods of reducing RX beam sweeping scaling factor number. We believe that based on the above early measurement, it is feasible for UE to obtain some prior information about the beam and reuse the previous measurement results, so as to avoid a more complicated measurement process. At the same time, the network can use the prior information to configure the UE in advance before the UE performs the measurement, so that the RX beam sweeping scaling factor number can be reduced.

Proposal 5: Based on early measurement, it is feasible for UE to obtain some prior information about the beam and reuse the previous measurement results, so as to reduce RX beam sweeping scaling factor number.

- Summary

In this paper, we provide our views on improvement on FR2 SCell/SCG setup/resume. From this discussion we have derived the following observations and proposals:

Observation 1: Option 1 strictly limits the time period for performing enhanced measurement, and the available measurement time may be too short, may causing the UE to be insufficient to perform enhanced measurement.

Proposal 1: It is suggested to at least define the earliest enhanced measurement time point.

- For MT calls, the UE perform enhanced measurement cannot be earlier than paging reception.

- For MO calls, the UE perform enhanced measurement cannot be earlier than first RACH preamble transmission.

Proposal 2: UE could continue measurements on one or more carriers during the connection setup and potentially during a period of time while in connected mode.

- Whether and how to define the end time point of measurement enhancement need further discussion.

Observation 2: The key to the discussion is to attribute the possible behavior of UE to ‘the enhanced measurement’ or ‘enhancement on Rel-16 EMR’ when using EMR measurement results during RRC connection.

Proposal 3: The possible behavior that UE uses EMR measurement results during RRC connection should belong to the discussion of ‘the enhanced measurement’ rather than ‘enhancement on Rel-16 EMR’.

Proposal 4: For the enhanced measurement (if feasible), it at least applies to scenarios that the EMR measurement results are unavailable or invalid.

- When EMR measurement results are valid, NW could use EMR results, so it is unnecessary to introduce new measurements.

- When EMR measurement results are unavailable, do not need to further tighten Rel-16 EMR measurement delay.

Proposal 5: Based on early measurement, it is feasible for UE to obtain some prior information about the beam and reuse the previous measurement results, so as to reduce RX beam sweeping scaling factor number.

- References

- R4-2214348, WF on Further NR Mobility Enhancements, MediaTek Inc.

- RP-222332, Revised WID on Further NR mobility enhancements, MediaTek Inc.

- R4-2214286, Email discussion summary for [104-e][237] NR_Mob_enh2, MediaTek Inc.

- R4-2211685, Discussion on FR2 Scell/SCG setup/resume, CATT