# 3GPP TSG-RAN WG4 Meeting # 104bis-e   									R4-2215479

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.7.3.1

Source: 	CMCC

Title: 	Beam correspondence requirements for initial access

Document for:	Discussion

# 1. Introduction

In RAN#96 meeting, a new WID on FR2 enhancement is approved with the objective of defining Beam correspondence requirements for RRC_INACTIVE and initial access to cover spec hole. In last RAN4 meeting, the agreements for initial access state are not much, we still need to further study about beam correspondence requirements.

In this contribution, we focus on the discussion of beam correspondence requirement for initial access.

# 2. Discussion

R16 beam correspondence requirements include three parts, min peak EIRP, spherical coverage and tolerance requirements. Only when all three requirements are met, UE is assumed to support beam correspondence.

There are two kinds of UE capability of beam correspondence in R16. For the UE supporting BC without beam sweeping, they don’t need to test the tolerance requirements. in other words, there is actually no tolerance requirements for such kind of UE. The other kind of UE supporting BC with beam sweeping, they have to meet tolerance requirements. For R18, although current spec support BC at initial access state without beam sweeping, actual UE’s BC capability is still not clear. There is possibility that certain R18 UE still need beam weeping in RRC_CONNECTED as the R16 UE. although in theory such UE should support BC without beam sweeping at initial access state, its actual performance is still not that good and tolerance requirement may still be needed. So it’s suggested to focus on the discussion of whether R18 UE has the same performance as R16 UE in RRC_CONNECTED, i.e. some UE support BC without beam sweeping while other UE support BC with beam sweeping in RRC_CONNECTED.

There are two kinds of R18 UE BC capability compared with R16.

- Option 1: assuming no BC capability enhancement compared with R16 UE. If so, there are still two kinds of R18 UE, one with better performance without tolerance requirements like UE without beam sweeping in R16, another with relatively bad BC performance like R16 UE with beam sweeping which still need the tolerance requirement, although in theory such UE should support BC without beam sweeping at initial access state.

- Option 2: assuming BC capability enhancement compared with R16 UE. Our understanding is that if R18 UE could achieve BC capability enhancement at initial access state, it could also achieve BC capability enhancement at RRC_CONNECTED state. If so this means BC requirements should be updated accordingly and we don’t need UE with beam sweeping even in RRC_CONNECTED state.

From our understanding option 1 is more reasonable that R18 UE has the same capability as R16 UE. If so, we have following observation and proposal.

Observation 1: there are still two kinds of R18 UE at initial access and RRC_INACTIVE

- one with better performance without tolerance requirements like UE without beam sweeping in R16

- another with relatively bad BC performance like R16 UE with beam sweeping which still need the tolerance requirement although in theory such UE should support BC without beam sweeping at initial access state.

Therefore, tolerance requirement is still needed. For the UE with bad BC performance, UL EIRP toward gNB is less and such UE may fail to get into RRC_CONNECTED. The result is that operators have to deploy more gNB sites to enhance edge coverage, which increasing deployment cost. From our point of view, it’s better to define tolerance requirements especially for the R16 like UE with beam sweeping and avoid bad UL performance especially at cell edge.

Proposal 1: it’s suggested to study the tolerance requirements especially for UE support BC with beam sweeping in RRC_CONNECTED.

In theory, the tolerance requirements for initial state is the tolerance between best-matched UL beam based on UL beam sweeping and automatically chosen UL beam. As for the best-matched UL beam, we may need to allow UE to sweep UL beam to find the best matched beam at initial access state, which may need further study during the test procedure.

Observation 2: to make the tolerance testing operational, we may need to allow UE to sweep UL beam during the test to find the best-matched beam at initial access state or at RRC_INACTIVE state.

# 3. Conclusions

In this contribution, beam correspondence requirements for initial access are discussed with following observations and proposals:

Observation 1: there are still two kinds of R18 UE at initial access and RRC_INACTIVE

- one with better performance without tolerance requirements like UE without beam sweeping in R16

- another with relatively bad BC performance like R16 UE with beam sweeping which still need the tolerance requirement although in theory such UE should support BC without beam sweeping at initial access state.

Proposal 1: it’s suggested to study the tolerance requirements especially for UE support BC with beam sweeping in RRC_CONNECTED.

Observation 2: to make the tolerance testing operational, we may need to allow UE to sweep UL beam during the test to find the best-matched beam at initial access state or at RRC_INACTIVE state.

# 4. Reference

[1] R4-2214454, WF on BC in RRC_INACTIVE and initial access, Nokia