3GPP TSG-RAN WG4 Meeting # 104-bis-e                              R4-2215530

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.9.2.1, 6.9.2.2

Source:	China Telecom

Title:	Discussion on A-TRS based unknown SCell activation

Document for:	Discussion

- Introduction

The WID of even further RRM enhancement for NR and MR-DC on Rel-18 was approved in [1] during RAN #95e meeting and revised in [2] during RAN #96 meeting. One of the objectives is to study the enhancement of FR2 SCell activation delay. In RAN4 #104-e meeting, the FR2 SCell activation delay reduction solutions were discussed for the first time with the WF [3] approved, and the solutions will be further discussed in detail.

In this paper, we discuss the solutions of FR2 SCell activation delay reduction and present our views about these issues.

- Discussion

Status in the WF [3]:

Issue 2-1-1: Baseline scope for FR2 SCell activation enhancement Agreement: - 1st phase: RAN4 starts the discussion of the R18 unknown FR2 SCell activation enhancement based on baseline FR2 SCell activation requirement in TS38.133 section 8.3.2 - Discuss the feasibility and method of SCell activation enhancement based on AP-CSI-RS and/or A-TRS - 2nd phase: the other SCell activation cases, e.g., multiple SCell activation, direct SCell activation, and PUCCH SCell activation, to be discussed after baseline case (i.e. single FR2 SCell activation) is completed. - 2nd phase will start from RAN4 #107 meeting. Issue 2-1-2: Prioritization for initial phase Agreement: - RAN4 to prioritize at least FR2 unknown Scell delay reduction in the 1st phase of the WI. - FR2 unknown Scell without intra-band serving cell is considered for 1st phase. - The extension of the enhancement solutions to FR1 can also be discussed.

In the WF, it was agreed to first study the unknown FR2 SCell activation enhancement without intra-band serving cell and discuss the feasibility and method of SCell activation enhancement based on AP-CSI-RS and/or A-TRS.

The requirement of baseline FR2 SCell activation in TS 38.133 section 8.3.2 is summarized in the following Table 1.

Table 1. Requirement of baseline FR2 SCell activation in TS 38.133

| FR2 SCell activation | Scenario |  | Delay (Tactivation_time) |
| --- | --- | --- | --- |
| At least one active serving cell on the FR2 band | UE is provided with SMTC |  | TFirstSSB+ 5ms |
|  | UE is not provided with any SMTC |  | 3 ms |
| No active serving cell on the FR2 band | Target SCell is known to UE (FR1-FR2 CA or FR2 CA) | Semi-persistent CSI-RS is used for CSI reporting | 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |
|  |  | Periodic CSI-RS is used for CSI reporting | max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) |
|  | Target SCell is unknown to UE (FR1-FR2 CA or FR2 CA with IBM) | Semi-persistent CSI-RS is used for CSI reporting | 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |
|  |  | Periodic CSI-RS is used for CSI reporting | 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)} |

As seen in Table 1, for the unknown FR2 SCell case, the baseline FR2 SCell activation delay is very long compared to the known cases and having active serving cell cases. During the unknown FR2 SCell activation process, AGC tuning, cell search, L1-RSRP measurement and report, fine timing tracking process are required, which cause the long delay, as illustrated in the following Figure 1.

Figure 1. Unknown FR2 SCell activation delay

In order to reduce the unknown FR2 SCell activation delay, the following options are presented in this paper.

A-TRS based unknown FR2 SCell activation

Status in the WF [3]:

RS related enhancement for L3 part Issue 2-2-4: RS related enhancement of FR2 unknown SCell activation - FFS: AP-CSI-RS and/or A-TRS based fast SCell activation may also apply to unknown FR2 SCell activation enhancement Aperiodic RS related enhancement for L1 part Issue 2-3-8: Aperiodic RS related enhancement during FR2 unknown SCell activation FFS: - Proposal 1 (Apple, Ericsson): RAN4 to discuss: use AP RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement - Proposal 2a(Apple, Xiaomi, MTK, Ericsson, ZTE): RAN4 to discuss: use A-TRS corresponding to the TCI state for fine timing tracking after TCI activation command - Proposal 2b(Xiaomi): If there is no any QCL information of reference signal for target SCell, A-TRS is configured for fine timing tracking, and the A-TRS is QCL-ed with the selected SSB index. - Proposal 3: Any other solutions to enable A-TRS usage to unknown SCell activation is not precluded.

In the WF, it was agreed to study RS related enhancement for L3 and L1 part. In our views, A-TRS based method is able to be used for both L3 and L1 part to reduce the whole procedure delay, provided that some conditions are fulfilled. In legacy RRM requirement, the A-TRS is used for fast FR2 SCell activation, as shown in Table 2.

Table 2. Requirement of A-TRS based fast FR2 SCell activation in TS 38.133

| A-TRS based fast FR2 SCell activation | Scenario |  | Delay (Tactivation_time) |
| --- | --- | --- | --- |
| At least one active serving cell on the FR2 band | Intra-band CA |  | TFirstATRS+ 5ms |
| No active serving cell on the FR2 band | Target SCell is known to UE (FR1-FR2 CA or FR2 CA) | Semi-persistent CSI-RS is used for CSI reporting | 3ms + max(TFirstATRS + 2ms, Tuncertainty_SP) |
|  |  | Periodic CSI-RS is used for CSI reporting | max(TFirstATRS + 5ms, Tuncertainty_RRC + TRRC_delay-THARQ). |

As seen in Table 2, A-TRS based fast FR2 SCell activation for the known case has been defined in TS 38.133 when there is no active serving cell on that FR2 band. Based on the configured A-TRS resource, UE is able to obtain the information of the SCell, including timing, frequency, beam information and so on. Then, the process of AGC tuning, cell search, L1-RSRP measurement and report are not required, which bring significant reduction of FR2 SCell activation delay, as illustrated in the following Figure 2.

Figure 2. A-TRS based fast FR2 SCell activation

However, the unknown case isn’t covered in the A-TRS based fast FR2 SCell activation requirements. We think that UE is able to obtain timing/frequency/beam information of SCell based on the A-TRS provided that the A-TRS has same QCL information with reference signals of any other active serving cell in another band. Therefore, the network is able to configure and trigger A-TRS for UE to inform the QCL information for the fine timing tracking, and meanwhile the complex and long AGC tuning, cell search, L1-RSRP measurement and report process are not required, which reduce the most of delay in the unknown FR2 SCell activation scenario.

Proposal 1: In unknown FR2 SCell activation scenario, UE is able to obtain timing/frequency/beam information of SCell based on the A-TRS provided that the A-TRS has same QCL information with reference signals of any other active serving cell in another band.

Extension of A-TRS based unknown FR1 SCell activation into inter-band scenario

In the WF, it was agreed that the extension of the enhancement solutions to FR1 can also be discussed to avoid the duplicated discussion. In legacy RRM requirement, the A-TRS is used for fast FR1 SCell activation, as shown in Table 3.

Table 3. Requirement of A-TRS based fast FR1 SCell activation in TS 38.133

| A-TRS based fast FR1 SCell activation | Scenario | Delay (Tactivation_time) |
| --- | --- | --- |
| Target SCell is known to UE | The measurement period of the SCell is equal to or smaller than [2400ms] | TFirstATRS+ 5ms |
|  | The measurement period of the SCell is larger than [2400ms] | TFirstATRS + Tgap + TATRS + 5ms |
| Target SCell is unknown to UE | SCell is contiguous to an active serving cell in the same band | TFirstATRS + Tgap + TATRS + 5ms |

In A-TRS based unknown FR1 SCell activation scenario, the specification defines intra-band case but doesn’t cover inter-band case. We think that the extension of A-TRS based unknown FR1 SCell activation into inter-band scenario can also be introduced.

Proposal 2: The extension of A-TRS based unknown FR1 SCell activation into inter-band scenario can also be introduced.

- Conclusion

The contribution presents our views on the solutions of unknown SCell activation delay reduction, with the following proposals:

Proposal 1: In unknown FR2 SCell activation scenario, UE is able to obtain timing/frequency/beam information of SCell based on the A-TRS provided that the A-TRS has same QCL information with reference signals of any other active serving cell in another band.

Proposal 2: The extension of A-TRS based unknown FR1 SCell activation into inter-band scenario can also be introduced.

- References

- RP-220977, New WID: Even Further RRM enhancement for NR and MR-DC, Apple, OPPO, RAN Meeting #95e, March 2022.

- RP-221696, Revised WID: Even Further RRM enhancement for NR and MR-DC, Apple, OPPO, RAN Meeting #96, June 2022.

- R4-2214345, WF on R18 eFeRRM, Apple, RAN4 #104-e, August 2022.